3GPP TSG RAN WG2 #129bis                                                                              R2-2501922

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Issues to address for AI/ML Positioning stage-2

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

During the preparation of 38.305 AI/ML positioning running CR, some issues are identified for further RAN2 discussion. In this contribution, the corresponding resolutions are provided from our perspective.

- Discussion

  - Positioning case priority

According to WID [1], there are 5 AI/ML positioning cases to specify. And RAN2 also made the following agreements to not work on Case 2a/2b, and regarding any further spec impact for Case 3a/3b RAN2 needs to wait for other WG’s input, as a result RAN2 only focus on Case 1 for now.

RAN2#129 Agreements on positioning

RAN2 has not and will not study Case 2a and Case 2b in Rel-19

Respond to SA2 indicating our current situation

RAN2#125bis Agreements:

- For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs

- For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

Therefore based on RAN2 agreements, only the spec impact for AI/ML positioning Case 1 has been captured in 38.305 running CR.

Observation 1: only the spec impact for AI/ML positioning Case 1 has been captured in 38.305 running CR.

  - Open issue 1: applicability reporting

Regarding applicability reporting discussed for BM use case, RAN2 has made the following agreements:

RAN2#127 Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. <other part is omitted>

- RAN2#129 Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

Although there are some progress in the aspect of applicability reporting for positioning Case 1, i.e., the baseline procedure includes proactive and reactive reporting as shown below. The detail of how UE determines whether a functionality is applicable, and the detailed content in initial report and subsequent reports, are still missing.

RAN2#128 Agreements

1	For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline.

- 	Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF .

- 	Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF.  Configuration details are FFS

Since the RAN2 intention is to have the same design for BM case and positioning case as much as possible, the agreements made for BM applicability reporting can also be applicable to positioning case 1.

Proposal 1: the following agreements made for BM applicability reporting is also applicable to positioning case 1:

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

  - Open issue 2: assistance data transfer procedure between LMF and UE

RAN1 agreed that all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE for AI/ML positioning Case 1.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE.

As specified for DL-TDOA positioning method in 38.305, the information that may be transferred from the LMF to the UE are listed in table 8.12.2.1.0-1.

Table 8.12.2.1.0-1: Assistance data that may be transferred from LMF to the UE

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | Yes | Yes |
| Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes | Yes |
| DL-PRS configuration of candidate NR TRPs | Yes | Yes |
| Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | Yes | Yes |
| SSB information of the TRPs (the time/frequency occupancy of SSBs) | Yes | Yes |
| Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | No | Yes |
| Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | No | Yes |
| Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | No | Yes |
| PRS-only TP indication | Yes | Yes |
| The association information of DL-PRS resources with TRP Tx TEG ID | No | Yes |
| LOS/NLOS indicators | No | Yes |
| On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation | Yes | Yes |
| Validity Area of the Assistance Data | Yes | Yes |
| PRU measurements together with the location information of the PRU | No | Yes |
| Data facilitating the integrity results determination of the calculated location | No | Yes |

Except for info #7 TRP locations, all other information captured in table 8.12.2.1.0-1 for DL-TDOA has been agreed by RAN1 as assistance data provided from LMF to UE for AI/ML positioning Case 1.

Observation 2: Except for info #7 TRP locations, all other information captured in table 8.12.2.1.0-1 for DL-TDOA has been agreed by RAN1 as assistance data provided from LMF to UE for AI/ML positioning Case 1.

But the corresponding assistance Data Transfer Procedure between LMF and UE has not been discussed in RAN2. As specified for DL-TDOA, there are four procedures respectively for this purpose:

- LMF initiated Assistance Data Delivery

- LMF initiated Periodic Assistance Data Delivery

- UE initiated Assistance Data Transfer

- UE initiated Periodic Assistance Data Transfer

Considering RAN1 agreed “For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE”, the existing assistance Data Transfer Procedure for DL-TDOA between LMF and UE should be reused for AI/ML positioning Case 1.

Proposal 2: Considering RAN1 agreed “For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE”, the existing assistance Data Transfer Procedure for DL-TDOA between LMF and UE should be reused for AI/ML positioning Case 1.

  - Open issue 3: Positioning Integrity

As a part of general NG-RAN UE Positioning procedures, positioning Integrity is supported for the following positioning methods for reporting confidence level purpose:

-	GNSS positioning methods as specified in clause 8.1;

-	Multi-RTT positioning as specified in clause 8.10;

-	DL-AoD positioning as specified in clause 8.11;

-	DL-TDOA positioning as specified in clause 8.12;

-	UL-TDOA positioning as specified in clause 8.13;

-	UL-AoA positioning as specified in clause 8.14.

Positioning integrity operation requires LMF to provide the error information to UE, and the mapping of integrity parameters in assistance data for DL-TDOA has been specified in clause 8.12.2.1.1 of TS 38.305.

Table 8.12.2.1.1-1: Mapping of Integrity Parameters

| Error | NR Assistance Data | Integrity Fields |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Integrity Alerts | Integrity Bounds (Mean) | Integrity Bounds (StdDev) | Residual Risks | Integrity Correlation Times |
| TRP location | NR-TRP-LocationInfo | TRP DNU | Mean TRP/ARP location error | Standard deviation TRP/ARP location error | Probability of Onset of TRP fault Mean TRP fault duration | TRP/ARP location error correlation time |
| Inter-TRP synchronization | NR-RTD-Info | RTD DNU | Mean RTD error | Standard deviation RTD error |  | RTD error correlation time |

For example, “Timing relative to the serving (reference) TRP of candidate NR TRPs” has been agreed by RAN1 as assistance data for AI/ML positioning Case 1, which means Inter-TRP synchronization error information is included in assistance data if positioning Integrity is supported for AI/ML positioning Case 1.

The following ASN.1 part is the excerpt from TS 37.355, and the Inter-TRP synchronization error nr-IntegrityRTD-InfoBounds-r18 is included in assistance data NR-PositionCalculationAssistance-r16 as shown in the following ASN.1 code.

– NR-PositionCalculationAssistance The IE NR-PositionCalculationAssistance is used by the location server to provide assistance data including integrity information to enable UE-based downlink positioning. -- ASN1START NR-PositionCalculationAssistance-r16 ::= SEQUENCE { nr-TRP-LocationInfo-r16 NR-TRP-LocationInfo-r16 OPTIONAL, -- Need ON nr-DL-PRS-BeamInfo-r16 NR-DL-PRS-BeamInfo-r16 OPTIONAL, -- Need ON nr-RTD-Info-r16 NR-RTD-Info-r16 OPTIONAL, -- Need ON ..., <unnecessary part omitted> } – NR-RTD-Info The IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs together with integrity information. -- ASN1START NR-RTD-Info-r16 ::= SEQUENCE { referenceTRP-RTD-Info-r16 ReferenceTRP-RTD-Info-r16, rtd-InfoList-r16 RTD-InfoList-r16, ... } <unnecessary part omitted> RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF RTD-InfoListPerFreqLayer-r16 RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPsPerFreq-r16)) OF RTD-InfoElement-r16 RTD-InfoElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON subframeOffset-r16 INTEGER (0..1966079), rtd-Quality-r16 NR-TimingQuality-r16, ..., [[ nr-IntegrityRTD-InfoBounds-r18 NR-IntegrityRTD-InfoBounds-r18 OPTIONAL -- Need OR ]] } NR-IntegrityRTD-InfoBounds-r18 ::= SEQUENCE { resolution-r18 ENUMERATED {mdot1, m1, m10, m30, ...}, meanRTD-r18 INTEGER (0..255), stdDevRTD-r18 INTEGER (0..31), ... } -- ASN1STOP

As a result, the following confidence level information can be reported to LMF along with the UE location information.

IntegrityInfo-r17 ::= SEQUENCE {

horizontalProtectionLevel-r17		INTEGER (0..50000),

verticalProtectionLevel-r17			INTEGER (0..50000)				OPTIONAL,

achievableTargetIntegrityRisk-r17	INTEGER (10..90)				OPTIONAL,

...

}

Considering RAN1 agreed “For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE”, the positioning integrity operation can be supported similar to DL-TDOA, i.e., LMF includes error information in assistance data and UE reports the confidence level information along with UE location information.

Proposal 3: positioning Integrity is supported for AI/ML positioning Case 1.

- Conclusion

In this contribution, we provide our views on the open issues for AI/ML positioning Case 1, and the corresponding observations are as below:

Observation 1: only the spec impact for AI/ML positioning Case 1 has been captured in 38.305 running CR.

Observation 2: Except for info #7 TRP locations, all other information captured in table 8.12.2.1.0-1 for DL-TDOA has been agreed by RAN1 as assistance data provided from LMF to UE for AI/ML positioning Case 1.

And we propose:

Proposal 1: the following agreements made for BM applicability reporting is also applicable to positioning case 1:

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

Proposal 2: Considering RAN1 agreed “For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE”, the existing assistance Data Transfer Procedure for DL-TDOA between LMF and UE should be reused for AI/ML positioning Case 1.

Proposal 3: positioning Integrity is supported for AI/ML positioning Case 1.

- References

- RP-243244, Revised WID for NR_AIML_air, Qualcomm