3GPP TSG RAN WG2 #129bis          	                                                          R2-2501923

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Consideration on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], some agreements have been achieved for NW side data collection in the aspect of BM use case:

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

In this contribution, we further discuss the data collection for NW-side model training for BM and Pos use cases.

- Discussion

  - Beam management

Mechanism related to NW side data collection

- Dynamic activation/deactivation

In previous meeting, it is FFS whether dynamic activation/deactivation of training data collection is supported. Since the activation/deactivation of measurements can be controlled by the add/modify/release operation of the related logged measurement configuration, dynamic activation/deactivation is not needed.

Proposal 1: Activation/deactivation of measurements for AI/ML training data collection is not needed, as it can be implicitly achieved by the add/modify/release operation of the related logged measurement configuration.

- Configuration of data logging

It is agreed in previous RAN2 meeting that both periodic logging and event-triggered logging will be supported.

For periodic logging, it is more like the current periodic CSI measurement in legacy CSI-ResourceConfig.

For event-triggered logging, the UE can use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. Therefore the parameters of l3-Threshold, hysteresis and timeToTrigger can be configured, similar as eventL1 in logged MDT configuration:

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

Proposal 2: Parameters of l3-Threshold, hysteresis and timeToTrigger can be configured for event-triggered training data logging, similar as eventL1 in logged MDT configuration.

- Condition of event triggered reporting

For training data collection, it has been agreed that UE should report to the network when the power state is low or when buffer is or may become full. This information can be part of availability indication. After the network receives such indication, on one hand the network could de-configure the data collection in UE; on the other hand the data retrieval procedure could be performed by sending the UEInformationRequest message to the UE. Therefore, when the UE power state is low, or the UE AS buffer has reached/exceeded a configured or specified threshold, could be considered as trigger events for sending the AI/ML training data availability indication. For determining whether AS buffer is (almost) full, an explicit threshold can be configured or specified to trigger the availability indication reporting.

Proposal 3: Define trigger condition of sending the availability indication for UE, such as when the UE power state is low, or the storage level of UE AS buffer has reached a configured/specified threshold.

- Data content report by UE

It is agreed in last RAN2 meeting that on-demand reporting principles as for logged MDT will be used for AI/ML data collection of BM use case, and the UEInformationRequest/UEInformationResponse is used. The format could also follow the logged MDT report. Besides the agreed availability indication for further data available for transmission, following contents should also be included in the reporting:

- For training data collection time marker, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE. And the time of each measurement can be represented as a relative time from the absoluteTimeStamp;

- For training data marker, the associated ID received in the training data configuration may be used;

- For training data collection node, collection node index such as TCE ID for MDT could be used;

- The main content of training data collection could be a list similar as logMeasInfoList, and the detailed content can be revised according to further RAN1 input.

Proposal 4: The report of collected training data includes:

- For training data collection time marker, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE. And the time of each measurement can be represented as a relative time from the absoluteTimeStamp;

- For training data marker, the associated ID received in the training data configuration may be used;

- For training data collection node, collection node index such as TCE ID for MDT could be used;

- The main content of training data collection could be a list similar as logMeasInfoList, and the detailed content can be revised according to further RAN1 input.

- Handling of configuration/data during mobility/state transition

The availability indicator is agreed to be carried in RRCReconfigurationComplete for retaining data during HO . Similar to that, the availability indicator is also needed in RRCReestablishmentComplete message for RLF/HOF scenario.

Proposal 5: UE retains logged data when RLF/HOF occurs and includes the availability indicator in RRCReestablishmentComplete message if any.

Similar to HO and RLF/HOF occurring, the UE can be released to IDLE/INACTIVE state at any time. Discard the AI/ML training data before reporting may result a large scale of data be cleared unnecessarily. The collection efficiency is low and the UE power and memory are also wasted. Therefore, the UE should retain the logged AI/ML training data during RRC state transition, and request to report it when UE connects to network next time.

Proposal 6: UE retains logged data during RRC IDLE/INACTIVE state and includes the availability indicator in RRCSetupComplete/RRCResumeComplete message.

For training data configuration, it is agreed by RAN2 that Immediate MDT is the baseline framework for OAM-centric data collection for network-sided model. The configuration of immediate MDT reuses the RRM configuration, and the principles are [2]:

5.1.2.3 MDT context handling during handover and UE context retrieval The measurements configured in the UE for Immediate MDT should fully comply with the transferring and reconfiguration principles for the current measurements configured in the UE for RRM purpose during handover (including conformance with Rel-8 and Rel-9). The target node releases the measurements configured in the UE for immediate MDT which are no longer needed based on any MDT trace configuration it receives or does not receive. In addition, MDT configuration handling during handover and UE context retrieval depends on MDT initiation from OAM defined in clause 5.1.3: - The MDT configuration configured by management based trace function will not propagate during handover. [Omitted part for LTE and UMTS cases] - For NR, the MDT configuration received by signalling based trace messages for a specific UE will propagate during intra-PLMN handover, intra-PLMN UE context retrieval, and may propagate during inter-PLMN handover or inter-PLMN UE context retrieval if the Signalling Based MDT PLMN List is available and includes the target PLMN. This behaviour applies also for MDT configuration that includes area scope, regardless of whether the source or target cell is part of the configured area scope. This behaviour applies also for Xn inter-RAT handover.

Since for management based MDT in NR, the UEs who perform MDT measurement are selected by the serving gNB, there is no need to propagate the UE context of MDT configuration during HO in the NW side.

For AI/ML training data configuration, it is also controlled by the network. Therefore at least for NW side data collection, the management based solution is more appropriate. And since the AIML training data is collected in CONNECTED, the new training configuration could be sent to the UEs by e.g. RRCReconfiguration message during HO.

Proposal 7: UE discards/clears the AIML training data collection configuration for NW-side model training during HO.

Since immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model, the AIML training data collection configuration should also be discarded/cleared when UE entering IDLE/INACTIVE state.

Proposal 8: UE discards/clears the AIML training data collection configuration for NW-side model training when UE entering IDLE/INACTIVE state.

Memory for NW side training data

It is agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. But it is FFS on the memory size. For logged MDT, the minimum memory size of MDT logged measurements is 64KB. Therefore 64KB could also be assumed as the minimum memory size of AI/ML training data.

Proposal 9: Set the minimum memory size of AI/ML training data logging to be 64KB.

Transmission priority of NW side training data

It is agreed in previous RAN2 meeting that low priority SRB will be used. But whether to use new SRB or use SRB4 is FFS. The legacy logged MDT report is transmitted on SRB2. The priority between SRBs is described in TS38.331 as below:

- SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel (except SRB1 of L2 U2N Remote UE); - SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel (except SRB2 of L2 U2N Remote UE). SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation; - SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel; - SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation. - SRB5 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB5 has a lower priority than SRB1 and SRB3 and can only be configured by the SN serving the SCG when the UE is in NR-DC, after AS security activation.

From the spec, the SRB2/SRB4/SRB5 has a lower priority than SRB1. But there is no fixed priority comparison between SRB2 and SRB4/SRB5. SRB2 could be configured with higher/lower/equal priority with SRB4/SRB5.

Observation 1: There are no fixed priorities among SRB2 and SRB4/SRB5. SRB2 can be configured to have a higher, lower, or equal priority as that of SRB4/SRB5

On one hand, the NW is capable on when to request the UE to report the large amount of AI/ML training data or logged MDT data to avoid the conflict with NAS message transmission, on the other hand, the UE could decide how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. Since the AI/ML training data may have the same minimum memory size as for logged MDT measurements, SRB2 could also be used to transmit the AI/ML training data. There is no need to define a new SRB or use SRB4/5 since the NW could require the data when the traffic load is low, e.g. in the midnight.

Observation 2: NW is capable to schedule the UE to report the large amount of AI/ML training data of logged MDT data and avoid the conflict with NAS message transmission.

Proposal 10: Utilize SRB2 as in the logged MDT report for AI/ML training data transmission.

  - Positioning accuracy enhancements

Training data collection for BM case is enhanced on MDT mechanism which involves UE, gNB, and TCE node. Some CN node may also be utilized for MDT configuration transmission without information change, i.e. for signaling based MDT. Since training data collection is non-time sensitive, the UE can log the training data over a period of time, and then send them to the training node to reduce the air-interface overhead and to avoid frequent connection establishment. PoS case1 is UE-based positioning with UE-side model, and UE side data collection is still under discussion in SI phase. For PoS case1, two options may be used for training data collection with 3GPP protocol implications:

- Option 1: To collect training data by OAM first, similar as BM case, and OAM sends all data to training node.

- Option 2: To collect training data by LMF first, using LPP signaling;

By using option1, the similar training data collection mechanism agreed for BM can be applied for Pos use case between UE and OAM, but the configuration for PoS training data collection, the report content should all be defined in Uu as new parameters which seem more complex.

Observation 3: It is complex and many PoS related parameters should be defined in Uu, if using a similar training data collection mechanism in BM case between UE and OAM for Pos use case.

By using option2, training data for Pos use case should be transmitted from UE to LMF. The location in Deferred 5GC-MT-LR procedure as specified in TS 23.273 can be reported by UE by periodic, triggered and UE available location events [3]. For a request for the UE available location event, a one-shot report will be performed. If periodic or triggered location was requested, the LMF sends a supplementary services LCS Periodic-Triggered Invoke Request to the UE, and the UE should monitors for occurrence of the trigger or periodic event requested. When a trigger or periodic event is detected, the UE should report the event and perform the positioning procedure if fulfill all the conditions (e.g. access an allowed access type, and inside the event report allowed area). So if we choose this option, the data logging, the available indication and on-demand report mechanism agreed for training data collection in BM case cannot be applied to PoS case without SA2 impact based on current LCS procedure.

But since the LCS procedure is not as frequent as RSRP/beam-id measurement in BM case, some legacy NW request LCS procedures could be utilized for training data collection, e.g. NI-LR procedure, 5GC-MT-LR procedure, Deferred 5GC-MT-LR procedure, as shown in Annex.

Proposal 11: Some legacy LCS procedures initiated by NW could be utilized for training data collection, e.g. NI-LR, 5GC-MT-LR, Deferred 5GC-MT-LR.

- Conclusion

In this contribution, we provide our views on the training data collection for NW side model, and the observation and proposals are summarized as follows:

For beam management

- Mechanism related to NW side data collection

Proposal 1: Activation/deactivation of measurements for AI/ML training data collection is not needed, as it can be implicitly achieved by the add/modify/release operation of the related logged measurement configuration.

Proposal 2: Parameters of l3-Threshold, hysteresis and timeToTrigger can be configured for event-triggered training data logging, similar as eventL1 in logged MDT configuration.

Proposal 3: Define trigger condition of sending the availability indication for UE, such as when the UE power state is low, or the storage level of UE AS buffer has reached a configured/specified threshold.

Proposal 4: The report of collected training data includes:

- For training data collection time marker, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE. And the time of each measurement can be represented as a relative time from the absoluteTimeStamp;

- For training data marker, the associated ID received in the training data configuration may be used;

- For training data collection node, collection node index such as TCE ID for MDT could be used;

- The main content of training data collection could be a list similar as logMeasInfoList, and the detailed content can be revised according to further RAN1 input.

Proposal 5: UE retains logged data when RLF/HOF occurs and includes the availability indicator in RRCReestablishmentComplete message if any.

Proposal 6: UE retains logged data during RRC IDLE/INACTIVE state and includes the availability indicator in RRCSetupComplete/RRCResumeComplete message.

Proposal 7: UE discards/clears the AIML training data collection configuration for NW-side model training during HO.

Proposal 8: UE discards/clears the AIML training data collection configuration for NW-side model training when UE entering IDLE/INACTIVE state.

- Memory

Proposal 9: Set the minimum memory size of AI/ML training data logging to be 64KB.

- Priority

Observation 1: There are no fixed priorities among SRB2 and SRB4/SRB5. SRB2 can be configured to have a higher, lower, or equal priority as that of SRB4/SRB5

Observation 2: NW is capable to schedule the UE to report the large amount of AI/ML training data of logged MDT data and avoid the conflict with NAS message transmission.

Proposal 10: Utilize SRB2 as in the logged MDT report for AI/ML training data transmission.

For positioning accuracy enhancements

Observation 3: It is complex and many PoS related parameters should be defined in Uu, if using a similar training data collection mechanism in BM case between UE and OAM for Pos use case.

Proposal 11: Some legacy LCS procedures initiated by NW could be utilized for training data collection, e.g. NI-LR, 5GC-MT-LR, Deferred 5GC-MT-LR.

- References

- R2-2501702, RAN2#129 Meeting Report

- TS37.320 Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2

- TS23.273, 5G System (5GS) Location Services (LCS); Stage 2

- Annex - Legacy LCS types and procedures in TS23.273

4.1a	Types of Location Request

- 4.1a.1	Network Induced Location Request (NI-LR)

With a Network Induced Location Request (NI-LR), a serving AMF for a UE initiates localization of the UE for a regulatory service (e.g. an emergency call from the UE) or for verification of a UE location (country or international area) for NR satellite access.

- 4.1a.2	Mobile Terminated Location Request (MT-LR)

With a Mobile Terminated Location Request (MT-LR), an LCS client or AF external to or internal to a serving PLMN sends a location request to the PLMN (which may be the HPLMN or VPLMN) for the location of a target UE.

- 4.1a.3	Mobile Originated Location Request (MO-LR)

With a Mobile Originated Location Request (MO-LR), a UE sends a request to a serving PLMN for location related information for the UE.

- 4.1a.4	Immediate Location Request

With an immediate location request, an LCS client or AF sends or instigates a location request for a target UE (or group of target UEs) and expects to receive a response containing location information for the target UE (or group of target UEs) within a short time period which may be specified using QoS. In regulatory cases, one or more responses of the target UE's location information can be expected. An immediate location request may be used for an NI-LR, MT-LR or MO-LR.

- 4.1a.5	Deferred Location Request

With a deferred location request, an LCS client or AF sends a location request to a PLMN for a target UE (or group of target UEs) and expects to receive a response containing the indication of event occurrence and location information if requested for the target UE (or group of target UEs) at some future time (or times), which may be associated with specific events associated with the target UE (or group of target UEs). In this version of the specification, only deferred location requests for an MT-LR are supported.