3GPP TSG RAN WG2 #129bis                                                                         	  R2-2501924

Wuhan, China, April 7th – 11th, 2025

Source:	CATT

Title:	Consideration on UE side data collection

Agenda Item:	8.1.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we further discuss the control and configuration of UE side data collection and the UE side data transfer solution 2/3.

- Discussion

  - Control and configuration of UE side data collection

In RAN2#129 meeting, RAN2 discussed and agreed the following agreements for data collection configuration for UE-side model training.

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Based on the agreements above, the UE can send a request for data collection and the network can configure whether UE is allowed to initiate the request for data collection. When the UE is configured to allow initial request data collection configuration, if the UE e.g., has no model for a functionality or has no model for a NW-side additional condition or triggered by OTT server etc., the UE can send the request information to network to request network to provide the data collection configuration. However, it is still FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

From our perspective, if the UE only sends the signal request information e.g., start/stop to network, the network can have no idea what type data does the UE expect to collect, e.g., the UE has no model for NW-side additional condition associated ID=1, NW sends the data collection configuration to UE after receiving the UE’s request, but current NW-side additional condition is associated ID=2, the UE will collect the data under associated ID=2, which is not the UE’s expectation. Instead, the UE can include the desired configuration in the request to network, e.g., desired NW-side additional condition, desired measurement resource configuration etc., in this case, the network can get to know what configuration the UE expects and provide the suitable configuration to UE.

In short, it is better to provide some assistance information (e.g., the desired NW-side additional condition, desired measurement resource configuration etc.) for the network configuration to avoid the unsuitable configuration from network. The detailed request information can be discussed and decided by RAN1.

Proposal 1: Assistance information e.g., the desired NW-side additional condition, desired measurement resource configuration etc. can be reported to network upon UE requests the data collection configuration for training. The detailed request information is up to RAN1.

In addition, an FFS issue was left in RAN2#128 meeting, i.e., whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration. From our perspective, it seems to be unnecessary that the UE reports the indicator to network to indicate UE can’t perform data collection based on received configuration. On the one hand, the UE can provide the sufficient information in the data collection request procedure, to make sure the network can get the sufficient information to configure the effective data collection configuration. Even if the configuration from network is invalid, the UE can send the request again to network to request the suitable configuration. One the other hand, if the UE can’t perform data collection based on received configuration, e.g., low power or low memory, the UE can stop the data collection, and continue to collect data when the UE has high power or sufficient memory again.

Proposal 2: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

  - RAN aspects related to data transfer over UP/CP

In RAN#106 meeting, RAN has sent a LS to SA/SA2/SA5 about the AI/ML UE sided data collection [1] and asked TSG SA to start studying the transfer of data over UP for solution 2 in WG SA2 and transfer of data over UP for solution 3 in WG SA5:

TSG RAN respectfully thanks WG SA2/SA5 for their response LSs on AI/ML UE side data collection. TSG RAN continued further discussions in RAN#106 and has agreed to a way forward in RP-243307. More specifically, TSG RAN has agreed to the following: From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements. Transfer of data over UP for Solution 3 can be studied by SA5 RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer. RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now. TSG RAN has also agreed that RAN2 will continue to discuss and study UE sided data collection in WG RAN2 during Rel-19 time frame. TSG RAN will make a final decision on the way forward taking into account SA inputs when available. TSG RAN will have a checkpoint in Q2, 2025

    - RAN aspects related to data transfer over UP

Solution 2 UP

For data transfer Solution 2 over UP, RAN2 asks SA2 to provide feedback on fulfillment of visibility and controllability requirements.

In previous meetings of SA2, there was no discussion on UE data collection. However, in the R20 FS_AIML_CN_Ph2 SID that was approved in February [2], UE data collection was listed as WT#1. Discussions on the study report (TR) will begin at SA4#168 meeting:

Specifically, the objectives include: - WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training. Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389. Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase. Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

From RAN2 point view, since the legacy UP tunnel is between UE and UPF, the data content is visible to CN, and the visibility to the MNO can be discussed between SA2 and SA5. RAN could control and configure the UE side data collection, e.g. to decide the resource allocation and configure the DRBs, but the data visibility to the gNB seems not required.

Observation 1: SA2 lists UE side data collection over UP as one WT in Rel-20 study.

Proposal 3: Data visibility to gNB is not required, and fulfillment of visibility and controllability for data transfer over UP for Solution 2 could wait for the study by SA2 in Rel-20.

Solution 3 UP

In the previous LS between RAN and SA5 [3], SA5 has provided their view on solution 3:

For option 3: CP tunnel: - The feasibility for the functionality of “Server for data collection for UE-side model training” to be carried by TCE as defined in SA5 MDT mechanism or MCE as defined in SA5 QoE mechanism for CP tunnel can be studied in SA5. SA5 will need to evaluate potential enhancement that may be needed based on the progress in RAN. UP tunnel: - The feasibility of UP tunnelling methods covering UE, gNB, OAM, and Server for data collection for UE-side model training needs to be studied, along with potential coordination with SA2.

For data transfer Solution 3 over UP, the feasibility needs to be studied by SA5.

And SA5 has replied the RAN LS [1] after SA5#159 meeting [4]:

1 Overall description SA5 would like to thank TSG RAN for their LS in RP-243316 on AI/ML UE sided data collection. SA5 has discussed the questions about the feasibility for transfer of data over UP for Solution 3 (UE-> gNB->OAM -> Server). SA5 has made the following analysis about the termination points of the UP tunnel for this solution: 1. If the UP tunnel is between UE and Server, SA5 does not see any difference between solution 3 and solution 1a); 2. If the UP tunnel is between UE and 5GC, SA5 does not see any difference between solution 3 and solution 2. SA5 assumes that the UP tunnel associated with data transfer is between UE and gNB for the solution 3. SA5 would like to get confirmation of this assumption from TSG RAN and RAN2, as well as RAN’s perspective on the UP tunnel termination points. Based on the assumption above, SA5 would like to confirm that it is feasible to develop the OAM solution with the necessary controllability and different levels of visibility for transfer of UE side data over UP using solution 3. 2 Actions To: TSG RAN, RAN2 ACTION: SA5 kindly asks TSG RAN and RAN2 to confirm if SA5 assumption is correct and take the SA5’s reply above into consideration.

The LS mentioned the SA5 assumes that the UP tunnel associated with data transfer is between UE and gNB for the solution 3, and based on such assumption, it is feasible to develop the OAM solution with the necessary controllability and different levels of visibility for transfer of UE side data over UP using solution 3.

SA5 assumption is not an existing mechanism. The UP tunnel can only be established between the UPF and the UE currently. For the PDU session establishment, CN should send PDU SESSION RESOURCE SETUP REQUEST message to the gNB, and then the gNB decides the resource allocation and reconfigures DRBs for the PDU session. The DRB configuration in RadioBearerConfig should mandatory include the sdap-Config (when connected to 5GC), and pdu-Session should be mandatory provided in the SDAP-Config IE. It is not possible for the UP tunnel terminated in the gNB now.

Observation 2: SA5 assumption of UP tunnel between UE and gNB is incorrect based on current mechanism.

Proposal 4: Clarify the UP tunnel terminated node and send reply LS to SA5.

In addition, if a tunnel is established between OAM and gNB/UE, the SA2 impact is needed, e.g. the authorization should be performed in CN, and CN to decide whether and how to establish a session for the specific service. Since SA2 has already postpone the UP Data Collection Solution 2 issue to Rel-20, the likelihood of discussing Solution 3 over UP in Rel-19 is also slim.

Observation 3: Data transfer over UP for Solution 3 also needs SA2 impact, e.g. authorization, session establishment.

Currently there is no UP tunnel involving OAM in the traditional sense. To compare the CP and the UP solutions:

  - RAN2 has no specific QoS requirements that need to be met by the UP solution compared with the CP solution;

  - Even it is feasible by SA5 LS answer, a bran-new protocol stack may need to be designed for introducing tunnel, since currently OAM does not even have the concept of protocol stack. The complexity is high and the spec impact is great;

  - Reuse of existing CP solutions (enhanced with MDT or QoE) is simple which could already fulfill the requirement of feasibility;

Observation 4: Data transfer over UP for Solution 3 may need to design bran-new protocol stack for UP tunnel which has great RAN and SA impact.

Based on the analyses above, we do not see the benefit to use the solution3 UP, or to postpone such discussion to Rel-20 to align with SA2 timeline.

Proposal 5: RAN2 to deprioritized or postpone the Data transfer over UP for Solution 3 to Rel-20 to align with SA2 timeline.

    - RAN aspects related to data transfer over CP

Solution 2 CP

For solution2 over CP, NAS message will be used for AI training data transmission. RAN has already concluded that transfer of data for Solution 2 over CP can meet the visibility and controllability RAN requirements. NAS message will be carried in UL/DL direct transfer message in RRC, so the 9000 bytes PDCP SDU size limit is also applicable for NAS message transmission in the air-interface.

Observation 5: The 9000 bytes PDCP SDU size limit is also applicable for NAS message transmission in the air-interface.

NAS message will be transferred via SRB2 if SRB2 has been established, otherwise SRB1 will be used. Since RRC cannot see the content of the NAS message, it is hard to control the transmission priority for large scale of UE side AI training data by RAN.

Observation 6: RAN is hard to control the transmission priority for large scale of UE side AI training data transmission, if the solution2 over CP is adopted.

And even RAN has already concluded that transfer of data for Solution 2 over CP can meet the visibility and controllability RAN requirements, the applicability of solution2 over CP should be studied by SA2.

Proposal 6: The applicability of solution2 over CP to transfer large scale of UE side AI training data should be studied by SA2.

Solution 3 CP

To transmit large AI training data over CP, we think the TR about segmentation of UE capability information could be considered as background:

There are two options to split and transmit large data in Uu control plane based on study on optimizations of UE radio capability signaling [5]:

6.2.1.3.1 Segmentation at RRC level In the segmentation of UE capabilities in the RRC level, for each segment, a type (e.g. notLastSegment, lastSegment) as well as a segment number is added. The RRC segmentation can be done in different ways as described below. Option 1: "hard" split RRC Segmentation: This approach is similar to ETWS/CMAS in LTE and NR. The total UE radio capability information is encoded as one structure (message or information element) similar to non-segmented case. This is then split into multiple segments in bits. The network can only decode the complete capabilities after assembling all parts, rather than decoding each part. Option 2: "soft" split RRC Segmentation: UE radio capability information is encoded separately in parts and carried by multiple messages or information elements (smaller than 9000 bytes for NR or 8188 bytes for LTE). The network can decode and process every segment and combine the content after all the segments are received.

Considering that it is meaningless to decode each segment separately of the UE capability information, the UL UE capability transmission uses the “hard” split option similar as ETWS/CMAS at last. And network can only decode the complete capabilities after assembling all parts.

For AI training data, the differences from the UE capability information report are:

  - AI training data does not need to create packages based on NW request requirement, and it can directly slice content from UE memory, which does not need large RRC processing delay;

  - UE AS layer collects measurement data from PHY layer based on samples. Therefore, each sample entry may be small and can be stored in cleartext which is more like MDT measurement result.

As in reply LS from SA5[3], the feasibility for the functionality of “Server for data collection for UE-side model training” to be carried by TCE as defined in SA5 MDT mechanism or MCE as defined in SA5 QoE mechanism for CP tunnel can be studied in SA5. To discuss Solution 3 over CP on air-interface, we take reusing MDT mechanism as an example:

- The MDT report is in cleartext on the air-interface. gNB packages the received MDT report as a file and sent it to the OAM through the IP packet. The transmission mode on the cable network is not critical, how to organize the data transmission on the air-interface is more important;

- At least for BM use case, the AI training data is collected from the PHY layer periodically which is visible for RRC layer. If MDT mechanism is reused, the AI data transmitted on the air-interface can be cleartext.

Observation 7: At least for BM use case, the AI training data is collected from the PHY layer periodically and is visible to RRC layer which can be transmitted on the air-interface in cleartext.

Based on the analysis above, The AI training data collection solution over CP is more suitable to use “soft” split option which is similar to the logged MDT report. Each segment can be decoded independently, and then packet loss has no significant impact in the limit case.

For the CP solution scalability issue concerned by some companies, if  “soft” split option is used, the UE could send the AI training data segments one by one until all the stored data is transmitted successfully, each segment should include as many samples as possible, not more than the PDCP SDU limit size. The last segment should be marked with a 1 bit indicator similar as the logged MDT report. Based on this mechanism, the scalability does not need to be considered, since each segment could be decode individually.

Observation 8: To use a “soft” split RRC segmentation method similar as logged MDT report, the AI training data transmission does not need to consider the scalability.

Proposal 7: To use a “soft” split RRC segmentation method similar as logged MDT report for solution3 over CP to avoid the scalability problem.

For NW side data collection, it is agreed that:

Agreements on NW side data collection

- Low priority SRB will be used.    FFS new SRB or use of SRB4

For the transmission priority of large scale of UE side AI training data, the same solution used for NW side data collection could be used. It is, to use a low priority SRB, either a new SRB or legacy SRB4.

Proposal 8: The low transmission priority for NW side data collection is also used for UE side data collection, if the solution3 over CP is adopted.

- Conclusion

In this contribution, we provide our views on the UE side data collection, and the observation and proposals are summarized as follows:

Control and configuration of UE side data collection

Proposal 1: Assistance information e.g., the desired NW-side additional condition, desired measurement resource configuration etc. can be reported to network upon UE requests the data collection configuration for training. The detailed request information is up to RAN1.

Proposal 2: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

RAN aspects related to data transfer over UP/CP

  - For solution2 over UP:

Observation 1: SA2 lists UE side data collection over UP as one WT in Rel-20 study.

Proposal 3: Data visibility to gNB is not required, and fulfillment of visibility and controllability for data transfer over UP for Solution 2 could wait for the study by SA2 in Rel-20.

  - For solution3 over UP:

Observation 2: SA5 assumption of UP tunnel between UE and gNB is incorrect based on current mechanism.

Proposal 4: Clarify the UP tunnel terminated node and send reply LS to SA5.

Observation 3: Data transfer over UP for Solution 3 also needs SA2 impact, e.g. authorization, session establishment.

Observation 4: Data transfer over UP for Solution 3 may need to design bran-new protocol stack for UP tunnel which has great RAN and SA impact.

Proposal 5: RAN2 to deprioritized or postpone the Data transfer over UP for Solution 3 to Rel-20 to align with SA2 timeline.

  - For solution2 over CP:

Observation 5: The 9000 bytes PDCP SDU size limit is also applicable for NAS message transmission in the air-interface.

Observation 6: RAN is hard to control the transmission priority for large scale of UE side AI training data transmission, if the solution2 over CP is adopted.

Proposal 6: The applicability of solution2 over CP to transfer large scale of UE side AI training data should be studied by SA2.

  - For solution3 over CP:

Observation 7: At least for BM use case, the AI training data is collected from the PHY layer periodically and is visible to RRC layer which can be transmitted on the air-interface in cleartext.

Observation 8: To use a “soft” split RRC segmentation method similar as logged MDT report, the AI training data transmission does not need to consider the scalability.

Proposal 7: To use a “soft” split RRC segmentation method similar as logged MDT report  for solution3 over CP to avoid the scalability problem.

Proposal 8: The low transmission priority for NW side data collection is also used for UE side data collection, if the solution3 over CP is adopted.

- References

- RP-243316, LS on AI/ML UE sided data collection, TSG RAN

- S2-2502788, New SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)_Ph2, vivo (Moderator)

- S5-247219, Reply LS on AIML Data Collection, SA5

- S5-250828, Reply LS on AI/ML UE sided data collection, SA5

- TR37.873, Study on optimizations of UE radio capability signalling; (Release 16)