3GPP TSG RAN WG2 #129bis		R2-2501926

China, Wuhan, Apr. 7th – 11th, 2025

Source:	CATT, Turkcell

Title:	Simulation results of Model Generalization

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#129 meeting, regarding model generalization in cell configuration aspect, the following agreements were agreed [1].

Agreements 1: two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study. 2: FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2). 3: agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2). Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 2: generalization parameters for FR1 Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 3: generalization parameters for FR2

In this document, the further simulation results are provided for FR1 based on the configurations agreed in RAN2#129.

- Discussion

In our simulation, we focus on the following simulation combinations in Table 1 and Table 2. The parameters of simulation assumption, AI model and simulation results are provided in Table 3.

Table 1: Generalization cases of Configuration #1

| Case | Training dataset | Inference dataset |
| --- | --- | --- |
| Baseline | Configuration #1 | Configuration #1 |
| GC#1 | Configuration #2 |  |
| GC#2 | Configuration#1+ Configuration#2 |  |

Table 2: Generalization cases of Configuration #2

| Case | Training dataset | Inference dataset |
| --- | --- | --- |
| Baseline | Configuration #2 | Configuration #2 |
| GC#1 | Configuration #1 |  |
| GC#2 | Configuration#1+ Configuration#2 |  |

Table 3: Simulation report table for different cell configurations for FR1

| Report parameters |  | CATT |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 30 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | No |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | N/A |
|  | Number of TX beams | 8 |
|  | Number of RX beams | 2 |
|  | Measurement reduction rate(50%,…Note2) | 50% |
|  | Observation window(Note3) | N/A |
|  | Prediction window (Note3) | N/A |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 61600/7700 |
|  | Testing | 7700 |
| AI/ML model input/output | Model input (Note 5) | Cell-level predicted RSRP of all cells |
|  | Model output(Note 6) | Cell-level predicted RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | MLP |
|  | Model complexity in a number of parameters(M) | 916.797M |
|  | Model complexity in model size (e.g. Mbyte) | 0.086MB |
|  | Computational complexity [FLOPs] | 19.68M |
| Metrics | Average L3 cell level RSRP difference (dB) | See Table 4-5 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |

Table 4: Simulation results for Configuration #1

| Case | Training dataset | Inference dataset | RSRP-Differ (dB) | ABS of RSRP Difference between GC#n and Baseline (dB) |
| --- | --- | --- | --- | --- |
| Baseline | Configuration #1 | Configuration #1 | 0.321 | N/A |
| GC#1 | Configuration #2 |  | 0.770 | 0.449 |
| GC#2 | Configuration#1+ Configuration#2 |  | 0.313 | 0.008 |

Table 5: Simulation results for Configuration #2

| Case | Training dataset | Inference dataset | RSRP-Differ (dB) | ABS of RSRP Difference between GC#n and Baseline (dB) |
| --- | --- | --- | --- | --- |
| Baseline | Configuration #2 | Configuration #2 | 0.315 | N/A |
| GC#1 | Configuration #1 |  | 0.620 | 0.315 |
| GC#2 | Configuration#1+ Configuration#2 |  | 0.332 | 0.017 |

Like the simulation of speed and frequency, the RSRP difference in GC#1 is larger than that of baseline, which increases approximate 0.449dB for Configuration#1 and 0.305dB for Configuration#2. And the reason is that in GC#1, the model is trained using the data set of different cell configuration, which lacks the features of the data set in baseline. The model can’t learn some of the specific features of the data set of different cell configuration which results in higher RSRP difference.

Observation 1: The RSRP difference in GC#1 is the highest among all generalization cases. I.e. the GC#1 shows the worst predication performance among these 3 cases for different cell configurations for FR1, but it’s still acceptable from prediction accuracy perspective.

The RSRP difference of GC#2 is close to that of baseline for both configurations, since the AI model of GC#2 can learn the features both in Configuration#1 and Configuration#2.

Observation 2: Using a mixed dataset (GC#2) improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size).

Even though the RSRP difference in GC#1 and GC#2 is higher than that of the baseline, it does not bring significant decline on performance. I.e. the RSRP difference growth is still under 0.02dB. Hence, the results of generalization with different cell configurations are applicable to AI predication.

Observation 3: The simulation results prove that generalization for different cell configuration is feasible for FR1.

- Conclusion

In this contribution, we provide our views on the simulation of generalization for different cell configurations, and the observations are summarized as follows:

Observation 1: The RSRP difference in GC#1 is the highest among all generalization cases. I.e. the GC#1 shows the worst predication performance among these 3 cases for different cell configurations for FR1, but it’s still acceptable from prediction accuracy perspective.

Observation 2: Using a mixed dataset (GC#2) improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size).

Observation 3: The simulation results prove that generalization for different cell configuration is feasible for FR1.

- References

- R2-2501702, RAN2#129 Meeting Report