3GPP TSG-RAN WG2 Meeting #129b	R2-2501933

Wuhan, China, 7 - 11, Apr, 2025

Agenda Item:	8.1.3

Source:	Xiaomi

Title:	Discussion on NW-side data collection

Document for:	Discussion

Introduction

For NW side model, data collection is essential to collect data to train AI model. In this contribution, we discuss open issues of NW-side data collection.

Discussion

Network-side data collection for BM

Data report

Buffer Full Indication

During previous RAN2 discussion, low power bit indication and data availability indication were agreed to assist network to decide when/whether/how UE performs NW-side data collection data logging and reporting. It was also agreed in RAN2 #127 meeting that UE will stop measuring and logging for NW-side data collection when UE reaches its buffer limitation.

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

To avoid losing necessary data collection for NW-side model training due to temporary measurement stop at UE-side, RAN2 further agreed UE can report to NW when its AS buffer for data logging becomes full, so that the network could timely request UE to report its logged data upon receiving the buffer full indication properly. UE can resume measurement and log for NW-side data collection after reporting logged data to network.

- The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after).

During the discussion, it was not clear when the buffer full indication should be reported. In our understanding, similar as low power state (which is determined up to UE implementation), determination of buffer full at UE-side can also be up to UE implementation.

Proposal 1: Buffer full indication is supported to allow network timely retrieve logged data when UE buffer is almost full. It’s up to UE to decide whether buffer is full, i.e. before or after buffer is full.

Availability Indication Trigger Condition

Data availability indicaiton is agreed to be supported in RAN2 #129 meeting, while its trigger condition is FFS.

- Data availability indication is supported. FFS when this would be triggered

In our understanding, it is not desirable for a UE to frequently report its data availability indication, which is power-consuming. Data availability indication should be triggered by the UE upon certain trigger condition(s) is met or based on network request.

Recalling that it’s agreed NW can configure radio condition-based logging event, which means UE will only collect data if radio condition event is fulfilled. When UE reports low power indication to network, it is possible that radio condition event has not been fulfilled at UE side before UE sending the indication, which lead to no data logging/stored at UE side. However, in such case, NW is not aware whether UE has any logged data. NW will send another request to check whether UE has stored data or not, followed by UE's response with a data availability indication. Alternatively, to avoid the additional signalling overhead and delay, UE can report low power indication together with a data availability indication in data availability reporting. With that, NW can decide whether to request UE to report stored data or not immediately.

Proposal 2: UE triggers data availability indication upon 1) network requests, 2) reporting low power bit indication.

As discussed earlier, data availability indication, low power bit indication and buffer full indication can be used to assist network deciding whether to retrieve logged data collected by the UE. Therefore, similar as other UE assistance information, UAI can be used by UE to report those indications.

Proposal 3: UE reports data availability indication, low power bit indication, and buffer full indication via UAI.

Release of logged data

As captured in the RRC running CR [4], new UE variable is introduced to store the logged data, similar as VarMeasReportList , UE can release the logged collected data after reporting the data to network. UE doesn’t need to store the old data anymore. Old data can be released to clear the buffer and UE can collect new data.

Proposal 4: UE releases the logged data after reporting.

If NW has collected enough data, NW may release the data collection configuration. In this scenario, UE shall also release the logged data. However, if NW modifies training data collection configuration, e.g. modify the CSI-ReportConfig for set A of beams, it’s not clear whether UE shall release or keep the logged data. It could be beneficial for UE to keep the logged data, considering NW may still need the old data to train a generic model. On the other hand, network can directly release configuration if network sees no need for UE keeps logging the data. In this case, NW has the flexibility to control whether UE can keep the data implicitly.

Proposal 5: If NW releases the data collection configuration, UE releases the logged data. Otherwise, UE keeps the logged data.

Mobility handling

During the mobility, source cell would provide the data collection configuration to target cell in AS-Config via HandoverPreparationInformation. If target cell is not interested in the data collection, target cell can release the data collection configuration. According to P6, the data would also be released. On the other hand, if the source cell doesn’t want to expose logged data to target cell, source cell can also choose to release the data collection configuration before handover. UE would release the data accordingly. Therefore, we don’t see the need to introduce specific handling to indicate UE keep or release the data. NW can control UE to keep or release logged data by keep or release of the data collection configuration.

Proposal 6: If data collection configuration is released during handover, UE releases the logged data as well. Otherwise, UE keeps the logged data.

Another open issue is the data content, it’s FFS what to log,

Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log

We believe the data content should at least contains L1 measurement results and maybe timing information for temporal prediction. The detail can be decided by RAN1. But after handover, UE shall report mobility related information.

RAN1 agreed the NW side additional condition can be associated with a ID. This ID is unique within a cell.

Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell

Therefore, same functionality in different cells may be associated with different IDs. Furthermore, it’s difficult to ensure the ID is global unique. Therefore, it’s possible the same ID means different NW side additional condition in two cells. When UE moves from one cell to another, UE may log data collected from multiple cells. These cells may be associated with different NW side additional condition. Therefore, when UE reports the data, UE shall indicate the cell and NW-side additional condition that the data is collected. The associated NW information can be cell ID and associated ID.

Proposal 7: If logged data are collected from multiple cells, UE logs and reports cell ID and associated ID associated with the L1 measurement results.

Unified handling for RLF and RRC_IDLE/INACTIVE

During last RAN2 #129 meeting, whether a unified handling logged data method in idle/inactive and RLF cases is FFS.

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

Unlike handover, RLF is not controlled by NW and RRC reestablishment is up to UE. There might be a risk to expose logged data to an unintended cell, if logged data is kept upon RLF. However, this depends on the NW deployment and implementation. If all cells are deployed by the same NW vendor around the nerighbourhood, there may be no concern on data exposure to other cells. However, on the other hand, we understand this may not be practical in reality. It can be considered as network implementation on whether source cell prefers such logged data to be maintained or not, which can be up to network control/configuration. Since NW has no idea on when RLF will happen to a UE, this configuration can be configured as part of data collection configuration.

Proposal 8: NW can configure UE whether to keep the logged data upon RLF, as part of the data collection configuration.

Another open issue is whether logged data is released upon UE leaving RRC_CONNECTED. Except for RLF, UE leaving RRC_CONNECTED is under NW control by RRCRelease. NW can request UE to report logged data before sending RRCRelease. However, if NW is overloaded, it’s also beneficial for UE to keep the data and report later when entering RRC_CONNECTED again. Since the whole procedure is under NW control, a compromised way is to enable NW to configure UE whether to release logged data upon leaving RRC_CONNECTED. The indication can be carried in RRCRelease.

Proposal 9: NW can configure UE whether to keep logged data upon leaving RRC_CONNECTED in RRCRelease.

If NW didn’t configure UE to keep the data in collection configuration or UE leaves RRC_CONNECTED without reception of RRCRelease, the default behaviour should be UE releases the logged data.

Proposal 10: If NW didn’t configure UE to keep the data, UE releases the logged data upon RLF and leaving RRC_CONNECTED.

Data collection control

It is not clear that whether multiple data collection configurations can be provided to UE simultaneously or not.

As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

We understand the data would be use cases specific, e.g. BM and CSI, or even sub use case specific. For example, one data sample of spatial BM prediction consists measurement results of set A and B beam at the same time. While, another data sample of temporal BM prediction requires historic measurement results of set B beam and one measurement result of set A beam. Therefore, NW should be allowed to provide multiple data collection configurations for different (sub) use cases.

Proposal 11: Multiple NW-side training data collection configurations can be configured to UE.

A subsequent question is whether dynamic activation/deactivation of data collection is supported. We understand the real question is whether NW-side training data collection needs to be controlled by L1/L2 signaling. It’s always possible for NW to setup/release the data collection configuration by RRC. According to LS from RAN1 [1], it is observed that the latency requirement of training data collection is rather relaxed. There is no need to introduce L1/L2 dynamic control from latency point of view.

Proposal 12: L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

Even if UE supports the data logging, it’s possible that different UE chipset/vendor has different implementation. The supported storage size may be different. As indicated in [1], different types of training data may require different storage sizes. Also, the data reporting configuration may be associated with the storage size, e.g. longer report period would result in UE storing more data. UE can report the supported data logging storage size to NW. NW can decide which data is to be logged by UE and the appropriate configuration, e.g. report period. The reported logging storage size can be used across all use cases.

Proposal 13: UE can support different data logging storage size across all use cases, which can be indicated by UE capability.

Network-side data collection for Pos

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

For AI positioning, there are there use cases with the AI model deployed at Network side, however, we only consider the first priority use cases in this section.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Note:	For gNB- and OAM-centric data collection, there may be a need to consult with RAN3 and SA5 whether/how OAM is to be involved.

Note:	For possible impacts due to positioning use cases, there may be a need to consult with RAN3 whether/how NRPPa is to be involved.

According to the TR 38.843 [2], we had made some conclusions on NW-sided data collection and Model training for positioning as below.

Model Training:

o	For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.

	Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

o	For LMF-side model, the LMF is the termination point for training data.

For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection. However, if the termination point is the LMF, the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training. Since we haven’t agreed that the LMF is the termination point for training data in Case 3a, we won’t consider the LMF as the termination point for now.

For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 14: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 15: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

For the network side data collection for AI positioning, SA2 had made some progresses as below [3]:

Figure 5.18.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

3.	The LMF checks the user consent for data collection for model training.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

According to SA2 conclusion, the LME will request the UE in an area of interest to perform data collection. Which means that the UE should stop data collection if the UE is not in the area of the intertest. Therefore, we think the LMF should send the area of interest to the UE for UE performing data collection.

Proposal 16: For Case 3b, LMF sends area of interest of data collection to UE when requesting it to perform the data collection.

For non-AI/ML positioning methods, the UE measures PRS based on PRS configuration provided by LMF, which is common for all UEs. For AI/ML based training data collection, the same PRS configuration can also be used by the UE to collect data for NW-side model. On the other hand, considering offline training is used, LMF may configure UE with a different measurement configuration compared to non-AI/ML positioning method. Therefore, LMF may also indicate a different PRS configuration  to the UE for NW-side data collection.

Proposal 17: For Case 3b, LMF indicates the PRS configuration for UE performing NW-side data collection, which can be the same or different PRS configuration for non-AI/ML PRS configuration.

Conclusion

Based on the discussion in section 2, we have following proposals:

For Beam management

Proposal 1: Buffer full indication is supported to allow network timely retrieve logged data when UE buffer is almost full. It’s up to UE to decide whether buffer is full, i.e. before or after buffer is full.

Proposal 2: UE triggers data availability indication upon 1) network requests, 2) reporting low power bit indication.

Proposal 3: UE reports data availability indication, low power bit indication, and buffer full indication via UAI.

Proposal 4: UE releases the logged data after reporting.

Proposal 5: If NW releases the data collection configuration, UE releases the logged data. Otherwise, UE keeps the logged data.

Proposal 6: If data collection configuration is released during handover, UE releases the logged data as well. Otherwise, UE keeps the logged data.

Proposal 7: If logged data are collected from multiple cells, UE logs and reports cell ID and associated ID associated with the L1 measurement results.

Proposal 8: NW can configure UE whether to keep the logged data upon RLF, as part of the data collection configuration.

Proposal 9: NW can configure UE whether to keep logged data upon leaving RRC_CONNECTED in RRCRelease.

Proposal 10: If NW didn’t configure UE to keep the data, UE releases the logged data upon RLF and leaving RRC_CONNECTED.

Proposal 11: Multiple NW-side training data collection configurations can be configured to UE.

Proposal 12: L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

Proposal 13: UE can support different data logging storage size across all use cases, which can be indicated by UE capability.

For positioning

Proposal 14: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 15: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 16: For Case 3b, LMF sends area of interest of data collection to UE when requesting it to perform the data collection.

Proposal 17: For Case 3b, LMF indicates the PRS configuration for UE performing NW-side data collection, which can be the same or different PRS configuration for non-AI/ML PRS configuration.

Reference

[1] R2-2311720	Reply LS on Data Collection Requirements and Assumptions

[2] TR 38.843 v18.0.0 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[3] TS 23.273-j00, Technical Specification Group Services and System Aspects; 5G System (5GS) Location Services (LCS)