3GPP TSG RAN WG2 Meeting #129bis              	                	                          R2-2501942

Title:	[Draft] Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	R2-2500015 (R1-2410922)

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	Xiaomi, Ericsson (to be RAN2)

To:	RAN1, RAN3, SA2, SA5

Contact person:	Ziyi Li

liziyi5@xiaomi.com

Marco Belleschi

marco.belleschi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

RAN2 thanks RAN1 for the LS on signalling feasibility of dataset and parameter sharing. RAN2 discussed the candidate solutions and feasibility of standardized signaling for NW-side sharing dataset/model parameter to UE or UE-side training entity for below options:

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

Based on RAN1 LS, RAN2 studied the principles and approaches of NW-side sharing dataset/model parameter transfer to UE or UE-side training entity, and reached following agreements:

Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles:

- A1 - Size: From RAN2 point of view, RAN2 aims to define a unified solution (e.g. OTA, non-OTA, or a combined) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs);

- A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported;

- A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server);

- A4 - Latency: Relaxed latency requirement and infrequent update;

- A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server).

- A6: Proprietary information of the network should be respected and not disclosed.

RAN2 concluded the following approaches for NW-side sharing dataset/model parameter transfer asked by RAN1:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) RAN2 analyzed area data dataset/model parameter transfer CSI compression data collection at gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training UE-side training entity for two-sided UE part model training. Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) RAN2 analyzed area UE-side training entity for two-sided UE part model training dataset/model parameter for training dataset/model parameter transfer CSI compression data collection at gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training data. NOTE: The data transfer (e.g. raw data, dataset and/or model parameter, etc, up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view. Feasibility analysis of non-OTA solutions is required to be evaluated by RAN3, SA2, and SA5. It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| - OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| - CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| - gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

For OTA approaches, i.e., ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but no consensus on feasibility. Further feasibility analysis of OTA solution 2 and 3 is required to be evaluated by RAN3, SA2, and SA5.

- OTA solution 1a: gNB -> UE via CP (8 companies think it is feasible, 7 companies think it is not feasible)

- OTA solution 2: CN -> UE via gNB (7 companies think it is feasible, 10 companies think it is not feasible)

- OTA solution 3: OAM -> UE via gNB (8 companies think it is feasible, 2 companies think it is feasible but no benefit, 8 companies think it is not feasible)

RAN2 further have following observations on challenges and potential suitable scenarios for above OTA solutions:

| OTA Solution 1a | Challenges | - Not feasible to have RRC buffer >200Mbyte for UE in 5G - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) |
| --- | --- | --- |
|  | Potential suitable scenario | - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs. |
| OTA Solution 2 | Challenges | - Same challenges as OTA solution 1a, if OTA solution 2 via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desireable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution 2 via CP is the same as OTA solution 1a. - OTA solution 2 and its feasibility is required to be evaluated by RAN3 and SA2. |
| OTA Solution 3 | Challenges | - Same challenges as OTA solution 1a, if OTA solution 2 via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution 3 via CP is the same as OTA solution 1a. - OTA solution 3 and its feasibility is required to be evaluated by RAN3 and SA5. |

# Action

To RAN1:

Action: RAN2 respectfully requests RAN1 to take above replies into account in future discussion and ask the related WGs for further feasibility analysis.

To RAN3, SA2:

Action: RAN2 respectfully asks RAN3/SA2 to study the feasibility of non-OTA solutions and OTA solution 2 based on RAN2 agreed principles.

To RAN3, SA5:

Action: Action: RAN2 respectfully asks RAN3/SA5 to study the feasibility of non-OTA solutions and OTA solution 3 based on RAN2 agreed principles.

# Dates of the next TSG RAN WG2 meetings

TSG RAN2 Meeting #130		19th – 23rd May, 2025	Malta, MT

TSG RAN2 Meeting #131		25th – 29th  Aug, 2025	India

# Annex – RAN2 agreements