3GPP TSG RAN WG2 Meeting #129bis                                          R2-2501944

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	8.12.3

Source:	Xiaomi

Title:	Discussion on open issues of RRC spec impact

Document for:	Discussion and Decision

# Introduction

During [Post129][208][ MIMO_Ph5] Issues and proposals on RRC impact (Ericsson), following open issue is proposed to be discussed:

Proposal 4 Discuss which option to use for the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank: Option 2: (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2; Option 3: (N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING.

In this contribution, we continue discuss the remaining open issue for Rel-19 CBSR.

# Discussion

During the discussion, following two options were proposed for further down-selection:

Option 2:

Alternatively, (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2. Note there are many (X1, X2) values not applicable to type-II and type-II Doppler codebook, but only applicable to type-I single-panel codebook. This option does not need separate parameter/description for X1, X2, and can directly reflect applicable (X1, X2) pairs in the CHOICE of (X1, X2). An example is shown below.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-codebookSubsetRestriction-r19		typeI-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeI-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (384)), thirtytwo BIT STRING (SIZE (192)), sixtyfour BIT STRING (SIZE (1024))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (192)), thirtytwo BIT STRING (SIZE (256)), sixtyfour BIT STRING (SIZE (512))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-two-r19           CHOICE {twentyfour BIT STRING (SIZE (48)), thirtytwo BIT STRING (SIZE (64)), sixtyfour BIT STRING (SIZE (128))},

four-four-r19          CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))}

}

typeII-codebookSubsetRestriction-r19		typeII-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeII-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (12)), thirtytwo BIT STRING (SIZE (16)), sixtyfour BIT STRING (SIZE (32))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-two-r19           CHOICE {thirtytwo BIT STRING (SIZE (4)), sixtyfour BIT STRING (SIZE (8))}

}

Option 3:

(N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING. Variable BIT STRING is widely used in UE capability signaling, which can be utilized to simplify the signaling of CBSR. This option needs extra parameters for (X1, X2) and capture nonapplicable pairs in FD. An example is shown below.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two, four-four}	OPTIONAL    -- Need R

typeII-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two}	OPTIONAL    -- Need R

typeI-codebookSubsetRestriction-r19		BIT STRING (SIZE (24, 32, 48, 64, 96, 128, 192, 256, 384,   512, 1024))							 OPTIONAL    -- Need R

typeII-codebookSubsetRestriction-r19		BIT STRING (SIZE (4, 6, 8, 12, 16, 24, 32, 64))		OPTIONAL    -- Need R

In our understanding, the key difference between two options is whether CBSR is signalled as a variable BIT STRING or signalled as a fixed size with CHOICE structure. Compared with Option 2, Option 3 can significantly save signaling overhead.

During the discussion, the main concern from proponent of Option 2 is that it is not clear whether variable bit string length is feasible in ASN.1.

We observed there are already some implementations with variable string length in current ASN.1:

For example, QoE application layer measurement configuration container has a variable size of 1 to 8000.

-- ASN1START

-- TAG-APPLAYERMEASCONFIG-START

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowedSRB4-r17               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...,

[[

rrc-SegAllowedSRB5-r18               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

idleInactiveReportAllowed-r18        ENUMERATED {enabled}                                                       OPTIONAL  -- Need R

]]

}

MeasConfigAppLayer-r17 ::=           SEQUENCE {

measConfigAppLayerId-r17             MeasConfigAppLayerId-r17,

measConfigAppLayerContainer-r17      OCTET STRING (SIZE (1..8000))                                              OPTIONAL, -- Need N

serviceType-r17                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}   OPTIONAL, -- Need M

pauseReporting-r17                   BOOLEAN                                                                    OPTIONAL, -- Need M

transmissionOfSessionStartStop-r17   BOOLEAN                                                                    OPTIONAL, -- Need M

ran-VisibleParameters-r17            SetupRelease {RAN-VisibleParameters-r17}                                   OPTIONAL, -- Cond ServiceType

...,

[[

reportingSRB-r18                     ENUMERATED {srb4, srb5}                                                    OPTIONAL, -- Cond QoENRDC

appLayerMeasPriority-r18             INTEGER (1..16)                                                            OPTIONAL, -- Need M

appLayerIdleInactiveConfig-r18       SetupRelease {AppLayerIdleInactiveConfig-r18}                              OPTIONAL  -- Need M

]]

}

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity-r17           ENUMERATED {ms120, ms240, ms480, ms640, ms1024}                            OPTIONAL, -- Need S

numberOfBufferLevelEntries-r17       INTEGER (1..8)                                                             OPTIONAL, -- Need R

reportPlayoutDelayForMediaStartup-r17 BOOLEAN                                                                   OPTIONAL, -- Need M

...,

[[

ran-VisibleReportingSRB-r18          ENUMERATED {srb4, srb5}                                                    OPTIONAL  -- Cond QoENRDC

]]

}

-- TAG-APPLAYERMEASCONFIG-STOP

-- ASN1STOP

Another example is that TCI activated Config also have a variable bit string configuration:

-- ASN1START

-- TAG-TCI-ACTIVATEDCONFIG-START

TCI-ActivatedConfig-r17 ::= SEQUENCE {

pdcch-TCI-r17               SEQUENCE (SIZE (1..5)) OF TCI-StateId,

pdsch-TCI-r17               BIT STRING (SIZE (1..maxNrofTCI-States))

}

-- TAG-TCI-ACTIVATEDCONFIG-STOP

-- ASN1STOP

Therefore, we think there’s no issue of using variable bit string length in ASN.1 for configurations.

Proposal 1: Option 3 (i.e. (N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING) is use for the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank.

# Conclusion

According to the above analysis, we have the following proposal:

Proposal 1: Option 3 (i.e. (N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING) is use for the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank: