3GPP TSG-RAN WG2 Meeting #129bis											R2-2501945

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.11.2

Source: 	Sharp

Title:  	Discussion on Random Access in SBFD

Document for: 	Discussion

# Introduction

In RAN2#129 meeting, RAN2 made the following agreements on SBFD random access:

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

This document discusses remaining open issues on SBFD random access.

# Discussion

  - Early Indication

In RAN2#128 meeting, whether an early indication of SBFD capability during the random access using legacy RO was discussed but no conclusion has been reached, as follows:

Early indication via msg3 R2-2410385 Random access for SBFD Operation Sony discussion Rel-19 NR_duplex_evo-Core Noted Proposal 2: Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure. R2-2410088 RA Aspects for SBFD Sharp discussion Rel-19 NR_duplex_evo-Core Noted Proposal 1 Msg3-based early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability. DISCUSSION - Sony do not think this is critical and think we should not spend too much time on this. - Apple think today we have many features that use msg3 based EI and if we do so here it requires more time to discuss.

In RAN2#129 meeting, there was no discussion on this issue. We expect a relevant discussion in this meeting.

We see that an early indication of SBFD capability during the random access is beneficial. One of the most important benefits of SBFD is latency reduction compared to legacy TDD. An early indication enables the benefit of latency reduction during the random access.

Regarding how to support the early indication, RAN2 concluded that there is no need to introduce SBFD as a feature combination. It means that Msg1-based early indication is not supported. Instead, we can consider Msg3-based early indication which still has a benefit to start transmission and reception early on SBFD symbols. It can be realized by using LCID field.

Msg3-based early indication with using LCID field in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

  - RO-Type Selection Threshold for Initial Transmission

For RO selection among additional RO in SBFD symbols and legacy RO, an issue is not to cause too strong cross link interference due to PRACH transmission in SBFD RO. If the UE selects an additional RO without consideration of the interference or coverage, it will lead unnecessarily many RA attempts with power ramp-up. We think that potential cross link interference aggressor UEs should not utilize additional ROs in SBFD symbols. Those are most likely cell-edge UEs whereas cell-center UEs usually have no problem on CLI. We understand this is a main reason why RAN2 agreed to have RSRP-based criteria for RO selection.

To achieve this, if a measured RSRP is above the threshold, we can allow UEs to use an SBFD RO. Otherwise, legacy RO should be used. With this approach, SBFD-aware UE will not select additional ROs if the UE is expected to use high transmission power for PRACH by checking the RSRP threshold for RO selection. We see that the opposite case does not have a clear motivation. Thus, the network does not need to further indicate whether to select the additional RO below or above the threshold.

If an SSB RSRP threshold for RO type selection is configured and SSB RSRP is above the threshold, the UE selects an SBFD RO.

If an SSB RSRP threshold for RO type selection is configured and SSB RSRP is below the threshold, the UE selects a legacy RO.

NW does not indicate whether to select the additional RO type below or above this SSB RSRP threshold.

  - RO-Type Change from Legacy RO to SBFD RO

In RAN2#127bis meeting, RAN2 agreed to support RO fallback from SBFD RO to legacy RO. The opposite direction of RO-type change was discussed in RAN2#129 meeting, but no consensus was reached, as follows:

RACH procedure, fallback behaviour R2-2500098 Impacts on the random access by the evolution of duplex operation Huawei, HiSilicon discussion Rel-19 NR_duplex_evo-Core Noted Proposal 6: Support that the UE is allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbol. Support to switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols. R2-2500274 Discussion on Random Access in SBFD symbols CATT discussion Rel-19 NR_duplex_evo-Core Noted Proposal 14: For CBRA, if legacy RO is selected for initial RACH transmission, it cannot fallback to SBFD RO after a number of times of RACH attempts. Discussion - ZTE do not see clear benefit of P6 from HW paper, e.g., ping-pong type of fallback. - Ericsson, Nokia, Charter, Sharp support HW proposal, and think we can have limitation on the maximum number of fallback times mentioned by ZTE. - Nokia think there is no ping-pong problem if we only allow one time fallback. Charter share this view. - OPPO think given what we already agreed it is better to avoid complicated fallback mechanism. - Sharp think either option has similar MAC spec impact. - Samsung think if we allow only fallback once then the specification complexity is similar for the two options. CMCC share this view. - Sony think we should limit the case for cell edge, e.g., only cell edge UEs can select RO and then do fallback. - Lenovo prefer CATT proposal, and is not sure what is the benefit to allow fallback from legacy RO to SBFD RO. Lenovo think already today we have a lot of tools to handle RACH performance, e.g., power control. HW think there is benefit in coverage limited case, since SBFD RO may give UE more UL resources. Lenovo still not sure about benefit in cell edge. - Nokia also see benefit in avoiding RA preamble collision. - Samsung and LG E not ready to making HW proposal as working assumption. LG E think the issue also depends on FFS on whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

We see that, regardless of the direction, the RO-type change procedure is beneficial when one RO is experiencing congestion that many UEs are attempting the preamble transmissions. For example, additional ROs in SBFD symbols can be less crowded as legacy UEs are not aware of them. In this case, RO-type change procedure from legacy RO to additional RO can be also useful from offloading perspective.

We do not see the fallback procedure requires a big complexity. The procedure will be almost symmetric for both directions. It is desirable that the network tries to maximize the system performance. Hence, we see that RO-type changes for both directions can be supported.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

  - Working Assumption on RA Trigger Events

In RAN2#127 meeting, RAN2 made a working assumption on RA trigger events in SBFD symbols, as follows:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

The issue is whether there is a big reason to exclude some events for random access in SBFD symbols. One could argue that some events may not be so essential for SBFD. However, none of these events will bring a big technical issue in any functionality related to SBFD. It means that relevant enhancements may be considered as an optimization but not considered as an essential feature.

RAN2 to confirm the working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- Msg3-based early indication with using LCID field in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability

- If an SSB RSRP threshold for RO type selection is configured and SSB RSRP is above the threshold, the UE selects an SBFD RO.

- If an SSB RSRP threshold for RO type selection is configured and SSB RSRP is below the threshold, the UE selects a legacy RO.

- NW does not indicate whether to select the additional RO type below or above this SSB RSRP threshold

- RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

- RAN2 to confirm the working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.