3GPP TSG-RAN WG2 Meeting #129bis																				R2-2501957

Wuhan, China, 7th - 11th April 2025

Agenda item:		8.5.3

Title: 	Remaining issues of on-demand SIB1

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some consideration on the remaining issues for on demand SIB1, with the latest higher parameter list [1] from RAN1 taken into consideration.

- Discussion

  - UL WUS configuration

- Issue 1: Confirm RAN1 agreed parameter list

The following SIB has been introduced for UL WUS configuration [2]:

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r19                       PhysCellId     OPTIONAL,        -- Need R

ul-WUS-Config-r19                        UL-WUS-Config-r19                               OPTIONAL,        -- Need R

...

}

UL-WUS-Config-r19  ::= {

sib1-rsrp-ThresholdSSB-r19                       RSRP-Range                                                              OPTIONAL,   -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                                   OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19               ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}  OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                     OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL, -- Need R

offsetToCarrier-r19                          INTEGER (0..2199)                                                          OPTIONAL, -- Need R

absoluteFrequencyPointA-r19                  ARFCN-ValueNR                                                              OPTIONAL, -- Need R

p-Max-r19                                    P-Max                                                                      OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                       INTEGER (-60..50)                                                          OPTIONAL, -- Need R

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19                                                     OPTIONAL  -- Need R

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

rach-OccasionsSIB1                    SEQUENCE {

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

sib1-RequestPeriod-r19                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

sib1-RequestResources-r19                 SEQUENCE (SIZE (1..maxSIB1-Message)) OF SIB1-RequestResources-r19

}

SIB1-RequestResources-r19 ::=             SEQUENCE {

sib1-ra-PreambleStartIndex-r19               INTEGER (0..63),

sib1-ra-AssociationPeriodIndex-r19           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

sib1-ra-ssb-OccasionMaskIndex-r19            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SIBxx-STOP

-- ASN1STOP

The following parameter is mentioned in RAN1 parameter list but not included in the running CR:

| Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213, clause 4.1. | SEQUENCE { inOneGroup BITSTRING (SIZE (8)), groupPresence BITSTRING (SIZE (8)) OPTIONAL-- Cond FR2-Only }, |
| --- | --- |

Observation 1: The ssb-PositionInBurst in RAN1 parameter list is missing in ASN.1.

Proposal 1: Include the following parameters in ul-WUS-Config-r19 in SIBxx:

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))								OPTIONAL  -- Cond FR2-Only

}

Apart from all the parameters listed above, the following issues need to be addressed, which will impact the UL WUS configuration.

- Issue 2: The start time of the on demand SIB1 acquisition window

At RAN2#129, we have agreed that Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In the latest RAN1#120 meeting, they agreed that:

-----------------------------RAN1#120 agreement-----------------------------

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the starting slot of the RAR window for UL WUS wherein UE successfully received a RAR.

Agreement

The time offset between the reference time point and the starting time for the time window of on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the time offset is in slot

-----------------------------RAN1#120 agreement-----------------------------

Proposal 2: The time offset between the reference time when UE successfully received a RAR and the start time of the on-demand SIB1 acquisition should be included in UL WUS configuration.

  - Target NES cell selection information

The following agreements have been made before on the triggering condition of WUS transmission and reselection priority for NES:

RAN2#126

Triggering condition of WUS transmission

If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

RAN2#127bis

NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

RAN2#128

New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

With the above agreement, the following options can be considered on how to provide the information for UE to select a target NES cell to send the SIB1 request.

- Option 1: Include all the cell reselection related parameters together with the WUS configuration for NES cells in the new SIB

- Option 2: Add the NES cells as neighbor cells along with the NES-specific reselection priority and IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES in SIB3/4 and reuse the existing cell reselection parameters in SIB2/3/4 in target NES cell selection

Option 2 would be better with less signaling overhead thus is preferred from our side.

Proposal 3: Add the NES cells as neighbor cells along with the NES-specific reselection priority and IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES in SIB2/3/4 and reuse the existing cell reselection parameters in SIB2/3/4 in target NES cell selection.

  - Validity of the WUS configuration

The following agreements have been made last meeting on the validity of the WUS configuration:

RAN2#128

SIB-x can be cell specific or area specific, as legacy.

As we agreed to introduced a new SIB to provide the WUS configuration, this SIB can be cell specific or area specific per the existing structure:

- Cell specific: The SIB is only valid in the current cell that provides the SIB

- Area specific: The SIB is valid in an area identified by systemInformationAreaID, which consists of one or several cells

The intention of supporting area specific SIB is that some system information maybe common in multiple cells, e.g. PWS information, which would also be possible for the WUS configuration.

Figure 1. Valid area of WUS configuration

As shown in the above figure, since we allow a cell to provide its own WUS configuration, the WUS configuration for all the four cells can be provided in cell A and any of the four cells and be valid in these cells.

The following options can be considered on supporting the area specific WUS configuration:

- Option 1: Reuse the idea of area specific SIB. The SIB containing WUS configuration is valid in an area identified by systemInformationAreaID, which consists of one or several cells

- Option 2: Define and provide valid area for WUS configuration.

For option 1, although we have the concept of system information area ID now, this id is common for all the SIBs while the valid area for the SIB containing WUS configuration would be quite different from other SIBs. Thus it is preferred to go for option 2.

Proposal 4: Define and provide valid area for WUS configuration.

The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells while the mapping between the area id and cells are maintained by the network.

Proposal 5: The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells.

  - SIB1 delivery in connected mode

We had some discussion on SIB1 delivery in connected mode with the following progress:

R2-2501465	Summary report of [AT129][105][NES] (Apple)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

Proposal 5: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1 as baseline.

- We will make conclusion on P5 next meeting.

Following the way forward agreed in RAN#106 [2], we understand on demand SIB1 procedure is supported in connected mode only when RLF happens and we do not intend to support other cases in connected mode.

- On RRC Connected mode

  - The scope of WID will not be updated to include RRC Connected.

  - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission.  No need to change the scope of WI to indicate this.

Observation 2: Following the RAN#106 way forward, on demand SIB1 procedure is supported in connected mode only when RLF happens and we do not intend to support other cases in connected mode.

For the case that searchSpaceSIB1 is configured and SIB1 is not broadcasting, the following description applies per the existing spec:

-----------------------------38.331 5.2.2.3.1----------------------------

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB or SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20].

-----------------------------38.331 5.2.2.3.1----------------------------

Per the existing spec, we only capture the handling for the case when searchSpaceSIB1 is configured and SIB1 is not broadcasting for idle/inactive and connected mode when T311 is running. While for UE in connected mode with searchSpaceSIB1 configured but SIB1 not broadcasting, we understand NW can still provide the SIB1 via dedicated signaling or broadcast SIB1 when needed.

Also the essential system information are defined for UE in idle/inactive and connected mode when T311 is missing, there is no sever impact if SIB1 acquisition is delayed for some time.

Since we have agree that There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1, we would still reply on the ssb-SubcarrierOffset, i.e. k_ssb, to indicate no SIB1, we understand the existing procedure applies for the case that searchSpaceSIB1 is configured and SIB1 is not broadcasting for UE in connected mode.

Proposal 6: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, it is up to NW implementation to deliver SIB1 via dedicated signalling or broadcasting, as legacy.

  - Essential System information

In the running CR discussion, one issue has been raised by the rapporteur on whether SIBxx containing on Demand SIB1 assistance information should be treated as essential SIB for R19 NES UE or not.

We understand those UEs supporting on demand SIB1 procedure and has selected a cell to trigger this procedure shall ensure having a valid version of SIBxx, which means:

- If the candidate target cell for cell reselection/selection is a normal cell with SIB1 broadcasting. UE can live without SIBxx;

- While if the candidate target cell for cell reselection/selection is a NES cell with no SIB1 broadcasting, UE has to trigger SIB1 request procedure with the assistance info from SIBxx.

Proposal 7: SIBxx containing the on demand SIB1 assistance information is essential SIB for UE capable of on demand SIB1 procedure and has selected a cell to trigger on demand SIB1 request procedure.

  - UE capability for on demand SIB1

We understand on demand SIB1 should be an optional feature without UE radio access capability parameters as it mainly impact the idle/inactive UE and connected mode UE when RLF happens, NW can broadcast SIBxx without knowing whether UE support it or not.

Proposal 8: On demand SIB1 should be an optional feature without UE radio access capability parameters

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Observation 1: The ssb-PositionInBurst in RAN1 parameter list is missing in ASN.1.

Proposal 1: Include the following parameters in ul-WUS-Config-r19 in SIBxx:

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))								OPTIONAL  -- Cond FR2-Only

}

Proposal 2: The time offset between the reference time when UE successfully received a RAR and the start time of the on-demand SIB1 acquisition should be included in UL WUS configuration.

Proposal 3: Add the NES cells as neighbor cells along with the NES-specific reselection priority and IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES in SIB2/3/4 and reuse the existing cell reselection parameters in SIB2/3/4 in target NES cell selection.

Proposal 4: Define and provide valid area for WUS configuration.

Proposal 5: The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells.

Observation 2: Following the RAN#106 way forward, on demand SIB1 procedure is supported in connected mode only when RLF happens and we do not intend to support other cases in connected mode.

Proposal 6: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, it is up to NW implementation to deliver SIB1 via dedicated signaling or broadcasting, as legacy.

Proposal 7: SIBxx containing the on demand SIB1 assistance information is essential SIB for UE capable of on demand SIB1 procedure and has selected a cell to trigger on demand SIB1 request procedure.

Proposal 8: On demand SIB1 should be an optional feature without UE radio access capability parameters

- Reference

- R1-2501645 Consolidated Rel-19 higher layers parameters list Post RAN1#120

- RP‑243297	Way forward on NES