3GPP TSG-RAN WG2 Meeting #129-bis	R2-2501966

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon, Turkcell

Title:	Further consideration on UL capacity enhancement

Document for: Discussion and Decision

# 1	Introduction

According to the RAN plenary #105 meeting, the following objectives on IoT-NTN [1] were agreed:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In the last meeting, RAN2 has made the following agreements:

Agreements:

1.	It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

2.	RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

3.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

4.	For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels.

5.	The UE shall at most have one ongoing CB EDT procedure at any time.

6.	The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT.

7.	The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels.

8.	CB EDT Config has three RSRP thresholds for eMTC for the four CE levels.

7.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

8.	RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature

Working assumption:

- One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

The UE first selects the next DSA transmission window and then randomly select K replicasinside the window.

RAN2 assumes that a pointer solution is not needed in Rel-19

In this contribution, we will further discuss the remaining details of SA and DSA mechanisms.

# 2	Discussion

## 2.1 SA

### 2.1.1 CB-msg3 EDT Configuration

During the previous meeting, it was agreed only system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. And for 3.75kHz SCS NB-IoT and eMTC CE mode A, TA validation is not needed.

However, for 15kHz SCS NB-IoT and eMTC CE mode B, UE still needs to be configured with at least TA validation parameters by RRC dedicated signalling, e.g., RRCRelease. Meanwhile, these UEs can share the resource configured in the system information.

Proposal 1: For 15kHz SCS NB-IoT UEs and eMTC CE mode B UEs, TA validation parameters need to be acquired via RRC dedicated signalling.

### 2.1.2 Contention resolution of CB-msg3 EDT

Since the contention-based MSG3 resource is shared by multiple UEs, contention can occur when different UEs choose the same resource, similar to the potential contention during the RACH procedure. As a result, contention resolution is required via Msg4 for the contention-based MSG3 transmission. To achieve this, a mechanism similar to the one used contention resolution during RACH can be applied. Specifically, the network retrieves the UE ID in Msg3 and identifies the winner of the contention. To inform the UE of the contention resolution result, the NW can send a MAC CE containing the UE ID via Msg4, allowing the UE to verify the result.

Proposal 2: A MAC CE containing UE ID information is used for contention resolution for CB-msg3 EDT. FFS on the detailed design of the MAC CE.

Msg3 transmission may fail again if different UEs select the same resource again. In the RACH procedure, a Backoff Indicator is introduced to address this issue. The UE selects a random backoff time according to a Backoff Indicator included in Random Access Response, delaying the subsequent Random Access transmission by the backoff time. A similar mechanism can be applied to handle Msg3 transmission failures in contention-based Msg3 EDT transmission. Specifically the backoff parameters can be configured for UE via system information, allowing the UE to randomly selects a backoff time based on these parameters.

Proposal 3: The backoff parameters are provided to UE via system information and UE randomly selects a backoff time based on the backoff parameters before another attempt.

### 2.1.3 Fallback

Another issue is whether the UE can fall back to 4-step random access EDT after a certain number of CB-msg3 EDT attempts. Generally, having this backup mechanism is beneficial in case the CB-msg3 EDT attempts are successful.

Proposal 4: After a specified number of failed CB-msg3 EDT attempts, the UE falls back to 4-step random access EDT.

### 2.1.4 RNTI for Msg4 reception

To complete the contention resolution, the UE needs the RNTI to monitor the PDCCH for MSG4 scheduling. In the last meeting, RAN2 has agreed that the RNTI used for Msg4 transmission will be derived based on the resource associated to the PUSCH occasion for CB-Msg3 EDT transmission. The detail is FFS.

In legacy RACH procedure, UE determines RA-RNTI according to selected PRACH resources.

36.321 5.1.4 Random Access Response reception The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI= 1 + t_id + 10*f_id where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), and f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6) except for NB-IoT UEs, BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10)) where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0. For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

RNTI used for Msg4 in CB-MSG3 EDT can use the similar mechanism as RA-RNTI.

Proposal 5: RNTI used for Msg4 in CB-MSG3 EDT can follow RA-RNTI calculation formula as baseline. FFS on the exact parameters.

During legacy RACH procedure, Msg2 contains Temporary C-RNTI. When UE receives Msg4 and considers this Contention Resolution successful, UE sets the C-RNTI to the value of the Temporary C-RNTI. However, Msg2 is omitted for CB-msg3 EDT. Which RNTI is used by the UE for further scheduling after MSG4 needs to be decided by RAN2. A straightforward approach would be for the network to is that NW allocates a C-RNTI in Msg4 to the UE so that UE can use it subsequently.

Proposal 6: NW should allocate a C-RNTI in Msg4 for the UE to receive the scheduling after MSG4.

## 2.2 DSA

### 2.2.1 Confirmation of the working assumption

In the last meeting, RAN2 has made the following working assumption:

Working assumption:

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

The UE first selects the next DSA transmission window and then randomly select K replicas inside the window.

RAN2 assumes that a pointer solution is not needed in Rel-19

For the sake of progress, we think RAN2 can confirm above working assumption in this meeting.

Proposal 7: RAN2 to confirm the following working assumption:

Working assumption: - For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today - The UE first selects the next DSA transmission window and then randomly select K replicas inside the window. - RAN2 assumes that a pointer solution is not needed in Rel-19

2.2.2 MSG4 monitoring for DSA

In the last meeting and the post-meeting email discussion, there is no consensus on whether the UE needs to start one or multiple monitoring windows for MSG4 reception. In our opinion, RAN2 should adopt the multiple window solution based on the following reasons:

1. Independent multiple windows are much simpler following the same logic as what is defined for SA. While for single window, extra effort is needed to define the start point and the duration of the window for DSA.

2. With multiple windows, UE is able to stop monitoring RATI for power saving if there are gaps between two windows. While for single window, UE has to monitoring the RNTI for the whole time until the ending of the window.

3. With single window, UE/NW has to wait until all the replicas have been sent/received to receive/send the feedback, which will bring unnecessary latency in case the first replica is already successful.

Besides, for NB-IoT, we don’t think the half-duplex is a big issue since the selection of replica occasions are up to UE and anyway UE can prioritize the reception of feedback by implementation.

Proposal 8: In DSA, multiple PDCCH monitoring windows are started for multiple replicas.

Besides, UE may need to monitor the feedback for different replicas at the same time if there are overlapping between different windows. In this case, if the RNTI is different for different replicas, The IoT UE will have to monitor multiple RNTIs at the same time. The maximum number of replicas configured by the NW is 4. So the UE needs to monitor a maximum of 4 RNTIs at the same time which will add complexity to especially low capability NB-IoT UEs. A better way is to define a common RNTI for all the replicas in DSA. Besides, if one common RNTI is used for monitoring MSG4s, NW can multiplex multiple MSG4s into one MAC PDU to reduce overhead.

Proposal 9: In DSA, one common RNTI is used for multiple MSG4 monitoring which corresponds to multiple MSG3 copies.

Since the MSG3 transmission window is fixed, the simplest way to derive the RNTI is based on a designated resource within the MSG3 transmission window, which is also aligned with what is adopted for SA.

Proposal 10: In DSA, the RNTI for MSG4 monitoring is derived based on a specified resource within the MSG3 transmission window.

# 3	Conclusion

This contribution analyses the UL capacity enhancement, and proposes the following:

SA

Proposal 1: For 15kHz SCS NB-IoT UEs and eMTC CE mode B UEs, TA validation parameters need to be acquired via RRC dedicated signalling.

Proposal 2: A MAC CE containing UE ID information is used for contention resolution for CB-msg3 EDT. FFS on the detailed design of the MAC CE.

Proposal 3: The backoff parameters are provided to UE via system information and UE randomly selects a backoff time based on the backoff parameters before another attempt.

Proposal 4: After a specified number of failed CB-msg3 EDT attempts, the UE falls back to 4-step random access EDT.

Proposal 5: RNTI used for Msg4 in CB-MSG3 EDT can follow RA-RNTI calculation formula as baseline. FFS on the exact parameters.

Proposal 6: NW should allocate a C-RNTI in Msg4 for the UE to receive the scheduling after MSG4.

DSA

Proposal 7: RAN2 to confirm the following working assumption:

Working assumption: - For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today - The UE first selects the next DSA transmission window and then randomly select K replicas inside the window. - RAN2 assumes that a pointer solution is not needed in Rel-19

Proposal 8: In DSA, multiple PDCCH monitoring windows are started for multiple replicas.

Proposal 9: In DSA, one common RNTI is used for multiple MSG4 monitoring which corresponds to multiple MSG3 copies.

Proposal 10: In DSA, the RNTI for MSG4 monitoring is derived based on a specified resource within the MSG3 transmission window.

# 4	References

- RP-242397, WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3