3GPP TSG-RAN WG2 Meeting #129bis	R2-2501967

Wuhan, China, April 07th – 11th, 2025

Agenda item:	8.4.3

Source:	Huawei, HiSilicon

Title:	Further discussion on the criteria for RRM measurement relaxation and offloading

Document for:	Discussion and Decision

- Introduction

The following agreements are reached on RRM measurement relaxation and offloading in previous RAN2 meetings.

RAN2#127 - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation RAN2#129 - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

The following agreements are reached in RAN4 for RRM measurement relaxation and offloading.

The following cases were discussed in RAN4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off; ON. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On with relaxation measurement; ON. #5 Exit case; On; On; Off. RAN4#111 Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4#112 Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. RAN4#114 Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

In this contribution, we further discuss the criteria for RRM measurement relaxation and offloading for IDLE/INACTIVE mode.

- Discussion

  - Criteria for RRM measurement relaxation

    - Entry/exit condition for RRM measurement relaxation

RAN4 has only agreed on case #1 and case #3 so far when serving cell measurement by LP-WUR is being performed:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

For now, RAN2 should focus on discussing the criteria for RRM measurement cases that have been agreed by RAN4, i.e., case#1 and case#3. In case#3, the intended behaviors are to allow UE to perform: LR measurement, relaxed MR serving cell measurement and relaxed MR neighboring cell measurement at the same time.

We understand the “RRM measurement relaxation” case discussed in RAN2 refers to case#3 agreed by RAN4. That means MR serving cell measurement and MR neighboring cell measurement should be relaxed at the same time, once the entry condition for “RRM measurement relaxation” is satisfied. In other words, entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation, while LR measurement is on.

Observation 1: The “RRM measurement relaxation” case discussed in RAN2 refers to case#3 agreed by RAN4 where UE performs LR measurement, relaxed MR serving cell measurement and relaxed MR neighboring cell measurement at the same time.

If the criteria for serving cell and neighbour cell RRM measurement relaxation are separate, it will create more cases such as: only MR serving cell measurement is relaxed but neighbour cell measurement is still performed normally, or only MR neighbour cell measurement is relaxed but serving cell measurement is still performed normally. On one hand, these cases are not agreed by RAN4. On the other hand, there is almost no power saving gain since UE still needs to turn on the MR frequently to perform the normal RRM measurement on either serving cell or neighbour cell, as stated in TR 38.869 [1].

Observation 2: If the criteria for serving cell and neighbour cell RRM measurement relaxation are separate, there is almost no power saving gain since UE still needs to turn on the MR frequently to perform the normal RRM measurement on either serving cell or neighbour cell.

Proposal 1: The entry/exit condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

Regarding the exit condition, a separate exit condition based on LR measurement is preferred. On the one hand, since the MR RRM measurement is relaxed and RAN4 agreed that the scaling factor for MR RRM relaxation is 16, MR measurement results cannot be obtained in a timely manner, which delays the exit of LP-WUS monitoring and may result in paging loss. On the other hand, with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism.

Observation 3: Defining a separate exit condition based on LR measurement has the following advantages:

- the LR measurement results is more timely compared with MR measurement results, since RAN4 agreed that the scaling factor for MR RRM relaxation is 16;

- with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism.

Proposal 2: The exit condition for RRM measurement relaxation is based on the LR measurement, i.e., when the LR measurement is below a threshold.

Based on the above analyses, the entry/exit conditions for RRM measurement relaxation (both serving cell and neighbour cell RRM measurement relaxation), i.e. case#3 agreed by RAN4, are summarized as below:

- entry condition: if the serving cell quality based on MR (and optionally serving cell quality based on LR, if configured), is(are) above threshold(s);

- exit condition: if the serving cell measurement result based on LR is below a threshold.

    - Relation between RRM measurement relaxation and LP-WUS monitoring

RRM measurement relaxation/fully offloading is tightly related to LP-WUS monitoring to ensure the power saving gain. We first discuss the UE behaviors for different cases of RRM measurement relaxation and LP-WUS monitoring as below.

Table 1. UE behaviors for different cases of RRM measurement relaxation and LP-WUS monitoring

| Cases |  |  | UE behaviors |
| --- | --- | --- | --- |
| No. | Criteria for RRM measurement relaxation | Criteria for LP-WUS monitoring |  |
| ① | Fulfilled | Fulfilled | UE can monitor LP-WUS, perform LR measurement and relaxed MR serving/neighbour cell measurement |
| ② | Not fulfilled | Not fulfilled | Legacy behaviors for paging reception and RRM measurements |
| ③ | Fulfilled | Not fulfilled | UE only performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS If UE can perform LR measurement, why cannot UE monitor LP-WUS? |
| ④ | Not fulfilled | Fulfilled | UE only monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (note that fully offloading criteria is not fulfilled either) There is no power saving gain for this case |

For case ④, it is not reasonable that UE enters LP-WUS coverage but still performs normal RRM measurement in which case there is no power saving gain, so case④ should not be supported. Then for case③, if the LR measurement can be used, then LP-WUS can also be used for further reducing power consumption for paging reception, otherwise, the thresholds for RRM measurement relaxation criteria and LP-WUS monitoring criteria are not properly set. Hence, it makes sense to only support case① and case②.

Proposal 3: RAN2 to confirm that the following two cases are not supported:

- UE performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS;

- UE monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (i.e., no RRM measurement relaxation or fully offloading).

Thus, UE monitors LP-WUS and performs LR measurement and relaxed MR serving/neighbour cell measurement at the same time, two options (or methods) can be considered:

Option 1) Reuse entry/exit condition of LP-WUS monitoring as entry/exit condition for RRM measurement relaxation. If the entry condition of LP-WUS monitoring is fulfilled, the UE starts LP-WUS monitoring, and performs RRM measurement relaxation (case#3). If the exit condition of LP-WUS monitoring is fulfilled, the UE starts legacy procedure by MR and performs legacy RRM measurement by MR. This option simplifies NW configurations and UE behaviors, and provides good power saving gain. In the meantime, it aligns with RAN4 agreed case#3, i.e., UE performs relaxed RRM measurement relaxation with LR measurement on.

Option 2) Define a separate entry/exit condition for RRM measurement relaxation, both LP-WUS monitoring entry/exit condition and RRM measurement relaxation entry/exit condition should be considered. If a separate entry/exit condition for RRM measurement relaxation is introduced, some restrictions need to be specified considering LP-WUS monitoring entry/exit condition to preclude some improper cases. Only if both LP-WUS monitoring entry condition and RRM measurement relaxation entry condition is fulfilled can the UE perform RRM measurement relaxation.

Proposal 4: RAN2 to discuss the following two options for defining RRM measurement relaxation criteria:

- Option 1: reuse entry/exit condition of LP-WUS monitoring as entry/exit condition for RRM measurement relaxation;

- Option 2: define a separate entry/exit condition, and UE considers both LP-WUS monitoring entry/exit condition and the separate condition for the entry/exit of RRM measurement relaxation.

  - Criteria for RRM measurement fully offloading

For RRM measurement fully offloading (RAN4 case#1), since neighbor cell of equal or lower priority (including intra-frequency, NR inter-frequency and inter-RAT frequency) is off, it means UE is not likely to perform cell re-selection. For example, UE is near the center of the cell. On the other hand, for RRM measurement relaxation (RAN4 case#3), UE may be near the cell edge so that neighboring cell measurement is on. So obviously the RSRP threshold for case#1 should be higher than for case#3.

Observation 4: RAN4 Case#1 is the RRM measurement behaviors for UEs in center of the cell and RAN4 Case#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage.

Proposal 5: The threshold(s) for RRM measurement fully offloading (RAN4 case#1) should be higher than threshold(s) for RRM measurement relaxation (RAN4 case#3).

Based on above, how LP-WUR and MR perform RRM measurement is illustrated in the figure below.

Fig. 1. UE behaviors for RRM measurement fully offloading and relaxation cases

In RAN2#127 meeting, RAN2 discussed the entry/exit condition for RRM measurement fully offloading (case#1) without taking case#3 (which was agreed in parallel by RAN4) and scaling factor for MR RRM relaxation (which was agreed by RAN4 in last meeting) into account. Currently, with both case#1 and case#3 supported, RAN2 needs to further think about the entry/exit condition considering: switch between case#1 and case#3.

If UE is in the RRM measurement relaxation case, RAN4 agreed that the scaling factor for MR RRM relaxation is 16. In this case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading case. Thus, the serving cell quality measured by LR is more appropriate to be used for the entry condition of fully offloading.

Observation 5: In the RRM measurement relaxation case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading.

Proposal 6: If UE is currently in RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

- Conclusion

In this paper, we discuss the criteria for RRM measurement relaxation and offloading, and have the following proposals.

Criteria for RRM measurement relaxation

Proposal 1: The entry/exit condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

Proposal 2: The exit condition for RRM measurement relaxation is based on the LR measurement, i.e., when the LR measurement is below a threshold.

Proposal 3: RAN2 to confirm that the following two cases are not supported:

- UE performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS;

- UE monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (i.e., no RRM measurement relaxation or fully offloading).

Proposal 4: RAN2 to discuss the following two options for defining RRM measurement relaxation criteria:

- Option 1: reuse entry/exit condition of LP-WUS monitoring as entry/exit condition for RRM measurement relaxation;

- Option 2: define a separate entry/exit condition, and UE considers both LP-WUS monitoring entry/exit condition and the separate condition for the entry/exit of RRM measurement relaxation.

Criteria for RRM measurement fully offloading

Proposal 5: The threshold(s) for RRM measurement fully offloading (RAN4 case#1) should be higher than threshold(s) for RRM measurement relaxation (RAN4 case#3).

Proposal 6: If UE is currently in RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

- References

- TR 38.869, Study on low-power wake up signal and receiver for NR.