3GPP TSG-RAN WG2 Meeting #129bis                                                              R2-2501972

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.19

Source:	vivo

Title:	Discussion on CSSF optimization for NR RRM Phase 5

Document for:	Discussion, Decision

- Introduction

This document aims to discuss the CSSF (carrier specific scaling factor) [1] optimization for NR RRM Phase 5, as presented in the RAN4 LS [2]. The optimization of CSSF outside gap is intended to reduce FR2-1 L3 measurement delay in CA/DC. This document will analyze the potential impact of the CSSF optimization on RAN2 work.

- Discussion

The CSSF enhancement, as proposed by RAN4, allows the UE to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band. If network indicates to enable this CSSF enhancement, UE is allowed to determine the CC in the band to be measured based on the following principle.

- Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band, - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

The principle of the LS shows that “Measure PCC if PCC in band” and “Measure PSCC if PSCC in band” are two parallel branches. If both PCC and PSCC are in one band, it needs to be clarified which carrier is measured by the UE. We understand that PCC should be measured when both PCC and PSCC are in one band.

Proposal 1:  The PCC should be measured when both PCC and PSCC are in one band. RAN2 sends an LS to RAN4  to confirm RAN2's understanding.

With regard to the following network indication, RAN2 need to enhance RRC configurations to indicate that UEs enable/disable the CSSF enhancement due to the infrequent enable/disable. For example, if the UE is in RRC_CONNECTED and in NR standalone (SA) mode, the network can configure the UE to enable or disable the CSSF enhancement through the measConfig field in the RRCReconfiguration message. The “cssf-Enhancement” parameter is used to indicate whether the CSSF enhancement is enabled or disabled. If the cssf-Enhancement is set to “enabled,” the UE is allowed to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band. If the cssf-Enhancement is set to “disabled,” the UE shall measure multiple configured serving CCs. It should be noted that the final applicability of CSSF enhancements is FFS. The NR standalone mode in the above example is is only an example. Potentially, CSSF enhancements  may also apply to EN-DC, NE-DC and NR-DC modes.

- Network indication of enable/disable this CSSF enhancement is per-UE.

Proposal 2 : In order to support the enhancement of CSSF outside gap, RAN2 needs to support the indication of enable/disable this CSSF enhancement per-UE and per CC level via RRC configuration for intra band and inter band case.

If the CSSF enhancement is enabled, the measurement reporting principle also needs to be clearly defined. For servicing CCs in the CSSF enhancements that allow UEs not to performance measurements,  there are two options for measurement reporting in cases where  the cell is configured with measurement reporting without measurement.

Option1: Consider no available measurement for the CC without measurement.

Option2: Copy the measurement result of the measurement CC into the reporting for the CC without measurement.

When the network enables this function, it implies that the conditions of the CCs are similar, making Option1 reasonable.

The CCs without measurements are considered as no available measurement even the measurement reporting is configured for this kind of CCs. Therefore, there is no measurement reporting.

Proposal 3: There is no measurement reporting for CCs that are allowed to be unmeasured in the CSSF enhacement.

For UE Capabilities, RAN2 need to consider the UE transfering its UE capability information on whether to support the CSSF enhancement. The final applicability of the CSSF enhancement is FFS as follows. Based on the final applicability and UE capabilities, the network can aquire whether UE support the CSSF enhancement under the applicable mode. For example, if the CSSF enhancement applies to NR standalone mode, the UE indicates its support for the CSSF enhancement in NR mode through the   nonCriticalExtension   field within the  UE-NR-Capability. This allows the network to determine whether to configure the UE to enable CSSF enhancements based on the UE's capabilities.

- Final applicability of the solutions can be FFS - EN-DC - EN-DC with FR2 only intra band CA - EN-DC with FR2 only inter band CA - EN-DC with FR1 +FR2 CA (FR1 PSCell) - EN-DC with FR1 +FR2 CA (FR2 PSCell) - NE-DC - NE-DC with FR2 only intra band CA - NE-DC with FR2 only inter band CA - NE-DC with FR1 +FR2 CA (FR1 PCell) - SA - FR1+FR2 CA (FR1 PCell) - FR1+FR2 CA (FR2 PCell) - FR2 only intra-band CA - FR2 only inter-band CA - NR-DC - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

Proposal 4:	In order to support the enhancement of CSSF outside gap, RAN2 needs to define UE capability IEs to indicate whether the UE supports the CSSF enhancement under the applicable mode, i.e., per SA, EN-DC NE-DC level.

3 Conclusion

Based on the discussion on the CSSF optimization for NR RRM Phase 5, we make the following proposals:

Proposal 1:  The PCC should be measured when both PCC and PSCC are in one band. RAN2 sends an LS to RAN4  to confirm RAN2's understanding.

Proposal 2 : In order to support the enhancement of CSSF outside gap, RAN2 needs to support the indication of enable/disable this CSSF enhancement per-UE and per CC level via RRC configuration for intra band and inter band case.

Proposal 3: There is no measurement reporting for CCs that are allowed to be unmeasured in the CSSF enhacement.

Proposal 4:	In order to support the enhancement of CSSF outside gap, RAN2 needs to define UE capability IEs to indicate whether the UE supports the CSSF enhancement under the applicable mode, i.e., per SA, EN-DC NE-DC level.

4 Reference

[1] TS 38.133 NR, Requirements for support of radio resource management.

[2] R4-2502662, LS on CSSF optimization for NR RRM Phase 5, RAN4.