3GPP TSG-RAN WG2 Meeting #129bis	R2-2501973

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item	: 8.1.4

Source	: LG Electronics Inc.

Title	: Discussion on UE-side data collection

Document for	: Discussion and Decision

Introduction

At the RAN#106 meeting, the new SID for AI/ML air Phase 2(RP-243245) [1] was agreed and the further discussion for UE-side data collection was approved in RP-243307[2] with the following guidance for RAN2:

- Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed

Based on the guidance from RAN#106 meeting, RAN2 had reached the following agreement at the RAN2#129 meeting [3].

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In this contribution, we will continue discuss the remaining FFS topics assigned to RAN2.

Discussion

Study RAN aspects related to data transfer over UP

In accordance with the RAN2#127 meeting agreement [4], data transfer over the UP tunnel is considered for Option 1a and Option 1b, while it remains as FFS for Option 2 and Option 3. The table below provides a baseline example of the data transmission process through the UP tunnel for each option, depending on whether the server for UE data collection is located inside or outside the MNO. As shown in the table, when the server is inside the MNO, the data transfer paths and the scope of the MNO for Option 1b and Option 2 are the same. Similarly, when the server is outside the MNO, the data transfer paths and the scope of the MNO for Option 1a, Option 1b, and Option 2 are identical.

| Location of the server Option | Server inside the MNO | Server outside the MNO |
| --- | --- | --- |
| Option 1a | N/A |  |
| Option 1b |  |  |
| Option 2 |  |  |
| Option 3 |  |  |

From the RAN2 perspective, since the UP tunnel is from the UE to the Core Network through the gNB and UPF, the UP solution for Option 1a, Option 1b, and Option 2 seems identical regardless of the server's location. Option 3, on the other hand, may be considered different as the UP tunnel from the UE to the Core Network goes through the gNB and OAM, making it distinct from the other options.

Therefore, the data transfer through the UP tunnel can be classified into two categories from the RAN2 perspective based on the analysis above.

- data transfer through UP tunnel via UPF (i.e., Option 1a, Option 1b, Option 2)

- data transfer through UP tunnel via OAM (i.e., Option 3)

Proposal 1. From the RAN2 perspective, UP solution can be classified into two : i) data transfer through UP tunnel via UPF(i.e., Option1a, Option1b, Option2), ii) data transfer through UP tunnel via OAM(i.e., Option3).

Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

Based on the RAN2#129 agreement [3], it is now clear that NG-RAN is responsible for the control and configuration of UE-side data collection for all RAN-centric use cases (e.g., beam management, CSI prediction). For both CP and UP solutions, configuring radio resources for UE-side data collection is essential. There is no specific reason to configure RS differently for each method, nor is there any need for unnecessary variations in the settings. Therefore, the RS configuration for methods aimed at UE-side data collection should be consistent across both CP and UP solutions. Since the configurations for both UE-side and NW-side data collection are related to CSI reporting configuration, the RS configuration framework used for NW-side data collection (including associated IDs if needed) can be applied. This approach will help avoid redundant discussions.

Proposal 2. RS configuration framework for NW-side data collection can be reused for RS configuration framework for UE-side data collection.

The RS configuration for UE-sided data collection is related to the CSI reporting configuration. The request for RS configuration is currently under discussion in the applicability report, and the framework for these requests is being addressed through Option A and Option B. Therefore, instead of introducing a new RS-related configuration framework for UE request, the existing RS configuration framework for applicability report can be utilized for the UE request. For instance, Option A can be used for the transmission of the preferred CSI configuration, while Option B can be used to request the preferred RS configuration.

Proposal 3. For the UE request configuration, the RS configuration for the applicability report can be applied.

If Proposal 3 is agreeable, RAN2 may consider several methods for the subsequent process of sending the UE request and assess the feasibility of each method. The following methods are examples that can be considered:

- Method 1: Specify a separate Information Element (IE) (e.g., DataCollectionPreference-r19)

- Method 2: Include the UE request within the applicability report for transmission.

The detailed configuration for each method can be established as outlined in the Annex of R2-2502020s.

Proposal 4: If proposal 3 is agreeable, RAN2 may consider the subsequent transmission procedure of UE request. (e.g., whether to transmit the UE request through a separate IE or include the UE request in the applicability report).

For the UE request, the following aspects can be considered as part of its content:

1) Necessity of UE-side data collection

- If the model is sufficient for inference and thus does not require additional data collection, the UE request for UE-side data collection would not be necessary.

- Conversely, if the model is insufficient and requires further training for the inference, the UE would request data collection to improve the model before attempting inference again.

2) UE preference for starting or stopping UE-side data collection

- UE preference based on the conditions: e.g., remaining battery/memory status, ongoing procedure that cannot be interrupted

3) UE preferred RS configuration/transmission

In terms of controllability, MNO ensures full control for the CP solution, as the transmission of SRBs is entirely under its management. This level of control enables MNO to effectively manage communication, optimize resource allocation, and maintain high-quality service for users. By overseeing SRB transmission, dynamic adjustment of parameters, management of signaling loads, and real-time respond to changing network conditions can be held within the MNO, ultimately enhancing overall network performance and reliability. Similarly, for the UP solution, NG-RAN also maintains full control as the SMF in the core network governs the UPF and OAM. Regardless of whether the data collection server is located within or outside the MNO, the MNO can achieve complete controllability by separating AIML data from non-AIML data, thereby creating different PDU sessions with distinct LCH, DRB, and QoS requirements.

Proposal 5. MNO can have full controllability over the data collection process through both CP solution and UP solution.

Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

In addition to controllability, NG-RAN guarantees full scalability for the transfer of standardized data through the CP solution, as the data transmission via SRBs is managed by NG-RAN. Similarly, full scalability for the transfer of standardized data through the UP solution within the core network is ensured, since each PDU session is under the control of the SMF. Furthermore, full visibility for standardized data collection is already attainable in legacy operations, as specified in the standard documents produced by the relevant working groups. For example, the TS 26.531 document states that when the data collection server is located within MNO, full visibility for standardized data can be guaranteed for UE-side model training. In this context, it is also expected that full visibility for standardized AIML data will be achievable.

Proposal 6. For both CP solution and UP solution, MNO can have full visibility over standardized data.

During the discussions regarding various data collection options for UE-side model training at the RAN2#127 meeting(R2-2407785), it was noted that "RAN2 has not reached a conclusion on the necessity of partial and no visibility options." For these options, it is essential to first address the content of partially standardized data and non-standardized data. Some companies advocate for the inclusion of partial or non-standardized data to protect proprietary information. However, the current data transfer requirements, such as L1-RSRP values, are sufficiently met with standardized data. Therefore, to facilitate discussions on partial/non-standardized data, it is crucial to clarify which data qualifies as partial/non-standardized and what specific information must be included for data transfer purposes. If, after this clarification, the introduction of partial/non-standardized data is deemed necessary, further discussions can be held at that time.

Proposal 7. For the discussion of transferring partial/non-standardized data of UE side data collection, RAN2 should first clarify which specific data needed as partial/non-standardized data.

Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

Adopting the UP solution for data collection in Options 2 and Option 3, the configuration of data collection should be managed by the network, specifically by either the CN or OAM, depending on which entity is responsible for the data collection. As such, this discussion falls within the purview of the SA working group, necessitating a feasibility review by that group. Initially, the request for data collection and transfer will be initiated by the server for the purpose of UE-side model training. These requests will be directed to either the CN or OAM, which will then handle the configuration and scheduling of the data collection and transfer. Subsequently, the CN/OAM will establish the UP tunnel between the UE and the CN/OAM and will also configure and schedule the UE for the data collection and transfer process. In this sense, RAN2 will need to wait for the discussions in SA2 regarding the feasibility of the UP tunnel between the CN and the server, as well as discussions in SA5 concerning the feasibility of the UP tunnel between the OAM and the server.

Proposal 8. RAN2 should wait for the SA2/SA5 discussion about the feasibility check of UP tunnel between CN/OAM and the server.

Discuss on the scalability aspects of CP

From the RAN2 perspective, the data transfer through the CP tunnel can be classified into two categories:

- CP solution via NAS from UE to AMF (i.e., Option 2)

- CP solution via AS from UE to gNB to OAM (i.e., Option 3)

  - The current MDT framework can be taken as a baseline for the Option3. Therefore, it shares the advantages of MDT, such as cost efficiency, extensive coverage, substantial data collection capability, real-time data acquisition, and user-centric analysis for precise network optimization.

Both the AS protocol and the NAS protocol are not well-suited for transferring large amounts of data, which is common when transferring data collected for AIML purposes. This limitation arises because both AS and NAS are ultimately transmitted through RRC messages, which have a size constraint of up to 9000 bytes. As a result, transferring large volumes of data necessitates multiple RRC transmissions, leading to increased signaling overhead. Furthermore, it is essential to specify the data type for each use case during transmissions via both AS and NAS. This requirement introduces scalability constraints. Whenever new use cases emerge from the development of AI/ML technology, the UE must be updated to support the new use case-specific data types. Additionally, the network must be fully deployed or updated to accommodate these new use case-specific data types.

The UP solution can effectively replace the CP solution by addressing the shortcomings mentioned above. This can be accomplished by configuring legacy PDU sessions without any enhancements. The process of transferring large volumes of data collected for AIML purposes can be managed like any other UP traffic. The network, based on the MNO configuration, can set up a PDU session and manage this traffic with a specific QoS flow and related QoS parameters. At this point, it is crucial to ensure that transmitting AIML data with relaxed latency requirement does not interfere with the transmission of other UP traffic data. Therefore, considerations such as dedicated bearers may be explored, provided they do not impose significant impacts on the specifications.

Proposal 9. The UP solution addresses the signaling overhead and scalability issues associated with the CP solution, allowing for the transmission of large volumes of AIML data using legacy methods without any spec impact.

Conclusion

Based on the above discussion, we made the following proposals:

[Study RAN aspects related to data transfer over UP]

Proposal 1. From the RAN2 perspective, UP solution can be classified into two : i) data transfer through UP tunnel via UPF(i.e., Option1a, Option1b, Option2), ii) data transfer through UP tunnel via OAM(i.e., Option3).

[Discuss level of NG-RAN involvement in the control and configuration of UE side data collection]

Proposal 2. RS configuration framework for NW-side data collection can be reused for RS configuration framework for UE-side data collection.

Proposal 3. For the UE request configuration, the RS configuration for the applicability report can be applied.

Proposal 4: If proposal 3 is agreeable, RAN2 may consider the subsequent transmission procedure of UE request. (e.g., whether to transmit the UE request through a separate IE or include the UE request in the applicability report).

Proposal 5. MNO can have full controllability over the data collection process through both CP solution and UP solution.

[Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion)]

Proposal 6. For both CP solution and UP solution, MNO can have full visibility over standardized data.

Proposal 7. For the discussion of transferring partial/non-standardized data of UE side data collection, RAN2 should first clarify which specific data needed as partial/non-standardized data.

[Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain]

Proposal 8. RAN2 should wait for the SA2/SA5 discussion about the feasibility check of UP tunnel between CN/OAM and the server.

[Discuss on the scalability aspects of CP]

Proposal 9. The UP solution addresses the signaling overhead and scalability issues associated with the CP solution, allowing for the transmission of large volumes of AIML data using legacy methods without any spec impact.

References

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm.

- RP-243307, “Way forward on AIML UE sided data collection,” Dec 2024

- R2_129_ChairNotes_25-02-21_final.docx

- R2_127_ChairNotes_24-08-23_17-00_eom.docx