3GPP TSG-RAN WG2 Meeting # 129bis	R2-2501976

# Wuhan, China, 7th – 11th April 2025

Agenda item:	7.2

Source:	Google

Title:	Clarification on LTM cell switch failure recovery

Document for:	Discussion & Decision

# Discussion

Upon initiating an LTM cell switch, the UE starts timer T304. As defined in section 5.3.5.8.3, expiration of T304 before the switch completion forces the UE to revert to the UE configuration used in the source PCell.

##### 5.3.5.8.3 T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure) The UE shall: 1> if T304 of the MCG expires; or <unrelated part omitted> 2> else: 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6: 4> revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG; 3> else: 4> revert back to the UE configuration used in the source PCell; 3> if the associated T304 was not initiated upon cell selection performed while timer T311 was running, as defined in clause 5.3.7.3: 4> store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5; 3> initiate the connection re-establishment procedure as specified in clause 5.3.7.

However, when an LTM cell switch is triggered on the MCG, the UE releases all current dedicated and common radio configurations, retaining only the specific configurations and variables described in section 5.3.5.18.6.

##### 5.3.5.18.6 LTM cell switch execution Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall: 1> if the LTM cell switch is triggered on the MCG: 2> release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following: - the radio bearer configuration (configured via RadioBearerConfig) - the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4]; - the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4]; - the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID; - the ltm-Config; - the MCG C-RNTI; - the AS security configurations associated with the master key; - the logged measurement configuration;

According to section 5.3.5.8.3, a UE experiencing T304 expiry during an LTM cell switch must restore the configuration of the source PCell. However, section 5.3.5.18.6 dictates that the UE shall release all current dedicated and common radio configurations, with exceptions for specified configurations and variables. This creates a procedural inconsistency, as section 5.3.5.18.6 inherently limits the reversion to a subset of the UE's configuration, conflicting with the complete reversion required by section 5.3.5.8.3.

Observation: 	There is a procedural inconsistency about managing UE configuration between section 5.3.5.8.3 and section 5.3.5.18.6.

To resolve this discrepancy, we propose two options for clarification:

- Modify Section 5.3.5.8.3: Add a NOTE clarifying that during T304 expiry in an LTM cell switch, the UE only reverts back to the configurations and variables retained as per section 5.3.5.18.6.

- Modify Section 5.3.5.18.6: Add a NOTE clarifying that the UE retains all current dedicated and common radio configurations for reverting back to the UE configuration upon T304 expiry as per section 5.3.5.8.3.

While both proposed opitons are technically feasible, option 1 presents a simpler implementation for the UE. Consequently, we prefer opiton 1 over option 2.

Proposal: 	RAN2 is requested to discuss the two opitons and decide option 1 or option 2 is adopted.

# Conclusion

RAN2 is requested to confirm the observatoin and discuss solutions presented in this paper. If RAN2 agrees to clarify the issue in our observation, we will draft a CR to clarify it.

Observation: 	There is a procedural inconsistency about managing UE configuration between section 5.3.5.8.3 and section 5.3.5.18.6.

Proposal: 	RAN2 is requested to discuss the two opitons and decide option 1 or option 2 is adopted.

- Modify Section 5.3.5.8.3: Add a NOTE clarifying that during T304 expiry in an LTM cell switch, the UE only reverts back to the configurations and variables retained as per section 5.3.5.18.6.

- Modify Section 5.3.5.18.6: Add a NOTE clarifying that the UE retains all current dedicated and common radio configurations for reverting back to the UE configuration upon T304 expiry as per section 5.3.5.8.3.