3GPP TSG-RAN WG2 Meeting # 129bis	R2-2501977

# Wuhan, China, 7th – 11th April 2025

Agenda item:	8.6.2

Source:	Google

Title:	Discussion on inter-CU LTM

Document for:	Discussion & Decision

- Introduction

To facilitate inter-CU MCG LTM, RAN3 has introduced several LTM-related signaling procedures within the Xn Application Protocol (XnAP) as outlined in the RAN3 baseline Change Request (CR) to 38.423 [1]. These procedures include:

- inter-CU LTM preparation (extend Handover Preparation procedure)

- inter-CU Cell Switch notification (new Cell Switch Notification procedure)

- inter-CU LTM cancel (new LTM Cancel procedure)

- subsequent LTM (new LTM Configuration Update procedure)

- inter-CU LTM with NR-DC (extend SN Addition and MN-initiated SN Modification procedures)

Concurrently, RAN2 is developing a running RRC for inter CU LTM [2]. This paper aims to provide observations on the current RAN2 and RAN3 running CRs, identify potential issues, and provide proposals to address these issues.

- Discussion

## 2.1 	Inter-CU MCG LTM

RAN3 has extended the Handover Preparation procedure within the XnAP to support inter-CU MCG LTM, as specified in the RAN3 baseline CR to 38.423 [1]. The following are the added signallings excerpted from the CR.

#### 9.1.1.1 HANDOVER REQUEST <unrelated part omitted> 9.1.1.2 HANDOVER REQUEST ACKNOWLEDGE <unrelated part omitted>

38.423 mandates the inclusion of the HandoverPreparationInformation inter-node message within the Handover Request during the Handover Preparation procedure. However, the current running RRC CR [2] does not define any LTM-specific signaling elements within the HandoverPreparationInformation message. In fact, the running RRC CR [2] introduces no new signaling for inter-CU MCG LTM within any inter-node messages. Consequently, we observe:

Observation 1: For inter-CU MCG LTM, LTM-related configurations, such as reference configuration, Channel State Information (CSI) resource configuration, and LTM candidate configuration, are exchanged via XnAP Information Elements (IEs). There is no signalling overlap (i.e., same IEs) in XnAP and HandoverPreparationInformation inter-node message.

## 2.2 	Inter-CU SCG LTM

The RAN3 baseline CR to 38.423 [1] adds LTM specific signalling elements in the SN Addition and SN Modification procedures for inter-CU SCG LTM in 38.423 as shown below.

#### 9.1.2.1 S-NODE ADDITION REQUEST <unrelated part omitted> #### 9.1.2.2 S-NODE ADDITION REQUEST ACKNOWLEDGE <unrelated part omitted> #### 9.1.2.5 S-NODE MODIFICATION REQUEST <unrelated part omitted> #### 9.1.2.6 S-NODE MODIFICATION REQUEST ACKNOWLEDGE <unrelated part omitted>

Distinct from the inter-CU MCG LTM handling, where no new RRC signaling is introduced in inter-node messages, the running RRC CR [2] incorporates new signaling elements for inter-CU SCG LTM within the CG-Config and CG-ConfigInfo inter-node messages as shown below.

#### – CG-Config <unrelated part omitted> #### – CG-ConfigInfo <unrelated part omitted>

As specified in 38.423, the CG-ConfigInfo is included in the SN Addition Request message and SN Modification Request message. As shown above, there is no signalling overlap in the CG-ConfigInfo, the SN Addition Request message and SN Modification Request message.

Observation 2: For inter-CU SCG LTM, there is no signaling overlap in the CG-ConfigInfo, the SN Addition Request message and SN Modification Request message.

As specified in 38.423, the CG-Config is included in the SN Addition Request Acknowledge message and SN Modification Request Acknowledge message. As shown above, the LTM-Config is added in the CG-Config in the running RRC CR [2]. The following table shows signalling overlaps in the LTM-Config in CG-Config, SN Addition Request Acknowledge message, and SN Modification Request Acknowledge message.

| LTM-Config in CG-Config in SN Addition Request Acknowledge message or SN Modification Request Acknowledge message | SN Addition Request Acknowledge message | SN Modification Request Acknowledge message |
| --- | --- | --- |
| The following configurations are included in each LTM-Candidate for a corresponding LTM candidate PSCell: - LTM-TCI-Info - Early UL sync configuration (for UL) - Early UL sync configuration for SUL - LTM-CandidateConfig | The following configurations are included for each LTM candidate PSCell: - LTM-TCI-Info - Early UL sync configuration (for UL) - Early UL sync configuration for SUL LTM-CandidateConfig | The following configurations are included for each LTM candidate PSCell: - SCG Configuration (i.e., LTM-CandidateConfig for PSCell) |

As shown in table above, we can oberserve:

Observation 3: For inter-CU SCG LTM, there are signalling overlaps in the CG-Config, the SN Addition Request Acknowledge message and SN Modification Request Acknowledge message.

The signalling overlaps lead to the following issues.

- In the SN Addition Request Acknowledge message, should the candidate SN use the LTM-Config or XnAP IEs to provide LTM-TCI-Info and Early UL synchronziatio configurations for UL and/or SUL?

- In the SN Modification Request Acknowledge message, should the candidate SN use the LTM-Config or XnAP IEs to provide a SCG Configuration for a LTM candidate PSCell?

From the standardization perspective, we should avoid specify mulitple options for the same purpose. The signaling overlaps just create complexicity in the gNB implementation, so we don’t prefer the signalling overlaps for the same purpose as indicated above. Therefore, we propose:

Proposal 1: 	RAN2 should discuss and decide on the specific configuration IEs to be included within XnAP IEs or the inter-node CG-Config message.

To maintain consistent design logic, we propose utilizing the same design approach for exchanging LTM-related information in inter-CU MCG LTM and inter-CU SCG LTM preparation. Besides, we prefer exchange of LTM-related information either via inter-node messages or XnAP IEs.

Proposal 2: 	If RAN2 decides to include LTM-Config in the CG-Config as the current running RRC CR, RAN2 should send a LS to RAN3 to inform the RAN2’s decision.

RAN3 baseline CR 38.423 supports prepare multiple LTM candidate PSCells in a single SN Addition procedure. If the SN Addition Request message requests mulitple LTM candidate PSCells, the candidate SN can prepare multiple LTM candidate PSCells. In accordance with the running RRC CR, the candidate SN generates a LTM-Config to include mulitple LTM candidate configurations for the repective LTM cadidate PSCells, generates a CG-Config to include the LTM-Config, and includes the CG-Config in a SN Addition Request Acknowledge message. The candidate SN then transmits the SN Addition Request Acknowledge messsage to the MN.

During immediate SN addition or change procedures, the CG-Config is used for preparing the target Primary Secondary Cell (PSCell) and zero, one, or more Secondary Cells (SCells). The CG-Config also includes coordination information between the Master Node (MN) and SN, such as DRX-Info, to align their DRX configurations. A similar coordination process may be applied between the MN and the candidate SN during inter-CU SCG LTM preparation.

However, when preparing multiple LTM candidate PSCells within a single SN Addition procedure, the candidate SN is required to use the same coordination information for all candidate PSCells, as specified in the running RRC CR [2]. This restriction may be undesirable, as it effectively prevents the use of a single SN Addition procedure for preparing multiple LTM candidate PSCells.

For SCPAC, a single SN Addition procedure can prepare multiple SCPAC candidate PSCells. This flexibility is due to the CG-CandidateList, which is defined for SCPAC and included in the SN Addition Request Acknowledge message, allowing the candidate SN to coordinate SCG configurations for multiple SCPAC candidate PSCells with the MN.

We propose extending this flexibility to inter-CU SCG LTM. Therefore, we recommend defining an LTM-CG-CandidateList, analogous to the CG-CandidateList, and including it in relevant XnAP messages. Specifically, these XnAP messages are the SN Addition Request Acknowledge, SN Modification Request Acknowledge, and SN Modification Required message.

The LTM-CG-CandidateList would serve two purposes: firstly, to prepare multiple LTM candidate PSCells during SN Addition, and secondly, to modify configurations of multiple LTM candidate PSCells during SN Modification. Additionally, the source SN could use the SN Modification Required message with the LTM-CG-CandidateList to release one or more LTM candidate PSCells.

LTM-CG-CandidateList ::=                SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ltm-CG-CandidateList-r17                LTM-CG-CandidateList-r17-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

LTM-CG-CandidateList-r17-IEs ::=        SEQUENCE {

ltm-CG-CandidateToAddModList-r17        SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF LTM-CG-CandidateInfo-r17    OPTIONAL,

ltm-CG-CandidateToReleaseList-r17       SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF LTM-CG-CandidateInfoId-r17  OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                          OPTIONAL

}

LTM-CG-CandidateInfo-r17 ::=            SEQUENCE {

ltm-cg-CandidateInfoId-r17             LTM-CG-CandidateInfoId-r17,

ltm-candidateCG-Config-r17             OCTET STRING (CONTAINING CG-Config)

}

LTM-CG-CandidateInfoId-r17::=           SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR,

physCellId-r17                      PhysCellId

}

Proposal 3: 	Define LTM-CG-CandidateList, analogous to CG-CandidateList, and request RAN3 to include it in the relevant XnAP messages.

## 2.3 	Issue in Running RRC CR

As shown above, the CG-Config-v19-xy-IEs include the ltm-ReferenceConfiguration-r19 (i.e., ReferenceConfiguration-r18) and the LTM-Config. As specified in 38.331, the LTM-Config also includes ReferenceConfiguration-r18. Again, such signalling overlap should be avoided with similar reasoning as described above. If RAN2 decides to include LTM-Config in the CG-Config, we propose:

Proposal 4: 	Remove ltm-ReferenceConfiguration-r19 from CG-Config-v19-xy-IEs.

# Conclusion

We provide the following observations in this paper.

Observation 1: For inter-CU MCG LTM, LTM-related configurations, such as reference configuration, Channel State Information (CSI) resource configuration, and LTM candidate configuration, are exchanged via XnAP Information Elements (IEs). There is no signaling overlap (i.e., same IEs) in XnAP and HandoverPreparationInformation inter-node message.

Observation 2: For inter-CU SCG LTM, there is no signalling overlap in the CG-ConfigInfo, the SN Addition Request message and SN Modification Request message.

Observation 3: For inter-CU SCG LTM, there are signalling overlaps in the CG-Config, the SN Addition Request Acknowledge message and SN Modification Request Acknowledge message.

Based on the observations, we identifies the issues and provides the following proposals. RAN2 is kindly to discuss and agree the following proposals.

Proposal 1: 	RAN2 should discuss and decide which node (i.e., either the CU or the DU) configures the L1 conditions.

Proposal 2: 	If RAN2 agrees that the DU generates the L1 conditions and provides to the CU, RAN2 sends the agreement in a LS to RAN3.

Proposal 3: 	Define LTM-CG-CandidateList, analogous to CG-CandidateList, and request RAN3 to include it in the relevant XnAP messages.

Proposal 4: 	If RAN2 agrees that the DU generates the L1 conditions, “OCTET STRING” should be used for l1-Conditions-r19.

# Reference

[1] R3-250935 (BL CR to 38.423) Xn support for inter-CU LTM

[2] Running RRC CR for inter-CU and conditional LTM v14

[3] R3-250933 (BL CR to 38.300) Support for Inter-CU LTM

[4] R3-250931 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

# Annex 1: inter-CU MCG LTM (excerpted from R3-250933)

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. Up to implementation, late data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs.

Editor’s Note: The new source gNB may initiate to setup the UE associated Xn signalling connection with the candidate gNBs. Details are FFS.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

# Annex 2: SN initiated inter-SN SCG LTM (excerpted from R3-250931)

### 10.5.2	MR-DC with 5GC

<unrelated text ommited>

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

1.  The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required  to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells to the MN.

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides a list of KSN and associated sk-Counter values for each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may select one of the candidate SN(s) and request providing the reference SCG configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s). The candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, the early UL sync configuration, and the TCI-state configuration.

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message.

7.	The MN transfers, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to each candidate SN via the SN Modification Request message.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN via the SN Modification Request Acknowledge message.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared.

Editor’s Note: FFS up to RAN3.

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN via the TA Information Transfer procedure.

Editor’s Note: Details for TA information transfer procedure is up to RAN3 discussion.

The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9.

16.	The source SN decides to execute cell switch to a target PSCell.

transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID including PSCell change.

18/19.	The source SN sends a notification to the target SN via the MN by Cell Switch Notification procedure, to indicate the triggering of LTM to the UE.

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.