3GPP TSG-RAN WG2 Meeting #129b	 R2-2502003

Wuhan, China, April 7-11, 2025

Agenda Item:	8.3.4

Source: 	NEC

Title:  	Discussion on AIML mobility LCM procedure

Document for:	Discussion and Decision

- 1 Introduction

Since this is the first time to discuss LCM procedure for AIML mobility, we give our understanding regarding AIML mobility specific model management.

- 2 Discussion

- 2.1 Model management

The general framework for LCM procedure for AIML PHY is given as the following:

Fig.1 Functional framework for AI/ML for NR Air Interface

Basically, the agreed functional framework for AIML PHY can be reused for AIML mobility. Model management may include details about the model/functionality selection, (de)activation, switching or fallback. Here we consider some mobility specific operation for model management for UE-side model.

AIML mobility use case can facilitate RRM measurement configuration, basically we can achieve two goals by operating AIML mobility model/functionality. One is for measurement reduction and the other one is for handover performance. For the latter one, by AIML RRM measurement prediction, the UE/NW can observe potential handover earlier compared with legacy reactive handover procedure. Therefore handover performance is improved.

However on the other hand, operating AI/ML model/functionality comes at a cost, e.g., UE power consumption, AIML specific computing resources could be increased. Since this is for mobility use case, if one UE is just staying cell centre, or has a stable signal quality, it is more likely that such UE does not have a need for handover. In this scenario, operating AIML RRM measurement prediction and event prediction is more or less useless, the AIML model/functionality can be deactivate for power saving. It is noted that in this contribution, model/functionality activation/deactivation only means whether the UE needs to perform inference or not for prediction, for other LCM procedures like data collection, model training, model monitoring, can be further studied.

Fig.1 AIML model (de)activation scenario

Proposal-1: AIML mobility model/functionality can be activated/deactivated (i.e., whether the UE performs model inference or not) based on not-at-cell-edge and/or stationary criterion.

Furthermore, in TR 38. 843, multiple solutions for model management (e.g., model/functionality selection, (de)activation, switching or fallback) is given:

Opt-1: network initiated, decision by network

Opt-2: UE initiated and requested, decision by network

Opt-3: UE decision, and report to network

Opt-4: UE auto decision, and without report to network

Based on AIML PHY discussion, the opt-4 is precluded as network should have some control. Therefore, we further consider opt 1-3 for AIML mobility model (de)activation. Below is taken model deactivation as an example.

For option 1 (network initiated, decision by network), even though the UE is not involved with the decision, NW still need to configure some RRM measurement configuration to UE so that the UE could provide some assistance information for facilitating decision. For example, NW could configure periodical measurement report to UE, once NW, based on implementation, think the AIML model should be deactivated, the NW could directly send e.g., RRCReconfiguration to deactivate the model/functionality.

Proposal-2: for network initiated, decision by network, the NW could directly deactivate AIML mobility model/functionality based on assistance information from UE, e.g., periodical measurement report.

For option 2 (UE initiated and requested, decision by network), the UE is involved with the decision, based on the discussion above, not-at-cell-edge and/or stationary criterion can be configured to the UE, once the UE is fulfilling these criteria, the UE does not deactivate the AIML model by itself, instead, the UE could send request to NW for deactivating the AIML model via e.g., UAI.

Proposal-3: for UE initiated and requested, decision by network, not-at-cell-edge-like and/or stationary-like criteria can be configured to UE, once these criteria are fulfilled, the UE requests NW for deactivating AI/ML model/functionality.

For option 3 (UE decision, and report to network), slight different from option 2 that once the UE is fulfilling not-at-cell-edge and/or stationary criterion, the UE could directly deactivate AIML model/functionality and after that the UE send the decision report to NW for information.

Proposal-4: for UE decision, and report to network, not-at-cell-edge-like and/or stationary-like criteria can be configured to UE, once these criteria are fulfilled, the UE deactivates AIML model/functionality and reports the decision to NW.

- 3 Conclusions

Proposal-1: AIML mobility model/functionality can be activated/deactivated (i.e., whether the UE performs model inference or not) based on not-at-cell-edge and/or stationary criterion.

Proposal-2: for network initiated, decision by network, the NW could directly deactivate AIML mobility model/functionality based on assistance information from UE, e.g., periodical measurement report.

Proposal-3: for UE initiated and requested, decision by network, not-at-cell-edge-like and/or stationary-like criteria can be configured to UE, once these criteria are fulfilled, the UE requests NW for deactivating AI/ML model/functionality.

Proposal-4: for UE decision, and report to network, not-at-cell-edge-like and/or stationary-like criteria can be configured to UE, once these criteria are fulfilled, the UE deactivates AIML model/functionality and reports the decision to NW.

- 4 References