3GPP TSG-RAN WG2s #129bis	R2-2502010

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.13.2

Source:	NEC

Title:	Discussion on multi-hop relay discovery and relay reselection

Document for:	Discussion, Decision

Introduction

So far up to RAN2 #129 meeting, the principle of multi-hop U2N relay discovery and reselection has almost been settled down. Yet still there are some unsolved issues, which shall be discussed in this paper.

Discussion

Previously, RAN2 has made the following agreement:

RAN2 #129 Agreements: Agreement: The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell. FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive. Agreement: From RAN2 perspective, when an intermediate/last relay UE serves two different downstream UEs, there is no assumption on whether it presents the same or different L2IDs for connection establishment. Agreements: Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay. Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay. Agreements: For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient). For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied. FFS if the PC5 link is not already established. Agreements: For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE. For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition: - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold. - When U2N Remote UE receives a PC5-S link release message from current first relay UE. - When U2N Remote UE detects PC5 RLF with the current first relay UE. - Indicated by upper layer. L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS: link quality degradation of the upstream links resulting in a notification. - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. - FFS applicability to L3. Agreements: The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification. FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger) FFS if the notification is required in all cases FFS the content in the notification FFS the child node behaviour after receiving this notification FFS other condition/timing that the intermediate relay UE notifies its child node

Currently it has been agreed that the U2N intermediate relay UE can perform relay (re)selection as a U2N remote UE upon the following condition:

- PC5 RRC link release between U2N intermediate relay UE and U2N last relay UE

- Indication from upper layer to trigger relay reselection

- Cell reselection, handover, Uu RLF or Uu RRC connection establishment/resume failure between the last relay UE and network

In addition, in order to maintain a stable multi-hop U2N relay link, one may challenge that the link quality between U2N intermediate relay UE and U2N last relay UE is also critical, so if there is a link quality degradation between U2N intermediate relay UE and U2N last relay UE, one notification shall be sent towards U2N remote UE to trigger its relay reselection. However, the link shall be maintained by U2N intermediate relay UE, i.e., if there is a link quality degradation between U2N intermediate relay UE and U2N last relay UE, the relay reselection shall be triggered by U2N intermediate relay UE.

The relay reselection shall be triggered by U2N intermediate relay UE if there is a link quality degradation between U2N intermediate relay UE and U2N last relay UE.

Also, according to the previous agreement, once the relay (re)selection is triggered by U2N intermediate relay UE, it shall notify the U2N remote UE. In order to allow the U2N remote UE to have a better understanding of the link condition between U2N intermediate relay UE and U2N last relay UE, one cause value can be carried within the notification message, of which the following cause value can be considered:

- PC5 RLF between the multi-hop relay UEs

- Cell reselection of the U2N last relay UE

- Handover of the U2N last relay UE

- Uu RLF/Uu RRC connection establishment/resume failure of the U2N last relay UE

The following value can be considered to set as cause within the notification message upon U2N intermediate relay UE triggering relay reselection:

- PC5 RLF between the multi-hop relay UEs

- Cell reselection of the U2N last relay UE

- Handover of the U2n last relay UE

- Uu RLF/Uu RRC connection establishment/resume failure of the U2N last relay UE

Furthermore, there are some particular cases where the notification message shall also be considered to be sent. For some time, the Uu link quality of the U2N intermediate relay UE may become better, in that case, the U2N intermediate relay UE shall perform cell reselection.

Thereafter, one notification message shall be sent towards the U2N remote UE. The U2N remote UE shall decide whether to maintain the PC5 link with U2N intermediate relay UE. If the U2N remote UE decide to maintain the PC5 RRC link with the U2N intermediate relay UE which is performing cell reselection, the multi-hop U2N relay link is actually become to the single hop U2N relay link.

When the Uu link quality of the U2N intermediate relay UE become better, the U2N intermediate relay UE shall perform cell reselection and send notification message towards U2N remote UE.

Considering some companies raised the concern of double-triggering of relay reselection during last meeting, one timer can be configured together with the notification message for both U2N intermediate relay UE’s relay reselection and cell reselection. In details, upon U2N intermediate relay UE send the notification message, the U2N remote UE can start the timer to give some time towards U2N intermediate relay UE for its relay reselection or cell reselection. Once the timer expired while the multi-hop relay link is not recovered, it shall be up to U2N remote UE implementation to perform relay reselection or maintain the link.

One timer can be configured together with the notification message for both U2N intermediate relay UE’s relay reselection and cell reselection.

Conclusion

In this contribution, we have primarily discussed further enhancement at RAN2 for U2N multi-hop relay. A brunch of proposals has been made as following:

Proposal 1	The relay reselection shall be triggered by U2N intermediate relay UE if there is a link quality degradation between U2N intermediate relay UE and U2N last relay UE.

Proposal 2	The following value can be considered to set as cause within the notification message upon U2N intermediate relay UE triggering relay reselection:

-	PC5 RLF between the multi-hop relay UEs

-	Cell reselection of the U2N last relay UE

-	Handover of the U2n last relay UE

-	Uu RLF/Uu RRC connection establishment/resume failure of the U2N last relay UE

Proposal 3	When the Uu link quality of the U2N intermediate relay UE become better, the U2N intermediate relay UE shall perform cell reselection and send notification message towards U2N remote UE.

Proposal 4	One timer can be configured together with the notification message for both U2N intermediate relay UE’s relay reselection and cell reselection.

Reference