3GPP TSG-RAN WG2 Meeting #129bis	R2-2502020

Wuhan, China, Apr. 7th – 11th , 2025

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	Functionality based LCM for BM

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #129 meeting, RAN2 reached the following agreements.

- Agreements Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). The full inference configuration is sent in CSI-ReportConfig. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

This contribution aims to address issued regarding applicable functionality reporting, data collection for UE sided training, and functionality continuity.

2. Discussion

Applicability report

UE Capability Information for applicability configuration/report

The network will refer to the information within the UE Capability Information to set the applicability report configuration. If only the maximum values of specific capabilities related to measurement prediction are reported in UECapabilityInformation message, it may lead to the inclusion of configurations for the applicable report that are not actually applicable for the UE. Therefore, to ensure an appropriate configuration for the applicability report, it would be beneficial to report the capability information that can update as value ranges.

For example, consider the following Functional Group (FG) that takes into account the sizes of Set A and Set B:

- FG: BM-Case1

  - Component#1: the max size of Set B

    - Values: 16, 32, 64, 128, …

  - Component#2: the max size of Set A

    - Values: 32, 64, 128, 256, …

  - Component#3: the supported ratio(s) of Set B size divided by Set A size

    - Values: 1/2, 1/4, 1/8, …

Component#1 is related to memory requirement at UE side thus the reported value in Step 2 may not be updated later according to applicable functionality updates. However, the supported values related to component#2 and component#3 can be updated according to applicable functionality updates, e.g. based on updates on NW or UE-side additional condition.

For example, a UE may report the following in Step 2(UECapabilityInformation).

- Support of the FG

- Component#1: 32

- Component#2: 64, 128

- Component#3: 1/2, 1/4

The network may configure the UE for applicability reporting in step 3 based on the UE capability information as components potentially applicable to the UE. The UE may then report the set of currently applicable components (e.g. (64, 1/2) and/or (128, 1/4)) in step 4.

Proposal 1: For the components of a Functional Group (FG) in UECapabilityInformation message, the UE reports value ranges that are potentially applicable. FFS detailed capability components are required for RAN1 input

Explicit Inapplicability report

RAN2 has agreed to support the explicit reporting of applicability/inapplicability in the initial report, and in subsequent reports.

Regarding the reporting method, there are two approaches: index type and bit masking type. The original proposal to report explicit inapplicability was intended to inform the network of the reasons for inapplicability. For example, some companies have suggested that the applicable functionality report can be used for specific purposes, such as requesting a particular configuration when retraining is necessary. In this context, they argue that non-applicable functionalities should be explicitly reported along with the reasons, allowing the network to understand which configuration should be used for training instead of activating a specific functionality.

Therefore, according to the original intent, the bitmap masking method may not effectively convey the reasons for individual functionalities being inapplicable. Additionally, interpreting a bitmap requires decoding each bit for specific configurations, which can introduce overhead, especially if configurations change frequently, potentially complicating network operations.

Observation 1. Inapplicable functionalities can be explicitly reported along with reasons, enabling the network to determine which configuration should be used for training instead of activating a particular functionality. The bitmap masking method may not effectively convey the reasons for individual functionalities being inapplicable

Proposal 2. To use index type information to indicate applicable and inapplicable functionalities.

However, in cases where it is unnecessary to inform the network of specific reasons for inapplicability—such as when the purpose is to activate or deactivate a particular functionality—reporting both applicable and non-applicable functionalities simultaneously may be redundant and could introduce signaling overhead. The applicable functionalities already indicate what can be activated.

In practice, from the perspective of (de)activation, the network only needs to manage what can be activated, not what cannot. As long as the UE provides a list of applicable functionalities, the network can implicitly infer that the rest are inapplicable. Therefore, consistently reporting inapplicable functionalities in these cases, even when there is no specific reason, could lead to inefficiencies, such as unnecessary signaling overhead.

To address this, RAN2 should separate the procedures for reporting non-applicable functionalities based on the purpose of the report:

- For deactivating a inapplicable functionality:

  - There is no need to explicitly report inapplicable functionality. Simply reporting the applicable functionality is sufficient.

- For informing the network about a preferred configuration (e.g., in cases related to training purposes):

  - The UE may explicitly report inapplicable functionality only when it intends to request a specific transmission/configuration

Observation 2. When there is no reason for inapplicability, reporting both applicable and non-applicable functionalities may be redundant and introduce signalling overhead.

Proposal 3. Inapplicable functionalities should only be reported only when specific reasons exist (Annes 4.1)

Data collection for UE sided training

RS configuration

The RS configuration for UE-sided data collection is related to the CSI reporting configuration. The request for RS configuration is currently under discussion in the applicability report, and the framework for these requests is being addressed through Option A and Option B. Therefore, instead of introducing a new RS-related configuration framework for UE request, the existing RS configuration framework for applicability report can be utilized for the UE request. For instance, Option A can be used for the transmission of the preferred CSI configuration, while Option B can be used to request the preferred RS configuration, as outlined in Annex 4.2.

RAN2 can also consider several methods for the subsequent process of sending the UE request and assess the feasibility of each method. The following methods are examples that can be considered:

- Method 1: Specify a separate Information Element (IE), such as DataCollectionPreference-r19

- Method 2: Include the UE request within the applicability report for transmission.

Proposal 4: RS configuration for UE request (for UE sided data collection) can reuse RS configuration framework for applicability report (Annex 4.2)

Proposal 5. To discuss the subsequent transmission procedure of UE request (e.g., transmit UE request through a separate IE(e.g., DataCollectionPreference-r19) or whether to include the UE request in the applicability report) (Annex 4.2)

3. Conclusion

Proposal 1: For the components of a Functional Group (FG) in UECapabilityInformation message, the UE should report value ranges that are potentially applicable. FFS detailed capability components are required for RAN1 input

Observation 1. Inapplicable functionalities can be explicitly reported along with reasons, enabling the network to determine which configuration should be used for training instead of activating a particular functionality. The bitmap masking method may not effectively convey the reasons for individual functionalities being inapplicable

Proposal 2. To use index type information to indicate applicable and inapplicable functionalities.

Observation 2. When there is no reason for inapplicability, reporting both applicable and non-applicable functionalities may be redundant and introduce signalling overhead.

Proposal 3. Inapplicable functionalities should only be reported only when specific reasons exist (Annes 4.1)

Proposal 4: RS configuration for UE request (for UE sided data collection) can reuse RS configuration framework for applicability report (Annex 4.2)

Proposal 5. To discuss the subsequent transmission procedure of UE request (e.g., transmit UE request through a separate IE(e.g., DataCollectionPreference-r19) or whether to include the UE request in the applicability report) (Annex 4.2)

4. Annex

4.1 ASN.1 for P3

#### –	ApplicabilityReportList

The IE ApplicabilityReportList comprises information that the UE reports to gNB related to the applicability of the radio measurement prediction configurations at UE.

ApplicabilityReportList information element

-- ASN1START

-- TAG-APPLICABILITYREPORTLIST-START

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..FFS)) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19               ServCellIndex,

applicabilityReportConfigIdList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdList-r19     OPTIONAL,

...

}

ApplicabilityReportConfigIdList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19        CHOICE {

CSI-ReportConfigId                        NULL,//e.g., utilized to request transmission using option A config

applicabilityReportConfigId-r19           NULL,// e.g., utilized to request configuration using option B config

...

},

applicabilityStatus-r19                        ENUMERATED {true}, OPTIONAL,

reason-r19                                     FFS OPTIONAL, // FFS for details (e.g., training)

...

}

-- TAG-APPLICABILITYREPORTLIST-STOP

-- ASN1STOP

Editor's Note: FFS the structure of the IE ApplicabilityReportList and other content in ApplicabilityReport, e.g. for option B.

ApplicabilityReportList field descriptions

applicabilityCellId Index of the serving cell that the ApplicabilityReport refers to. Editor's Note: FFS whether a cell ID (e.g. ServCellIndex) should be included in ApplicabilityReport to unambiguously identify the prediction configuration across serving cells.

applicabilityReportConfigIdList Indicates the list of applicability reports, each associatied with a configuration ID for a configuration subject to the applicability determination procedure.

applicabilityReportConfigId Indicates a CSI-ReportConfigId associated to a CSI report configured for radio measurement predictions.

applicabilityStatus Indicates whether the CSI report configured for radio measurement predictions and associated to applicabilityReportConfigId is applicable. This field absent when the CSI report configured for radio measurement predictions and associated to applicabilityReportConfigId is non-applicable.

reason Indicates reason for report FFS for details (e.g., training) This field absent when the CSI report configured for radio measurement predictions and associated to applicabilityReportConfigId is applicable.

<Text Omitted>

4.2 ASN.1 for P4

Method 1. Specify a separate Information Element (IE), such as DataCollectionPreference-r19

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

<skip>

DataCollectionPreference-r19 ::= SEQUENCE {

DC-preferenceConfigList-r19    SEQUENCE (SIZE (1..maxNrofDC-PreferenceConfig)) OF DC-PreferenceConfigList-r19     OPTIONAL,

...

}

DC-PreferenceConfigList-r19  :: = SEQUENCE {

DC-PreferenceConfigId-r19        CHOICE {

CSI-ReportConfigId                        NULL,//e.g., to request transmission using option A config

applicabilityReportConfigId-r19           NULL,// e.g., to request configuration using option B config

...

},

DC-termination-r19      ENUMERATED {true}     OPTIONAL, // data collection stop ind

...

}

LoggedDataCollectionAssistance-r19 ::=    SEQUENCE {

lowBatteryState-r19                         ENUMERATED {true}                                                   OPTIONAL,

memoryFull-r19                              ENUMERATED {true}                                                   OPTIONAL,

csi-LogMeasAvailable-r19                    ENUMERATED {true}                                                   OPTIONAL

...

}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

Method 2. Include the UE request within the applicability report for transmission.

#### –	ApplicabilityReportList

The IE ApplicabilityReportList comprises information that the UE reports to gNB related to the applicability of the radio measurement prediction configurations at UE.

ApplicabilityReportList information element

-- ASN1START

-- TAG-APPLICABILITYREPORTLIST-START

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..FFS)) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19               ServCellIndex,

applicabilityReportConfigIdList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdList-r19     OPTIONAL,

...

}

ApplicabilityReportConfigIdList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19        CHOICE {

CSI-ReportConfigId                        NULL,//e.g., utilized to request transmission using option A config

applicabilityReportConfigId-r19           NULL,// e.g., utilized to request configuration using option B config

...

},

applicabilityStatus-r19                        ENUMERATED {true}, OPTIONAL,

reason-r19                                     FFS OPTIONAL, // FFS for details (e.g., training (i.e., RS transmission req)/training stop (i.e data collection stop))

...

}

-- TAG-APPLICABILITYREPORTLIST-STOP

-- ASN1STOP

Editor's Note: FFS the structure of the IE ApplicabilityReportList and other content in ApplicabilityReport, e.g. for option B.

ApplicabilityReportList field descriptions

applicabilityCellId Index of the serving cell that the ApplicabilityReport refers to. Editor's Note: FFS whether a cell ID (e.g. ServCellIndex) should be included in ApplicabilityReport to unambiguously identify the prediction configuration across serving cells.

applicabilityReportConfigIdList Indicates the list of applicability reports, each associatied with a configuration ID for a configuration subject to the applicability determination procedure.

applicabilityReportConfigId Indicates a CSI-ReportConfigId associated to a CSI report configured for radio measurement predictions.

applicabilityStatus Indicates whether the CSI report configured for radio measurement predictions and associated to applicabilityReportConfigId is applicable. This field absent when the CSI report configured for radio measurement predictions and associated to applicabilityReportConfigId is non-applicable.

reason Indicates reason for report FFS for details (e.g., training (i.e., RS transmission req)/training stop (i.e data collection stop)) This field absent when the CSI report configured for radio measurement predictions and associated to applicabilityReportConfigId is applicable.

<Text Omitted>