3GPP TSG-RAN WG2 Meeting #129bis	R2-2502028

Wuhan, China, Apr. 7th –11th, 2025

Agenda item:	8.2.2

Source: 	Fujitsu

Title: 	Discussions on AIoT Paging

Document for:	Discussion and decision

1. Introduction

According to the objective specified in the New WID [1], RAN2 needs to specify A-IoT Paging to enable DO-DTT and DT data transmission. At last RAN2 meeting, some agreements related to A-IoT Paging are reached [2]:

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

This contribution will provide our considerations on the Ambient IoT Paging (including subsequent paging for the same service).

2. Discussion

During Rel-19 SI phase, RAN2 has made some agreements and reported them in [3]. According to the WID, the definitions provided in the TR and decisions made during the Rel-19 SI are taken into this WI. Therefore, the discussions can be started from the decision already made in the TR.

Determination to skip the response

Regarding the A-IoT Paging email discussion, some proposals are made in the summary report based on companies’ comments [4]. Device behaviour can be discussed based on these proposals in this summary report.

In the Rel-19 TR, it is captured that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. Based on a transaction ID, the A-IoT device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). Regarding the determination to skip sending the response, the A-IoT device should compare the stored transaction ID with the received one in the A-IoT paging message. In this way, the A-IoT device will store the transaction ID if it had successfully responded the A-IoT paging message associated with the transaction ID. Alternatively, the A-IoT device also stores the status, i.e. whether it has successfully responded the A-IoT paging message associated with a transaction ID and takes the stored transaction ID and the corresponding status into account to determine whether to skip sending the response. We prefer the former slightly since less information will be stored.

Proposal 1: RAN2 to agree that the A-IoT device will store a transaction ID if it had successfully responded the A-IoT paging message associated with the transaction ID.

Content and format for A-IoT paging

In the TR, the following information can be contained in the A-IoT paging message:

- information to identify the device/group of devices, considering:

  - no any identifier

  - an identifier(s) of a single A-IoT device

  - multiple identifier(s) as requested by WID

  - a group ID

- information from which the device(s) can determine the resource(s) to be used for D2R response message(s) and random access type

- information for D2R scheduling (FFS whether MAC signalling is used)

- in case of multiple (subsequent) A-IoT paging message

  - information to avoid duplicated response from the device to a reader

- FFS on the following information:

  - information required for multiple readers case

  - information to indicate service type/command type (inventory/read/write/disable/enable) or to indicate whether there will be subsequent messages

It is observed that both A-IoT upper layer data (e.g. identifier or group ID, service type/command type) and MAC control information (e.g. random access related information) need to be contained in the A-IoT paging message.

Observation: A-IoT paging message needs to include both A-IoT upper layer data and MAC control information.

There are two options for the transmission of the A-IoT paging message. For example, a R2D transmission including A-IoT paging message contains both A-IoT upper layer data and MAC control information. Or, the A-IoT paging message involves two R2D transmissions, i.e. one for A-IoT upper layer data and the other for MAC control information. If two R2D transmissions are used for the A-IoT paging message, more complexity could be introduced. For example, how to determine whether the transmission of the A-IoT paging message fails and the multiple/subsequent A-IoT paging message.

Proposal 2: RAN2 to agree that A-IoT paging message contains both A-IoT upper layer data (SDU) and MAC control information, e.g. a transaction ID.

Paging identifier

RAN2 assumed that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. During the email discussion [4], the need for visibility in the MAC layer was discussed. It seems that the following use cases require the paging identifier visible in A-IoT MAC layer:

- Case 1: the paging identifier is used to associate the AS ID with the A-IoT device

- Case 2: AS grouping

For case 1, the issue can be discussed in AI 8.2.4. Basically, we think that the issue can be solved by e.g. deterministic D2R resource or 16-bit random number in Msg1.

For case 2, AS grouping incurs more complexity and standization effors. Considering the timeline, at least AS grouping will not be supported in this release.

Proposal 3: RAN2 to confirm the current assumption that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.

3. Conclusion

In this contribution, we had some discussions on the Ambient IoT Paging and made the following observation and proposals:

Determination to skip the response

Proposal 1: RAN2 to agree that the A-IoT device will store a transaction ID if it had successfully responded the A-IoT paging message associated with the transaction ID.

Content and format for A-IoT paging

Observation: A-IoT paging message needs to include both A-IoT upper layer data and MAC control information.

Proposal 2: RAN2 to agree that A-IoT paging message contains both A-IoT upper layer data (SDU) and MAC control information, e.g. a transaction ID.

Paging identifier

Proposal 3: RAN2 to confirm the current assumption that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.

Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106, December 9-12, 2024

- RAN2 Chair notes, RAN2 Chair (InterDigital), RAN2#129

- 3GPP TR38.769, “Study on solutions for ambient IoT (Internet of Things)”

- R2-2502211, “Email discussion report: [POST129] [035] [AIoT] Paging”, Qualcomm Incorporated, RAN2#129bis