3GPP TSG-RAN WG2 Meeting #129bis	R2-2502037

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.7.6

Source: 	Fujitsu

Title: 	Discussions on XR rate control

Document for:	Discussion and decision

Introduction

On XR rate control, there are following agreements in RAN2#128 meeting [1]:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In RAN2#129 meeting, the following agreements are made [2]:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

In this contribution, we would like to discuss the details of the rate control MAC CE from gNB to UE, the potential format of the rate control MAC CE and confirm the working assumption on the rate query MAC CE.

Discussion

Per QoS flow or per DRB rate control

In RAN2#128 meeting, there are two options for rate indication from the gNB to the UE on a per QoS flow level as following:

Option 1) flows are indicated by MAC CE.

Option 2) by RRC while MAC CE is per DRB.

In Option 1), the MAC CE from gNB to UE would indicate the rate control information for one or more QoS flows, and QFI related information of the QoS flows should be included in the MAC CE. At the gNB side, the QFI related information may need to be transmitted from CU to DU in CU-DU split scenario, and some impact on F1AP is foreseen. At the UE side, upon receiving the rate control MAC CE, MAC layer may need to indicate the QFI related information to SDAP layer or higher layer, since PDCP, RLC and MAC cannot see QoS flows. In our opinion, Option 1) may need more inter-layer interactions between MAC and SDAP/higher layers, which may have some specification impact or increase UE complexity. In addition, Option 1) has more overhead, since the QFI related information included in MAC CE may include QFI and PDU session ID for each indicated QoS flow.

In Option 2), the MAC CE from gNB to UE would indicate the rate control information for one or more DRBs, and DRB related information should be included in the MAC CE. Since DRB related information is already known at gNB-DU and MAC layer, Option 2) requires less specification impact and is simpler than Option 1), like the PDCP duplication activation/deactivation MAC CE.

In the reply LS from RAN3 last meeting [3], RAN3 agrees that both options an impact on the F1 interface. RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available. In RAN#107 meeting, the necessary update of the WID objectives and TU allocations for RAN3 is discussed. The support of exposure of available data rate is added as an objective in RAN3. From our perspective, Option 2) is preferred for its simplicity and smaller specification impact.

Proposal 1: Rate control MAC CE from the gNB to the UE indicates rate control information per DRB and QoS flows are indicated via RRC as in legacy.

Potential rate control MAC CE format

The format of the current Recommended bit rate MAC CE is shown in the following figure. It has a fixed size indicating the recommended bit rate for only one logical channel. In this MAC CE, LCID, UL/DL, Bit rate and X bits are included.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

To save the signalling overhead of indicating rate control information for multiple DRBs or QoS flows (depending on which option will be adopted by RAN2) by multiple MAC CEs where each MAC CE indicates rate control information for a single DRB/QoS flow, the rate control MAC CE can indicate the rate control information of multiple DRBs/QoS flows at a time. To be simple, a bitmap can be used at the beginning of the MAC CE, indicating the index(indices) of the DRBs/QoS flows for which the rate control information is included in the MAC CE. For each indicated DRB or QoS flow, the bit rate needs to be indicated.

In the SA4 LS [4], SA4 reply is as follows:

The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps. The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

In this LS, SA4 replies that the range of bit rate varies from 100kbps to 40Mbps and finer granularity, e.g., 1%, is generally preferred but coarser granularity may also be sufficient, e.g., 10%.

If the granularity is 1%, we suppose that 604 values are needed to indicate the bit rates from 100kbps to 40Mbps and the length of the bit rate field would be 10 bits. If coarser granularity can be used, 256 values may be enough to indicate the bit rates in this range and the length of the bit rate field can be reduced to 8 bits. If the direction (UL or DL), the bitRateMultiplier and other information also need to be indicated in the MAC CE, the rate control information for each DRB/QoS flow would be 2-byte long. An example of the potential MAC CE format is shown in Figure 1 where the length of the bit rate field is 8 bits. The length of each field in the MAC CE can be determined based on further RAN2 discussions.

Figure 1 Potential format of rate control MAC CE

Proposal 2: The rate control MAC CE indicates the rate control information of multiple DRBs/QoS flows.

UL rate query

In RAN2#129 meeting, a	working assumption is made on rate query MAC CE as following:

a.	Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE.

b.	The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy).

We think this working assumption can be confirmed as agreements, since it has very small impact on the specifications.

Proposal 3: The working assumption is changed to agreements on rate query MAC CE.

The trigger of rate query can be similar to the trigger of Recommended bit rate query, e.g., requested by upper layer. After a rate query is triggered, it can be considered as pending until it is cancelled. After the generation or transmission of the rate query MAC CE, the triggered (pending) rate query should be cancelled if the rate query MAC CE contains rate query information associated with this triggered rate query. In addition, we think that the upper layer can also cancel the triggered rate query if the rate query is pending, e.g., when the upper layer has knowledge that the UL congestion is relieved.

Proposal 4: After the generation or transmission of the rate query MAC CE, the triggered rate query is cancelled.

Proposal 5: The upper layer can cancel the pending rate query.

Conclusion

In this contribution, we have discussed the details of the rate control MAC CE from gNB to UE, the potential format of the rate control MAC CE and whether to support the rate query MAC CE. We have the following proposals:

Proposal 1: Rate control MAC CE from the gNB to the UE indicates rate control information per DRB and QoS flows are indicated via RRC as in legacy.

Proposal 2: The rate control MAC CE indicates the rate control information of multiple DRBs/QoS flows.

Proposal 3: The working assumption is changed to agreements on rate query MAC CE.

Proposal 4: After the generation or transmission of the rate query MAC CE, the triggered rate query is cancelled.

Proposal 5: The upper layer can cancel the pending rate query.

References

- R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#128 meeting

- Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#129 meeting

- R2-2501508, Reply LS on XR UL Rate Control	RAN3, LS in, RAN2#129 meeting

- R2-2500056, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications, SA4, LS in, RAN2#129 meeting