3GPP TSG-RAN WG2 Meeting #129bis	R2-2502040

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.2

Source:	OPPO

Title:	Discussion on paging procedure for Ambient IoT

Document for:	Discussion, Decision

Introduction

In RAN2#129, we agreed that paging ID can be either device ID or group ID.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

and provide different ways to capture Msg1 resource(s) as follows:

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this discussion, we will delve further into how can different ways work and what the MAC PDU format will be.

Discussion

Initial paging message

For both Way-1 and Way-2, the initial message is included. Although the length of device ID has not been decided, one thing for sure is that the length would be relatively long to ensure uniqueness of the identifier. Consequently, when the paging message contains numerous device IDs for CBRA, its message size could exceed TB size. Currently, R2D segmentation is not supported. Therefore, we should allow multiple initial paging messages to be present before R2D transmission message that determines the Msg1 resource(s).

Proposal 1: In one paging round, multiple initial paging messages containing different device IDs to be paged, which contest for same access occasions, are allowed to be transmitted before the R2D trigger message that provides Msg1 resources.

Given that the device ID can have a variable length, the number of device IDs each initial paging messages can include may differ with a given TB size. Therefore, the number of paged devices in each initial paging messages can vary.

For CFRA, only one device ID will be included in the paging message. Since the paging message would be relatively short, there is no need to use separate initial paging message and R2D trigger message for CFRA.

Observation 1: For CFRA, only one device ID will be included in the paging message. Given the relatively short size of the message, there is no need to use separate initial paging message and R2D trigger message for CFRA.

Proposal 2: The R2D trigger message following the initial paging message applies only to CBRA.

Contents of paging message

In RAN2#129, it was agreed that a paging ID can be either a device ID or a group ID. The following agreements were reached:

- Agreements on MAC PDU format design

- 2. Support multiplexing of information for multiple devices in R2D message for msg2.  FFS others for multicast messages

Currently, paging messages containing multiple device IDs are supported. This raises the question of whether group IDs can also be multiplexed within the same paging message. There will be the scenarios where a group of devices and some individual devices share the same UL resources for access. If multiplexing is not possible, two separate messages linking to the same UL resources would need to be sent, which would increase signalling overhead compared to the multiplexing method.

If those individual devices can also be grouped (i.e., paged by group ID), it makes sense to multiplex different group IDs into a single paging message.

Proposal 3: Group IDs and/or device IDs can be multiplexed into a single paging message for CBRA.

According to the agreement from SA2, the procedures/messages supported by AIoT Device NAS layer do not include the inventory request, i.e., the paging message does not contain AIoT Device NAS layer data.

4. AioT Device NAS protocol is supported between the AioT Device and the AIOTF. The AioT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

From a reader perspective, if the paging ID coming from CN is a NAS ID (i.e., invisible to MAC layer), how could a reader without NAS layer associate it to multiple AS-ID would be an issue as raised in [POST129][035].

That means the paging ID should be visible to the MAC layer. If group IDs can be multiplexed with device IDs, there should be ways for devices to distinguish whether the paging ID is a group ID or a device ID. Given that device IDs may have variable lengths and that the exact format of group IDs is supposed to be designed by SA2, we may need some ways to make the distinction.

Proposal 4: Devices should have a way to differentiate between group ID and device ID.

For CFRA, it has been agreed that group ID is not supported.

Agreements - CFRA is not supported for group ID

Therefore in this WID, CFRA only considers the case that UL resource(s) is allocated for a single device, meaning only one device ID will be included in the paging message for CFRA. Consequently, the number of access occasions for CFRA is always 1. There is not need to explicitly state this in the MAC PDU. For CBRA, things change since at least two paging IDs will be included in the paging message.

Observation 2: For CFRA, only one device ID will be included in the paging message. In contrast, for CBRA, multiple device IDs can be included.

In CBRA, devices need to know the number of access occasions to randomly choose their access occasion within the following assigned UL resources. Without this information, devices have to count how many access occasions are attached in the paging messages, which increases the complexity and processing delay. What’s worse, if there are multiple subsequent trigger message used for providing Msg1 resources, a device has to count all the UL access occasions in every trigger messages. Therefore, it would be good if the paging message of CBRA can capture a field for number of access occasions.

Observation 3: Without a field for number of access occasions for CBRA, devices have to count how many access occasions are attached in the paging messages, adding complexity and processing delay.

Additionally, if there is a field for number of access occasions in CBRA but not in CFRA, it becomes natural for devices to distinguish whether the following random access is contention-based or contention-free.

Proposal 5: The paging message for CBRA includes a field for the number of access occasions, but such a field is not needed for CFRA.

For Way-2 of CBRA, it is clear that no paging ID will be included in the R2D trigger message. Therefore, the ID should be included in the initial paging message. However, for Way-1, there is some ambiguity about which message should carry the paging ID. In our understanding, the initial paging message shall trigger the A-IOT device to determine the resource index, and only the targeted A-IOT device shall do this job, so the initial paging message shall indicate which set of A-IOT devices are paged. However, the R2D trigger message is only for providing access occasions to them, so there is no need to include the paging ID in the R2D trigger message for Way-1.

Observation 4: Initial paging message and R2D trigger message provides different functionalities, i.e., the initial paging message shall trigger targeted A-IOT devices to determine the resource indices, and the R2D trigger message provides access occasion opportunities.

Proposal 6: If there are initial paging message(s) and R2D trigger message(s) in the paging round, for both Way-1 and Way-2, paging ID is only carried in the initial paging message.

Transaction ID

The “transaction ID” can be generated by reader based on CN correlation ID. Currently, we lack details on what the corelation ID will look like and how it can be mapped to a transaction ID. However, there are some general requirements for the transaction ID that should be adhered to:

Different service requests originating from the same reader need to use distinct transaction IDs to differentiate between services. Otherwise, devices may mistakenly treat the subsequent request as duplicates of the previous, leading to the discarding of new service request with identical transaction IDs. Similarly, when there is a same service request coming from different readers. If reader ID is not attached in the paging message, devices would be confused whether to respond to one or both requests. To address these concerns, it is proposed that

Proposal 7: The following principles of transaction ID need to be followed:

- Consecutive service requests from the same reader need to have different transaction IDs.

- When the same service request is sent from different readers and the reader ID is not included in the paging message, each request must use a distinct transaction ID.

- When the same service request is sent from different readers and the reader ID is included in the paging message, a same transaction ID can be used.

This approach ensures clear differentiation between service requests and prevents confusion within the network.

Conclusion

In this contribution, we have shared our simulation results on RRM measurement prediction and have the following observations and proposals:

Initial paging message:

Observation 1: For CFRA, only one device ID will be included in the paging message. Given the relatively short size of the message, there is no need to use separate initial paging message and R2D trigger message for CFRA.

Proposal 1: In one paging round, multiple initial paging messages containing different device IDs to be paged, which contest for same access occasions, are allowed to be transmitted before the R2D trigger that provides Msg1 resources.

Proposal 2: The trigger message following the initial paging message applies only to CBRA.

Contents of paging message:

Observation 2: For CFRA, only one device ID will be included in the paging message. In contrast, for CBRA, multiple device IDs can be included.

Observation 3: Without a field for number of access occasions for CBRA, devices have to count how many access occasions are attached in the paging messages, adding complexity and processing delay.

Observation 4: Initial paging message and R2D trigger message provides different functionalities, i.e., the initial paging message shall trigger targeted A-IOT devices to determine the resource indices, and the R2D trigger message provides access occasion opportunities.

Proposal 3: Group IDs and/or device IDs can be multiplexed into a single paging message for CBRA.

Proposal 4: Devices should have a way to differentiate between group ID and device ID.

Proposal 5: The paging message for CBRA includes a field for the number of access occasions, but such a field is not needed for CFRA.

Proposal 6: If there are initial paging message(s) and R2D trigger message(s) in the paging round, for both Way-1 and Way-2, paging ID is only carried in the initial paging message.

Transaction ID:

Proposal 7: The following principles of transaction ID need to be followed:

- Consecutive service requests from the same reader need to have different transaction IDs.

- When the same service request is sent from different readers and the reader ID is not included in the paging message, each request must use a distinct transaction ID.

- When the same service request is sent from different readers and the reader ID is included in the paging message, a same transaction ID can be used.