3GPP TSG-RAN WG2 Meeting #129bis	R2-2502045

Wuhan, China, April 7 – 11 2025

Agenda item:	8.6.2

Source:	Nokia

Title:	On remaining issues for inter-CU LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the baseline procedure for inter-CU LTM support and identify the related key issues.

# 2	Discussion

We analyze the following key issues related to Inter-CU LTM and provide our proposals on way forward on these issues in this paper.

- Security Key Management for Inter-CU LTM

  - Discussion on further RAN2 impacts based on SA3 LS Response on security key change.

  - Inter-CU MCG LTM and SK-counter configuration for key generation for SCG.

  - Inter-CU Fast recovery issues.

- Remaining issues related to non-DC LTM

  - UE based TA related configuration

  - Support for non-subsequent LTM

  - Shared usage of configured grant for Inter-CU LTM

- Remaining Inter-CU DC LTM issues

  - Analysis on MCG-LTM with SCG addition scenario

  - Inter-SN SCG LTM configuration structure and associated impacts for co-existence with MCG-LTM

## 2.1	Security Key Management for Inter CU LTM

### 2.1.1 RAN2 Impacts for SA3 proposed security key change solution.

SA3 has replied in [2] on the security handling for Inter-CU LTM. Following is the SA3 conclusion on the proposed solution,

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC lead to increased probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

In our view SA3 has reached compromise for transmission of NCC as plain text in MAC CE based on RA2 request for avoiding RRC Reconfiguration as possible option to achieve the same and does not preclude other options that RAN2 can consider without impacting the actual requirements of subsequent LTM. In this section we further analyse the possible options for RAN2 for security key handling for Inter-CU LTM.

In case of subsequent Inter-CU cell switch the LTM switching towards UE cannot be triggered until the following steps are completed.

- Path-switch procedure with AMF should be completed and Target node need to receive the new NCC value.

- The new K-GNB* needs to be generated and informed to other candidate cells for modification of Master key parameters and acknowledged from all the candidate GNBs.

Hence subsequent cell-switch with new NCC value cannot be delivered until the candidate configurations are updated with new security key values and these steps already introduce the delay which cannot be avoided in Inter-CU LTM.

Observation 1: Subsequent Inter-CU LTM cannot be triggered until all the candidate nodes are prepared with new security keys based on new NCC for subsequent LTM.

In the LS to SA3 RAN2 has clearly indicated that RRC-Reconfiguration with modified candidate configuration is to be avoided between cell switches NOT the RRC messages itself. Because the control plane and user plane interactions can continue until next cell switch. In case of Inter-CU LTM with bearer configuration mismatch during preparation additional RRC Reconfiguration is anyhow needed prior to triggering subsequent switching.

Observation 2:  RRC Reconfiguration and other RRC procedures are expected to continue after Inter-CU cell switch until subsequent Inter-CU LTM.

As the Inter-CU MCG LTM involves change of PDCP anchor point the network is not expected to trigger this subsequent switching immediately to minimize the frequent interruption. The NW may configure L3 measurements for Inter-CU candidates in such scenarios for more stable switching decision. In such scenarios, delivery of security key parameter via RRC message does not cause additional delay in subsequent switching.

Observation 2A:  NW may configure L3 measurement-based LTM for Inter-CU switching for stable switching to minimize frequent change of PDCP anchor. In these cases security key information transfer via RRC does not impact subsequent switching delay.

If the delivery of NCC value in CSC command in unprotected way is adapted the failure handling related to UE receiving wrong NCC value by intruder needs to be addressed in RAN2 signaling procedure. Mainly the failure on completion of cell switch due to mismatch of NCC and ways to recover from this impact.

Observation 3: Mismatch of NCC value on LTM switch execution possible due to security threat and impacts the LTM switching if NCC is delivered via unprotected MAC CE. Detection of such failure and recovery requires additional RAN2 investigation.

As part of path switch procedure, the AMF may generate new master-key and provide NAS-Container along with NH and NCC values. In this case the Master-key update for the remaining part anyhow needs to be delivered to UE via RRC signaling message. This message need not be RRC Reconfiguration. The UE can use these parameters for new key generation at the time of Inter-CU cell switching.

Observation 4: Delivery of Master-key update via RRC message is anyhow needed in Inter-CU LTM for other security key update scenarios of Inter-CU mobility. In this case delivery of security key information via RRC message prior to cell switch is unavoidable.

Above observations indicates that delivery of all the Master-key update parameters via RRC message does not bring additional delay for subsequent LTM. Moreover, this approach is more reliable and secured for other scenarios related to key change and future extensions for Inter CU C-LTM. Based on the above analysis delivery of all the parameters related to Master-key change via RRC message prior to LTM cell switch is better option for security key change for Inter-CU LTM.

Proposal 1: RAN2 to support delivery of master key update related parameters via RRC message prior to LTM switch.

### 2.1.2 Remaining issues for Inter-CU LTM Fast Recovery handling.

In RAN2#127bis, the following agreement was made:

RAN2#127bis agreement: The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

The SA3 LS proposes that cell-switching command is used to indicate the LTM security configuration to the UE. In case UE has received the security parameters from the agreed method, UE can use the received parameters for security update during LTM recovery procedure after RLF or L3 based HOF.

Proposal 2: The UE can use the configured inter-CU LTM security parameters for inter-CU LTM recovery triggered after RLF or L3 based HOF.

For the scenario of LTM recovery after LTM cell switch failure, we have the following scenarios,

- recovery to inter-CU LTM candidate after intra-CU LTM failure,

- recovery to inter-CU LTM candidate (with respect to the failed target and initial source cell) after inter-CU LTM failure,

- recovery to intra-CU LTM candidate (with respect to the failed target) after inter-CU LTM failure.

- recovery to intra-CU LTM candidate (with respect to initial source cell) after inter-CU LTM failure,

For scenario 1, UE can use the stored LTM security configuration. These configurations will be untouched by the UE as the failed configuration is intra-CU.

For scenario 2, the UE uses the LTM security configuration for the initial inter-CU LTM procedure that has failed, UE can re-use the stored LTM security configuration to generate the keys for the new candidate CU from scratch.

For scenario 3, the UE uses the LTM security configuration for the initial inter-CU LTM procedure that has failed. UE can re-use the same security configuration to generate the new key for the recovery target.

For scenario 4, UE can re-use the Rel. 18 procedure.

Proposal 2A: In case the LTM security configuration is configured from the current serving MN, the UE can re-use the LTM security configuration for inter-CU LTM recovery for all other cases.

Finally, if UE does LTM recovery before the security parameters are configured to the UE, the solution for scenario 1, 2 and 3 cannot work. For these scenarios there are two options.

- UE triggers re-establishment.

- UE is pre-configured to do horizontal key derivation as default behavior when the LTM security configuration is not available at the UE side.

Proposal 2B: RAN2 discuss and agree among the below options:

- Option 1: A default behavior for the UE to use horizontal key generation in case if new LTM security configuration is not received from current serving MN

- Option 2: A default behavior to trigger re-establishment when the LTM security configuration is not received from current serving MN for scenario 1, 2 and 3.

### 2.1.3 SK-counter configuration for Inter-MN MCG LTM with same SCG

The security key generation principles described in TS33.501 indicates that whenever master key associated with MCG is changed the network and UE need to regenerate new S-KgNB and the derived keys associated with SCG bearers.

In classic handover scenario, above requirement is achieved by MN including explicit SK-counter value in the RRC Reconfiguration message that includes master-key change or NCC change.

Observation 5: In classic Inter-MN handover scenario without SCG change, the SK-counter value needs to be included along with master-key change configuration to enable regeneration of new security keys for SN .

In case of Inter-MN MCG LTM with same SCG, whenever inter MN LTM is executed the UE and MN can apply the new K-GNB based on NCC value provided in the MAC CE command or based on stored NCC values. The actual solution is discussed in SA3. However, how the SN keys are regenerated in this scenario is not yet discussed in RAN2. This issue is not indicated as key issue in the LS to SA3.

Observation 6: Issues related to regeneration of SN-Keys during Inter-MN MCG LTM is not discussed within RAN2. Not included in the list of issues for SA3 to address in the previous LS to SA3.

There can be multiple solutions to enable regeneration of SN keys without additional RRC configuration. Some of the options are as follows:

- O1: LTM Configuration includes SK-counter value for each Rel-19 ID (the identifier to differentiate Inter-MN MCG LTM). Whenever inter MN MCG LTM is executed both UE and MN make use of this SK-counter for generation of SN keys. One SK counter value is sufficient to handle multiple MN changes because the master key is changed in this case and same SK-counter value can be reused. O2: Without explicit SK-counter value, for inter-MN LTM with SCG, the UE and MN should regenerate new SN-keys with default SK-counter value =1. For further inter-SN change without MN change, MN should use the value starting from 2. In this case LTM with SCG unchanged the LTM candidate configuration need not include any SK-counter value.

Proposal 3:  RAN2 discuss and agree upon the potential options to trigger SN key regeneration for Inter-MN MCG-LTM

## 2.2	Remaining Issues for non-DC LTM

### 2.2.1	UE-based TA estimation support for inter-CU LTM

In RAN2#125bis, the following was agreed

1. Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. 2. PDCCH ordered early RACH is supported for inter-CU LTM.

In their LS (R4-2314455) RAN4 informed the other WGs that UE-based TA measurement can be supported in Rel. 18. From RAN2 viewpoint, UE knows that it is configured with UE-based TA measurements if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. Moreover, in case of RACH-less LTM, if the LTM CSC does not give a valid TA value, and UE is configured with UE-based TA measurement for the target cell, UE can use the TA value it has measured itself,

Observation 7: UE-based TA measurement is supported for Rel. 18 intra-CU LTM and can be used by the UE during RACH-less when the LTM CSC does not provide a valid TA value.

As Rel-18 only supports Intra-CU LTM, the serving CU determines the UE-based TA estimation configuration based on its serving cell.  According to the RAN4 LS (R4-2314455), the support for UE-based TA measurement is dependent on TAE, DL timing estimation error (of both serving and target cell), serving cell TA resolution and TA adjustment error.  For Inter-CU LTM in Rel-19 if this field needs to be extended across all the candidates across different CU, additional coordination will be required to exchange the information related to possibility of TA-estimation across Inter-CU cells.  Depending on support of such coordination NW may decide to restrict the UE-based TA within same CU or extend it across CU

Proposal 4: RAN2 to discuss how the UE-based-TA-measurement-ID is coordinated across Inter-CU candidate cells based on the knowledge of cells of candidate CU that can be considered for UA-based TA estimation.

### 2.2.2	Support for Non-Subsequent LTM

In RAN2#125bis the following were agreed with respect to subsequent LTM

Agreements on scenarios: - RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios. - Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. - UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

The Inter-CU LTM involves security key change and also PDCP re-establishment as additional steps. Subsequent LTM is beneficial for fast switching without any higher layer impacts. Usage of subsequent LTM in Inter-CU is required for cases where a high number of ping-pongs is expected. For instance, consider generic movement, wherein the UE may move between different cells without any pattern. In this case, the user may or may not go back to the same cell. In such cases, preparing multiple candidates would likely lead to inefficient resource usage.

However, in some scenarios the NW may need to support one time mobility towards some candidates and subsequent mobility across other candidates. This case is similar to support of CPAC and subsequent CPAC together in single configuration. In such cases non-subsequent LTM towards selected candidate is needed to avoid maintaining the other configurations for which no further subsequent mobility is expected.  This support becomes more relevant for Inter-CU LTM where there could be some issues in candidate CU supporting reference configurations and exchange of information related to Early-sync procedures. It is better to configure non-subsequent mobility towards these candidate cells

Observation 8:  In some Inter-CU LTM scenarios subsequent mobility is not needed at some candidate cells. Release of configuration of other candidates is preferred in this case.

Proposal 5:  Non-subsequent LTM towards selected candidates is supported for Inter-CU LTM.

### 2.2.3	Support of Configured Grant-based RACH-less LTM

In RAN2#126, the following agreement was made on the support of the first UL transmission for RACH-less inter-CU LTM

RAN2#126 Agreement: Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

Compared to dynamic grants (DGs), configured grants (CGs) enable faster access to the target cell, as the UE does not need to monitor PDCCH to receive the UL grant and can directly perform its first UL transmission on one of the valid occasions of the provided CG.

Nevertheless, a known issue of the use of CGs in LTM is the potential excessive resource reservation to support a single RACH-less LTM switch. For each UE prepared for RACH-less LTM and for each candidate cell, a CG must be included in the candidate’s RRC configuration provided to the UE during the preparation phase. Therefore, the PUSCH resources of these grants are reserved for a long period of time stretching from the preparation phase to the execution of one or multiple cell switches and they may never be used, if the UE does not switch to the corresponding cell. The issue may be even more pronounced in inter-CU LTM, as it cannot be solved or mitigated with intra-gNB resource scheduling optimizations.

Observation 9: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

. To optimise the PUSCH resource reservation issue,  the probabilistic nature of the use of the provided PUSCH resources can be exploited in a similar way as it is done for CFRA resources. PUSCH resources can be shared by multiple UEs, as it is unlikely that they switch to the same candidate cell at the same time. Still, it is possible that UEs sharing the same PUSCH resource will use the same CG occasion for their first UL transmission to the candidate. Therefore, coordination of the use of shared PUSCH resources is required. As cell switch execution is controlled by the source node, shared PUSCH resources can be provided by the candidate node to the source, which in turn can coordinate their use during cell switch.

Proposal 6: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

## 2.3	Remaining issues for DC LTM

### 2.3.1		Discussion on RRC running CR

In the latest version of RRC running CR, the following topics in our view needs further discussion:

- When LTM-Config for Inter-SN SCG is received through LTM-Config-NRDC where the UE maintains this configuration for further LTM procedures including early TA and switching needs further clarification.

- The reference configuration and candidate configurations handling for Inter-CU SCG LTM also requires further clarification.

Handling of ltm-ConfigNRDC at the UE

In Rel-18 LTM, for NR-DC, UE receives two independent ltm-Config, one associated with MCG and the other associated with SCG, and UE maintains two independent variables to store ltm-Config associated with MCG and SCG.

One approach could be that the ltm-ConfigNRDC may be associated with MCG as it is signalled via RRCReconfiguration message received via SRB1. Furthermore, the inter-CU SCG LTM configuration must have both MCG and SCG parts. This would be similar to Rel-18 S-CPAC.

In another approach, the LTM-config-SCG received via LTM-Config-NRDC can be maintained as SCG-Config for Secondary cell group as this parameter contains other parts that are relevant for LTM switching within SCG

In an alternate approach, the inter-CU SCG LTM configuration received in ltm-Config-SCG--r19 can be stored in a new variable different from MCG and SCG. This approach may be considered for the co-existence of intra-CU and inter-CU SCG LTM.

Proposal 7A: RAN2 to discuss clarify how the LTM-Config-SCG received in LTM-Config-NRDC is maintained at UE.

Reference configuration for inter-CU SCG LTM

RAN2 made the following agreements related to reference configuration and candidate configuration for Inter-CU SCG-LTM:

The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

In the case of inter-CU SCG LTM, whether and how to include the reference configuration for MCG was an FFS. In the RRC running CR, it is unclear how the reference configuration for MCG may constructed and signalled to the UE.

Proposal 7B: RAN2 to clarify that the reference configuration and candidate configuration for Inter-CG SCG LTM always include MCG part and update running CR accordingly.

### 2.3.2 	Inter-CU DC LTM with unchanged MCG/SCG scenarios

In the case of inter-CU SCG LTM, it is likely that the SCG configuration (related to bearers mapped to the SCG resources) prepared by all or some of the candidate SNs may be identical to the SCG configuration at the source; in such a case, the MCG part of the inter-CU SCG LTM candidate configuration prepared by the source MN shall have identical MCG configuration. When MCG configuration is identical across source and target configurations the UE handling of MCG related configuration during LTM switch is unnecessary. These operations includes release of current dedicated configurations such as release of secondary cells and also default MAC configuration. If the UE is aware of the information about two candidate configurations have same MCG these steps can be skipped.

Observation 10: In the case of inter-CU SCG LTM, it is likely that in many scenarios the SCG configuration prepared by all or some of the candidate SNs may have identical SCG configuration. In such a case, the MCG part of the inter-CU SCG LTM candidate configuration prepared by the source MN remains unchanged similar to Rel-18 SCG LTM.

Proposal 8A:  RAN2 to discuss the inter-CU SCG LTM scenario with identical SCG configuration and agree to have a new indication in ltm candidate configuration to enable the UE to avoid replacing the current MCG configuration during inter-CU SCG LTM cell switch.

RAN2 made the following agreement in #129:

The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

As per the above agreement, even though SCG configuration is same across different candidate configuration of MCG-LTM, the SCG configuration is expected to be included in the MRDC-SecondaryCellGroupConfig IE, and the candidate configuration shall include MRDC-SecondaryCellGroupConfig ReleaseAndAdd; which results SCG release procedure followed by SCG addition procedure in these scenarios eventhough SCG is not at all affected between these two candidate configurations.

This issue can be avoided if the NW signals a new indication in ltm candidate configuration to enable the UE to decide on whether to follow the release and add procedure for SCG during switching or ignore this parameter.

Observation 11: For the intra-CU MCG LTM with SCG unchanged, MRDC-SecondaryCellGroupConfig ReleaseAndAdd need not have to be performed.

Proposal 8B: RAN2 to discuss the intra-CU MCG LTM scenario with SCG unchanged and agree on introducing a new indication in ltm candidate configuration to avoid SCG release and add procedure for SCG unchanged scenario

### 2.3.3	Inter-CU MCG LTM for candidate configurations with different SCG

RAN2 made the following agreement related to SCG Addition in the case of inter-CU MCG LTM.

[RAN2#128]: RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

When there are multiple candidate PSCells prepared associated with a candidate inter-MN target PCell, then an inter-CU MCG LTM execution which may result in SCG or PSCell addition (after an earlier SCG release) may result in a blind PSCell addition, which may potentially result in RL failure of UE’s link to the PSCell or sub-optimal DC throughput. In this case if the switching decision is only based L1 measurements of PCell the DU may need to choose between two configurations having same PCell but different PSCell blindly.

To enable the DU to choose the right candidate configuration additional coordination related to PSCell L3 measurements will be required. These impacts of MCG-LTM with SCG addition were not yet analyzed completely in RAN2. We propose further discussion on this scenario and required signalling changes.

Proposal 9: RAN2 to discuss signalling procedure to handle blind PSCell addition and PSCell change when two candidate LTM configuration have same MCG(PCell) but different SCG(PScell).

# 3	Conclusion

In this paper, we analyzed the remaining open issues for Inter-CU LTM and made the following observations and proposals.

Remaining issues for security key handling

Observation 1: Subsequent Inter-CU LTM cannot be triggered until all the candidate nodes are prepared with new security keys based on new NCC for subsequent LTM.

Observation 2:  RRC Reconfiguration and other RRC procedures are expected to continue after Inter-CU cell switch until subsequent Inter-CU LTM.

Observation 2A:  NW may configure L3 measurement-based LTM for Inter-CU switching for stable switching to minimize frequent change of PDCP anchor. In these cases security key information transfer via RRC does not impact subsequent switching delay.

Observation 3: Mismatch of NCC value on LTM switch execution possible due to security threat and impacts the LTM switching if NCC is delivered via unprotected MAC CE. Detection of such failure and recovery requires additional RAN2 investigation.

Observation 4: Delivery of Master-key update via RRC message is anyhow needed in Inter-CU LTM for other security key update scenarios of Inter-CU mobility. In this case delivery of security key information via RRC message prior to cell switch is unavoidable.

Proposal 1: RAN2 to support delivery of master key update related parameters via RRC message prior to LTM switch.

Proposal 2: The UE can use the configured inter-CU LTM security parameters for inter-CU LTM recovery triggered after RLF or L3 based HOF.

Proposal 2A: In case the LTM security configuration is configured from the current serving MN, the UE can re-use the LTM security configuration for inter-CU LTM recovery for all other cases.

Proposal 2B: RAN2 discuss and agree among the below options:

- Option 1: A default behavior for the UE to use horizontal key generation in case if new LTM security configuration is not received from current serving MN

- Option 2: A default behavior to trigger re-establishment when the LTM security configuration is not received from current serving MN for scenario 1, 2 and 3.

Observation 5: In classic Inter-MN handover scenario without SCG change, the SK-counter value needs to be included along with master-key change configuration to enable regeneration of new security keys for SN .

Observation 6: Issues related to regeneration of SN-Keys during Inter-MN MCG LTM is not discussed within RAN2. Not included in the list of issues for SA3 to address in the previous LS to SA3.

Proposal 3:  RAN2 discuss and agree upon the potential options to trigger SN key regeneration for Inter-MN MCG-LTM

Remaining Issues for Non-DC LTM

Observation 5: In classic Inter-MN handover scenario without SCG change, the SK-counter value needs to be included along with master-key change configuration to enable regeneration of new security keys for SN .

Observation 6: Issues related to regeneration of SN-Keys during Inter-MN MCG LTM is not discussed within RAN2. Not included in the list of issues for SA3 to address in the previous LS to SA3.

Proposal 3:  RAN2 discuss and agree upon the potential options to trigger SN key regeneration for Inter-MN MCG-LTM without SCG change.

Observation 7: UE-based TA measurement is supported for Rel. 18 intra-CU LTM and can be used by the UE during RACH-less when the LTM CSC does not provide a valid TA value.

Proposal 4: RAN2 to discuss how the UE-based-TA-measurement-ID is coordinated across Inter-CU candidate cells based on the knowledge of cells of candidate CU that can be considered for UA-based TA estimation.

Observation 8:  In some Inter-CU LTM scenarios subsequent mobility is not needed at some candidate cells. Release of configuration of other candidates is preferred in this case.

Proposal 5:  Non-subsequent LTM towards selected candidates is supported for Inter-CU LTM.

Observation 9: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

Proposal 6: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

Remaining Issues for DC-LTM

Proposal 7A: RAN2 to discuss clarify how the LTM-Config-SCG received in LTM-Config-NRDC is maintained at UE.

Proposal 7B: RAN2 to clarify that the reference configuration and candidate configuration for Inter-CG SCG LTM always include MCG part and update running CR accordingly.

Observation 10: In the case of inter-CU SCG LTM, it is likely that in many scenarios the SCG configuration prepared by all or some of the candidate SNs may have identical SCG configuration. In such a case, the MCG part of the inter-CU SCG LTM candidate configuration prepared by the source MN remains unchanged similar to Rel-18 SCG LTM.

Proposal 8A:  RAN2 to discuss the inter-CU SCG LTM scenario with identical SCG configuration and agree to have a new indication in ltm candidate configuration to enable the UE to avoid replacing the current MCG configuration during inter-CU SCG LTM cell switch.

Observation 11: For the intra-CU MCG LTM with SCG unchanged, MRDC-SecondaryCellGroupConfig ReleaseAndAdd need not have to be performed.

Proposal 8B: RAN2 to discuss the intra-CU MCG LTM scenario with SCG unchanged and agree on introducing a new indication in ltm candidate configuration to avoid SCG release and add procedure for SCG unchanged scenario.

Proposal 9: RAN2 to discuss signalling procedure to handle blind PSCell addition and PSCell change when two candidate LTM configuration have same MCG(PCell) but different SCG(PScell).