3GPP TSG-RAN WG2 Meeting #129-bis	 R2-2502054

Wuhan, China, April 7th – 11st, 2025

Source:	CATT

Title:	Further consideration on UL capacity enhancements

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

Based on the agreements made in previous meetings [1][2], in this document we further discuss the remaining issues on UL capacity enhancements in IoT NTN, with our observations and proposals provided.

Discussion

Terminology

In order to facilitate discussion in this document, the following terminologies are used for CB-Msg3 EDT in this document.

- CB-Msg3-EDT Occasion: the time occasion to transmit CB-Msg3. In the time domain, it corresponds to UL transmissions in N consecutive UL slots with N = NRepNslots, where NRep is the repetition number which can be determined by the CE level (NRep=2 in Figure 1), and Nslots is the length of basic resource unit, e.g. slot number within one RU in NB-IoT. In frequency domain, the maximum frequency resource number is 4 in Figure 1.

- Replica: a number of consecutive transmissions within a CB-Msg3-EDT occasion according to the CE level. The number of the consecutive transmissions is NRep. Each replica covers consecutive resources in time domain for repetitions (as determined by RSRP for intended CE level).

- CB-Msg3 window: the timer duration in which the UE monitors PDCCH for network response for CB-Msg3.

- CB-Msg3-RNTI: The RNTI used to scramble PDCCH to schedule network response for CB-Msg3.

Figure 1: Example for illustration of CB-Msg3-EDT occasion

Issues of CB-Msg3 EDT transmission

OCC resources

The following agreements about CB-Msg3 resources were made in RAN2#129: [2]

RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - - Frequency domain resources for (N)PUSCH occasions - - repetition number - - (N)PDCCH resource - - MCS RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

We think non-trivial RAN1 workload is needed to evaluate RSRP ranges that enable OCC during CB-msg3 procedure and to design the resources configuration respectively for CB-msg3 using and not using OCC, as discussed in last meeting.

Note that RAN2 had already tasked RAN1 on a number of discussions for this CB-Msg3 EDT design which is however not an objective with RAN1 as a responsible WG as per the WID. Therefore, we do not think asking RAN1 to do even more job is a feasible way, and thus do not want to further pursue the CB-Msg3 EDT with OCC in Rel-19.

Proposal 1: For CB-msg3 (N)PUSCH, OCC solution will not be applied in Rel-19.

Maximum TBS definition

In RAN2#128 meeting, the following agreement about maximum TBS was reached, with the FFS left regarding how to define a maximum TBS for CB-Msg3 EDT procedure trigger: [1]

The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

Different CE levels imply different channel quality, which may require different repetition number, MCS and also different TBS. Intuitively, the maximum TBS that can be supported at different CE level are typically different. Therefore, it seems more reasonable to have per CE level maximum TBS levels as the trigger condition for the UE at different CE levels. This per CE level maximum TBS level can simply be included as one of the parameters in CB-Msg3 EDT resources configurations which are also concluded as per CE level.

If the per CE level maximum TBS is agreed, then the UE, when there is available data for transmission, may need to first determine the current CE level. If the available data volume is within the maximum TBS associated with the current CE level determined, CB-Msg3 EDT procedure is triggered.

Proposal 2a: For CB-msg3 (N)PUSCH, different maximum TBS values are configured for different CE levels.

Proposal 2b: RAN2 discusses the specific CB-Msg3 EDT trigger condition, if maximum TBS is configured per CE level as in Proposal 2a (e.g. UE first selects CE level, and then compare data volume with the associated maximum TBS).

Issues of Msg4 reception

The design of CB-Msg3 RNTI

The following working assumption for DSA transmission: [2]

Working assumption: One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

Generally speaking, the network does not know which DSA occasions have been used by UE for CB-Msg3 replicas' transmission. So even with the working assumption that multiple UEs' response can be multiplexed into one CB-Msg4, it is not intended to multiplex the response for the UEs whose replicas are transmitted on different time resources, because otherwise some of the UE's response may have to be delayed. This is to say that the CB-Msg3-RNTI should be computed for different (D)SA occasions in time domain respectively, meaning that at least the time domain information (including repetition information) should be used for CB-Msg3 RNTI calculation.

Proposal 3a: CB-Msg3 RNTI should be calculated for each replica respectively in CB-Msg3 EDT, and at least the time domain information of each replica should be used for CB-Msg3 RNTI calculation.

The working assumption was made to reduce the PDCCH monitoring, i.e. the response to the CB-Msg3 replicas on different DSA occasions with the same time resource but different frequency resources can be included in the same CB-Msg4. Therefore, depending on whether the working assumption is confirmed or not, RAN2 discusses whether the frequency domain information is needed for CB-Msg3 RNTI calculation or not.

Proposal 3b: RAN2 discusses whether the frequency domain information is necessary to calculate CB-Msg3 RNTI (pending the confirmation of WA for Msg4 multiplexing).

The determination of monitoring window

For the response reception (PDCCH for CB-Msg4 monitoring), the following agreements were made: [1]

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

Firstly, we think for the DSA case, the UE should also start a window for response reception taking UE-eNB RTT into account, after the end of all repetition of each CB-Msg3 replica transmission. This can enable a unified design with the SA case for simplicity.

Proposal 4: For DSA case, after the end of all repetition of each CB-Msg3 replica transmission, UE starts a window for response reception taking UE-eNB RTT and potential the processing time into account.

For DSA case of multiple replica transmission, it is still FFS on whether one or multiple (i.e. one for each replica) PDCCH monitoring window(s) will be used for PDCCH monitoring. The following two options can be considered:

Option 1: Multiple windows: CB-Msg3 window is started for each Replica transmission respectively

In this option, the UE monitors PDCCH scrambled by the CB-Msg3-RNTI calculated according to the corresponding replica per replica, which is shown in Figure 2.

Figure 2: CB-Msg3 window running in Option 1

Option 2: One window: the UE starts only one CB-Msg3 window for all the replicas.

In this option, only one window is maintained, which is shown in Figure 3. Since there is no boundary of each replica for PDCCH monitoring, when the CB-Msg3-window overlaps, the UE has to monitor the CB-Msg3-RNTIs for all the replicas. This brings extra complexity to the procedure of the "blind" PDCCH detection.

Figure 3: CB-Msg3 window running in Option 2

Comparing these 2 options, it can be found that Option 1 is friendlier to blind detection procedure, especially when the CB-Msg3 windows overlap. Therefore, it is proposed to handle the window for NW response as follows.

Proposal 5: A CB-Msg3 window is started for each Replica transmission respectively, and the UE only monitors PDCCH scrambled with the CB-Msg3 RNTI for the corresponding replica within the CB-Msg3 window.

The issue of contention resolution

In CB-Msg3 EDT, if the UE selects the same PUSCH/NPUSCH resource, there is no way to distinguish them except for contention resolution. It has been agreed that broadcast signaling is used to deliver the CB-Msg3 configuration. In this case, the UE could calculate CB-Msg3 RNTI based on the resource associated with the PUSCH occasion used for CB-Msg3 EDT transmission. Similar to legacy Msg4 reception, the UE monitors the PDCCH scrambled by the CB-Msg3 RNTI. Then, in order for contention resolution, a CB-Msg3 contention resolution MAC CE should be introduced and included in the DL message as a NW response to the UE (especially if UEs transmit Msg3 on the same DSA occasion but using different frequency resources). If the CB-Msg3 contention resolution MAC CE corresponds to the beginning part of the UL message (e.g. first 48 bits), the UE will consider the contention resolution for this CB-Msg3 transmission successful.

We think the 48-bit contention resolution identity is long enough to identify UE exclusively, with no need to consider any other information (e.g. the frequency information of the DSA occasion for a CB-Msg3 replica), even though CB-Msg4 multiplexing is supported.

Proposal 6: For CB-Msg3 EDT, the similar mechanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3 by the UE. Not any additional information (e.g. frequency resource information) is needed.

Failure handling

The UE may not receive any response from the network after the CB-Msg3 transmission, because of, for example, the collision of the CB-Msg3 transmission, the lack of transmission power, the failure of Msg4 reception, etc. For simplicity, if the CB-Msg3 EDT fails, the RACH-like failure handling mechanism can be considered, for example: the UE could make another CB-Msg3 transmission attempt(s), fallback to RA-EDT or declare CB-Msg3 transmission failure to upper layers.

As a result, the network may need to configure the maximum CB-Msg3 attempt number for each CE level, with the UE selecting SA or DSA occasions within each CB-Msg3 EDT attempt. Another issue needs to be discussed is that, if the CB-Msg3 EDT procedure still fails after all the attempts, the UE will fall back to RA-EDT, or just declare CB-Msg3 transmission failure to upper layers, or can select one of them based on the network configuration.

Therefore, we propose that RAN2 discusses how to perform CB-Msg3 EDT failure handling, taking possible ways shown above into account, as well as the NW configurations/trigger conditions to enable them.

Proposal 7: RAN2 discusses how to deal with the CB-Msg3 EDT failure, considering the following possible ways and corresponding NW configuration/trigger conditions to enable them: trigger of another CB-Msg3 transmission attempt, fallback to RA-EDT and/or declaration of CB-Msg3 EDT failure to upper layers.

- 3.	Conclusion

In this document, we analyse the potential impacts for UL capacity enhancements in IoT NTN, and provide the following proposals:

Proposal 1: For CB-msg3 (N)PUSCH, OCC solution will not be applied in Rel-19.

Proposal 2a: For CB-msg3 (N)PUSCH, different maximum TBS values are configured for different CE levels.

Proposal 2b: RAN2 discusses the specific CB-Msg3 EDT trigger condition, if maximum TBS is configured per CE level as in Proposal 2a (e.g. UE first selects CE level, and then compare data volume with the associated maximum TBS).

Proposal 3a: CB-Msg3 RNTI should be calculated for each replica respectively in CB-Msg3 EDT, and at least the time domain information of each replica should be used for CB-Msg3 RNTI calculation.

Proposal 3b: RAN2 discusses whether the frequency domain information is necessary to calculate CB-Msg3 RNTI (pending the confirmation of WA for Msg4 multiplexing).

Proposal 4: For DSA case, after the end of all repetition of each CB-Msg3 replica transmission, UE starts a window for response reception taking UE-eNB RTT and potential the processing time into account.

Proposal 5: A CB-Msg3 window is started for each Replica transmission respectively, and the UE only monitors PDCCH scrambled with the CB-Msg3 RNTI for the corresponding replica within the CB-Msg3 window.

Proposal 6: For CB-Msg3 EDT, the similar mechanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3 by the UE. Not any additional information (e.g. frequency resource information) is needed.

Proposal 7: RAN2 discusses how to deal with the CB-Msg3 EDT failure, considering the following possible ways and corresponding NW configuration/trigger conditions to enable them: trigger of another CB-Msg3 transmission attempt, fallback to RA-EDT and/or declaration of CB-Msg3 EDT failure to upper layers.

- 4.	Reference

- R2-2410913	Report from Break-out session on NR-NTN and IoT-NTN	Session Chair (ZTE Corporation)

- R2-2501333	Report from Break-out session on NR-NTN and IoT-NTN 	Session Chair (ZTE Corporation)