3GPP TSG-RAN WG2 Meeting #129bis	R2-2502066

Wuhan, China, Apr. 7th – 11th, 2025

Source: 	Xiaomi

Title:  	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM), as shown below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, we provide our understandings on inter-CU LTM.

Discussion

  - Security issues for MCG LTM

To support subsequent LTM for the inter-CU LTM, how to handle the security key upon cell switch needs to be discussed, as captured in the WID [1]. In the previous meeting, RAN2 has sent an LS [2] to SA3, including many potential solutions as follows: Option 1 to Option 4, and sent an LS [3] to SA3, to declare RAN2 thinks avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection.

Option 1: Use a new information in MAC CE to deliver the security information. Whether the UE uses horizontal or vertical key derivation is derived from this new information in MAC CE (which is currently, neither integrity protected nor ciphered). Option 1A: the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE. Option 1B: the UE is preconfigured with a list of NCC values in a ciphered and integrity protected RRC message and the index of an NCC value in the list is included in the LTM cell switch command MAC CE. Option 2: Similar to Rel-18 S-CPAC key update mechanism, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling (that is both integrity protected and ciphered). It is expected that the participating gNBs (CUs) would need to be aware of the list and how the UE applies the list during LTM cell switches: Option 2A: UE chooses the first unused NCC for the target CU upon inter-CU LTM execution. Option 2B: As an alternative to choosing the next unused NCC (as in option 2A), horizontal key derivation is used in this option if the LTM cell switch is between the same two CUs. Option 3: After the execution of inter-CU LTM cell switch, the participating gNBs are expected to be updated with new K-gNB* to be used for the next inter-CU LTM cell switch. The UE and CN are aware of how the UE would use the next NCC value. Option 3A: The UE determines the following NCC value to use by itself (e.g., increase by 1) after subsequent inter-CU LTM execution. Option 3B: UE is pre-configured by the CN (via source gNB RRC signalling) with a list of NCC values and the UE chooses the first unused NCC value as the next NCC value. Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, the Reply LS from SA3 [4] indicates the NCC can be transmitted to the UE in plaintext via MAC CE message, as follows:

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

According to the LS from SA3 [4], Option 1A can be considered without protection. And, RAN2 can confirm the method for the sake of completing WI.

Proposal 1: RAN2 confirms the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE.

Whether to introduce a new MAC CE for inter-CU LTM CSC

If P1 is agreed, the first open issue is whether to introduce a new MAC CE for inter-CU LTM CSC.

In 3GPP TS 38.321 #### 6.1.3.75 LTM Cell Switch Command MAC CE The LTM Cell Switch Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields (Figure 6.1.3.75-1): - R: Reserved bit, set to 0; …… - TCI state ID: This field indicates and activates the TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The TCI state is identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList as specified in TS 38.331 [5]. If the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is joint, this field is for joint TCI state, otherwise, this field is for downlink TCI state. The length of the field is 7 bits; - UL TCI state ID: This field indicates and activates the uplink TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The UL TCI state is identified by TCI-UL-StateId in ltm-UL-TCI-StateToAddModList as specified in TS 38.331 [5]. The octet containing this field (i.e. this field and the two reserved bits in the same octet) is included if the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is separate. The length of the field is 6 bits; - C: This field indicates the presence of the contention-free Random Access Resources fields. If the value of this field is set to 1, the following fields are present: Random Access Preamble index field, S/U field, SS/PBCH index field, PRACH Mask index field, Repetition number field and the reserved bits in the same octet. If the value of this field is set to 0, these fields are absent. …… Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

In 38.321, only one R bit in Oct 3 (highlighted in red) is always present in Rel-18 LTM CSC MAC CE. Other R bits in Oct4 and Oct7 (highlighted in green) are absent under certain conditions:

- If the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is joint, the two reserved bits in Oct4 are absent.

- If the value of the C is set to 0, the 5 reserved bits in Oct7 are absent.

Consequently, only one R bit in Oct 3 (highlighted in red) can be used to configure the NCC value. Since at least 3 bits are needed for NCC, one R bit in LTM CSC MAC CE is not enough. To include the NCC value in LTM CSC MAC CE, an extra octet (e.g., Oct 8) is needed. Hence, we support to introduce a new MAC CE (Enhanced LTM Cell Switch Command MAC CE) including NCC value for inter-CU MCG LTM, and the Figure 1 shows an example of Enhanced LTM Cell Switch Command MAC CE.

Figure 1: Enhanced LTM Cell Switch Command MAC CE

For intra-CU MCG LTM and intra/inter-CU SCG LTM, the Rel-18 LTM Cell Switch Command MAC CE can be reused.

Proposal 2: Introduce a new MAC CE (Enhanced LTM Cell Switch Command MAC CE) including NCC value for inter-CU MCG LTM.

Whether to revise Rel-19 ID for security key update

Agreement-RAN2#127 6. If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

In the previous meeting, RAN2 has agreed to use Rel-19 ID to indicate whether to perform PDCP re-establishment and security key update. However, SA3 has reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, and NCC value in LTM CSC MAC CE can also implicitly indicate the UE to perform security key update. Rel-19 ID is redundant for security key update.

Nonetheless, as shown in section 2.2, Rel-19 ID can be used for LTM based failure recovery to distinguish intra-CU LTM and inter-CU LTM. And Rel-19 ID is useful even if NCC value in LTM CSC is supported. In addition, many L2 reset procedures are agreed and captured in running CR based on the Rel-19 ID. If we revised the previous agreement, it will result in extra work. Hence, it is not needed to revisit the previous agreements and Rel-19 ID in RRC is used to indicate whether to update the security key.

Observation 1: It is not needed to revisit the previous agreements and Rel-19 ID in RRC is used to indicate whether to update the security key.

To avoid the misalignment, the network shall ensure the LTM CSC always aligns with the Rel-19 ID in RRC: i.e., if the Rel-19 ID is different for the source cell and the target cell, the network shall send the LTM CSC with NCC to UE; otherwise, the network shall send the LTM CSC without NCC to UE.

Proposal 3: For MCG LTM, the network shall ensure the LTM CSC always aligns with the Rel-19 ID in RRC:

- i.e., if the Rel-19 ID is different for the source cell and the target cell, the network shall send the LTM CSC with NCC to UE; otherwise, the network shall send the LTM CSC without NCC to UE.

  - LTM based failure recovery

In previous meetings, LTM based fast failure recovery has been discussed, with the following agreement:

RAN2#127bis meeting 1. The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs). RAN2 #128 meeting RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

The Rel-18 LTM based failure recovery can also be used for both radio link failure and MCG LTM failure, as specified in TS 38.331：

3GPP TS 38.331 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

In Rel-19, the following 7 cases can be discussed for LTM based failure recovery in the table 1, and we provide 3 types of failure, including RLF, intra-CU LTM failure and inter-CU LTM failure and provide three types of selected cell, including the selected cell from source/serving CU (CU1), the selected cell from target CU (CU2) and the selected cell from non-source/serving CU and non-target CU (CU3).

Table 1: Rel-19 LTM based failure recovery cases

| Failure type: Selected cell for recovery: | RLF (serving cell CU1 Cell) | Intra-CU LTM failure (source CU1 cell ->target CU1 cell*) | Inter-CU LTM failure (source CU1 cell 0->target CU2 cell*) |
| --- | --- | --- | --- |
| Selected CU1 cell** (selects an intra-CU candidate cell) | Case 1 | Case 2 (Already supported) | Case 3 |
| Selected CU2 cell** (selects an inter-CU candidate cell) |  |  | Case 4 |
| Selected CU3 cell** (selects an inter-CU candidate cell) (CU3: non-source/serving CU and non-target CU) | Case 5 | Case 6 | Case 7 |

For Case1：

Case 1 has been supported in Rel-18 LTM. In Rel-19, Case 1 can also be considered, if the UE selects an intra-CU LTM candidate cell after radio link failure of the MCG, the Rel-18 handling on failure can also be reused.

Observation 2a: The Rel-18 handling on failure can be reused in Rel-19 for case 1.

For Case 2:

Case 2 has been confirmed in RAN2#127bis meeting: The Rel-18 handling on failure is reused in Rel-19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure.

For Case 3~4:

Figure 2a: Case 3 of LTM based failure recovery

Figure 2b: Case 4 of LTM based failure recovery

The Figure 2a and Figure 2b show the Case 3 and Case 4 separately. The difference between Case 3 and Case 4 is whether the selected candidate cell is from the source CU1 or target CU2.

- For Case 3, the selected LTM candidate cell** for recovery can reuse the security key of source cell.

- For Case 4, the selected LTM candidate cell** for recovery can reuse the security key of targer cell, which is generated based on the NCC in LTM CSC.

In 3GPP TS 38.331 5.3.5.8.3 T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure) The UE shall: …… 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6: 4> revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG; 3> else: 4> revert back to the UE configuration used in the source PCell;

In TS 38.331, upon detection an LTM Failure, the UE shall revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG. In other words, after LTM failure, the UE has reverted back to the security key of source cell (CU1) and the Rel-19 ID stored in VarLTM-ServingCellNoSecurityChange is reverted back to the Rel-19 ID of source cell (CU1). For Case 3 and Case 4, after inter-CU LTM Failure, the UE maintains the PDCP state variables for SRB(s) associated to the failed target cell.

For Case 3, the security key of source cell (CU1) can be used, but the PDCP state variables for SRB(s) need to continue from the status of source cell rather than the failed target cell. Hence, the Rel-18 handling on failure cannot be reused in Rel-19 for case 3.

Observation 2b: The Rel-18 handling on failure cannot be reused in Rel-19 for case 3. The PDCP state variables for SRB(s) need to continue from the status of source cell rather than the failed target cell for Case 3.

If we want to support case 4, some enhancement may be needed: e.g., upon inter-CU LTM failure, the UE shall store the NCC value provided in LTM CSC and the UE shall compare the Rel-19 ID between the failed target cell and selected cell to determine the failed target cell and the selected cell are from the same CU (CU2). It is very complicated for UE and RAN2 may not have enough time to discuss these details. So, case 4 can not be supported in Rel-19.

Observation 2c: The Rel-18 handling on failure cannot be reused in Rel-19 for case 4. Extra enhancements are needed to support case 4, e.g., upon inter-CU LTM failure, the UE shall store the NCC value provided in LTM CSC and the UE shall compare the Rel-19 ID between the failed target cell and selected cell to determine the failed target cell and the selected cell are from the same CU.

For Case 5~7:

In Case 5~7, the UE selectes the candidate cell from non-source and non-target CU (for LTM failure) or non-serving CU (for RLF), and the key update is needed. If p1 is agreed for key update, for the LTM cell switch triggered for failure recovery by the UE, the UE has no possibility to get NCC for key update in case 5~7. Hence, Case 5~7 shall not be supported in Rel-19.

Observation 2d: For case 5,6 and 7, the key update is needed, but the UE has no possibility to get NCC for the key update.

In summary:

Based on the above analysis, the Rel-18 handling on failure can be reused in Rel-19 for Case 1 and Case 2. Case 1 and Case 2 shall be supported. Case 3 and Case 4 can be supported with some enhancements. Case 5, Case 6 and Case 7 have no possibility of being supported in Rel-19. For Case 5, Case 6 and Case 7, the UE shall perform RRC re-establishment.

A simple method is to only support Case 1 and Case 2 for LTM based fast failure recovery.

Proposal 4a: RAN2 supports the LTM based fast failure recovery for the following case in Rel-19:

- if the UE selects an intra-CU LTM candidate cell after RLF of the MCG (case 1).

Proposal 4b: RAN2 can discuss whether to support the LTM based fast failure recovery for the following cases in Rel-19:

- if the UE selects an intra-CU LTM candidate cell, after inter-CU LTM failure (case 3)

- if the UE selects an inter-CU LTM candidate cell, after inter-CU LTM failure, where the failed cell and the selected cell are from the same CU (case 4)

Proposal 4c：The UE shall perform RRC re-establishment for the following cases in Rel-19:

- if the UE selects an inter-CU LTM candidate cell, after RLF of the MCG (case 5)

- if the UE selects an inter-CU LTM candidate cell, after intra-CU LTM failure (case 6)

- if the UE selects an inter-CU LTM candidate cell, after inter-CU LTM failure, where the failed cell and the selected cell are from different CUs (case 7)

Other issues for LTM based failure recovery is how to determine whether the failed LTM is intra-CU LTM failure or not and whether the UE selects an intra-CU LTM candidate cell or not after failure for recovery. For these issues, reusing Rel-19 ID is the straightforward method.

Specifically, for case 1, after detecting RLF of the MCG, if the Rel-19 ID of the selected cell is equal to the Rel-19 ID of the serving cell, UE can perform LTM cell switch for the selected cell for fast failure recovery. ‘

For case 2, the intra-CU LTM failure can be recovered through intra-CU LTM, according to the current agreement. Hence, after detecting LTM failure, if the Rel-19 ID of the selected LTM cell is equal to the Rel-19 ID of the serving cell and the failed cell, UE can perform LTM cell switch for the selected cell for fast failure recovery.

Proposal 5: Rel-19 ID can be used to differentiate intra-CU and inter-CU case for LTM based fast failure recovery:

- (For Case 1) After detecting RLF of the MCG, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the serving cell, UE can perform cell switch for the selected cell for fast failure recovery.

- (For Case 2) After detecting LTM failure, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the serving cell and the failed cell, UE can perform cell switch for the selected cell for fast failure recovery.

  - SCG LTM

How to configure the coexistence of intra-CU and inter-CU SCG LTM.

From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

The coexistence case of intra-CU SCG LTM and inter-CU SCG LTM has been supported and RAN2 to support intra-CU SCG LTM in MN RRC message, in addition to SN RRC message. Based on current progress, one open issue is whether SCG LTM in MN RRC message (i.e., LTM-ConfigNRDC-r19) and SCG LTM in SN RRC message (i.e., LTM-Config-r18) can be configured to UE simultaneously.

If SCG LTM in MN RRC message and SCG LTM in SN RRC message are configured to UE at the same time, there might be two different LTM candidate ID pools, which can cause ambiguity regarding LTM configuration ID. Because SCG LTM can only be triggered by the Cell Switch Command MAC CE from the serving SN, the UE cannot determine whether the LTM configuration ID indicated in the SN Cell Switch Command MAC CE is from the MN RRC message ID pool or the SN RRC message ID pool. Therefore, the common LTM candidate ID pool is needed for all SCG LTM configurations. In Rel-18 SCPAC, for one UE, only either MN format or SN format (only intra-SN case is possible) is used. The similar restriction, that the mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message (i.e., LTM-ConfigNRDC-r19), can address the common LTM candidate ID pool issue for all SCG LTM configuration.

Proposal 6: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message (i.e., LTM-ConfigNRDC-r19).

Reference configuration for inter-CU SCG LTM

For the reference configuration of inter-CU SCG LTM, RAN2 has reached the following agreements:

RAN2 #127bis meeting 4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. RAN2 #128 meeting It's up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM. RAN2 #129 meeting 3. The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field. 4. UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG. 5. FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

An open issue for complete indication can be discussed, there are two options:

Option 1: For inter-SN LTM, one indication is needed to indicate LTM configuration as complete configuration in LTM-ConfigNRDC-r19.

Option 2: For inter-SN LTM, two independent complete indicators are used for MCG part and SCG part LTM configuration in LTM-ConfigNRDC-r19, respectively.

For option 2, if one indicator is “true” and another indicator is “false”, the UE behaviours for SCG LTM execution will be very complicated and the UE should apply the MCG and SCG configuration independently. For example, if the MCG part indicator is “false” but the SCG part indicator is “true”, the UE shall consider the MCG part of reference configuration to be the MCG part of current UE configuration and ignore the SCG part of reference configuration. Option 1 is a simple and straightforward method. For Option 1, if the indication is “false”, the UE shall consider the reference configuration to be the whole current UE configuration and the UE don’t need to differentiate MCG part and SCG part configuration. Hence, Option 1 can be confirmed.

Proposal 7: For inter-SN SCG LTM, RAN2 confirms only one indication is needed to indicate LTM configuration as complete configuration.

Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	Security issues for MCG LTM

Proposal 1: RAN2 confirms the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE.

Proposal 2: Introduce a new MAC CE (Enhanced LTM Cell Switch Command MAC CE) including NCC value for inter-CU MCG LTM.

Observation 1: It is not needed to revisit the previous agreements and Rel-19 ID in RRC is used to indicate whether to update the security key.

Proposal 3: For MCG LTM, the network shall ensure the LTM CSC always aligns with the Rel-19 ID in RRC:

- i.e., if the Rel-19 ID is different for the source cell and the target cell, the network shall send the LTM CSC with NCC to UE; otherwise, the network shall send the LTM CSC without NCC to UE.

2.2 LTM based failure recovery

Table 1: Rel-19 LTM based failure recovery cases

| Failure type: Selected cell for recovery: | RLF (serving cell CU1 Cell) | Intra-CU LTM failure (source CU1 cell ->target CU1 cell*) | Inter-CU LTM failure (source CU1 cell 0->target CU2 cell*) |
| --- | --- | --- | --- |
| Selected CU1 cell** (selects an intra-CU candidate cell) | Case 1 | Case 2 (Already supported) | Case 3 |
| Selected CU2 cell** (selects an inter-CU candidate cell) |  |  | Case 4 |
| Selected CU3 cell** (selects an inter-CU candidate cell) (CU3: non-source/serving CU and non-target CU) | Case 5 | Case 6 | Case 7 |

Observation 2a: The Rel-18 handling on failure can be reused in Rel-19 for case 1.

Observation 2b: The Rel-18 handling on failure cannot be reused in Rel-19 for case 3. The PDCP state variables for SRB(s) need to continue from the status of source cell rather than the failed target cell for Case 3.

Observation 2c: The Rel-18 handling on failure cannot be reused in Rel-19 for case 4. Extra enhancements are needed to support case 4, e.g., upon inter-CU LTM failure, the UE shall store the NCC value provided in LTM CSC and the UE shall compare the Rel-19 ID between the failed target cell and selected cell to determine the failed target cell and the selected cell are from the same CU.

Observation 2d: For case 5,6 and 7, the key update is needed, but the UE has no possibility to get NCC for the key update.

Proposal 4a: RAN2 supports the LTM based fast failure recovery for the following case in Rel-19:

- if the UE selects an intra-CU LTM candidate cell after RLF of the MCG (case 1).

Proposal 4b: RAN2 can discuss whether to support the LTM based fast failure recovery for the following cases in Rel-19:

- if the UE selects an intra-CU LTM candidate cell, after inter-CU LTM failure (case 3)

- if the UE selects an inter-CU LTM candidate cell, after inter-CU LTM failure, where the failed cell and the selected cell are from the same CU (case 4)

Proposal 4c：The UE shall perform RRC re-establishment for the following cases in Rel-19:

- if the UE selects an inter-CU LTM candidate cell, after RLF of the MCG (case 5)

- if the UE selects an inter-CU LTM candidate cell, after intra-CU LTM failure (case 6)

- if the UE selects an inter-CU LTM candidate cell, after inter-CU LTM failure, where the failed cell and the selected cell are from different CUs (case 7)

Proposal 5: Rel-19 ID can be used to differentiate intra-CU and inter-CU case for LTM based fast failure recovery:

- (For Case 1) After detecting RLF of the MCG, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the serving cell, UE can perform cell switch for the selected cell for fast failure recovery.

- (For Case 2) After detecting LTM failure, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the serving cell and the failed cell, UE can perform cell switch for the selected cell for fast failure recovery.

2.3	SCG LTM

Proposal 6: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message (i.e., LTM-ConfigNRDC-r19).

Proposal 7: For inter-SN SCG LTM, RAN2 confirms only one indication is needed to indicate LTM configuration as complete configuration.

Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] R2-2404037, “LS on security handling for inter-CU LTM in non-DC cases”

[3] R2-2411136, “Reply LS on security handling for inter-CU LTM in non-DC cases”

[4] S3-251124, “Reply LS on security handling for inter-CU LTM in non-DC cases”