3GPP TSG-RAN WG2 #129bis	R2-2502072

Wuhan, China, April 7th – 11th 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on NR NTN downlink coverage enhancements

Document for:        	Discussion and decision

Agenda Item:         	8.8.2		Downlink coverage enhancements

- Introduction

Based on the chair notes for last 2 meetings RAN2#128[1] / #129[2], the following agreements are made for downlink coverage enhancement:

RAN2 #128:

Agreements:

1.	If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

1.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

1.	For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

RAN2 #129:

Agreements:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

- The number of SMTC/gaps a UE needs to consider at any time will not be increased further

This paper attempts to investigate about consideration on cell barring for downlink coverage enhancement based on the above agreements.

- Discussion

2.1 Cell barring

In RAN2#128, it was agreed to introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE”, if the UEs not supporting DL-CE should be barred. In the most cases, the UEs not supporting may not find SSB with longer periodicity than 20 ms during cell search phase, however, it is possible for the UEs to find the SSB if the scanning timing matches the SSB transmission. In the case, the UE may continue to access the cell unexpectedly. Therefore, we think UEs not supporting DL-CE should be somehow barred explicitly.

For signalling details, firstly we would like to propose that UEs not supporting DL-CE (including legacy and Rel-19) could be barred using legacy barring IE (e.g. cellBarredNTN-r17) as long as there is no need to differentiate between legacy UE and Rel-19 UE not supporting DL-CE.

Proposal 1: UEs not supporting DL-CE (including legacy and Rel-19) could be barred using legacy barring IE (e.g. cellBarredNTN-r17)

Regarding UEs supporting DL-CE, we would like to confirm a precondition whether there is a case where only UEs supporting DL-CE is barred, since it may affect to the signalling details.

Proposal 2: RAN2 to confirm if there is a case where only UEs supporting DL-CE should be barred

We suppose that UEs supporting DL-CE (extended SSB periodicity) should also support non-extended SSB periodicity. Therefore, we think there would be no case where only UEs supporting DL-CE should be barred. With this assumption, legacy barring IE could also be used to bar UEs supporting DL-CE.

Observation 1: Legacy barring IE could also be used to bar UEs supporting DL-CE if there is no case where only UEs supporting DL-CE is barred

Based on the assumptions above, “barred” bit is not necessary in the new barring IE. Therefore, we would like to propose to include only “notBarred” bit in the new barring IE to exceptionally allow the UEs supporting DL-CE.

Proposal 3: RAN2 to include only “notBarred” bit in the new barring IE to exceptionally allow the UEs supporting DL-CE

Additionally, we slightly prefer to add explicit functionality (extended SSB periodicity) in the name of the new barring IE, so that readers of the specification can easily understand the requirement.

Proposal 4: RAN to add explicit functionality (extended SSB periodicity) in the name of the new barring IE

- Summary and proposal

This paper discussed about consideration on cell barring for downlink coverage enhancement and proposed the following:

Proposal 1: UEs not supporting DL-CE (including legacy and Rel-19) could be barred using legacy barring IE (e.g. cellBarredNTN-r17)

Proposal 2: RAN2 to confirm if there is a case where only UEs supporting DL-CE should be barred

Observation 1: Legacy barring IE could also be used to bar UEs supporting DL-CE if there is no case where only UEs supporting DL-CE is barred

Proposal 3: RAN2 to include only “notBarred” bit in the new barring IE to exceptionally allow the UEs supporting DL-CE

Proposal 4: RAN to add explicit functionality (extended SSB periodicity) in the name of the new barring IE

- References

[1] R2-2410913, “Report from Break-out session on NR-NTN and IoT-NTN”

[2] R2-2501333, “Report from Break-out session on NR-NTN and IoT-NTN”