3GPP TSG-RAN WG2 Meeting #129-bis                                      R2-2502085

Wuhan, China, Apr 7th – 11th, 2025

Source:        ZTE Corporation

Title:           Discussion on the higher layer spec impact of positioning SRS FH for non-RedCap UE

Agenda item:   8.18

Document for:  Discussion and Decision

- Introduction

RAN1 has sent a LS R1-2501573[1] in TEI19 with the following agreement:

- Overall description In RAN1#120 meeting TEI agenda, RAN1 has made the following agreement for enabling non-RedCap UE performing UL SRS frequency hopping for positioning: Agreement - Extend Rel-18’s UL frequency hopping UL SRS for positioning transmission to non-RedCap UEs in a single carrier - UE capability for non-RedCap UEs for UL SRS frequency hopping for positioning transmission Send LS to RAN2 to inform this agreement, whether new parameter is needed is up to RAN2 discussion.. As this agreement may relate to RAN2 specification, this liaison informs RAN2 about this agreement and whether new parameter for non-RedCap UE UL frequency hopping is needed is up to RAN2 discussion. - Actions To RAN2 ACTION: RAN1 respectfully asks RAN2 to take the agreement into account in their Rel-19 specification, and feedback if there is any spec impact.

This paper discusses the RAN2 specification impact of this Rel-19 new feature, and the identified RAN3 specification impact.

- Discussion

RAN2 specification impact

Current 38.331 has the following specified for positioning SRS frequency hopping:

- pSRS frequency hopping configuration including UTW and separate BWP in SRS-PosTx-Hopping-r18 in both ServingCellConfig and RRCRelease;

- pSRS frequency hopping detailed parameters in txHoppingConfig-r18 in SRS-Resource.

It can be seen that the related parameters are not specified dedicated for RedCap UE even in field description. That is to say, the current 38.331 already provides a common description for all kinds of UEs. There is no need to introduce new parameters to support positioning SRS FH feature for non-RedCap UE.

Proposal 1: There is no need to introduce new RRC parameters for extending pSRS FH to non-RedCap UEs.

In Rel-18, RAN1 has agreed the maximum bandwidth per hop should be 20MHz in FR1 for RedCap UE:

Conclusion For the positioning of redcap UEs, for the DL PRS reception and UL SRS transmission, the maximum hopping bandwidth for a single hop is 20MHz for FR1 and 100MHz with FR2.

In the 38.331 IE frequHopping, the corresponding restriction added in Rel-18 saying that the c-SRS (bandwidth per hop) should have a maximum bandwidth as 20MHz (represented by 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs for 15, 30, 60, 120 KHz respectively):

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area. c-SRS Indicates the maximum bandwidth per hop. When TxHoppingConfig is configured the valid values for c-SRS are such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively. The same value for c-SRS is configured for all the hops when TxHoppingConfig is configured.

The highlighted part is only correct when the c-SRS is for RedCap UE, since RedCap UE only supports a maximum bandwidth as 20MHz, so the maximum bandwidth per hop should be 20MHz. However when the feature is extended to non-RedCap UE, the non-RedCap UE supports a maximum bandwidth as 100MHz in FR1 and 400MHz in FR2, so the maximum bandwidth per hop of the non-RedCap UE should not be restricted to 20MHz anymore.

Therefore, to introduce/extend the feature to non-RedCap UE in Rel-19, the current Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE. See the following example of change:

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area. c-SRS Indicates the maximum bandwidth per hop. When TxHoppingConfig is configured for RedCap UE, the valid values for c-SRS are such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively. The same value for c-SRS is configured for all the hops when TxHoppingConfig is configured.

Proposal 2: The current 38.331 Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE. RAN2 to adopt the corresponding RRC CR R2-2502087.

38.305 does not specify any positioning SRS frequency hopping dedicated for RedCap UE since Rel-18, so this feature is not well reflected/captured in RAN2’s specification/stage-2 spec. We suggest to add this into 38.305.

Proposal 3: RAN2 to adopt the 38.305 CR R2-2502086 which reflects the pSRS FH feature for both RedCap UE and non-RedCap UE in stage-2.

RAN3 specification impact

Current NRPPa signaling contains pSRS hopping configuration provision from gNB to LMF in Positioning SRS Resource IE, but LMF cannot request pSRS hopping configuration from LMF to UE, this is because in Rel-18, gNB can only configure this feature to the RedCap UE, but LMF does not know whether the current target UE is a RedCap UE or not (only gNB receives the UE capability of supporting RedCap), so LMF cannot make request to gNB asking gNB to configure such pSRS with FH.

Since the feature is extended to all UEs in Rel-19, LMF should be able to request pSRS hopping configuration to gNB regardless of UE type, for example in Requested SRS Transmission Characteristics:

Requested SRS Transmission Characteristics This IE contains the requested SRS configuration for the UE. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Number Of Periodic Transmissions; C-ifResourceTypePeriodic; INTEGER (0..500,…); The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions.; -. Resource Type; M; ENUMERATED (periodic, semi-persistent, aperiodic, …); -. CHOICE Bandwidth; M; -. >FR1; ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ... , 160mHz, 200mHz, 15mHz, 25mHz, 30mHz, 60mHz , 35mHz, 45mHz, 70mHz, 90mHz). >FR2; ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…, 600mHz, 800mHz, 1600mHz, 2000mHz). SRS Resource Set List; 0.. 1; -. >SRS Resource Set Item; 1..< maxnoSRS-ResourceSets>; -. >>Number of SRS Resources Per Set; O; INTEGER (1..16,...); The number of SRS Resources per resource set for SRS transmission.; -. >>Periodicity List; 0.. 1; -. >>>Periodicity List Item; 1..<maxnoSRS-ResourcePerSet>; -. >>>>PeriodicitySRS; M; ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …); Milli-seconds; -. >>Spatial Relation Information; O; 9.2.34; This IE is ignored if the Spatial Relation Information per SRS Resource IE is present.; -. >>Pathloss Reference Information; O; 9.2.53; -. >>Spatial Relation Information per SRS Resource; O; 9.2.60; -. SSB Information; O; 9.2.54; -. SRS Frequency; O; INTEGER(0..3279165); NR ARFCN The carrier frequency of SRS transmission bandwidth.; YES; ignore. Bandwidth Aggregation Request Indication; O; ENUMERATED(true, …); YES; ignore. Positioning Validity Area Cell List; O; 9.2.93; YES; ignore. Validity Area Specific SRS Information; O; 9.2.96; YES; ignore. SRS Frequency Hopping Request Indication; O; ENUMERATED(true, …); YES; ignore.

Since the LS from RAN1 does not include RAN3, so we suggest to indicate the identified NRPPa spec impact in the reply of LS to RAN1 and RAN3.

Proposal 4: Send LS to RAN1 and RAN3 to indicate the identified RRC/stage-2 and NRPPa spec impact of pSRS FH for non-RedCap UEs. Take the draft LS out R2-2502260 as baseline.

The draft reply LS is in Annex also submitted in R2-2502260.

- Conclusion

In this contribution, we propose the following observation and proposals:

Proposal 1: There is no need to introduce new RRC parameters for extending pSRS FH to non-RedCap UEs.

Proposal 2: The current 38.331 Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE. RAN2 to adopt the corresponding RRC CR R2-2502087.

Proposal 3: RAN2 to adopt the 38.305 CR R2-2502086 which reflects the pSRS FH feature for both RedCap UE and non-RedCap UE in stage-2.

Proposal 4: Send LS to RAN1 and RAN3 to indicate the identified RRC/stage-2 and NRPPa spec impact of pSRS FH for non-RedCap UEs. Take the draft LS out R2-2502260 as baseline.

- Reference

- R1-2501573, LS on non-RedCap UE UL SRS frequency hopping for positioning, RAN1, LS in, 2025-02

- Annex: draft reply LS to RAN1/RAN3

3GPP TSG-RAN WG2 Meeting#129-bis								R2-25xxxxx

Wuhan, China, 7th – 11st Apr. 2025

Title:	[DRAFT] Reply LS on the non-RedCap UE UL SRS frequency hopping for positioning

Response to:	R2-2501714 (R1-2501573)

Release:	Rel-18

Work Item:	TEI19

Source:	ZTE [to be RAN WG2]

To:	RAN WG1, RAN WG3

Cc:

Contact Person:

Name:	Yu Pan

E-mail Address:	pan.yu24@zte.com.cn

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		-

1. Overall Description:

RAN2 thanks RAN1 for the LS indicating that non-RedCap UE can also support positioning SRS frequency hopping from Rel-19. Regarding the spec impact:

- There is no need to introduce new RRC parameters for this feature;

- RAN2 has identified the specification impact of this feature in TS38.331, wherein the field description of freqHopping should be changed because the current Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE. RAN2 has agreed the TS38.331 CR R2-2502087;

- RAN2 has identified the specification impact of this feature in TS38.305. RAN2 has agreed the TS38.305 CR R2-2502086.

Further, RAN2 has identified a potential NRPPa specification impact that, LMF can request positioning SRS frequency hopping configuration to gNB via Requested SRS Transmission Characteristics IE, since all kinds of UE can support this feature from Rel-19, LMF can be able to request pSRS frequency hopping configuration to gNB for all kinds of UEs.

2. Actions:

To RAN WG1:

ACTION: RAN2 kindly asks RAN WG1 to take the above reply into account.

To RAN WG3:

ACTION: RAN2 kindly asks RAN WG3 to determine whether there is such specification impact in NRPPa.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #130		19 - 23 May 2025					Malta, EU

TSG RAN2 Meeting #131		25 - 29 Aug 2025					Bangalore, India