3GPP TSG-RAN WG2 Meeting #129bis	R2-2502100

Wuhan, China, 07 – 11 April 202

Agenda item:	8.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on support of TDD mode for IoT-NTN

WID/SID:	IoT_NTN_TDD - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN #106 the work item on NB-IoT TDD NTN mode [1] was agreed with the following objective:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

At RAN2 #129 the following was agreed:

Agreements: 1. RAN2 will continue studying paging aspects based, on RAN1 progress 2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. 3. RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. 4. RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. 5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. 6. RAN2 can continue the discussion also on RAR window 7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

In this contribution we discuss aspects of TDD NTN mode, which impacts RAN2.

# 2	Discussion

According to the WID and RAN1 discussions the NB-IoT TDD NTN frame structure is as illustrated in Figure 1.

Figure 1 NB-IoT TDD NTN frame structure.

Furthermore, RAN1 has agreed that the UL and DL occasions are paired, i.e. if UE has received scheduling information in DL1 the corresponding UL occasion is UL1.

Agreement (RAN1 #119) Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraint

In general, the sparsity of DL and UL subframes impacts the network capacity, especially when considering the mandatory signalling (NPSS, NSSS, NPBCH, SIB1, SIB31). According to RAN1 conclusion the overhead of transmitting the aforementioned, excluding SIB31, is about 40 % of the anchor carrier.

Conclusion (RAN1 #119) For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits. - When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16: - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier. - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier. - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included. - The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB

Thus the number of subframes, which can carry control- and user-plane data is limited. For example, the NB-IoT PRACH preamble has a high capacity due to the use of 3.75 kHz subcarrier. However, the RAR/Msg2 will require a larger bandwidth and thus it may be challenging for the eNB to respond timely to all UEs if many preambles were transmitted in the preceding UL occasion.

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

Based on the RAN1 agreements, if the UE has transmitted in UL1 the UE will have to monitor for a response in DL1. According to the frame structure definition, the time-separation is 3*8.28 ms + 3 guard periods which approximates 25 ms (see also Figure 1).

The WID assumptions define LEO 600 km and 1200 km orbits and thus the RTT is in the range 4-12.88 ms for a regenerative LEO600 deployment and 8-25.76 ms for a transparent LEO600 deployment depending on the UE’s elevation angle to the satellite. This means that the UL-DL occasion gap in some scenarios is significantly longer than the RTT and in other scenarios the RTT and the gap is quite comparable.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

The large UL-DL time separation impacts the RAN2 specification, because some MAC procedures are based on the UE-eNB RTT. For example, the 4-step Random Access procedure utilize the UE-eNB RTT when defining the start of the RAR window after Msg1 transmission and for start of the Contention Resolution timer after Msg3 transmission.

An example is given for the RAR window (TS 36.321), which starts based on X subframes + UE-eNB RTT.

### 5.1.4 Random Access Response reception … If the UE is an NB-IoT UE: - if the random access preamble was transmitted in a non-terrestrial network: - RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions; … Table 5.1.4-1: Subframes between preamble transmission and RA Response Window in NB-IoT TDD/FDD mode; Preamble format; Number of NPRACH repetitions; X. FDD; 0 or 1; >= 64; 41. FDD; 0 or 1; < 64; 4. FDD; 2; >= 16; 41. FDD; 2; < 16; 4. TDD; Any; Any; 4.

Starting the RAR window based on X (of table 5.1.4-1) and the UE-eNB RTT may result in most of the window occurring during the UL-DL occasions gap. This is problematic from UE energy consumption point of view and also limits the eNB scheduling flexibility, because it may reduce the overlap between the RAR window and the DL subframes in the DL occasion.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

A similar issue also exists for the Contention Resolution timer, because it is started based on the UE-eNB RTT as well.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

Based on this issue we propose that RAN2 discuss how to better align the UE’s PDCCH monitoring, based on the RAR window and Contention Resolution timer, with the DL occasions. We consider a feasible solution is to ensure the RAR window and Contention Resolution timer does not start before the next DL occasion. This can be achieved by adjusting the  “starts at the subframe that contains the end of the last … repetition plus X subframes plus UE-eNB RTT” to “starts at the subframe that contains the end of the last … repetition plus max(X subframes plus UE-eNB RTT, distance from last UL transmission repetition to next DL subframe)”. Using this update, the RAR window and Contention Resolution timer will either start at the first DL subframe of the next DL occasion or start later if the UE-eNB RTT is long.

Proposal 1: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap.

Within the 90ms, for one cell, only one pair of UL and DL occasions can be used for the UEs served by the cell. For the other occasions and the gaps between the occasions, it can be regarded as unavailable subframes. For UE in RRC idle state, the UE may however still be able to measure neighbor cells in the subframes, which are unavailable on the serving cell, because the neighbor cell(s) may use different pair(s) of UL and DL.

For UE in RRC Connected state, we think one approach can be to base the UE behavior on what was agreed for GNSS measurement gaps for NB-IoT. The stage 2 defines the following:

NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS acquisition and resumed when the GNSS acquisition is completed.

Differently from the NOTE, for the NB-IoT TDD NTN operation the neighbor cell measurement can still be performed outside the UL-DL pair of the serving cell, while the other UE AS operations such as RLM and RACH shall be suspended.

Proposal 2: RAN2 to discuss reusing the AS operation from GNSS measurement gap for RRC Connected UE working in NB-IoT TDD NTN, except the neighbor cell measurement which is possible outside the serving cell’s UL-DL pair.

In NB-IoT, SIB1-NB is scheduled with a fixed periodicity of 2560ms, it is transmitted on subframe 4 of every other frame and 8 subframes makes up one transport block. For TDD mode, RAN1#120 discussed how to handle the SIB1-NB repetition and segmentation falls in non-D NB-IoT subframes. However, there is no conclusion reached yet.

Agreement (RAN1 #120) NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - FFS: handling for SIB1-NB

If the SIB1-NB repetitions and segmentations are dropped in the non-DL subframes, it is not clear whether the SIB1-NB coverage performance can be guaranteed. Therefore, it is not clear whether more transmission opportunities should be scheduled for SIB1-NB. We propose RAN2 to wait for RAN1 conclusion to decide whether the SIB1-NB scheduling scheme should be changed.

Proposal 3: RAN2 to wait for RAN1 conclusion on how to handle SIB1-NB in non-DL subframes, to decide whether the SIB1-NB scheduling scheme should be changed.

At RAN2 #129, the extension of the Kmac value was discussed. Currently, the Kmac (Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB) is in the range 1-512 ms, and 0 ms when not configured by RRC. In our view, 512 ms allows for a very long propagation distance (including one or more hops via inter-satellite links). For example, assuming a satellite altitude of 600 km and Earth radius of about 6400 km, the circumference is approximately 44,000 km, which results in a propagation time of less than 150 ms with multiple hops of inter-satellite link. Thus, with the current Kmac value range it is already possible for the signal to travel around the Earth and still have hundreds of ms (512 ms - 150 ms ≈ 350 ms) for on-board processing in the satellites.

We therefore kindly ask the proponents to further clarify the scenario where this Kmac value extension is needed, in particular noting that it will mean legacy release 17 and 18 NTN UEs cannot access the cell using an extended Kmac value. Based on that, RAN2 can identify the scenarios to be supported with the extended Kmac.

Proposal 4: RAN2 to confirm that extending the Kmac beyond 512 ms will block the cell for legacy release 17 and 18 NTN UEs.

Proposal 5: Further details about the scenarios requiring an extended Kmac should be provided to justify the Kmac value extension.

# 3	Conclusion

This document has made the following observations:

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

And proposed the following:

Proposal 1: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap.

Proposal 2: RAN2 to discuss reusing the AS operation from GNSS measurement gap for RRC Connected UE working in NB-IoT TDD NTN, except the neighbor cell measurement which is possible outside the serving cell’s UL-DL pair.

Proposal 3: RAN2 to wait for RAN1 conclusion on how to handle SIB1-NB in non-DL subframes, to decide whether the SIB1-NB scheduling scheme should be changed.

Proposal 4: RAN2 to confirm that extending the Kmac beyond 512 ms will block the cell for legacy release 17 and 18 NTN UEs.

Proposal 5: Further details about the scenarios requiring an extended Kmac should be provided to justify the Kmac value extension.

# Reference

- RP-243293, Iridum, “Revised WID on introduction of IoT-NTN TDD mode”, December 2024