3GPP TSG-RAN WG2 Meeting#129bis                               	R2-2502101

Wuhan, China, April 7th – 11st, 2025

Agenda item:	8.9.2

Source: 	Google

Title: 	On the support for the S&F operation

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the revised WID of Rel-19 IoT-NTN [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Significant progress was made in RAN#128 and RAN2#129, on the S&F indication, the time information associated with the S&F status, and the list of satellite IDs provided by MME [2][3].

Agreements in RAN2#128: - Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start) - It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable - “S&F operation” indication is provided in SIB1 - RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN) - RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation - RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite - RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list

Agreements in RAN2#129: - RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change - The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) - Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). - A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In this paper, we discuss several open issues relevant to the agreements highlighted above, and also the impact of the S&F Monitoring List (provided by the MME) on RAN. Based on the discussion, several way forwards are proposed in this paper.

# Discussion

## Time information associated with the S&F mode transition

It was agreed in RAN2#128 [2] that the time information can be broadcasted to indicate when the current satellite operation mode will transit from S&F mode to normal mode (i.e., non-S&F mode), and the usage of such time information is to allow UE to at least delay some NAS procedures until the feeder link is resumed. The time information associated with the opposite transition (from normal mode to S&F mode) is still FFS. We believe the time information indicating when the “S&F mode” will start is beneficial to at least the Rel-19 UEs not supporting the S&F operation, as these UEs can determine whether to initiate/trigger an access attempt based on whether there is sufficient time to fulfill the access purpose. Without this information, the UEs not supporting the S&F operation may initiate an access attempt right before the satellite turns into the S&F mode, and then will be released by the network soon after the satellite turns into the S&F mode, which consumes considerable radio resources and UE energy.

In addition, for the UEs not supporting the S&F operation, the principle is to make these UEs stay in the cells operating in the normal mode for as long as possible, even if they are allowed to camp on the cell in the S&F mode. Therefore, while camping on the cell operating in the normal mode and indicating when S&F mode will start, the Rel-19 UEs not supporting the S&F operation should trigger the intra-/inter-frequency measurement before the S&F mode starts. Similar to the case of t-service in Rel-18, these UEs should trigger the intra-/inter-frequency measurement regardless of other triggering conditions (e.g., the serving cell quality, the distance to the reference location).

It is beneficial for the Rel-19 UE not supporting S&F operation to know when the S&F mode will start, as based on the time information the UE can determine: 1) whether to initiate an access attempt, and 2) when to trigger the neighbor cell measurement.

In RAN2#129, it was also discussed whether to reuse the existing t-Service to indicate when the S&F mode will start. For the UE not supporting the S&F operation, this probably can work (i.e., it may trigger the neighbor cell measurement and cell reselection at intended time). However, for the Rel-19 UE supporting the S&F operation, this would trigger the neighbor cell measurement and cell reselection too early, resulting in unnecessary UE power consumption. Figure 1 illustrates such a case, where the UE capable of the S&F operation triggers the neighbor cell measurement at t1, which is much earlier than the timing that the UE is supposed to perform the measurement (i.e., right before t2).

If t-Service is used to indicate when the S&F mode will start, the UE capable of the S&F operation could trigger the neighbor cell measurement and cell reselection too early, resulting in unnecessary UE power consumption.

Figure 1. The UE capable of S&F operation would trigger the neighbor cell measurement too early, if t-Service is used to indicate when the S&F mode will start.

Consequently, to enhance access control, facilitate cell reselection for Rel-19 UEs lacking S&F capability, and prevent negative impacts on Rel-19 UEs with S&F, the start time of "S&F mode" should be broadcast in SIB31, keeping its application separate from t-Service.

The time information indicating when the S&F mode will start is provided in SIB31.

If proposal 1 can be agreed, there is no need to introduce a new IE in addition to the agreed IE indicating when the normal mode will start. Both time information can share the same IE and be differentiated based on the presence or absence of the S&F operation indication. That is, if the time information is signaled together with the S&F operation indication, the time information indicates when the normal mode will start; if the time information is signaled but the S&F operation indication is absent, the time information indicates when the S&F mode will start.

When the S&F operation indication is present, the agreed time information in SIB31 indicates when the normal mode will start.

When the S&F operation indication is absent, the agreed time information in SIB31 indicates when the S&F mode will start.

To increase the likelihood for the UE to detect, measure, and reselect a cell operating in the normal mode (i.e., non-S&F mode), the network can provide a list of cell (PCIs) that are not operating in the S&F mode as part of cell reselection information. The UE can then prioritize the cells in the list while making the cell reselection decision.

As part of cell reselection information, a list of cells not operating in the S&F mode can be provided in the system information.

## The S&F Monitoring List

SA2 has discussed a list of satellite IDs over which the UE may re-attempt a NAS procedure if the procedure cannot be completed within the coverage time of a single satellite due to the S&F operation, where the list of satellite IDs can be provided during the Attach or TAU procedure. In the normative phase, the agreed CR S2-2502513 [4] formally introduced the list (i.e., the S&F Monitoring List) into TS 23.401 and briefly described the potential usage of the list.

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE’s EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with asatellite of the same PLMN that is operating in S&F Mode. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). - NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

In the reply LS to RAN2 [5], SA2 also made it clear that if a satellite is included in the S&F Monitoring List, the satellite has the UE context and must support the S&F operation.

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s): - the satellite has the UE context but does not necessarily support store-and-forward operation; or - the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)? [SA2 Answer]: SA2 confirms that option 2 is the correct understanding.

We understand, however, that the S&F Monitoring List is ineffective unless used during the AS cell selection and reselection procedure. While the SA2 specification leaves the UE's behavior upon receiving this list to its implementation, the UE's NAS layer cannot be expected to refrain from attempting or reattempting a NAS procedures when camped on an S&F satellite not included in the list. This is because the NAS layer doesn't have visibility of the selected/reselected cell's satellite ID. Furthermore, imposing such a prohibition would be impractical, effectively preventing the UE from performing any actions when camped on a S&F satellite not included in the list.

The S&F Monitoring List is ineffective unless used during the AS cell selection and reselection procedure.

To leverage the S&F Monitoring List, RAN2 could redefine "suitable cell" to include the satellite ID (obtained from SIB31) as a determining factor. This wouldn't prevent the UE from camping on a satellite not included in the S&F Monitoring List, as it could still qualify as an "acceptable cell". Alternatively, we could explore increasing the frequency priority of carriers or the cell ranking of cells linked to satellite IDs within the S&F Monitoring List.

RAN2 to work on the possible impact of the S&F Monitoring List on the cell selection/reselection procedure, among the following options:
1) Redefining the suitable cell taking into account the satellite ID.
2) Prioritizing the frequencies or cells linked to satellite IDs in the S&F Monitoring List during cell reselection.

# Conclusion

In this paper, we discuss the time information associated with the S&F mode transition and the impact of the S&F Monitoring List on RAN. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Time information associated with the S&F mode transition

Observation 1	It is beneficial for the Rel-19 UE not supporting S&F operation to know when the S&F mode will start, as based on the time information the UE can determine: 1) whether to initiate an access attempt, and 2) when to trigger the neighbor cell measurement.

Observation 2	If t-Service is used to indicate when the S&F mode will start, the UE capable of the S&F operation could trigger the neighbor cell measurement and cell reselection too early, resulting in unnecessary UE power consumption.

Proposal 1	The time information indicating when the S&F mode will start is provided in SIB31.

Proposal 2	When the S&F operation indication is present, the agreed time information in SIB31 indicates when the normal mode will start.

Proposal 3	When the S&F operation indication is absent, the agreed time information in SIB31 indicates when the S&F mode will start.

Proposal 4	As part of cell reselection information, a list of cells not operating in the S&F mode can be provided in the system information.

The S&F Monitoring List

Observation 3	The S&F Monitoring List is ineffective unless used during the AS cell selection and reselection procedure.

Proposal 5	RAN2 to work on the possible impact of the S&F Monitoring List on the cell selection/reselection procedure, among the following options: 1) Redefining the suitable cell taking into account the satellite ID. 2) Prioritizing the frequencies or cells linked to satellite IDs in the S&F Monitoring List during cell reselection.

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

Draft Report of 3GPP TSG RAN WG2 meeting #128, ETSI MCC.

Draft Report of 3GPP TSG RAN WG2 meeting #129, ETSI MCC.

S2-2502513, Support of S&F Monitoring List in S&F Mode, Huawei, HiSilicon, Samsung.

S2-2502450, Reply LS on Satellite IDs for store-and-forward operation, SA2.