3GPP TSG-RAN WG2 Meeting#129bis                               	R2-2502103

Wuhan, China, April 7th – 11st, 2025

Agenda item:	8.9.4

Source: 	Google

Title: 	On the support for PWS in NB-IoT

Document for:	Discussion and Decision

# Introduction

The support for the broadcast of PWS messages in NB-IoT was discussed in the RAN2#129 meeting [1], resulting in the following agreements.

Agreements in RAN2#129: - Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells. - Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information. - Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB. - We don’t introduce a new SIB for carrying geographical information for emergency areas. - Send a LS to CT1 and CT4 indicating that RAN2 currently supports a maximum of 64 segments for SIB11-NB and SIB12-NB with a maximum TBS of 680bits and asking to provide feedback if they think we need to work on changes to allow longer messages (that would require more segments). Also indicate some possible drawbacks.

Based on the current RAN2 status on the PWS support for NB-IoT, several important improvements are explored in this paper.

# Discussion

## Handling of PWS segments after switching to a new cell

It has been agreed that the LTE SIBs for PWS are used as a baseline for designing NB-IoT SIBs for PWS. In LTE, if a UE (in the idle or connected mode) switching to a new cell has not fully received a segmented PWS message in the original cell, it discards any previously buffered segments, as specified below in 3GPP TS 36.331[2]. This is because the segmentation of the same PWS message can vary across base stations.

- 5.2.2.4 System information acquisition by the UE <unrelated part omitted> 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11; <unrelated part omitted> 1> if the UE is CMAS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

An LTE UE switching to a new cell would discard any previously buffered PWS segments, if it has not fully received a segmented PWS message in the original cell.

However, in NB-IoT NTN, it is very likely the segmentation of the PWS message in the original cell and in the new cell are performed by the same base station, especially in the transparent satellite payload architecture and when the cells are earth-moving cells. Therefore, discarding previously buffered PWS segments is not always a preferable approach, as it would prolong the PWS reception time significantly, resulting in greater UE power consumption. A more reasonable handling is to allow UE to keep the previously buffered PWS segments and resume the PWS reception after switching to a new cell in some circumstances. For instance, the UE can resume the reception of a segmented PWS message in the new cell if the new cell and the original cell share the same tracking area identity, or simply if the UE is operating in the NTN mode.

Under certain circumstances (e.g., operating in NTN mode), the NB-IoT UE may resume the reception of a segmented PWS message after switching to a new cell.

## Inclusion of PWS area information in Paging

The Cell Broadcast Entity (CBE) first takes care of all the aspects of formatting alerts messages, including the splitting of an alter message into a number of pages. The alerts messages are then passed all the way to the MME, and then eventually reach the eNB operating an NTN cell. The UE, under the coverage of the NTN cell and upon receiving the paging message including the CMAS indication, proceeds to acquire SIB12 and obtains the information in SIB12. If the geo-fencing information (i.e., warningAreaCoordinatesSegment) is also included in SIB12, the UE is able to determine whether to take actions (e.g., display the alert messages) based on whether the UE is within the alert area(s) or not. Since the cell coverage of the NTN cell is much larger than the alert area, most of the UEs within the NTN cell do not need to respond to the CMAS message, due to out of the alert area. However, as these UEs (within the NTN cell) still receive the paging message including a CMAS notification, they need to further acquire and store SIB1/SIB12 to determine whether to take further actions, which is a significant power waste for these UEs.

The likelihood of paging an irrelevant UE in an NTN cell for a PWS alert is very high.

To alleviate the aforesaid UE power waste, the simplest way is to duplicate the geo-fencing information (i.e., warningAreaCoordinatesSegment) in SIB12 and put the same geo-fencing information also in the paging message. However, as the geo-fencing information can include the coordinates of up to 100 vertices, embedding such detailed geo-fencing information in the paging message could bring significant signalling overheads. To achieve good trade-off between the UE power saving and the signalling overheads, the paging message may include the coarse information regarding the affected area of the PWS, where the coarse information is able to indicate affected area in a coarse level, but requires much less bits. For example, as illustrated in Figure 1, an NTN cell can be divided equally into four quadrants in accordance with the reference location of the NTN cell. The paging message includes the coordinate of the reference location and also a 4-bit long bitmap indicating whether there is any alert area within each quadrant, if the CMAS indication (i.e., cmas_Indication) is also present. The four quadrants are mapped to the 1st bit, 2nd bit, 3rd bit, and 4th bit in the bitmap, respectively, and the value ‘1’ in each bit indicates there is at least one alert area while the value ‘0’ indicates there is no alert area in the corresponding quadrant. The reference location can be even omitted in the paging message, and in such case, the UE will acquire and use the reference location in SIB31 as the basis to divide the NTN cell. In the example illustrated in Figure 1, as the alert area falls within the 3rd quadrant, the values of the bitmap are {0, 0, 1, 0}.

PWS area information in coarse level can be signaled together with a PWS indication in Paging-NB. The UE not within the area determined by the PWS area information can skip acquiring the system information relevant to the PWS.

Figure 1.The coarse geographical information in the Paging-NB divides an NTN cell into four quadrants.

## PWS reception in RRC_CONNECTED via dedicated messages

Unlike LTE UEs, NB-IoT UEs cannot receive broadcast SIBs while operating in RRC_CONNECTED. To make it possible for the NB-IoT UE in the connected mode to receive broadcast PWS messages, some RAN1 efforts are required to enable the UE reception of the PWS notification included in the paging, direct indication, or system information in the connected mode. However, this path has been officially blocked according to the following note in the revised WID [3], which was updated in the RAN#107 meeting.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome. Note: PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported.

Consequently, to ensure the successful reception of a broadcast PWS message, the network may compel NB-IoT devices to enter the idle state. This transition, however, disrupts ongoing services and impacting user experience. This disruption is particularly problematic during emergency calls or messaging, where any interruption could have severe consequences.

Compelling connected UEs to enter idle mode for the reception of PWS messages is problematic, especially when the UEs are conducting emergency calls or messaging.

Therefore, to ensure seamless PWS reception without service interruption, it is pivotal to support the delivery of PWS messages through dedicated signalling in NB-IoT, given that the path through the broadcast signalling is blocked now.

Support transmission of PWS messages via dedicated signalling for NB-IoT UEs in the connected state.

To simplify the implementation, we propose reusing an existing DL RRC message for transmitting PWS messages to NB-IoT UEs in the RRC_CONNECTED state. Given that the PWS message is an upper layer message, the DLInformationTransfer-NB message is a suitable candidate for this purpose.

Utilize the DLInformationTransfer-NB message to transmit PWS messages to NB-IoT UEs operating in the RRC_CONNECTED state.

# Conclusion

In this paper, we discuss several extended issues for supporting the broadcast of PWS messages in NB-IoT, based on the current progress in RAN2. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Handling of PWS segments after switching to a new cell

Observation 1	An LTE UE switching to a new cell would discard any previously buffered PWS segments, if it has not fully received a segmented PWS message in the original cell.

Proposal 1	Under certain circumstances (e.g., operating in NTN mode), the NB-IoT UE may resume the reception of a segmented PWS message after switching to a new cell.

Inclusion of PWS area information in Paging

Observation 2	The likelihood of paging an irrelevant UE in an NTN cell for a PWS alert is very high.

Proposal 2	PWS area information in coarse level can be signaled together with a PWS indication in Paging-NB. The UE not within the area determined by the PWS area information can skip acquiring the system information relevant to the PWS.

PWS reception in RRC_CONNECTED via dedicated messages

Observation 3	Compelling connected UEs to enter idle mode for the reception of PWS messages is problematic, especially when the UEs are conducting emergency calls or messaging.

Proposal 3	Support transmission of PWS messages via dedicated signalling for NB-IoT UEs in the connected state.

Proposal 4	Utilize the DLInformationTransfer-NB message to transmit PWS messages to NB-IoT UEs operating in the RRC_CONNECTED state.

# Reference

Draft Report of 3GPP TSG RAN WG2 meeting #129, ETSI MCC.

3GPP TS 36.331 v18.5.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek.