3GPP TSG-RAN WG2 Meeting#129bis                               	R2-2502104

Wuhan, China, April 7th – 11st, 2025	Revision of R2-2500463

Agenda item:	8.5.3

Source: 	Google

Title: 	Remaining issues on the support for OD-SIB1

Document for:	Discussion and Decision

# Introduction

The second phase (i.e., Release-19) of the NES WI was kicked off in February 2024, and the revised Work Item Description (WID) RP-242354 [1] was updated after the overall progress was checked at the checkpoint in the RAN#105 meeting. The WID indicates that 3GPP will specify the procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, with the focus on the scenario where the UE obtains UL WUS configuration from Cell A, transmits UL WUS on NES Cell, and receives on-demand SIB1 from NES Cell (i.e., Case 2).

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

Based on the current progress in RAN2 and RAN1, this paper discuss several important open issues from the signaling perspective, to support the Case 2 on-demand SIB1 procedure.

# Discussion

## The reception of the RAR

RAN2 has agreed in the RAN2#126 meeting [2] that the UE is expected to receive the RAR responding to the UL WUS, which is then followed by RAN1 in the RAN1#118 and #119 meeting [3][4] as follows.

Agreement in RAN2#126 RAR monitoring for MSG1 based OD-SIB1 REQ - RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement in RAN1#118 RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline. Agreement in RAN1#119 At least the contents of RAR in response to SI request are included in RAR in response to UL WUS

As the RAR message is merely served as an acknowledgement to the UL WUS that the UE has transmitted earlier, the contents (i.e., the temporary C-RNTI, the uplink grant, and the TAC value) within the RAR message are not really useful and hence can be ignored by the UE.

The UE ignores the TAC value, temporary C-RNTI, and uplink grant in the RAR message acknowledging an UL WUS.

Alternatively, we can adopt a similar mechanism used in confirming the completeness of the MSG1-based other SI request procedure, where the UE considers the MSG1-based other SI request procedure as being completed successfully, when the UE receives an RAPID MAC sub-header matching the PRACH preamble transmitted by the UE earlier.

UE considers the RACH procedure of an UL WUS transmission is completed successfully, upon receiving an RAPID MAC sub-header matching the PRACH preamble of the UL WUS.

To monitor the RAR message after transmitting the UL-WUS, UE has to use a specific RNTI to decode the PDCCH, where the specific RNTI can be the RA-RNTI as used in the legacy RACH procedure, or a common RNTI that is common to all UEs no matter which PRACH resource the UE picked to transmit the UL-WUS. In our view, using the common RNTI can facilitate reducing the signal overheads as the network is able to use a single RAR to acknowledge multiple UL-WUS transmitted by different UEs at different time, and hence is more preferable.

RAN2 to discuss whether the UE shall use the RA-RNTI as in legacy or use a new common RNTI, to monitor the RAR responding to the preamble transmission for the on-demand SIB1 procedure.

## Monitoring window for SIB1 reception

RAN1 has agreed that the UE shall monitor the type 0 PDCCH monitoring occasions for the OD-SIB1 transmission after receiving the RAR in response to UL WUS, and the duration of the time window for monitoring OD-SIB1 is indicated in the UL WUS configuration. RAN2 also acknowledged such a design in the RAN2#129 meeting [6].

Agreement in RAN2#129 6. Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

However, from RAN2 perspective, what is more important is the UE behavior after the expiry of the SIB1 monitoring window. In our view, if UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, the UE shall consider the cell as being barred and pick the next candidate cell to evaluate the cell reselection criteria again. This is to avoid UE stranding too long in a cell that does not intend to response to the UL WUS immediately, otherwise the long waiting time will prolong the cell reselection procedure, prevent the UE from reselecting to another cell, and eventually compromise the idle/inactive mode mobility performance. This behavior also aligns with the following RAN2 agreement made in RAN2#126 [2].

Agreement in RAN2#126 Barring for R19 NES UE - If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred.

## Area scope of the UL WUS configuration

RAN1 has agreed in the RAN1#117 meeting [5] that a UL-WUS configuration can apply to either a single NES cell or multiple NES cells, and a unified configuration format is needed for supporting these two cases.

Agreement in RAN1#117 From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Apart from the RAN1 agreement, we think it is beneficial from the network perspective to use a single UL WUS configuration across multiple cells, as it saves the configuration burdens for the network/operator and has a potential merit of reducing the signaling overheads. It is even more needed in the case when the UE is configured with the UL WUS via the RRC Release message, as the UE may move to another cell after entering the idle/inactive state.

The network can apply the same uplink WUS configuration to more than one cells.

RAN2 agreed that the PCI and frequency of a NES Cell can be used to associate a UL WUS configuration with the NES cell. That means for each UL WUS configuration there can be at least one PCI (and one frequency) being included in the UL WUS configuration.

Agreement in RAN2#126 Information of WUS configuration - Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

To extend to the cases where the same UL WUS configuration is applied to multiple cells, the simplest way is to include a PCI list containing more than one PCIs in the UL WUS configuration. However, if these cells happen to be within the same SI area, the same tracking area, or the same PLMN, there are more efficient ways to signal the UL WUS configuration. For instance, the UL WUS configuration may include an SI area ID, a TAI, or a PLMN ID within it instead of including a cell list. Alternatively, we may also index each UL WUS configuration with a configuration ID, and then associate this configuration ID to the cells listed in SIB3 or SIB4.

RAN2 to consider an efficient and unified method to signal the UL WUS configuration that can be applied to multiple cells. For instance, an UL WUS configuration can be associated with a list of PCIs, an SI area ID, a TAI, or a PLMN ID.

## The provision of the UL-WUS configuration in the LTE cell

Since a Rel-19 UE camping on an LTE cell may measure and then reselect an NR cell based on the NR frequency information provided in SIB24, it is possible for such a UE to reselect an NR NES cell that does not transmit SIB1. Without the UL-WUS configuration being provided in the LTE cell, the UE reselecting the NR OD-SIB1 cell will have to bar the NR OD-SIB1 cell due to the absence of SIB1, and then have to reselect another cell that may consume quite considerable time and energy from the UE. To address the issue, one way is to refrain the LTE cell from listing the NR frequency of the OD-SIB1 cell in SIB24, but this would also prevent the UE from reselecting other non-NES cells operating in the same NR frequency. A more desirable approach is to provide the UL-WUS configuration also in the LTE cell, so that the Rel-19 UE can use this configuration to request the on-demand SIB1 transmission from the NR OD-SIB1 cell.

The UL-WUS configuration can be also provided in E-UTRAN.

## The provision of the neighbouring cell information

Since the idle/inactive UE may need to check the PLMN ID and certain feature-specific barring indication (e.g., cellbarredNTN-r17, cellBarredRedCap1Rx-r17, etc.) in SIB1 to determine whether the UE can camp on the cell or not, the entire cell reselection process may take much longer time if the cell the UE attempts to reselect to is an OD-SIB1 cell. The worst case is that the UE may find an OD-SIB1 cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

The UE may find an OD-SIB1 cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

Therefore, we think it is beneficial if the PLMN ID and the feature-specific barring indications of an OD-SIB1 cell can be provided to UEs in other mean (e.g., provided through the SIB4/5 in the neighbor cells). In addition, if the UL WUS configuration can be applicable across multiple cells within a specific region (e.g., TAI or SI Area), it is better if the tracking area code (TAC) as well as the SI area identity of the OD-SIB1 cell can be provided to the UEs in the same way as the PLMN ID, so that a UE can know whether the UL-WUS configuration is applicable in that OD-SIB1 cell or not.

Serving cell can provide the cell access related information (e.g., PLMN IDs and feature-specific barring indications) of a neighboring OD-SIB1 cell via the system information.

## Impact on the cell reselection

It has been agreed in RAN2 that the UE should first perform the cell reselection procedure, and then request the on-demand SIB1 transmission from the cell UE picks, if the picked cell happens to be an NES cell not transmitting SIB1.

Agreement in RAN2#126 Triggering condition of WUS transmission - If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. - After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

In the beginning of the cell reselection procedure, the UE perform the serving and neighboring cell measurement  by measuring the SSBs transmitted by the serving and neighboring cells. As the MIB payload within a SSB is able to indicate (through the ssb-SubcarrierOffset IE) whether a cell is transmitting SIB1 or not, and RAN1 assumes the always-on SSB is transmitted on the NES cell with OD-SIB1, the UE should be able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on that cell.

Agreement in RAN1#117 For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

UE is able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on the neighboring cell.

Given that the UE is able to know whether a neighboring cell is transmitting SIB1 or not while measuring the neighboring cell, reselecting to the neighboring cell having the highest ranking but not transmitting SIB1 may not be a best practice as it may introduce overhead and consume more power at the UE compared to reselecting to another cell with already-activated SIB1 transmissions. To achieve good balance among the mobility performance, the signaling overheads, and the power consumption, we may consider to introduce an NES version of the rangeToBestCell parameter, which allows UE to reselect to a non-NES cell among the non-NES cells whose R value is within rangeToBestCell of the R value of the highest ranked cell that happens to be an NES cell.

The network can configure whether the UE is allowed to reselect to a non-NES cell whose ranking is smaller than but within a ranking margin of the ranking of the best-ranked NES cell.

# Conclusion

In this paper, we discuss several open issues for supporting the on-demand SIB1 mechanism based on the current progress in RAN1 and RAN2. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

The reception of the RAR:

Proposal 1	The UE ignores the TAC value, temporary C-RNTI, and uplink grant in the RAR message acknowledging an UL WUS.

Proposal 2	UE considers the RACH procedure of an UL WUS transmission is completed successfully, upon receiving an RAPID MAC sub-header matching the PRACH preamble of the UL WUS.

Proposal 3	RAN2 to discuss whether the UE shall use the RA-RNTI as in legacy or use a new common RNTI, to monitor the RAR responding to the preamble transmission for the on-demand SIB1 procedure.

Monitoring window for SIB1 reception:

Proposal 4	If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred.

Area scope of the UL WUS configuration:

Observation 1	The network can apply the same uplink WUS configuration to more than one cells.

Proposal 5	RAN2 to consider an efficient and unified method to signal the UL WUS configuration that can be applied to multiple cells. For instance, an UL WUS configuration can be associated with a list of PCIs, an SI area ID, a TAI, or a PLMN ID.

The provision of the UL-WUS configuration in the LTE cell:

Proposal 6	The UL-WUS configuration can be also provided in E-UTRAN.

The provision of the neighboring cell information:

Observation 2	The UE may find an OD-SIB1 cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

Proposal 7	Serving cell can provide the cell access related information (e.g., PLMN IDs and feature-specific barring indications) of a neighboring OD-SIB1 cell via the system information.

Impact on the cell reselection:

Observation 3	UE is able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on the neighboring cell.

Proposal 8	The network can configure whether the UE is allowed to reselect to a non-NES cell whose ranking is smaller than but within a ranking margin of the ranking of the best-ranked NES cell.

# Reference

RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.

R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #118, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #119, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #117, ETSI MCC.

Draft Report of 3GPP TSG RAN WG2 meeting #129, ETSI MCC.