3GPP TSG-RAN WG2 Meeting #129bis                           R2-2502108

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.2.2

Source:	Mediatek Inc.

Title:	Further Discussion on LCM for UE-side Model for AI-BM

Document for:	Discussion, Decision

Introduction

During the RAN2#127 meeting, the following procedures were agreed upon for the applicability report concerning the functionality-based LCM for the UE-side model, specifically for the use case of beam management. Additionally, one LS[1] was sent to RAN1, containing questions whose answers are essential for facilitating further progress in RAN2.

According to the reply LS[2] from RAN1 (R1-2410898), RAN1 provided agreements related to the questions from RAN2, although there was limited consensus on the answers to RAN2’s specific questions.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

In the RAN1#120 meeting, RAN1 halted discussions on the applicability report and deferred the decision to RAN2 on whether to support Option A, Option B, or both. During the RAN2#129 meeting, RAN2 reached the following agreements regarding Option A and Option B for the applicability report.

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

In this contribution, we further discuss how inference configurations and their applicability are managed and reported by the UE, with a particular focus on supporting Option B.

Discussion

Signaling Flow for Option B)

The signaling flows of Option A and Option B are illustrated in Figure 1. Each method has its unique advantages and drawbacks.

Figure 1 Signaling flow for Option A) and Option B)

In Option A) the proactive approach, the network proactively sends the full inference configurations to the UE without any assistance information from the UE, which then responds with the applicability or inapplicability of these configurations.The main benefit of Option A is the simplicity it offers to the network, as it does not need to identify which parameter subset of the full inference configuration will be used by the UE to judge applicability. However, this approach can lead to resource waste and potential performance degradation due to the blind activation of periodic CSI resources.

In Option B) the reactive approach, the network sends inference parameters, and the UE provides feedback on the applicability of those inference parameters. This allows the network to configure and activate only the necessary full inference configuration and resources. The signaling flow for this approach involves the following steps:

- Step 3: The UE receives an RRCReconfiguration message that includes one or more sets of inference parameters, which can be included in OtherConfig or a new IE. It is assumed that each set of inference parameters is identified by an index.

- Step 4: The UE responds in the RRCReconfigurationComplete message as initial report, and sends a UEAssistanceInformation message for subsequent updating. Those messages include applicable and/or inapplicable sets of inference paremeters (e.g., indexes of the sets of inference parameters).

- Step 5: The network sends a RRCReconfiguration message that provides the full inference configuration, i.e. CSI-ReportConfig.

The Option B) reactive approach allows the network to make informed decisions based on the UE's assistance information, leading to more efficient resource utilization. However, the drawback is a slight delay in the initial full inference configuration as the network waits for the UE to report the applicability status.

The reactive approach ensures that only necessary periodic CSI resources are configured and activated based on the actual applicability reported by the UE. By selectively configuring and activating resources based on UE's assistance information, the network can improve overall efficiency.

Observation 1: Option A provides simplicity and immediate configuration and activation of the inference configuration but may lead to inefficiency issues due to resource waste. Option B, although more complex and delay, offers efficient resource utilization based on UE's assistance information.

Proposal 1: In addition to Option A, Option B is optionally supported, i.e., the set(s) of inference parameters is optionally signaled in the RRCReconfiguration message in Step 3.

In last RAN2 meeting, it was agreed that upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. In order to have an unified processing on option A) and Option B) to simplify UE implementation, the UE sends the initial applicability report for the set(s) of the inference parameters in RRCReconfigurationComplete message. UAI can be sent to update applicability of the set(s) of the inference parameters. Similar as Option A that the inference configuration is idenfied by CSI-ReportConfigId, each set of inference parameters is identified by an index.

Proposal 2: Each set of inference parameters is identified by an index.

Proposal 3: Upon receiving the set(s) of inference parameters, the UE sends the initial applicability report for the set(s) of inference parameters in RRCReconfigurationComplete. UAI can be sent to update applicability of the set(s) of inference parameters.

Placement of the Inference parameters for Option B)

The placement of the set(s) of inference parameters needs to be for efficient signaling and applicability reporting. There are three primary options for where to include these parameters: in CSI-ReportConfig, in OtherConfig, or in a new IE.

Alternative 1: Including Inference Parameters in CSI-ReportConfig

If the set(s) of inference parameters is included in CSI-ReportConfig, a flag is introduced to indicate whether the CSI-ReportConfig contains a full inference configuration or a set of inference parameters.

Benefits:

- Consolidation: Both full inference configurations and inference parameters are consolidated into the same IE (CSI-ReportConfig), simplifying the signaling process.

Drawbacks:

- Use-Cases specific placement: The placement of inference parameters will be use-case specific, as they are embedded at a detailed level within the IE. This approach is not modular and is not easily extensible for future use cases.

- Complexity: For example, inference parameters for AI-BM use cases are included in CSI-ReportConfig, while inference parameters for AI mobility would be included in RRM-related IEs. The inference parameters for different use cases will be spread in different IEs.

Alternative 2: Including Inference Parameters in OtherConfig

If the set(s) of inference parameters is sent in OtherConfig, the UE should respond with the applicability of these parameters via UAI instead of RRCReconfigurationComplete, based on the current RRC process.

Benefits:

- Separation: This approach separates the inference parameters from the full inference configuration, potentially reducing the complexity of each individual configuration.

Drawbacks:

- Latency: This introduces additional latency for configuring and activating the full inference configuration for the UE if the UE responds the applicability in UAI.

One possible solution is to allow the UE to respond to the applicability of the set(s) of inference parameters in RRCReconfigurationComplete, reducing the latency impact.

Alternative 3: Introducing a New IE for Inference Parameters

The third option is to introduce a new IE specifically for inference parameters, which would be at the same level as OtherConfig in the RRCReconfiguration message.

Benefits:

- Unified signaling design: This new IE can include the set(s) of inference parameters for different use cases (e.g., AI-BM, AI-CSI Prediction, AI mobility), promoting a more unified signaling design for various use cases for inference parameters configuration and applicability reporting.

- Modularity and extensibility: As this new IE can be commonly used for different use cases, it supports modularity and is easily extensible for future scenarios.

Drawbacks:

- Introduction of New IE: The introduction of a new IE may initially require updates to the existing RRC signaling framework, which could involve some implementation effort.

Option 2 via OtherConfig shares similar benefits of modularity and extensibility as Option 3, but the UE should be allowed to report the applicability of the set(s) of inference parameters in RRCReconfigurationComplete message.

Observation 2: Including inference parameters in OtherConfig separates them from the full inference configuration, modularizing the configuration and providing more extensibility. However, this introduces additional latency for applicability reporting, which can be mitigated by allowing the UE to respond in RRCReconfigurationComplete.

Observation 3: Introducing a new IE for inference parameters provides a unified and modular signaling design that is easily extensible for various use cases. This approach ensures clear organization for the inference parameters configuration and subsequent applicability reporting.

RAN2 can considered both of Alternative 2 and Alternative 3, although alternative 3 is preferred from our prespective.

Proposal 4a: The set(s) of inference configuration is included in OtherConfig and the UE is allowed to report the applicability in the RRCReconfigurationComplete message.

Proposal 4b: The set(s) of inference configuration is included in a new IE at the same level as OtherConfig in RRCReconfiguration message.

Inference Configuration Handling after Non-applicability

In previous RAN2 meetings, following agreements were made regarding the applicability change from applicable to non-applicable, and vice versa.

- UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). - When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

Considering the dynamic changes in the UE-side conditions, it is possible that none of the full inference configurations (Option A) and/or the set(s) of inference parameters (Option B) provided in step 3 or step 5 are applicable to the UE.

- Step 3 Scenario: If none of the full inference configurations (Option A) and/or the set(s) of inference parameters (Option B) provided in step 3 are applicable, the UE should keep the configurations and report them as inapplicable in step 4. It is expected that the network will reconfigure the full inference configurations and/or the set(s) of inference parameters later.

- Step 5 Scenario: If none of the full inference configurations provided in step 5, which are intended to be the final configurations, are applicable, the UE should behave similarly as in step 3 and not trigger the re-establishment procedure. Instead, the network may initiate a fallback procedure and provide legacy configurations. The UE should report all inference configurations as inapplicable in the RRCReconfigurationComplete message.

Proposal 5: If none of the full inference configurations (Option A) and/or the set(s) of inference parameters (Option B) provided in step 3 are applicable, the UE should retain these configurations and report them as inapplicable in step 4.

Proposal 6: If none of the full inference configurations provided in step 5 are applicable, the UE should retain these configurations and report all as inapplicable in the RRCReconfigurationComplete message.

Proposal 7: The UE should not trigger re-establishment if none of the full inference configurations are applicable.

Reporting the cause of inapplicable inference configurations to the network is not reasonable. Firstly, the UE's internal conditions, such as memory, compute power, and power state, are highly dynamic and specific to each device's implementation. These resources may or may not be shared with non-AI features, further complicating the situation. The frequent changes in these conditions make it impractical to standardize and consistently report them. Additionally, the network lacks a comprehensive understanding of these internal conditions, rendering it unable to effectively utilize the reported information to adjust configurations appropriately.

Moreover, there is no consensus on whether network-side additional conditions are mandatory or optional, which complicates the applicability determination process. Given these factors, the cause of inapplicability may not be useful for the network, as it cannot take effective action based on this variable and implementation-specific data. Therefore, it is more practical to allow the UE to determine applicability without the need to report detailed causes to the network.

Proposal 8: For the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting of applicability if changed, the explicit cause should not be reported.

Conclusion

Signaling Flow for Option B)

Observation 1: Option A provides simplicity and immediate configuration and activation of the inference configuration but may lead to inefficiency issues due to resource waste. Option B, although more complex and delay, offers efficient resource utilization based on UE's assistance information.

Proposal 1: In addition to Option A, Option B is optionally supported, i.e., the set(s) of inference parameters is optionally signaled in the RRCReconfiguration message in Step 3.

Proposal 2: Each set of inference parameters is identified by an index.

Proposal 3: Upon receiving the set(s) of inference parameters, the UE sends the initial applicability report for the set(s) of inference parameters in RRCReconfigurationComplete. UAI can be sent to update applicability of the set(s) of inference parameters.

Placement of the Inference parameters for Option B)

Observation 2: Including inference parameters in OtherConfig separates them from the full inference configuration, modularizing the configuration and providing more extensibility. However, this introduces additional latency for applicability reporting, which can be mitigated by allowing the UE to respond in RRCReconfigurationComplete.

Observation 3: Introducing a new IE for inference parameters provides a unified and modular signaling design that is easily extensible for various use cases. This approach ensures clear organization for the inference parameters configuration and subsequent applicability reporting.

Proposal 4a: The set(s) of inference configuration is included in OtherConfig and the UE is allowed to report the applicability in the RRCReconfigurationComplete message.

Proposal 4b: The set(s) of inference configuration is included in a new IE at the same level as OtherConfig in RRCReconfiguration message.

Inference Configuration Handling after Non-applicability

Proposal 5: If none of the full inference configurations (Option A) and/or the set(s) of inference parameters (Option B) provided in step 3 are applicable, the UE should retain these configurations and report them as inapplicable in step 4.

Proposal 6: If none of the full inference configurations provided in step 5 are applicable, the UE should retain these configurations and report all as inapplicable in the RRCReconfigurationComplete message.

Proposal 7: The UE should not trigger re-establishment if none of the full inference configurations are applicable.

Proposal 8: For the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting of applicability if changed, the explicit cause should not be reported.

Reference

[1] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model

[2] R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model

[3] R2-2501517, Summary of offline discussion [AT129][026][AI Phy] Signaling for BM, Ericsson