3GPP TSG-RAN WG2 Meeting #129bis                           R2-2502110

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.4

Source:	Mediatek Inc.

Title:	Further Discussion on UE-side Data Collection

Document for:	Discussion, Decision

Introduction

In the RAN#106 meeting, the Way forward on AIML UE sided data collection was approved [1]:

For SA From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfilement of visibility and controllability requirements. Transfer of data over UP for Solution 3 can be studied by SA5. RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer. RAN is not expecting SA2 to study the transfer of data over CP for now. For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed General - RAN will make a final decision on way forward taking into account SA inputs when available. RAN will have a checkpoint in Q2, 2025

In this contribution, we divide the overall data collection procedure into different phases to facilitate discussion and examine NG-RAN involvement in each phase. It should be noted that the discussion in this contribution is within the context of Solution 2 and Solution 3.

Discussion

In RAN2#128 meeting, RAN2 clarfied the terminilogy of ‘data collection’ and ‘data transfer’ with following agreements:

RAN2 wants to clarify the following for the discussion of the UE side data collection for UE side model training: - Data collection refers to the UE collecting data that involves the UE performing measurements (e.g., radio measurements). - Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server (as capture in Table 7.2.1.3.2-1) in TR38.843.

The overall procedure from initiating data collection to transfering dataset to the server for data collection for UE-side model training/OTT server can be divided into the following phases: Data collection initiation, configuration for UE measurement and data collection/transfer data collection, and data transfer, just as illustrated in Fig.1.

Fig.1 The overall procedure for data collection and data transfer

Data Collection Initiation

In RAN2#128 meeting, following agreement was made for data collection initiation:

Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server))

There are two alternatives to initate data collection procedure, one alternatve is via UE request, just as agreed in RAN2#128 meeting that for data collection configuration UE-side model training, the UE can send a request for data collection.

The other alternative discussed is via UE server, i.e. the server for data collection for UE-side model training or the OTT server, just as illustrated in Fig. 2. The UE server sends the request and coordinates with CN or OAM (for option2/3) on the data collection policy. The UE context and content needs to be verified to determine whether data collection for the UE can be initiated. The NG-RAN then provides the data collection configuration and initiates the data collection procedure towards the UE based on the outcome of the coordination between the UE server and CN or OAM.

The alternative offers clear benefits in centralized management and scalability for data collection for UE-side model training. Centralized management lets the UE server efficiently coordinate with the CN/OAM to optimize data collection across the network. Furthermore, it enables the possibility to perform data collection from groups of UEs at once, rather than just one specific UE. In this case, the network can provide the data collection configuration at any point in time without UE direct request.

The configuration for data collection can be coordinated during the data collection initiation phase. There are two levels of configurations to consider:

- For Data Collection: This involves configuring the measurement resources and parameters related to the targeted scenarios, RAN configurations, and network conditions under which the data needs to be collected.

- For Data Transfer: This involves establishing the necessary tunnel for data transfer, such as setting up the UP tunnel from the UE to the termination endpoint within the CN or OAM. These parameters are use case independent and could include details like the FQDN/IP address of the termination point, or any other relevant information (subject to SA2 assessment).

Provision of these configurations can be integrated or handled as separate steps, using Option 3(data transfer via OAM) as example.

Fig.2 Data Collection Initiation via UE-Server request over network interfaces (using Opton 3 as example)

The CN/OAM can reject requests from the UE server for data collection for UE-side model training/OTT server under certain conditions, such as lack of measurement resources or network overload. Data collection initiation via UE server request is not exclusive with the approach via UE request directly but complementary between each other. This alternative requires SA2/SA5 involvement to determine the procedure to initiate the data collection procedure via the UE server request.

Proposal 1: RAN2 assumes that data collection can be initiated via the UE server, following which the NG-RAN provides the data collection configuration to the UE. The procedure for data collection/transfer initiation and configuration via UE server request is within SA2/SA5 scope.

Data Collection

Common procedure for network-side and UE-side model

Following the completion of the Rel-18 SI on AI/ML for the NR Air Interface, a set of principles for data collection for network-side model training has been established and documented in the TR [1].

A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging, - UE to report the collected data periodically, event-based, and on-demand, - The UE memory, processing power, energy consumption, signalling overhead should be considered. Note: The above principles can be revised depending on RAN1 requirements.

The principles outlined for data collection for network-side model training are equally applicable to UE-side model training. Data collection for UE-side model training also needs to consider the support of data logging, flexible data reporting, resource efficiency, UE energy consumption, UE processing power, UE memory, etc. The most important benefit to apply the same set of principle is to apply unified data collection procedure over the air interface including configuration for data collection, UE measurement/data logging and measurement/log reporting. Applying the same data collection procedure to both UE-side and network-side model training creates a standardized approach，which facilitates easier design, implementation and maintenance of data collection procedures, leading to better compatibility and less complexity in both UE and network side. The only problem needs to be addressed for data collection for UE-side model training is how to transfer the data collected by the network to the UE-side server.

Proposal 2: The general data collection principles for network side model training are applicable to UE-side model training:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

Proposal 3: Common procedures over the air interface including configuration for data collection, UE measurement/data logging and measurement/log report are preferred for data collection for UE-side model training and network-side model training.

If proposal 3 is agreeable, the agreement achieved for data collection for network-side model training can be largely applied to UE-side model training. What we need to do is to check the whether the agreements made for data collection for network-side model training is feasible for UE-side model training.

Network Configuration and Control for data collection

In RAN2#128 meeting, following methods for network control of the initation and configuration for data collection were agreed:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In RAN2#129 meeting, RAN2 agreed to extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration:

- Agreements - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

No matter whether CP or UP tunnel will be used for data transfer, the network configuration and control (start/stop) can be realized by CP e.g. RRC messages. Since the configuration is provided by the network, the UE needs to indicate to the network when the UE can’t perform data collection based on the received configuration. The problem is similar as the discussion in network-side data collection that the UE can’t performs data logging when in low power state or the buffer is full. According to current agreement, the UE reports to the network when the power state is low or when buffer is or may become full. Then the network should stop the data collection procedure.

Proposal 4: Network control and configuration for data collection for UE-side model training is realized by control plane, e.g. RRC protocol and messages.

Proposal 5: Same as network-side data collection, the UE reports to the network when the UE can’t perform data collection based on the received configuration.

For UE-side model training, it is essential for the UE-side (e.g., the UE training entity within an MNO or OTT server) to know the specific configurations that the network may later use for model inference. This allows for offline training of AI models targeted to these configurations. For instance, in AI-BM, the Set A/Set B configurations need to be known to the UE-side for effective model training. There are two possible methods to inform the UE-side of the Set A/Set B configurations for model training:

- Option 1: The gNB provides Set A and Set B configurations to the UE over the air interface. The UE then forwards these configurations, along with the collected data, to the UE-side server.

- Option 2: The gNB shares Set A and Set B configurations directly with the UE-side server via the CN/OAM.

Considering that data collection for model training targets a group of UEs rather than a specific UE, Option 2 aligns more on this purpose and can be supported with UE-side data collection solutions 2 and 3, assuming there is an interface between the UE-side server and CN/OAM for data collection and model training.

Proposal 6: For data collection for UE-side model training, RAN2 considers the following options to inform the UE-side of specific configurations for offline training, such as Set A and Set B configurations:

- Option 1: The configurations (e.g., Set A and Set B) are provided to the UE by the gNB over the air interface. The UE forwards these configurations, along with the collected data, to the UE-side server.

- Option 2: The configurations (e.g., Set A and Set B) are shared directly with the UE-side server by the gNB via CN/OAM.

To enable the UE-side to train AI models targeted to specific configurations, the UE-side should be allowed to request or suggest data collection configurations to the network-side. The requested or suggested data collection configurations (e.g., Set A and Set B) can be communicated to the gNB in two ways: either the UE can suggest the data collection configuration directly to the gNB over the air interface, or the UE-side server can coordinate the data collection configuration with the gNB via CN/OAM.

Proposal 7: The UE-side should be allowed to suggest data collection configurations (e.g., Set A and Set B) to the network-side for model training. The UE can suggest the data collection configuration directly to the gNB over the air interface, or the UE-side server can coordinate and suggest the data collection configuration to the gNB via CN/OAM.

Data collection

Same as the network-side data collection, the UE performs measurement and data logging according to the network’s configuration. Therefore, the data content can be fully visible to MNO if the information conveyed, the values held, the data types used, and the data formats structured, as well as the encoding and decoding rules applied are standardized, like RRC messages with ASN.1 to encode data. The network can verify whether the reported data matches the configuration.

However, it is important to consider the benefits of non-standardized data content, which can still be made accessible to MNOs through alternative methods such as SLAs, or business contracts.

- Flexibility in Model Training and enhanced performance: AI/ML algorithms often require tailored datasets that are specific to the particular use case or environment in which they are deployed. Not standardizing data content allows for the customization of data collection and model training processes to suit specific needs, enhancing the effectiveness of the AI solutions and the performance of these models.

- Decreased Standardization Burden: The effort and resources required to standardize vast and varied data types across multiple industries and use cases can be immense. By not standardizing data content, this burden is significantly reduced, allowing standardization efforts to focus on more critical areas where universal standards can provide clear benefits.

- Scalability and Evolution: As AI technologies evolve and the variety of use cases expands, the need for data will similarly diversify. It is impractical to anticipate and standardize data for all potential future needs. Keeping data content non-standardized allows for greater scalability and adaptability as new requirements and use cases emerge.

- Vendor-Specific Innovation: Allowing for vendor-dependent information and proprietary data formats can spur innovation, as different vendors may develop unique, optimized ways of handling data that could lead to more advanced AI solutions.

Using partially standardized data content, leads to a balance between interoperability and flexibility.  On one hand, the standardized components of the data ensure a certain level of compatibility and visibility across different systems, addressing some MNO concerns about data transparency. On the other hand, the non-standardized segments allow for the tailoring of data to specific AI/ML use cases and environments, enhancing model performance and effectiveness. This approach also reduces the burden of having to standardize every aspect of data, which can be resource-intensive and impractical given the vast and varied nature of data types needed. MNOs have the ability to configure their systems to either allow or block the carriage of non-standardized data content, giving them control over the integration of such data within their networks.

Proposal 8: Support non-standardized and partially standardized data content in Solution 2 and Solution 3. Whether the non-standardized data is allowed to be carried is configured by the MNOs.

Data Transfer

In current agreement, there is a consensus that the CP tunnel can be used as baseline for data transfer from the UE to the network, provided the data volume remains within the RRC/NAS signalling capacity. The feasibility of using UP tunnel for data transfer is up to SA2 (for solution 2) and SA5 (for solution 5).

According to the replied LS from SA5[2], SA5 confirms that it is feasible to develop the OAM solution for transfer of UE-side data over UP in solution 3, where the UP tunnel associated with data transfer is between UE and gNB.

SA5 has discussed the questions about the feasibility for transfer of data over UP for Solution 3 (UE-> gNB->OAM -> Server). SA5 has made the following analysis about the termination points of the UP tunnel for this solution: 1. If the UP tunnel is between UE and Server, SA5 does not see any difference between solution 3 and solution 1a); 2. If the UP tunnel is between UE and 5GC, SA5 does not see any difference between solution 3 and solution 2. SA5 assumes that the UP tunnel associated with data transfer is between UE and gNB for the solution 3. SA5 would like to get confirmation of this assumption from TSG RAN and RAN2, as well as RAN’s perspective on the UP tunnel termination points. Based on the assumption above, SA5 would like to confirm that it is feasible to develop the OAM solution with the necessary controllability and different levels of visibility for transfer of UE side data over UP using solution 3.

The following alternatives can be considered to support large data transfers while fulfilling the requirement of standardized data visibility:

A1. Increasing the number of segments supported by RRC messages

A2. Specific radio bearer terminated between the UE and gNB

  - File format: ASN.1

  - New file format specified by 3GPP

B. Specific tunnel terminated between the UE and OAM/CN

  - File format: ASN.1

  - New file format specified by 3GPP

Option A1: Transmitting the dataset in a large number of RRC segments is not preferred due to the resulting complexity. This complexity arises from the need to manage and ensure the integrity of multiple segments, handle errors, and implement mechanisms for retransmission in case of lost or corrupted segments. Additionally, this approach would lead to significant latency in completing the data transfer, potentially delaying or interrupting the transmission of user data and slowing down control plane (e.g., RRC) procedures. The overhead associated with managing such a large number of segments would place a substantial burden on both the network and the UE, increasing the risk of errors and retransmissions, which could further exacerbate latency issues and degrade overall network performance. However, as Option 1A utilizes ASN.1 as the file format for data transfer, the data content is visible.

Option A2: The current DRB is not feasible for data transfer from the UE to the network-side because the DRB is terminated between the UE and the UPF, and the corresponding PDU session provides an end-to-end data path between the UE and the DN. Consequently, the data content is not visible to the network-side. However, terminating a DRB at the gNB instead of the UPF is feasible and can facilitate the development of an OAM solution with the necessary controllability and varying levels of visibility for transferring UE-side data over the user plane using solution 3. Therefore, we are considering terminating a specific bearer for data transfer between the UE and the gNB.

For Option A2, we may consider different file formats and encoding/decoding rules to make the data content visible. Formats such as ASN.1 or other file formats specified by 3GPP can be used. Regardless of the chosen file format, the specification for the file format should be maintained by RAN2.

Option B: It is possible to terminate a specific tunnel between the UE and OAM/CN, which should be studied by SA5/SA2. In this case, ASN.1 can also be used as the file format, or another file format specified by 3GPP can be used. In the former case, the specification for the file format should be maintained by RAN2.

Table 1-Different Options to support data transfer with large size with standardized visibility

| Option | Pro | Con | File format | Involved WGs |
| --- | --- | --- | --- | --- |
| Option A1 | - Utilizes existing RRC signaling - Low specification impact | - High complexity due to managing multiple segments - Significant latency, potentially delaying user data transmission and network responsiveness | ASN.1 | RAN2 |
| Option A2 | - Combines the merits of SRB's data visibility and DRB's unrestricted data amount - No impact on current CP and UP protocols | - Introduce a new type of bearer - Potential need for new file format specification (2b) - High specification impact | - ASN.1 | RAN2 |
|  |  |  | - New file format |  |
| Option B | - Combines the merits of SRB's data visibility and DRB's unrestricted data amount - No impact on current CP and UP protocols Note: the analysis is up to SA2/SA5 | - Introduce a new type of bearer - Potential need for new file format specification (3b) - High specification impact Note: the analysis is up to SA2/SA5 | - ASN.1 | RAN2, SA5/SA2 |
|  |  |  | - New file format | SA5/SA2 |

Option A2 combines the merits of both SRB and DRB. With SRB, the data content is visible to NG-RAN, allowing further processing of the collected data. With DRB, there is no restriction on the data amount that can be transferred. Importantly, SA5 has confirmed that it is feasible to develop an OAM solution for transferring UE-side data over the user plane using solution 3. Since the bearer is terminated at NG-RAN, NG-RAN can determine when to establish the radio bearer for data transfer. Additionally, this specific bearer can also be used for network-side data collection, providing a versatile solution.

Proposal 9: RAN2 consider to introduce a specific (data) radio bearer used for large data amount transfer, which is characterised by:

- Terminated between the UE and NG-RAN;

- The data content is visible and can be processed by NG-RAN;

- The data size is not restricted.

When the data from different UEs are collected and stored at NG-RAN/OAM or termination point within CN, it can further transfer the data file to the UE-server, which can be through the UP tunnel, just like the data transfer from the gNB to TCE in MDT or via interfaces between CN and AF/OAM (subject to SA2/SA5 assessment).

Proposal 10: RAN2 assumes that the data transfer from the NG-RAN to the UE-server is via interfaces between NG-RAN and CN/OAM (subject to SA2/SA5 assessment).

The following issues need to be addressed if a specific radio bearer terminated between the NG-RAN and the UE is introduced:

- End-to-End Protocol: An end-to-end protocol is required to support data transmission, reception, and formatting. This protocol should operate on top of the L2 protocol stack, utilizing existing mechanisms for error detection, correction, and flow control provided by the PDCP/RLC layer. The data formatting must ensure that: 1) all necessary information (e.g., timestamps when needed) of the collected data can be encapsulated within a PDU, and 2) the receiving end can accurately reconstruct the data from the received PDUs. The design of the end-to-end protocol and data format should be undertaken by RAN2.

- Mobility Management: When a large amount of data is stored at the UE, it is critical to support data transfer continuation during UE mobility. The UE should not discard the remaining stored data when a handover occurs and should continue the data transfer after handover to the target gNB. It is assumed that the source gNB and the target gNB should deliver the received dataset to the UE-side server via OAM or CN. The responsibility for reassembling and reconstructing the received dataset during mobility needs to be clearly defined, whether it lies with the gNB, CN/OAM, or the UE-side server. This consideration should be addressed by SA2/SA5. Additionally, it is important to determine whether data loss during handover is permissible or if a lossless handover is required to ensure the integrity and completeness of the data transfer.

Proposal 11: RAN2 assumes that the data transfer continuation during handover should be supported.

Proposal 12: RAN2 consider defining the data format for data transfer from the UE to NG-RAN.

Conclusion

Data Collection Initiation

Proposal 1: RAN2 assumes that data collection can be initiated via the UE server, following which the NG-RAN provides the data collection configuration to the UE. The procedure for data collection/transfer initiation and configuration via UE server request is within SA2/SA5 scope.

Common procedure for network-side and UE-side model

Proposal 2: The general data collection principles for network side model training are applicable to UE-side model training:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

Proposal 3: Common procedures over the air interface including configuration for data collection, UE measurement/data logging and measurement/log report are preferred for data collection for UE-side model training and network-side model training.

Network Configuration and Control for data collection

Proposal 4: Network control and configuration for data collection for UE-side model training is realized by control plane, e.g. RRC protocol and messages.

Proposal 5: Same as network-side data collection, the UE reports to the network when the UE can’t perform data collection based on the received configuration.

Proposal 6: For data collection for UE-side model training, RAN2 considers the following options to inform the UE-side of specific configurations for offline training, such as Set A and Set B configurations:

- Option 1: The configurations (e.g., Set A and Set B) are provided to the UE by the gNB over the air interface. The UE forwards these configurations, along with the collected data, to the UE-side server.

- Option 2: The configurations (e.g., Set A and Set B) are shared directly with the UE-side server by the gNB via CN/OAM.

Proposal 7: The UE-side should be allowed to suggest data collection configurations (e.g., Set A and Set B) to the network-side for model training. The UE can suggest the data collection configuration directly to the gNB over the air interface, or the UE-side server can coordinate and suggest the data collection configuration to the gNB via CN/OAM.

Data Collection

Proposal 8: Support non-standardized and partially standardized data content in Solution 2 and Solution 3. Whether the non-standardized data is allowed to be carried is configured by the MNOs.

Data Transfer

Proposal 9: RAN2 consider to introduce a specific (data) radio bearer used for large data amount transfer, which is characterised by:

- Terminated between the UE and NG-RAN;

- The data content is visible and can be processed by NG-RAN;

- The data size is not restricted.

Proposal 10: RAN2 assumes that the data transfer from the NG-RAN to the UE-server is via interfaces between NG-RAN and CN/OAM (subject to SA2/SA5 assessment).

Proposal 11: RAN2 assumes that the data transfer continuation during handover should be supported.

Proposal 12: RAN2 consider defining the data format for data transfer from the UE to NG-RAN.

Reference

[1] RP-243307, Way forward on AIML UE sided data collection, RAN2 Chair (InterDigital)

[2] RP-250030, Reply LS on AI/ML UE sided data collection, SA5

[3] 3GPP TR 38.843 V18.0.0