3GPP TSG-RAN WG2 Meeting #129bis                           R2-2502111

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.5

Source:	Mediatek Inc.

Title:	Feasibility Analysis on the Alternatives for Two-sided Model

Document for:	Discussion, Decision

Introduction

The RAN1 LS (R1-2410922)[1] requests RAN2 to evaluate the feasibility of standardized signaling for the RAN1 identified solution.

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

In the post-meeting email discussion [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)[2], the options for OTA and non-OTA approaches were discussed and the principles/aspects for the feasibility study were identified.

In this contribution, we further analyse the feasibility of OTA and non-OTA approaches from following identified aspects: size, continuity, controllability, latency, visibility, Uu interface impact, UE impact, UE selection for being reception of dataset/model parameter, E2E reliability and undisclosure of the proprietary information of NW topology and NW vendor. We then analyze the feasibility of these alternatives within the scope of RAN2 expertise.

Furthermore, we discuss the necessity and possibility of using a common framework and tunnels for both one-sided and two-sided models for the purposes of data transfer and sharing model parameters/datasets.

Discussion

Non-OTA and OTA Approaches

The non-OTA and OTA approaches were described in [2] as follows:

Alternative 1 (non-OTA approach): RAN2 analyzed area dataset/model parameters transfer dataset/model parameter transfer CSI compression data collection at gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided model training UE-side training entity for two-sided UE part model training gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside/outside of MNO or an OTT server) Alternative 2 (OTA approach): RAN2 analyzed area dataset/model parameter for training dataset/model parameters transfer dataset/model parameter transfer UE-side training entity for two-sided UE part model training CSI compression data collection at gNB dataset/model parameter transfer gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (OTT server inside/outside of MNO) gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided model training. NOTE: The data transfer (e.g. raw data, dataset and/or model parameter, etc, up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

For Alternative 1 (non-OTA approach), following solutions were identified:

- 1.1 OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity

- 1.2 CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity

- 1.3 gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity

Alternative 1.3, which involves the gNB -> OAM/CN -> UE-side training entity, is essentially a subset of Alternatives 1.1 and 1.2. For the CSI compression use case, the gNB is inherently involved in the lifecycle management (LCM) and data collection for model training. Both Alternatives 1.1 and 1.2 already require the involvement of the gNB for sharing the collected data with OAM or CN.

Including Alternative 1.3 as a separate candidate solution is redundant and does not offer a distinct approach beyond what is already covered by Alternatives 1.1 and 1.2. Therefore, removing Alternative 1.3 will streamline the evaluation process for other working groups and ensure a more focused discussion.

The feasibility study on Alternative 2 is not within RAN2 expertise scope, and should be evaluated by SA2 and SA5 depending whether the UE-side server is within the CN or OAM domain. Therefore, RAN2 should focus on the evaluation on Alternative2.

Observation 1: Alternative 1.3, which involves the gNB -> OAM/CN, is essentially a step within Alternatives 1.1 and 1.2, as both Alternatives 1.1 and 1.2 necessitate the involvement of the gNB for collecting and sharing data with OAM/CN.

Proposal 1: Removing gNB -> OAM/CN -> UE-side training entity from the list of candidate solutions for non-OTA approach.

For Alternative 2 (OTA approach), following candidate solutions were identified without consensus on feasibility:

- OTA solution 1a: gNB -> UE via CP

- OTA solution 2: CN -> UE via gNB

- OTA solution 3: OAM -> UE via gNB

In the study phase, RAN2 analyzed the current status and gaps, and potential RAN specification impact for Solutions 1a and 1b. It can be observed that both Solution 1a (CP) and Solution 1b (UP) require substantial changes to current protocols and procedures. During the email discussion, there are still several companies having interest to evaluate the feasibility of solution 1b with UP tunnel.

Observation 2: Both Solution 1a (CP) and Solution 1b (UP) require substantial changes to current protocols and procedures to support model parameter/dataset sharing between gNB and UE.

Proposal 2: Following candidate solutions can be considered for feasibility study:

- OTA solution 1: gNB -> UE

- OTA solution 2: CN -> UE via gNB

- OTA solution 3: OAM -> UE via gNB

OTA Solution 3: OAM -> UE via gNB is an extension of OTA Solution 1. There is no significant difference between OTA Solution 1 and OTA Solution 3, and the path between OAM and gNB falls outside RAN2's expertise.

- If the OTA -> UE via gNB is through a CP tunnel, it is fundamentally similar to OTA Solution 1a. The RRC messages between the UE and gNB are what is visible over the air interface.

- If OAM -> UE via gNB is through a UP tunnel, it is assumed that the UP tunnel is terminated between the UE and gNB. This assumption follows the same principle outlined in the LS from SA5 for data transfer, applicable to dataset/model parameter sharing.

Observation 3: OTA Solution 3: OAM -> UE via gNB is an extension of OTA Solution 1, with no significant difference between them. The path between OAM and gNB falls outside RAN2's expertise. If the OTA -> UE via gNB is through a CP tunnel, it mirrors OTA Solution 1a, with RRC messages visible over the air interface.

OTA Solution 2:

- If CN -> UE via gNB is through a UP tunnel, similar to UE-side data collection Option 2, SA2 needs to be involved.

- If CN -> UE via gNB is through a CP tunnel, NAS signaling is assumed in the feasibility evaluation. However, given that the CSI compression use case is a RAN use case, it is preferable that the LCM procedures are only performed between the UE and NG-RAN. For instance, performance monitoring to detect mismatches between NW-side data distribution and UE-side inference data distribution may trigger the gNB to share model parameters and/or datasets with the UE over the air interface for offline training. There is no strong rationale for the CN to share this information via NAS messages.

Observation 4: For OTA Solution 2, if CN -> UE via gNB is through a UP tunnel, SA2 needs to be involved. If it is through a CP tunnel, NAS signaling is assumed, but there is no strong reason for CN to share this information via NAS messages, considering the CSI compression use case always requires gNB involvement in LCM, e.g, data collection.

Furthermore, just as commented by many companies during the email discussion that OTA solution 2 and 3 have no benefit over non-OTA solutions. The inefficiency arises from the need to transmit the dataset and model parameters to the CN/OAM before they are sent back to the gNB. This process involves relaying the parameters from the gNB to the UE and then returning them to the UE training entity, resulting in added complexity, latency and resource consumption.

Proposal 3: RAN2 focus to evaluate the feasibility of OTA solution 1, i.e. gNB->UE (CP and UP) via air interface.

Feasibility from RAN2 Perspective

According to the RAN1 LS, the dataset size can be as large as hundreds of megabytes, making it infeasible to transmit through current control plane signaling, such as RRC or NAS messages in SRBs. Let's illustrate this with an example.

Consider a dataset size of 225MB for option 4-1. This size is 25,600 times larger than the maximum RRC message size of 9kB, resulting in 25,600 RRC segments. Given RRC processing latencies of 10ms, 15ms, or 16ms depending on the DL RRC message used, it would take 4m16s, 6m24s, and 6m50s respectively to transmit the entire dataset, assuming no additional overhead (e.g., transmission time, retransmissions, or network delays, etc).

In an urban deployment with a site-to-site distance of 200m, if a UE is moving at pedestrian speed (3km/h), it takes approximately 240 seconds (4 minutes) to travel 200 meters. This is slightly shorter than the optimal assumption of 4m16s. Therefore, it is typically impossible to transfer model parameters/datasets of hundreds of MBs without performing a handover. It is also unreasonable to keep the UE in a connected mode for several minutes just for receiving model parameters/datasets.

This is why the majority of companies agree that OTA solution 1a: gNB->UE via CP is not feasible for RRC buffers >200MB in 5G, even if the threshold of >200MB is arbitrary. In our understanding, transmitting datasets larger than tens of megabytes with hundreds of RRC segments is also not feasible due to the resulting complexity. This approach would lead to significant latency in completing dataset sharing, potentially delaying or interrupting user data transmission and slowing down control plane (e.g., RRC) procedures. Additionally, managing such a large number of segments would place a substantial burden on both the network and the UE, increasing the risk of errors and retransmissions, further exacerbating latency issues and degrading overall network performance.

Observation 5: Transmitting a 225MB dataset would take longer than the time a UE moving at pedestrian speed within the same cell in an urban area, making it impractical to transfer such large datasets without performing a handover or keeping the UE in CONNECTED mode for an extended period.

Observation 6: Transmitting large datasets/model parameters exceeding tens of megabytes via hundreds of RRC segments is impractical due to the complexity and inefficiency involved, making such an approach unfeasible.

Proposal 4: RAN2 reply to RAN1 that it is not feasible to use CP to exchange the AI model parameters and/or dataset between NW-side and the UE over the air interface.

If we consider the DRB, the current DRB is also not feasible for sharing AI model parameters and/or datasets from the network side to the UE. This is because the UP tunnel is terminated between the UE and the UPF, and the corresponding PDU session provides the end-to-end data path between the UE and the DN. As a result, the data content is not visible to the network side.

Terminating a DRB at the NG-RAN instead of the UPF would represent a significant architectural shift from the current system. While it is technically feasible, completing the standardization for it may not be achievable within the limited time left for Rel-19 due to the substantial specification impact. However, we are open to consider it in Rel-20.

Observation 7: UP tunnels are terminated between the UE and UPF, and the corresponding PDU sessions provide the end-to-end data path between the UE and the DN. The data content is not visible to the network side.

Proposal 5: RAN2 should inform RAN1 that it is not feasible to use the UP to exchange AI model parameters and/or datasets between the network side and the UE over the air interface in Rel-19.

Common Framework/Tunnel for UE-sided and Two-sided

The discussion on one-sided and two-sided models has progressed separately in both RAN1 and RAN2, with RAN2 not having advanced the two-sided model discussion for a long time. It is suggested that UE-sided and two-sided models should be considered together when discussing the system framework and tunnels to enable data collection and model transfer/delivery, as these are common issues for both cases.

It is preferred to have a common design for the framework and tunnel that can be used for both UE-sided and two-sided models to reduce complexity in both implementation and standardization. It should be noted that we are discussing potential standardized methods, and the framework for data collection and model delivery between the UE and the OTT server is not considered here.

Taking the UE-sided model in Figure 2a as an example, a tunnel going through the UE, RAN, OAM, and the UE-side server can be established between the UE and the UE-side server for both data collection for model training and model transfer. This is a bi-directional tunnel, where data collection is the upstream (in orange) and model transfer is the downstream (in purple).

Comparing the two-sided model in Figure 2b with the UE-sided model in Figure 2a, the tunnel for model parameter/dataset sharing and model transfer is quite similar. The NG-RAN can transfer the model parameter/dataset to the UE-side server e.g., via OAM for offline model training for the UE-side of the two-sided model if NW-first training is considered. Since the data used for NW-first training is collected from the UE, the mechanism for NW-side data collection over the air interface is applied. The UE-side of the two-sided model can be transferred from the UE-side server to the UE e.g., via OAM and RAN node. This is a bi-directional tunnel, where model parameter and dataset transfer is the upstream (in orange) and model transfer is the downstream (in purple).

It should be noted that Figures 2a) and 2b) use UE-side data collection option 3 (where the UE collects training data and transfers it to OAM) as an example. These figures are also applicable to option 2 (where the UE collects training data and transfers it to the Core Network) if OAM is replaced with CN.

Obervation 8: A common design for the framework and tunnel can be considered for both UE-sided and two-sided models to reduce complexity in both implementation and standardization, streamlining data collection and model transfer processes.

| Fig 2a) UE-sided model: Data collection (for model training) and model transfer (for model inference) | Fig 2b) Two-sided model: Model parameter/dataset sharing (for model traiing) and model transfer (for model inference) |
| --- | --- |

Proposal 6: A common bi-directional tunnel can be used for both the UE-sided model and the two-sided model:

- UE-sided model: Data collection for model training is the upstream (from the UE to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

- Two-sided model with NW-first training: Model parameter and dataset transfer for UE-sided model training is the upstream (from NW-side to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

For common bi-directional tunnel, how to make the dataset/model parameters visible and accessible to RAN and UE for two-sided model needs to be considered.

- One possible method is to introduce a specific radio bearer dedicated to model parameter and dataset transfer, which would be capable of handling large amount of data and be terminated between the UE and NG-RAN. For UE-sided model, the specific radio bearer can also be used for data collection from the UE to NG-RAN. It has the merit of SRB that the data content is visible to NG-RAN and the merit of DRB that the data amount is not restricted. Since it is terminated at NG-RAN, NG-RAN determines when to establish the radio bearer for model parameter and/or dataset transfer. Another benefit is that that such specific bearer can also be used for NW-side data collection. In order to make the data content visible, the file format needs to be specified. Current ASN.1 can be used to define the data content. Other file formats are not excluded.

- Another possible approach is to terminate the tunnel at an entity within the Core Network (CN), to which the RAN subscribes for the data service. This entity in the CN can then expose the data to the RAN. However, this falls under the jurisdiction of SA2.

Observation 9: A specific radio bearer dedicated to model parameter and data transfer, terminated between the UE and NG-RAN, combines the advantages of SRB (data content visibility to NG-RAN) and DRB (unrestricted data amount). Additionally, it can be used for NW-side data collection.

Proposal 7: RAN2 study the ways to make the dataset/model parameters accessible and visible to RAN and UE.

Conclusion

Non-OTA and OTA Approaches

Observation 1: Alternative 1.3, which involves the gNB -> OAM/CN, is essentially a step within Alternatives 1.1 and 1.2, as both Alternatives 1.1 and 1.2 necessitate the involvement of the gNB for collecting and sharing data with OAM/CN.

Observation 2: Both Solution 1a (CP) and Solution 1b (UP) require substantial changes to current protocols and procedures to support model parameter/dataset sharing between gNB and UE.

Observation 3: OTA Solution 3: OAM -> UE via gNB is an extension of OTA Solution 1, with no significant difference between them. The path between OAM and gNB falls outside RAN2's expertise. If the OTA -> UE via gNB is through a CP tunnel, it mirrors OTA Solution 1a, with RRC messages visible over the air interface.

Observation 4: For OTA Solution 2, if CN -> UE via gNB is through a UP tunnel, SA2 needs to be involved. If it is through a CP tunnel, NAS signaling is assumed, but there is no strong reason for CN to share this information via NAS messages, considering the CSI compression use case always requires gNB involvement in LCM, e.g, data collection.

Proposal 1: Removing gNB -> OAM/CN -> UE-side training entity from the list of candidate solutions for non-OTA approach.

Proposal 2: Following candidate solutions can be considered for feasibility study:

- OTA solution 1: gNB -> UE

- OTA solution 2: CN -> UE via gNB

- OTA solution 3: OAM -> UE via gNB

Proposal 3: RAN2 focus to evaluate the feasibility of OTA solution 1, i.e. gNB->UE (CP and UP) via air interface.

Feasibility from RAN2 perspective:

Observation 5: Transmitting a 225MB dataset would take longer than the time a UE moving at pedestrian speed within the same cell in an urban area, making it impractical to transfer such large datasets without performing a handover or keeping the UE in CONNECTED mode for an extended period.

Observation 6: Transmitting large datasets/model parameters exceeding tens of megabytes via hundreds of RRC segments is impractical due to the complexity and inefficiency involved, making such an approach unfeasible.

Observation 7: UP tunnels are terminated between the UE and UPF, and the corresponding PDU sessions provide the end-to-end data path between the UE and the DN. The data content is not visible to the network side.

Proposal 4: RAN2 reply to RAN1 that it is not feasible to use CP to exchange the AI model parameters and/or dataset between NW-side and the UE over the air interface.

Proposal 5: RAN2 should inform RAN1 that it is not feasible to use the UP to exchange AI model parameters and/or datasets between the network side and the UE over the air interface in Rel-19.

Common Framework/Tunnel for UE-sided and Two-sided

Obervation 8: A common design for the framework and tunnel can be considered for both UE-sided and two-sided models to reduce complexity in both implementation and standardization, streamlining data collection and model transfer processes.

Observation 9: A specific radio bearer dedicated to model parameter and data transfer, terminated between the UE and NG-RAN, combines the advantages of SRB (data content visibility to NG-RAN) and DRB (unrestricted data amount). Additionally, it can be used for NW-side data collection.

Proposal 6: A common bi-directional tunnel can be used for both the UE-sided model and the two-sided model:

- UE-sided model: Data collection for model training is the upstream (from the UE to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

- Two-sided model with NW-first training: Model parameter and dataset transfer for UE-sided model training is the upstream (from NW-side to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

Proposal 7: RAN2 study the ways to make the dataset/model parameters accessible and visible to RAN and UE.

Reference

[1] R1-2410922, LS on signalling feasibility of dataset and parameter sharing, RAN1

[2] 3GPP TR 38.843 V18.0.0

[3] RP-250030, Reply LS on AI/ML UE sided data collection, SA5