3GPP TSG-RAN WG2 Meeting #129bis                                        R2-2502112

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.3.2.2

Source:	Mediatek Inc.

Title:	Generalization performance evaluation

Document for:	Discussion, Decision

Introduction

Agreements 1: two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study. 2: FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2). 3: agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2). Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 2: generalization parameters for FR1

In RAN2 #129 meeting, the agreements on the model generalization are given as follows.

According to the agreement, the FR1 is the primary focus. Thus, in this contribution, we present the simulation results of model generalization in RRM temporal domain case B (FR1) with different channel configurations (configuration #1 and configuration #2). The observations and analysis of the generalization result are provided.

Discussion

Generalization Evaluation for Temporal Domain Case B

In this contribution, we examine the impact of model generalization for the RRM temporal domain case B. The AI model is trained and used to perform inference at different configurations. We consider two different measurement reduction ratios, i.e., MRRT = 0.5 and 0.8. The observation and prediction patterns are given below.

Figure 1: observation and prediction pattern for (a) MRRT = 0.5 (b) MRRT = 0.8

In this simulation, we consider the sliding case, sub-use case 2, and cluster-based approach where both AI model input and output are L3 cell level RSRP of 21 cells. The approach is selected because, in the sliding case, UE can update the measurement filter more frequently, the sub-use case 2 can achieve the highest accuracy among all sub-use cases, and the cluster-based approach generally outperforms the cell-based approaches in our previous simulation result. Detailed AI model parameters and settings are given in the below table. The general simulation setting can be found in the Appendix.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| Flop | 3.02M |
| Model parameters | 204KB |
| Cluster approaches setting | Input: 21 cells, output: 21 cells |
| AI input/output | Input: L3 cell level RSRP, output: L3 cell level RSRP |

Table 1:AI model parameters and settings for intra-frequency temporal domain case B

In the following simulation, we examine the impact of different configurations (including different deployment scenarios, ISD, BS height, and Tx power). The baseline refers to the case where the training dataset and inference dataset are generated with the same configuration. The GC#1 refers to the case where the training dataset and inference dataset are mismatched, e.g., configuration #1 for training and configuration #2 for inference. The GC#2 refers to the case where the training dataset is a hybrid dataset that contains both configurations. We consider two different experiments A and B, where Experiment A considers the inference dataset with configuration#2 and Experiment B considers the inference dataset configuration#1. A non-AI result, e.g., sample and hold, is also provided as a reference.

| FR1, Temporal domain case B, sliding, sub-use case 2, 30km/hr, MRRT = 0.5 | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (sample and hold) |
| --- | --- | --- | --- | --- |
| Baseline A | Configuration #2 (UMa) | Configuration #2 (UMa) | 0.063 | 0.13 |
| GC#1 A | Configuration #1 (UMi) |  | 0.073 |  |
| GC#2 A | Hybrid |  | 0.068 |  |
| Baseline B | Configuration #1 (UMi) | Configuration #1 (UMi) | 0.091 | 0.277 |  |
| GC#1 B | Configuration #2 (UMa) |  | 0.114 |  |  |
| GC#2 B | Hybrid |  | 0.089 |  |  |

Table 2:  Model generalization - RRM temporal domain case B with different configurations (MRRT=0.5, 30km/hr)

| FR1, Temporal domain case B, sliding, sub-use case 2, 30km/hr, MRRT = 0.8 | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (sample and hold) |  |
| --- | --- | --- | --- | --- | --- |
| Baseline A | Configuration #2 (UMa) | Configuration #2 (UMa) | 0.26 | 0.307 |  |
| GC#1 A | Configuration #1 (UMi) |  | 0.321 |  |  |
| GC#2 A | Hybrid |  | 0.268 |  |  |
| Baseline B | Configuration #1 (UMi) | Configuration #1 (UMi) | 0.44 | 0.677 |
| GC#1 B | Configuration #2 (UMa) |  | 0.55 |  |
| GC#2 B | Hybrid |  | 0.47 |  |

Table 3: Model generalization - RRM temporal domain case B with different configurations (MRRT=0.8, 30km/hr)

| FR1, Temporal domain case B, sliding, sub-use case 2, 90km/hr, MRRT = 0.5 | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (sample and hold) |
| --- | --- | --- | --- | --- |
| Baseline A | Configuration #2 (UMa) | Configuration #2 (UMa) | 0.06 | 0.15 |
| GC#1 A | Configuration #1 (UMi) |  | 0.07 |  |
| GC#2 A | Hybrid |  | 0.07 |  |
| Baseline B | Configuration #1 (UMi) | Configuration #1 (UMi) | 0.09 | 0.301 |  |
| GC#1 B | Configuration #2 (UMa) |  | 0.13 |  |  |
| GC#2 B | Hybrid |  | 0.10 |  |  |

Table 4:  Model generalization - RRM temporal domain case B with different configurations (MRRT=0.5, 90km/hr)

| FR1, Temporal domain case B, sliding, sub-use case 2, 90km/hr, MRRT = 0.8 | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (sample and hold) |  |
| --- | --- | --- | --- | --- | --- |
| Baseline A | Configuration #2 (UMa) | Configuration #2 (UMa) | 0.309 | 0.375 |  |
| GC#1 A | Configuration #1 (UMi) |  | 0.382 |  |  |
| GC#2 A | Hybrid |  | 0.325 |  |  |
| Baseline B | Configuration #1 (UMi) | Configuration #1 (UMi) | 0.56 | 0.72 |
| GC#1 B | Configuration #2 (UMa) |  | 0.62 |  |
| GC#2 B | Hybrid |  | 0.55 |  |

Table 5: Model generalization - RRM temporal domain case B with different configurations (MRRT=0.8, 90km/hr)

Our result shows that the generalization of different configurations including GC#1 and GC#2 are acceptable with the comparison of baseline and non-AI approaches. One possible reason is that the AI model may learn to predict measurements based on historical measurements with similar logic. Another possible reason is the short prediction distance in this case, e.g., 40ms or 160ms, where the channel does not vary much. In most cases, GC#1(mismatched dataset) has much higher performance degradation compared to the baseline (matched dataset), while GC#2 (hybrid dataset) has close performance as the baseline. It should be noticed that most generalization cases including GC#1 and GC#2 still outperform the non-AI approach, e.g., sample and hold.

Observation 1: AI approaches demonstrate robust generalization performance across different cell configurations (e.g., configuration #1 and #2) in the RRM temporal domain case B.

Observation 2: For temporal domain case B with different cell configurations, GC#1 suffers a higher performance degradation while GC#2 has a similar performance compared to the baseline. In almost all cases, both GC#1 and GC#2 still outperform the non-AI approach, e.g., sample and hold.

Proposal 1: RAN2 capture the above observations in the TR.

Conclusion

Observation 1: AI approaches demonstrate robust generalization performance across different cell configurations (e.g., configuration #1 and #2) in the RRM temporal domain case B.

Observation 2: For temporal domain case B with different cell configurations, GC#1 suffers a higher performance degradation while GC#2 has a similar performance compared to the baseline. In almost all cases, both GC#1 and GC#2 still outperform the non-AI approach, e.g., sample and hold.

Proposal 1: RAN2 capture the above observations in the TR.

- Reference

- Appendix: Simulation Setting

| parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@4GHz, SCS: 30KHz | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model (7 sites, 3 sectors/cells per site) | 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; Inter-frequency correlation model is optional. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60, 90 km/h for temporal domain case B | 60, 90,120 km/h for temporal domain case A |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 1, 1, 1) |
| UE Antenna pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| Tx/Rx beam | Tx: 4, Rx:1 | Tx:32, Rx:4 |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 with coarse update | Procedure A in TR38.901 with coarse update |
| UE trajectory model | Option 1 | Option 1 |
| UE trajectory boundary processing model | Option 2: circle-bouncing model, | Option 2: circle-bouncing model, |
| Sampling period | 40ms | 20ms *4 = 80ms |