3GPP TSG-RAN WG2 Meeting #129bis                                        R2-2502113

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.3.5

Source:	Mediatek Inc.

Title:	Discussion on LCM and Specification Impact for AI Mobility

Document for:	Discussion, Decision

Introduction

In TR 38.843 [1], a general functional framework for life cycle management (LCM) is provided, as shown in the following figure.

The general description of data collection, model training, management, inference, and model storage are provided in TR 38.843. In principle, the LCM for AI mobility should follow the common mechanisms where applicable, such as data collection, model training, inference, model storage, etc. However, other functions may need further study, especially for those use-case-specific functions and procedures. In this contribution, we clarify the general principles and study scope of LCM for AI mobility. Discussions about the AI mobility-specific functions and procedures, including AI model management, the content of data (including monitoring data and inference data), and corresponding configurations, are provided. We also further discuss the potential impact on RRM and HO procedure with AI prediction.

Discussion

General Principle and Study Scope

According to TR 38.843, the general functional framework consists of the following functions:

- Data collection: This function provides input data to the model training, monitoring, and inference processes.

- Model training: This function performs AI/ML model training, validation, and testing.

- Management: This function oversees the operation (e.g., selection, (de)activation, switching, fallback) and monitoring of AI/ML models. It is also responsible for making decisions to ensure performance.

- Inference: This function provides the output of AI/ML models.

- Model storage: This function is responsible for storing trained/updated models.

In principle, those functions that are general for different AI use cases should be reused for AI mobility, such as data collection, model training, inference, and model storage. Additionally, the corresponding procedures, e.g., trained/updated model, model transfer/delivery request, and model transfer/delivery, discussed in the AI/ML for NR air interface SID should be reused if applicable. The procedures and mechanisms of (UE-side/network-side) data collection, model training, model transfer/delivery and applicability report/update designed for AI PHY use cases can be reused for AI mobility, although the detailed content of the signalings for those procedures will be AI mobility specific.

Observation 1: Those functions and procedures that are general for all AI use cases should be used as the baseline for AI mobility, including data collection, model training, applicability report, and model transfer/delivery.

Proposal 1: The procedures of data collection, model training, and applicability report model transfer/delivery designed for AI PHY use cases can be reused for AI mobility. Detailed content of the signalings for those procedures are FFS.

In AI mobility, RAN2 should focus on the study of those functions that depend on the specific use case, as shown below:

- Management (including monitoring and decision like activation/deactivation, switching and fallback)

- Content of data (including training data/monitor data/inference data)

- Corresponding configuration and report

  - Configuration for applicability report

  - Inference Configuration

  - Configuration for measurement for data collection for model training

  - Configuration for measurement for performance monitoring

- UE Capability report to support AI mobility

The management function includes how to perform model monitoring, e.g., how to compare the inference output and monitoring data (ground-truth measurement) and what the monitoring criteria are. The monitoring criteria should be defined based on each use case. For AI PHY use cases, the model monitoring and activation/deactivation, switching and fallback were discussed in RAN1. For AI mobility, the model monitoring and control mechanism should be discussed in RAN2.

The content of data is obviously dependent on the use case. For example, in the AI mobility use case, we may consider the L3 cell RSRP as inference data, which is different from the AI BM case that uses the L1 beam RSRP as model input in general. Besides the content of data, the configurations to enable AI mobility operation is also use-case specific, including inference configuration, configuration of measurement for data collection and monitoring, as well as the configuration for applicability report.  Therefore, RAN2 should identify the measurement target, e.g., measurement object (MO), and the corresponding occasion, e.g., period and offset, of the observation MO, and the information of predicted targets as well. For example, in inter-frequency prediction, the network needs to configure which MO should be measured and where to find the corresponding reference signal. The network also needs to configure the predicted MO and the corresponding predicted occasion, at least for the UE-sided model such that it can perform AI inference.

Since the above functions are closely related to the AI use case and can not be described in a general way as given in TR38.843, in the AI mobility SID, RAN2 should focus on the study of these functions and specify the necessary modification for the AI mobility use case.

Proposal 2: In AI mobility SI, RAN2 should focus on the study of those use-case-specific functions, such as management (monitoring and control), content of data (including training data/monitor data/inference data), and the corresponding configuration and report (for inference, data collection and monitoring).

Model Management

In TR38.843, five different common procedures for model management are provided, which are

- NW decision, NW-initiated

- NW decision, UE-initiated

- UE decision, event-triggered as configured by NW

- UE-autonomous, UE’s decision is reported to NW

- UE-autonomous, UE’s decision is not reported to NW

The illustrations of first four cases are given below.

Figure 1: Common framework of (a) NW decision, NW-initiated (b)NW decision, UE-initiated

Figure 2: Common framework of (c) UE decision, event triggered as configured by NW (d) UE-autonomous, UE’s decision is reported to NW

The model management function is responsible for controlling and determining whether an AI model is suitable or not. It can be divided into two different parts: monitoring and decision. The monitoring part contains collecting monitoring data and inference output and comparing them with appropriate monitoring criteria. The monitoring criteria must be properly selected to reflect reasonable model performance. Based on the monitoring results, the decision-making device can make the final management decision, e.g., selection, (de)activation, switching, fallback, etc.

Since the entire management function is highly related to whether inference, monitoring, and decision are performed on the UE side or the NW side, we will discuss the management procedure with UE-side models and NW-side models separately.

NW-sided Model

For NW-sided model, RAN2 have the following agreement in AI/ML for NR air interface as follows.

RAN#125bis

Agreements

1    RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2    RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3    RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

According to the agreement, the UE will only be involved in providing the required data if configured by the network. Both the monitoring and decision operations can be fully implemented by the network and do not need to be specified in the standard. Thus, the management should follow the common framework of case (a) NW decision, NW-initiated.

The network first configures the measurement for purpose of monitoring to the UE. Based on this configuration, the UE performs measurement and sends the measurement report to the network. The network can then compare the inference output with the monitoring data according to a certain monitoring metric.

Proposal 3: For NW-sided model, RAN2 consider NW-decision, NW-initiated management. The monitoring and decision take place on the NW side and can be fully NW-implemented. UE is only involved in providing the measurement for monitoring based on the configuration provided by NW.

UE-sided Model

For the UE-side model, the management decision could be made on either the UE side or the NW side.

- UE-decision approaches can save the measurement report and inference report overhead since all the necessary information is measured/generated on the UE side. UE does not need to send the report to NW to make the decision.

- NW-decision approaches allow the network to fully control the AI performance.

Both of them are reasonable options and should be studied.

Observation 2: For UE-sided model, the decision can be made by either NW or UE side.

Network-decision:

- UE performs monitoring or NW performs monitoring. As mentioned above, if the entire management function, including monitoring and decision, is performed by the network side, there is a significant additional report transmission overhead.

- UE performs monitoring and only reports the monitoring results to the network. The monitoring criteria are configured by the network so that the network can still fully control the performance of the AI model.

Since the frequency of monitoring could be much less than inference, and the content of the monitoring report could be much less than the inference output and monitoring data, the transmission overhead can be reduced compared to the NW-monitor case.

Observation 3: For NW-decision case, it is more efficient to perform monitoring on the UE side, as it does not require sending the inference output and monitoring data to the NW side.

Thus, we propose to consider NW-decision, UE-initiated management, as shown in the following figure, where the monitoring is performed on the UE side in this case.

Figure 4: Procedure of model management for UE-sided model - NW-decision, UE-initiated

For the UE-side model, it is also possible to allow the UE to make the management decision directly. To ensure the decision meets the requirements defined by the network, the UE's measurement and monitoring need to rely on the configuration given by the network. Additionally, after the decision is made, the UE must transmit the decision report to the network, as some decisions would trigger additional network-related actions.

For example, in the Temporal domain case B, if the management decision suggests that the UE should not use AI for prediction because the performance is lower than a given threshold (e.g., the management decision is fallback), the UE needs to inform the network. The network can then reconfigure the related measurement settings so that the UE will measure the target measurement object without measurement reduction anymore. The proposed UE-autonomous with decision report management procedure is shown in the following figure.

Figure 5: Procedure of model management for UE-sided model - UE-autonomous with decision report

Observation 4: For UE-decision case, it is more reasonable to consider UE-autonomous with decision report, since NW may need to reconfigure the measurement behavior depending on the management decision.

Proposal 4: For UE-sided model, RAN2 consider following management procedures as the baseline

- NW decision, UE initiated management

- UE-autonomous, UE’s decision is reported to NW

Proposal 5: For both management procedures, UE performs the required measurement and monitors the AI performance based on the configurations given by NW. If the final management decision is made by UE, i.e., case (b), the decision report is sent by UE to NW.

Content of Data and Configuration

In this section, we discuss the content of data (for inference and monitoring) as well as the corresponding configuration. For the RRM prediction use case (e.g., Scenarios 2, 3, 4), the input and output of the AI model depend on sub-use cases 1, 2, and 3. Therefore, the content of inference data (i.e., the model input) and monitoring data (i.e., the model output) should be chosen accordingly. According to the content of model input/output for each sub-use, a direct mapping of the inference data and monitoring data could be summarized as shown in the following table:

| RRM prediction | Sub-use case 1 | Sub-use case 2 | Sub-use case 3 |
| --- | --- | --- | --- |
| Inference data | L1 cell RSRP | L3 cell RSRP | L1 cell RSRP |
| Monitoring data | L1 cell RSRP | L3 cell RSRP | L3 cell RSRP |

Table 1: Content of inference and monitoring data for each sub-use case.

Proposal 6: For RRM prediction use cases (e.g., Scenario 2,3,4), the inference data could be considered as L1/L3 beam level RSRP and/or L1/L3 cell level RSRP.

Proposal 7: For RRM prediction use cases (e.g., Scenario 2,3,4), the monitoring data could be considered as L1/L3 beam level RSRP and/or L1/L3 cell level RSRP.

In addition to the content of data, RAN2 needs to further discuss the configuration of inference data, monitoring data, and monitoring criteria. For the inference data configuration, the observed target and predicted target should be specified so that the UE can identify where to find the required measurements as model input and what the predicted target is. The legacy RRM measurement configuration can be reused if applicable, including the description of measurement objects (MOs), which includes the information of frequency, reference signal configuration, and report configuration. RAN2 needs to further discuss necessary modifications to provide inference configuration, such as the separation of observed and predicted targets, the observation/prediction pattern (for Temporal domain case B), the prediction window (for Temporal domain case A), etc. Additionally, if the prediction target is a specific cell instead of all the cells belonging to the MO, for example, the prediction of the current serving cell, further modifications, e.g., additional indicators to specify the prediction of serving cell, are needed to support this.

For monitoring data configuration, RAN2 needs to discuss the monitoring criteria for UE-monitor management procedures. The performance metric and the corresponding mechanism to check the model performance are needed. For the RRM prediction use case, considering the average L1/L3 cell-level RSRP difference as the performance metric can be a starting point. RAN4 needs to be informed and involved in defining the requirement. An A3 event-like mechanism could be an option to reflect the model performance. Specifically, the monitoring device compares the RSRP difference between the inference output and monitoring data. If the RSRP difference remains higher than a predefined threshold for a given time, we could assume the model is out-of-date or not suitable. A monitoring report can be generated and sent to the decision device to trigger the necessary action, e.g., fallback.

Proposal 8: The RRM measurement configuration can be used as the baseline for inference and monitoring configuration for RRM prediction.

Proposal 9: The inference configuration should contain the information of the observed target and predicted target in the form of MOs as well as the linkage between them.

Proposal 10: The measurement quantity of RSRP difference is introduced for monitoring the AI RRM prediction. RAN4 needs to be informed and involved to define the requirement to determine whether the AI prediction is accurate or not.

Potential Specification Impact of HO Procedure with AI Prediction

Besides LCM, there is potential specification impact for AI mobility. In this section, we discuss the impact of HO procedure when AI prediction, e.g., predicted MR, is made and received by NW.

Based on previous agreements reached in RAN2, two options have been identified for the network's transmission of handover commands:

- Option 1: The NW sends handover command based on the actual Measurement Report (MR). The predicted MR is used solely to initiate an early handover preparation process.

- Option 2: The NW performs handover preparation and decides to transmit handover command based on the predicted MR. Under this approach, the handover command is sent immediately upon satisfaction of the A3 event criteria after HO preparation.

Option 1, which utilizes both actual and predicted Measurement Reports (MRs), would require significant amendments to the current specifications. This approach's dependence on both predicted MR and actual MR poses considerable challenges and has a significant impact on the existing specification framework. Conversely, Option 2 allows the network to treat predicted MRs as equivalent to actual MRs, eliminating the need to distinguish between predicted and actual reports. This approach minimizes the impact on existing specifications, offering a more seamless integration path.

Furthermore, the performance gains from Option 1 are limited to the time saved in handover preparation. The requirement to wait for the reception of actual MRs may still lead to Radio Link Failure (RLF) in certain scenarios. Additionally, based on results presented in the previous meeting, Option 2 demonstrates superior performance improvements compared to Option 1.

Observation 6: Option 2 demonstrates less impact on existing specifications compared to Option 1, while also offering superior performance improvements.

Proposal 11: HO procedure, Option 2 is considered for the normative work, i.e. the NW performs handover preparation and decides to transmit handover command based on the predicted MR.

During the AI implementation process, the sub-use case is identified as a crucial simulation parameter. RAN2 has evaluated simulation results for various sub-use cases. However, given that this is an implementation-based setting, we believe that its execution will not have any impact on the specifications.

Proposal 12: The choice and operation of different sub-use cases in RRM prediction is implemented (at either UE or network) and has no impact on specifications.

We propose 2 options for event prediction in our contributions [2], for option 1 UE does continuous inference and for option 2 UE starts AI inference when the entering condition of event is fulfilled. We would like to emphasize that the two options regarding when the UE initiates AI inference are also implementation-based. The choice between these two methods does not have any impact on the specifications.

Proposal 13: When UE starts AI inference in AI mobility is UE-implementation and has no impact on specifications.

Conclusion

Life cycle management (LCM) Part:

General principle and study scope:

Observation 1: Those functions and procedures that are general for all AI use cases should be used as the baseline for AI mobility, including data collection, model training, applicability report, and model transfer/delivery.

Proposal 1: The procedures of data collection, model training, and applicability report model transfer/delivery designed for AI PHY use cases can be reused for AI mobility. Detailed content of the signalings for those procedures are FFS.

Proposal 2: In AI mobility SI, RAN2 should focus on the study of those use-case-specific functions, such as management (monitoring and control), content of data (including training data/monitor data/inference data), and the corresponding configuration and report (for inference, data collection and monitoring).

Model Management (NW-sided model)

Proposal 3: For NW-sided model, RAN2 consider NW-decision, NW-initiated management. The monitoring and decision take place on the NW side and can be fully NW-implemented. UE is only involved in providing the measurement for monitoring based on the configuration provided by NW.

Model Management (UE-sided model)

Observation 2: For UE-sided model, the decision can be made by either NW or UE side.

Observation 3: For NW-decision case, it is more efficient to perform monitoring on the UE side, as it does not require sending the inference output and monitoring data to the NW side.

Observation 4: For UE-decision case, it is more reasonable to consider UE-autonomous with decision report, since NW may need to reconfigure the measurement behavior depending on the management decision.

Proposal 4: For UE-sided model, RAN2 consider following management procedures as the baseline

- NW decision, UE initiated management

- UE-autonomous, UE’s decision is reported to NW

Proposal 5: For both management procedures, UE performs the required measurement and monitors the AI performance based on the configurations given by NW. If the final management decision is made by UE, i.e., case (b), the decision report is sent by UE to NW.

Content of Data and configuration:

Proposal 6: For RRM prediction use cases (e.g., Scenario 2,3,4), the inference data could be considered as L1/L3 beam level RSRP and/or L1/L3 cell level RSRP.

Proposal 7: For RRM prediction use cases (e.g., Scenario 2,3,4), the monitoring data could be considered as L1/L3 beam level RSRP and/or L1/L3 cell level RSRP.

Proposal 8: The RRM measurement configuration can be used as the baseline for inference and monitoring configuration for RRM prediction.

Proposal 9: The inference configuration should contain the information of the observed target and predicted target in the form of MOs as well as the linkage between them.

Proposal 10: The measurement quantity of RSRP difference is introduced for monitoring the AI RRM prediction. RAN4 needs to be informed and involved to define the requirement to determine whether the AI prediction is accurate or not.

Potential specification impact of HO procedure with AI prediction:

Observation 6: Option 2 demonstrates less impact on existing specifications compared to Option 1, while also offering superior performance improvements.

Proposal 11: HO procedure, Option 2 is considered for the normative work, i.e. the NW performs handover preparation and decides to transmit handover command based on the predicted MR.

Proposal 12: The choice and operation of different sub-use cases in RRM prediction is implemented (at either UE or network) and has no impact on specifications.

Proposal 13: When UE starts AI inference in AI mobility is UE-implementation and has no impact on specifications.

Reference

[1] 	TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2]   R2-2502106 Discussion and preliminary results for direct and indirect event prediction