3GPP TSG-RAN WG2 Meeting #129bis				           R2-2502115

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Discussion on LCM for UE-sided model for beam management

Document for:	Discussion and Decision

# Introduction

In RAN2 #129, based on RAN1 LS reply, RAN2 concluded details of applicable functionalities reporting procedure with option A.

Agreements

1.	Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

2.	The full inference configuration is sent in CSI-ReportConfig.

3.	Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

4.	FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

5. 	Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

6.	If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

7. 	The provided periodic CSI configuration should be consistent with reported UE capabilities

8.	FFS option B

9.	Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI

In this contribution, we would like to discuss details of option B and remaining issues for LCM for BM use case.

# Option B

# Advantage of option B

In the last meeting, there was a question on the advantage of option B. In our view, the main benefit of option B compared to option A is so clear that gNB doesn’t need to provide full configuration to determine applicable functionality. Especially, in case of periodic CSI report, in option A, UE needs to activate upon receiving RRCReconfiguration if the CSI report is considered as applicable.

For example, let us suppose that UE supports only 1 periodic CSI report for BM use case (maxNumberPeriodicCSI-PerBWP-ForCSI-Report). With option A, gNB would configure only one potential candidate to ask UE for applicability reporting in each RRC reconfiguration message in order to avoid the case where the number of periodic CSI reports is activated more than 1 which is beyond UE capability.  If the provided CSI-Report Config is not applicable, the gNB should provide another periodic CSI-Report Config in the next RRC message. This procedure could iterate until applicable configuration is found and will cause additional delay in enable inference.

With option B, gNB can provide preferred multiple number of sets of inference related parameters regardless of UE capability. Even if UE indicates the multiple functionalities as applicable, gNB could just decide one of them based on UE capability.

This would provide more flexibility in gNB to select or switch functionality based on performance or internal algorithm.

Observation 1: advantage of option B is that gNB can request/check applicability of multiple functionalities regardless of UE capability for CSI report.

# Procedure/signaling for option B

In this section, we provide potential procedure/signaling for option B to demonstrate how it works.

In step 3, as agreed previously, gNB provides set(s) of inference related parameters for applicability report only. The exact location of set(s) of inference related parameters would not be so critical. That is, it can be located in the same location with CSI-ReportConfig (i.e. under CSI-MeasConfig in ServingCellConfig) or can be located in OtherConfig as it requires to trigger UAI eventually. Given that there was majority support on using OtherConfig, RAN2 can proceed to introduce set(s) of inference related parameters in OtherConfig.

Proposal 1: gNB provides set(s) of inference related parameters in OtherConfig for option B.

In Step 4, if there is at least one set of inference related parameters, UE need to report applicability. In order to have common design, we prefer that UE indicates applicability of set of inference related parameters in RRCReconfigurationComplete message upon reception Step 3. This is aligned with the previous agreement RAN2 made and it is applied to both option A and option B.

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

Proposal 2: UE reports applicability of each set of inference related parameters in RRCReconfigurationComplete in Step 4 for option B.

In addition, same as option A, the UE can also send the change of applicability status in UAI when the applicability status has changed after UE provides report of applicability status in Step 4.

Proposal 3: UE reports applicability of each set of inference related parameters in UAI when the applicability status has changed in Step 5 for option B.

# Operation of option A and option B

In order to merge or combine option A and option B, it is necessary to decide whether option A and option B can be simultaneously configured.

Technically, option A and option B can be used concurrently from signaling perspective. However, given that option A and option B would be decided considering the different network strategy/operation scenario e.g. multiple functionalities are supported or limited number of functionates is supported.

Therefore, there is not much motivation to configure option A and option B simultaneously. gNB configures CSI-ReportConfig to request applicability reporting (i.e. using option A) or gNB configures set(s) of inference related parameters to request applicability reporting (i.e. using option B). Mixed configuration should not be allowed e.g. gNB should not request some CSI-ReportConfigs and some sets of inference related parameters simultaneously for the same UE.

Proposal 4: either option A or option B is configured for one UE i.e either option for all configured functionalities.

If we assume that set(s) of inference related parameters is configured in otherConfig. UE can assume that if set(s) of inference related parameters is configured, option B is used. Otherwise, option A is used.

Proposal 5: option A or option B is implicitly indicated with the presence of a set of inference related parameters in otherConfig.

A remaining question is whether UE can also indicate applicability status for CSI-ReportConfig in option B in Step 5. One can consider that it is necessary for UE to indicate when a configured CSI-ReportConfig is non-applicable similar to option A. However, as long as the associated a set of inference related parameter is configured, the UE can still indicate the change of applicability status in UAI and gNB can be aware of changing of applicability status of the configured CSI-ReportConfig.

Proposal 6: in option B, the UE doesn’t send applicability indication for a configured CSI-ReportConfig.

The following shows an example of how option B is included based on the current applicabilityReportList provided by the CR rapporteur.

#### –	ApplicabilityReportList

The IE ApplicabilityReportList comprises information that the UE reports to gNB related to the applicability of the radio measurement prediction configurations at UE.

ApplicabilityReportList information element

-- ASN1START

-- TAG-APPLICABILITYREPORTLIST-START

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..FFS)) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19               ServCellIndex,

applicabilityReportType  CHOICE {

applicabilityReportConfigIdList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdList-r19,

applicabilityReportConfigIdOptionBList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdBList-r19,

}

...

}

ApplicabilityReportConfigIdList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19                CSI-ReportConfigId,

applicabilityStatus-r19                        ENUMERATED {true},

...

}

ApplicabilityReportConfigIdOptionBList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19                inferenceParameterId,

applicabilityStatus-r19                        ENUMERATED {true},

...

}

-- TAG-APPLICABILITYREPORTLIST-STOP

-- ASN1STOP

# Associated ID

RAN1 introduced associated ID to ensure consistence between inference and training. RAN1 agreed about the need of associated ID and how to provide associate IDs as follows.

For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

One remaining issue is how NW determines associated IDs. It is our understanding that it should be discussed in RAN2 because it requires overall system operation in NW to decide the characteristics of associated ID.

First characteristic to decide regarding associated ID would be whether it is cell specific or multiple-cell specific. Main consideration would be the granularity of categorizing dataset for model training. If it is cell specific, it would mean that UE should categorize dataset per cell and per associated ID, which requires UE to accommodate a lot of models and consequently more complexity/burden in model training at UE side.

Observation 2: cell specific associated ID will require UE to train a huge number of models and consequently more complexity/burden in model training at UE side.

In addition, NW side additional conditions may not be different in each cell because it is more related to network deployment and antenna configuration. One model trained for a certain network deployment can be reused to multiple cells.

Observation 3: NW side addition condition is likely to be the same for multiple cells with the same NW configuration.

On the other hand, from NW point of view, cell specific ID may be more preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

Observation 4: cell specific associated ID may be preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

Considering pros and cons for each option, it would be desirable to provide flexibility to UE/NW implementation. That is, the signaling of associated ID should be defined to support both approaches i.e. cell specific ID and multiple cell-specific IDs.

Proposal 7: RAN2 agree to support both cell specific ID and multiple cell-specific IDs for associated ID

From signaling point of view, there could be two options to define associate ID to be applied for multiple cells.

- Area specific associated ID: associated ID(s) is provided with a certain area ID similar to system information area ID in NR. Different area ID can be used to differentiate the same associated ID. Therefore, the actual associated ID is the combination of associated ID and area ID. With this approach, the same associated ID can be applied across multiple cells with the same area ID. Allocation and mapping of area IDs to specific areas/cells should be left to implementation. At least, we can assume that area ID should be unique within a PLMN.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

- Globally unique associated IDs within PLMN: associated IDs are consistently unique as long as the cell belongs to the same PLMN. Allocation and mapping of associate IDs to specific NW sided additional conditions should be left to implementation.

The two options are quite similar from operation point of view i.e. requiring pre-allocation by operators to make it unique in PLMN. Area specific associated ID would require more specification efforts to update associated IDs with the received area ID. Nevertheless, both options are feasible from signaling point of view.

Proposal 8: RAN2 discuss two options (area specific associated ID and globally unique associated IDs within PLMN) for associated IDs.

# Conclusion

In this document, we discussed details of option B and remaining issues for LCM for BM use case. Based on the discussion, we propose the following points.

Observation 1: advantage of option B is that gNB can request/check applicability of multiple functionalities regardless of UE capability for CSI report.

<option B>

Proposal 1: gNB provides set(s) of inference related parameters in OtherConfig for option B.

Proposal 2: UE reports applicability of each set of inference related parameters in RRCReconfigurationComplete in Step 4.

Proposal 3: UE reports applicability of each set of inference related parameters in UAI when the applicability status has changed.

<Operation of option A and option B>

Proposal 4: either option A or option B is configured for one UE i.e either option for all configured functionalities.

Proposal 5: option A or option B is implicitly indicated with the presence of a set of inference related parameters in otherConfig.

Proposal 6: in option B, the UE doesn’t send applicability indication for a configured CSI-ReportConfig.

<Associated ID>

Observation 2: cell specific associated ID will require UE to train a huge number of models and consequently more complexity/burden in model training at UE side.

Observation 3: NW side addition condition is likely to be the same for multiple cells with the same NW configuration.

Observation 4: cell specific associated ID may be preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

Proposal 7: RAN2 agree to support both cell specific ID and multiple cell-specific IDs for associated ID

Proposal 8: RAN2 discuss two options (area specific associated ID and globally unique associated IDs within PLMN) for associated IDs.

# Reference

[1] R2-2500011, Reply LS on applicable functionality reporting for beam management UE-sided model, RAN1