# 3GPP TSG RAN WG2 Meeting #129b      	                           R2-2502128
Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.1.5

Source:	Apple

Title:	Further discussion on parameters/model transfer in two-sided model

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- CSI feedback enhancement [RAN1]:

  - For CSI temporal prediction: further update TR 38.843 with additional evaluations.

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

The new Rel-19 SID of AI/ML for air interface Phase 2 (SID RP-243245) was agreed in RAN#106 [1]. The SI objective on model transfer/delivery and two-sided model can be found below:

Although RAN2 is not listed in SI objective of CSI compression, RAN1 sent LS (R1-2410922) [2] to request RAN2 to evaluate feasibility of identified solutions on two-sided model. To make more progress, RAN2 had a post-meeting email discussion [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson) to identify solutions for OTA approaches and non-OTA approaches [3].

In this contribution. we provide our view on some important aspects in summary report [3]:

- Feasibility of non-OTA approaches

- Feasibility of OTA approaches

- Feasibility of 3 options on parameter and/or dataset transfer

# 2 Discussion

## 2.1 Feasibility of non-OTA approaches

For non-OTA approaches, [3] identify the following solutions:

Proposal 3 [phase 2 10/14]: For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view. Feasibility analysis of non-OTA solutions is required to be evaluated by RAN3, SA2, and SA5. It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| - OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5 | Up to SA5 (any intermediate node, if any, between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| - CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2 | Up to SA2 (any intermediate node, if any, between CN and UE-side OTT server is up to SA2) |
| - gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5 | Up to RAN3, SA2, SA5 (any intermediate node, if any, between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

During post-meeting email discussion [3], we observe that the only technique concern on non-OTA approaches seems to be security concern on direct transfer from NW training entity to UE-side server (e.g. exposure of NW proprietary information). However, we don’t think this issue is valid:

- If the UE-side server is within MNO domain, it is trusted by MNO and discovered by other NF (e.g. NWDAF).

- If UE training entity is outside of MNO domain (untrusted domain), SA2 had introduced Network Exposure Function (NEF) to securely expose the services and capabilities of 3GPP to external applications (e.g. UE-side server performs as AF and communicates with 3GPP network via NEF exposure).

  - Similarly, section 6.2.8.2.4.3 of TS 23.288 [4] has specified the procedure on how AF in untrusted domain connects NWDAF via NEF, as copied below. Here, AF can be an UE-side server.

Observation 1: During post-meeting email discussion, main technique concern on non-OTA approaches is whether secure direct transfer from NW training entity to UE-side server. However, (NEF) bad been introduced to securely expose the services and capabilities of 3GPP to external applications (e.g. UE-side server performs as AF and communicates with 3GPP network via NEF exposure).

Observation 2: Section 6.2.8.2.4.3 of TS 23.288 has specified the procedure on how AF in untrusted domain connects NWDAF via NEF.

Thus, we propose RAN2 to confirm no technique issue on non-OTA approaches.

Proposal 1: For non-OTA approaches, RAN2 confirm that it has no security issue on direct transfer from NW training entity to UE-side server because UE-side server can perform as AF and securely communicates with 3GPP network (e.g. CN/OAM) via NEF exposure.

With Proposal 1, we don’t identify any technique showstopper for non-OTA approaches from RAN2 point of view.

Proposal 2: RAN2 conclude non-OTA approaches have no technical showstopper from RAN2 point of view. Send LS to RAN3/SA2/SA5 for feasibility evaluation.

## 2.2 Feasibility of OTA approaches

For OTA approaches, [3] identify the following solutions:

Proposal 4 [easy agreeable, phase 1 14/14]: From RAN2 point of view, when gNB is the NW dataset/model parameter collection entity, solution 1b (i.e. gNB -> UE via UP) is not considered as a candidate solution in 5GA.

Proposal 5 [easy agreeable, phase 1 14/14]: For OTA solutions ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but there’s no consensus on feasibility:

- OTA solution 1a: gNB -> UE via CP

- OTA solution 2: CN -> UE via gNB

- OTA solution 3: OAM -> UE via gNB

For OTA solution 1a, the main technique concern is that it requires UE to have a large RRC buffer (e.g. 226 Mbyte) and handle thousands of segments. To resolve these concerns, the proponents of solution 1a further proposed the following solutions in [3]:

- Alt-1: Enhance existing hard RRC segmentation with increased maximum of segments

- Alt-2: Soft RRC segmentation

- Alt-3: Segmentation in a new layer above RRC

- Alt-4: Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity.

We discuss these solutions one by one.

### 2.2.1 Alt-1: Enhance existing hard RRC segmentation with increased maximum number of segments

Some companies proposed to increase maximum number of RRC segments in [3]. However, even if we only consider 11.6MB, it still needs around 1288 RRC segments, which is significantly larger than existing supported RRC segment (9 in DL). We think it is not feasible for 5G UE to have such a large RRC buffer:

- Please note that section 4.2.14 of TS 38.306 [5] clearly captured below:

4.2.14	RRC buffer size

The RRC buffer size is defined as the maximum overall RRC configuration size that the UE is required to store. The RRC buffer size is 45Kbytes.

This limit was specified in Rel-15, and UE vendors had designed their memory access and hardware accelerators for ciphering based on it. Supporting >10Mbyte RRC buffer would require major re-design of the user plane hardware architecture, which is not realistic in late 5G.

- RRC buffer utilizes RAM memory which is a scarce resource rather than UE internal storage. And limited RAM memory for baseband needs to share with RF buffer (e.g. for analog beamforming), L2 buffer (e.g. for RLC/PDCP record/reassemble), and L1 buffer (e.g. for massive MIMO processing). We think it is quite challenging for 5G UE to have 11.6Mbyte RRC buffer which will significantly increase UE cost.

From Section 5.3.5.3 of TS 38.331:

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

omit unrelated part

4>	if any application layer measurement report container has been received from upper layers for which the successful transmission of the MeasurementReportAppLayer message or at least one segment of the message via SRB4 (if reconfigurationWithSync was included in masterCellGroup) or SRB5 (if reconfigurationWithSync was included in secondaryCellGroup) has not been confirmed by lower layers:

5>	if RRC segmentation was used for the MeasurementReportAppLayer message:

6>	if RRC segmentation is enabled based on the field rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 for the reportingSRB (or SRB4 if reportingSRB is not configured):

7>	re-submit all segments of the MeasurementReportAppLayer message to lower layers for transmission via the reportingSRB (or SRB4 if reportingSRB is not configured);

omit unrelated part

Meanwhile, RRC segment continuity during handover is not supported in legacy RRC [6].

Thus, we think service continuity requirement can’t be satisfied, especially if parameter/dataset have a huge number of 1288 RRC segments.

Observation 3: Enhancing existing hard RRC segmentation with 1000+ segments is not feasible in late 5G:

- It requires the UE to have large RRC buffer (e.g. 11.6 MB). It needs major re-design of the user plane hardware architecture and significantly increases UE cost because RRC buffer utilizes RAM memory.

- Service continuity requirement can’t be satisfied for 1000+ RRC segments because RRC segment continuity during handover is not supported.

Thus, we propose:

Proposal 3: RAN2 conclude that enhancing existing hard RRC segmentation with 1000+ segments is not feasible for OTA solution 1a because large RRC buffer needs major re-design of the user plane hardware architecture and significantly increases UE cost due to expensive RAM memory.

### 2.2.2 Alt-2: Soft RRC segmentation

In Rel-16 WI of RACS, RRC segmentation is specified to be performed after ASN.1 encoding and thereby it is “hard” RRC segmentation (i.e. the RX can decode the RRC message only after assembling all segments). “Soft” RRC segmentation was studied in Rel-16 SI of RACS as captured in [7] but it was not adopted by 3GPP. It segments a large RRC information into several independent parts where each part can be individually decoded and processed.

We don’t think it can work for parameter/dataset delivery:

- Legacy RRC segmentation is already specified to be performed after ASN.1 encoding (i.e. “hard” segmentation). It doesn’t make sense to introduce two kinds of RRC segmentations for same purpose.

- As discussed in Rel-16 RACS, the “Soft” RRC segmentation will put a high requirement in TX side which needs to fully comprehend the raw data and smartly segment them to remove dependency across any two segments. Furthermore, ASN.1 encoding and decoding need to be performed multiple times which adds more complexity in both UE and network side.

Observation 4: As discussed in Rel-16 RACS, the “Soft” RRC segmentation will put a high complexity in TX side because it needs to ensure each segment can be be individually decoded. Furthermore, ASN.1 encoding and decoding need to be performed multiple times which adds more complexity in both UE and network side.

Thus, we propose:

Proposal 4: RAN2 conclude “Soft” RRC segmentation is not feasible for OTA solution 1a because it put a high requirement in gNB to ensure each segment can be be independently decoded. Furthermore, ASN.1 encoding and decoding need to be performed multiple times which adds more complexity in both UE and NW.

### 2.2.3 Alt-3: Segmentation in a new layer above RRC

This solution is proposed in [8][9]. It may address the concern of RRC buffer. However, we are concerned with its potential huge specification work, which is expected to at least include the following long list:

- Design a new SRB protocol stack with a new specified above RRC (e.g. AI/ML layer)

  - It at least includes design on how to generate AI/ML PDU, design AI/ML PDU header with segments information and status report PDU for loss-less delivery (as proposed in [8]).

- Introduce RRC segments transmission continuity mechanism:

  - As in Observation 3, RRC segment transmission continuity during HO is not supported in legacy RRC.

- Introduce SRB recovery mechanism:

  - Note that TS 38.322 only supports data recovery in DRB.

- New SRB design, including priority and how to avoid blocking other RRC message

  - TS 38.300 specify all segments of an RRC message are transmitted before sending another RRC message:

7.10	Segmentation of RRC messages

An RRC message may be segmented in case the size of the encoded RRC message PDU exceeds the maximum PDCP SDU size. Segmentation is performed in the RRC layer using a separate RRC PDU to carry each segment. The receiver reassembles the segments to form the complete RRC message. All segments of an RRC message are transmitted before sending another RRC message. Segmentation is supported in both uplink and downlink as specified in TS 38.331 [12].

- How to select UE only from one specific UE vendor, which requires UE vendor info to share with gNB.

- It may need a new stage 3 specification (e.g. similar to SDAP stage 3 specification TS 37.324),

Observation 5: Segmentation in a new layer above RRC is not feasible in late 5G:

- Design a new SRB protocol stack with a new specified layer above RRC and a new stage 3 specification.

- Introduce RRC segments transmission continuity mechanism.

- Introduce SRB recovery mechanism.

- New SRB design, including priority and how to avoid blocking other RRC message

- How to select UE only from one specific UE vendor, which requires UE vendor info to share with gNB.

Thus, we propose:

Proposal 5: RAN2 conclude “segmentation in a new layer above RRC (e.g. AI/ML layer)” is not feasible for OTA solution 1a because its required specification work is not manageable.

### 2.2.4 Alt-4: Split large dataset/model parameter into small pieces for multiple UEs

This solution is proposed in [8]. In detail, gNB splits the overall dataset into many subsets each with a limited number of data samples within RRC message payload limitation, which can be separately sent to different UEs and then gather by UE training entity.

This is an interesting approach. However, we are still questioned on its feasibility due to below concerns:

- It may work for splitting dataset (i.e. option 4-1) because there are some dependency / correlations between different small pieces of dataset. Even if some pieces are lost during OTA transmission, it may be recovered during re-training of UE part model. However, we understand it can’t work for model parameter (i.e. two variants of option 3a-1) because existing data formats of model parameters in industrial are significantly compressed due to consideration of storage and open sourcing. Thus, different small pieces of model parameter are almost independent, and missing any piece may lead to failure of reconstructing model.

Observation 6: “Split into small pieces for multiple UEs” may work for dataset (i.e. option 4-1) because there are some dependency / correlations between different small pieces. However, it can’t work for model parameter (i.e. two variants of option 3a-1) because existing data formats of model parameters in industrial are significantly compressed due to storage and open sourcing. Missing any piece may lead to failure of reconstructing model.

- Need to support selecting UE only from one specific UE vendor, which requires UE vendor info to share with gNB.

  - Please note that existing UE capability do not include the UE’s vendor information and the device type information because they are UE’s internal information and can’t be exposed to NW.

- Need to support aggregating pieces of multiple UEs in UE-side server, especially during HO and re-establishment

  - To support dataset aggregation, “subset ID” may be needed, but it is not clear whether such ID is in RRC or application layer. Furthermore, “subset ID” may not be sufficient if we have requirement of in-order delivery and transmission continuity in dataset aggregation.

Observation 7: “Split into small pieces for multiple UEs” needs to support selecting UE only from one specific UE vendor and aggregating pieces of multiple UEs in UE-side server, especially during HO and re-establishment.

Thus, we propose:

Proposal 6: RAN2 conclude that “Split into small pieces for multiple UEs” is not feasible for model parameter (i.e. two variants of option 3a-1) because existing data formats of model parameters in industrial are significantly compressed due to storage and open sourcing. Missing any piece may lead to failure of reconstructing model.

Proposal 7: RAN2 conclude that “Split into small pieces for multiple UEs” is not feasible for OTA solution 1a because it is challenging how to support selecting UE only from one specific UE vendor and aggregating pieces of multiple UEs in UE-side server (especially during HO and re-establishment).

### 2.2.5: Security concern on OTA transmission

For all variants of OTA solution 1a, we think the following security concerns are common:

- NW data leakage due to UE selection: NW may blindly select UE of wrong UE vendor and exposure NW proprietary information.

- OTA transmission on dataset/parameter from gNB to UE is not secure enough (e.g. OTA decoded by malicious UE nearby).

Proposal 8: If OTA solution 1a is considered, the following security concerns need to be addressed:

- NW data leakage due to UE selection: NW may blindly select UE of wrong UE vendor and exposure NW proprietary information.

- OTA transmission on dataset/parameter from gNB to UE is not secure enough (e.g. OTA decoded by malicious UE nearby).

## 2.3 Feasibility of 3 options on parameter and/or dataset transfer

According to our Proposal 1-8, we conclude that OTA approaches are not feasible for any of the 3 dataset/parameter transfer options listed by RAN1 LS [2]. Thus, we propose:

Proposal 9: OTA approaches are not feasible for any of the 3 dataset/parameter transfer options listed by RAN1 LS (i.e. option 4-1 and two variants of option 3a-1).

For non-OTA approaches, we understand that there is no much difference among the 3 dataset/parameter transfer options from RAN2 perspective because the tunnel between NW training entity and UE-side server (e.g. IP tunnel) can easily handle the maximum payload size identified by RAN1 (i.e. 225 MB in option 1).

Observation 8: For non-OTA approaches, there is no much difference among the 3 dataset/parameter transfer options from RAN2 perspective because the tunnel between NW training entity and UE-side server (e.g. IP tunnel) can easily handle the maximum payload size identified by RAN1 (i.e. 225 MB in option 1).

Thus, we propose:

Proposal 10: For non-OTA approaches, RAN2 conclude that all the 3 solutions identified by RAN1 (i.e. option 4-1 and two variants of option 3a-1) are feasible. It is up to RAN1 to determine which option to adopt.

# 3 Conclusion

In this contribution, we share our view on dataset/parameter transfer in two-sided model. Our observations are:

Observation 1: During post-meeting email discussion, main technique concern on non-OTA approaches is whether secure direct transfer from NW training entity to UE-side server. However, (NEF) bad been introduced to securely expose the services and capabilities of 3GPP to external applications (e.g. UE-side server performs as AF and communicates with 3GPP network via NEF exposure).

Observation 2: Section 6.2.8.2.4.3 of TS 23.288 has specified the procedure on how AF in untrusted domain connects NWDAF via NEF.

Observation 3: Enhancing existing hard RRC segmentation with 1000+ segments is not feasible in late 5G:

- It requires the UE to have large RRC buffer (e.g. 11.6 MB). It needs major re-design of the user plane hardware architecture and significantly increases UE cost because RRC buffer utilizes RAM memory.

- Service continuity requirement can’t be satisfied for 1000+ RRC segments because RRC segment continuity during handover is not supported.

Observation 4: As discussed in Rel-16 RACS, the “Soft” RRC segmentation will put a high complexity in TX side because it needs to ensure each segment can be be individually decoded. Furthermore, ASN.1 encoding and decoding need to be performed multiple times which adds more complexity in both UE and network side.

Observation 5: Enhance existing hard RRC segmentation with 1000+ segments is not feasible in late 5G:

- Design a new SRB protocol stack with a new specified layer above RRC and a new stage 3 specification.

- Introduce RRC segments transmission continuity mechanism.

- Introduce SRB recovery mechanism.

- New SRB design, including priority and how to avoid blocking other RRC message

- How to select UE only from one specific UE vendor, which requires UE vendor info to share with gNB.

Observation 6: “Split into small pieces for multiple UEs” may work for dataset (i.e. option 4-1) because there are some dependency / correlations between different small pieces. However, it can’t work for model parameter (i.e. two variants of option 3a-1) because existing data formats of model parameters in industrial are significantly compressed due to storage and open sourcing. Missing any piece may lead to failure of reconstructing model.

Observation 7: “Split into small pieces for multiple UEs” needs to support selecting UE only from one specific UE vendor and aggregating pieces of multiple UEs in UE-side server, especially during HO and re-establishment.

Observation 8: For non-OTA approaches, there is no much difference among the 3 dataset/parameter transfer options from RAN2 perspective because the tunnel between NW training entity and UE-side server (e.g. IP tunnel) can easily handle the maximum payload size identified by RAN1 (i.e. 225 MB in option 1).

Based on observations, our proposals can be found below:

Feasibility of non-OTA approaches

Proposal 1: For non-OTA approaches, RAN2 confirm that it has no security issue on direct transfer from NW training entity to UE-side server because UE-side server can perform as AF and securely communicates with 3GPP network (e.g. CN/OAM) via NEF exposure.

Proposal 2: RAN2 conclude non-OTA approaches have no technical showstopper from RAN2 point of view. Send LS to RAN3/SA2/SA5 for feasibility evaluation.

Feasibility of OTA approaches

Proposal 3: RAN2 conclude that enhancing existing hard RRC segmentation with 1000+ segments is not feasible for OTA solution 1a because large RRC buffer needs major re-design of the user plane hardware architecture and significantly increases UE cost due to expensive RAM memory.

Proposal 4: RAN2 conclude “Soft” RRC segmentation is not feasible for OTA solution 1a because it put a high requirement in gNB to ensure each segment can be be independently decoded. Furthermore, ASN.1 encoding and decoding need to be performed multiple times which adds more complexity in both UE and NW.

Proposal 5: RAN2 conclude “segmentation in a new layer above RRC (e.g. AI/ML layer)” is not feasible for OTA solution 1a because its required specification work is not manageable.

Proposal 6: RAN2 conclude that “Split into small pieces for multiple UEs” is not feasible for model parameter (i.e. two variants of option 3a-1) because existing data formats of model parameters in industrial are significantly compressed due to storage and open sourcing. Missing any piece may lead to failure of reconstructing model.

Proposal 7: RAN2 conclude that “Split into small pieces for multiple UEs” is not feasible for OTA solution 1a because it is challenging how to support selecting UE only from one specific UE vendor and aggregating pieces of multiple UEs in UE-side server (especially during HO and re-establishment).

Proposal 8: If OTA solution 1a is considered, the following security concerns need to be addressed:

- NW data leakage due to UE selection: NW may blindly select UE of wrong UE vendor and exposure NW proprietary information.

- OTA transmission on dataset/parameter from gNB to UE is not secure enough (e.g. OTA decoded by malicious UE nearby).

Feasibility of 3 options on parameter and/or dataset transfer

Proposal 9: OTA approaches are not feasible for any of the 3 dataset/parameter transfer options listed by RAN1 LS (i.e. option 4-1 and two variants of option 3a-1).

Proposal 10: For non-OTA approaches, RAN2 conclude that all the 3 solutions identified by RAN1 (i.e. option 4-1 and two variants of option 3a-1) are feasible. It is up to RAN1 to determine which option to adopt.

# 4 References

[1] RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm.

[2] R1-2410922, LS on signalling feasibility of dataset and parameter sharing.

[3] R2-2501939, Report of [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson), Xiaomi/Ericsson.

[4] TS 23.288- v19.1.0, Architecture enhancements for 5G System (5GS) to support network data analytics services (Release 19), 2024-12.

[5] TS 38.306-v18.0.0, User Equipment (UE) radio access capabilities, 2023-12.

[6] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[7] TR37.873, Study on optimizations of UE radio capability signalling; (Release 16)

[8] R2-2501111, Discussion on signalling feasibility of dataset and parameter sharing for CSI compression, Huawei.

[9] R2-2500128, Discussion on signaling feasibility of dataset and parameter, vivo.