# 3GPP TSG RAN WG2 Meeting #129b      	                           R2-2502130
Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.2

Source:	Apple

Title:	Remaining issues on on-demand SSB for SCell

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

By RAN2#129 [2], OD-SSB made some progress on MAC-CE format and L3 measurements. In this contribution, we continue to discuss remaining issues of OD-SSB.

# 2 Discussion

RAN2#127 agreed to first study Scenario 2/2A [3] and RAN1 precluded Scenario 3A [4] (i.e. only Scenario 3B is FFS).

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

We provide Figure.1 to illustrate Scenario 2/2A for ease of followed discussion.

Figure.1 Illustration of considered scenarios (scenario 2/2A) of OD-SSB

Meanwhile, RAN1 also agreed to support Case 1 and Case 2, which are illustrated in Figrue.2 for ease of followed discussion.

- Case 1: No always-on SSB on the SCell.

- Case 2: Always-on SSB is also periodically transmitted on the SCell besides OD-SSB.

Figure. 2 Illustration of Case 1 and Case 2 of OD-SSB

## 2.1 Further details of OD-SSB MAC CE

In RAN2#127b [5], below high-level agreements on OD-SSB MAC-CE format were made:

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Some detailed design options on MAC-CE format were discussed during RAN2#127b [5]. However, as RAN1 is still discussing what information needs to be included in OD-SSB MAC-CE, no conclusion was made. The related RAN1 progress includes below agreements:

Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Although RAN1 design is not finished, we think some high level principle of OD-SSB MAC-CE design can be discussed for progress.

First, we would like to discuss how to indicate the SCell(s) with OD-SSB activation. In RAN2#127b [3], there are mainly three kinds of solutions:

- Alt-1: Introduce a fixed sized MAC-CE with a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE)

- Alt-2: Introduce a variable sized MAC-CE with explicit Serving Cell ID(s) with activated OD-SSB

- Alt-3: Introduce a fixed sized MAC-CE with only one explicit Serving Cell ID with activated OD-SSB (i.e. multiple MAC-CEs are needed if more than one serving cells have activated OD-SSB)

We prefer Alt-1 due to below reason:

- Alt-3 is precluded by below RAN2#127b agreement:

NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

- Alt-2 has benefit of less overhead only when the number of SCells with activated OD-SSB is small. When the SCell number is large, its overhead is higher than Alt-1. Meanwhile, MAC-CE with variable size will bring extra complexity in UE processing.

Thus, we propose:

Proposal 1: The OD-SSB MAC-CE includes one fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

Please note that legacy SCell A/D MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells, according to TS 38.321 [6]. Similarly, the new OD-SSB MAC-CE should also support the two formats.

Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Then, we discuss how to include other information (e.g. periodicity of OD-SSB) in the new MAC-CE. As can be observed, RAN1 agreed to include periodicity of OD-SSB in MAC-CE, but other parameters were not precluded. In RAN2#127b [3], there are mainly two kinds of solutions to include periodicity of OD-SSB in MAC-CE:

- Alt-1: Include actual values of OD-SSB periodicities (5/10/20/40/80/160ms) in MAC-CE

- Alt-2: Include configuration indices of candidate SSB periodicities in RRC in MAC-CE

We prefer Alt-2 due to below reasons:

- OD-SSB periodicity is expected to be much shorter than always-on SSB. Thus, supporting all possible SSB periodicity values are not necessary.

- Alt-2 is more forward compatible: other agreed OD-SSB information can also be included in RRC together with periodicity under the same configuration index.

Proposal 3: OD-SSB MAC-CE includes another bitmap to indicate the index of candidate configurations of OD-SSB in RRC, where each candidate configuration may include SSB periodicity and other RAN1 agreed OD-SSB parameters (e.g. od-ssb-PositionsInBurst). FFS its detailed format.

One example of OD-SSB MAC-CE with 7 SCells and up to 2 candidate OD-SSB configurations in RRC can be found in Figure.3, where Bi (i=1-7) indicates the activation status of OD-SSB in SCell with SCellIndex i and Ci (i=1-7) indicates whether 1st or 2nd candidate value of OD-SSB periodicity is applied in SCell with SCellIndex i.

Figure.3 Example of OD-SSB MAC-CE with 7 SCells and up to 2 candidate values of OD-SSB periodicity

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

In RAN1#118 [7], it was agreed to support both implicit deactivation and explicit deactivation.

During online discussion of RAN2#129 [2], some companies raised the concern that if both explicit deactivation and implicit deactivation are signaled in one single MAC-CE, it may bring the additional complexities. We have some sympathy with this issue. However, we are not sure whether it is necessary to include both types of deactivation indication in the same MAC-CE. It is just making things complex but doesn’t bring any benefit.

Observation 1: RAN1 agreed to support both implicit and explicit deactivation. However, RAN1 didn’t agree both types of deactivation indications can be included in the same MAC-CE, and it doesn’t bring any benefit.

As RAN1 agreement don’t mention whether both can be included in same MAC-CE, we think RAN2 can go one step further to preclude it. If this can be agreed, we don’t need to introduce two separate MAC-CEs for implicit deactivation and explicit deactivation. Thus, we propose:

Proposal 4a: For one Rel-19 UE, the Network configures whether to use either implicit deactivation or explicit deactivation (but not both) of OD-SSB transmission in all its SCells (i.e. per UE rather than per SCell config).

Proposal 4b: If Proposal 4a can be agreed, RAN2 confirm that it is not necessary to introduce two separate MAC-CEs for implicit deactivation and explicit deactivation respectively.

## 2.2 L3 measurement on OD-SSB

L3 measurement on OD-SSB was discussed in RAN2#127 [3] and RAN2#129 [2], and below agreements were made:

Agreements on OD-SSB in RAN2#127

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB in RAN2#129

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

We discuss L3 measurement for case 1 and case 2, respectively

### 2.2.1 How to support L3 measurement in case 1

In case 1, there is only OD-SSB in SCell but A-SSB is not transmitted, which is simpler than case 2. In RAN2#127 [3], it was agreed that the UE doesn’t expect to measure SSB when OD-SSB is deactivated in case 1, i.e. the UE measures SSB only when on-demand SSB is activated.

Agreements on OD-SSB in RAN2#127

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

Since the activation / deactivation of OD-SSB transmission is indicated by MAC-CE / RRC, we think the simplest solution is described as follows:

- RRC based OD-SSB activation / deactivation

  - No specification change is foreseen because NW can de-configure servingCellMO via RRC when the OD-SSB is deactivated.

- MAC-CE based OD-SSB activation / deactivation

  - Specify the UE to start L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation MAC-CE, and the UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

    - Please note that we can’t reuse the solution of RRC based OD-SSB activation/deactivation because the UE is mandated to perform serving cell measurement when servingCellMO is configured, according to Section 5.3.3.1 of TS 38.331 [8].

From Section 5.3.3.1 of TS 38.331

omit unrelated part

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

omit unrelated part

Observation 2: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

Thus, we propose:

Proposal 5: In L3 measurement in OD-SSB case 1, if RRC based activation / deactivation, no specification change is foreseen because NW can de-configure servingCellMO via RRC when the OD-SSB is deactivated.

Proposal 6: In L3 measurement in OD-SSB case 1, if MAC-CE based activation / deactivation, introduce the following RRC specifications changes:

- The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation MAC-CE

- The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

### 2.2.2 How to support L3 measurement in case 2

In case 2, both OD-SSB and A-SSB are transmitted in SCell. Meanwhile, According to RAN1#120 agreement [4], we can further divide into the following two sub-cases:

- Sub-case 1: A-SSB and OD-SSB have the same frequency

- Sub-case 2: A-SSB and OD-SSB have different frequencies

#### 2.2.2.1 Sub-case 1: A-SSB and OD-SSB have the same frequency

When A-SSB and OD-SSB share the same centre frequency, the key issue of L3 measurement is whether either of them is measured or both are measured. In RAN2#129 [2], it was agreed to leave the study of case 2 to RAN4, but we believe the intention is only target for sub-case 1 (i.e. no intention to cover sub-case 2 as well).

Agreements on OD-SSB in RAN2#129

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

Proposal 7: When A-SSB and OD-SSB share the same center frequency in Case 2, RAN2 wait RAN4 conclusion before evaluation of RAN2 specification impacts on L3 measurement.

#### 2.2.2.2 Sub-case 2: A-SSB and OD-SSB have different frequency

Please note that RAN1#120 agreed that A-SSB and OD-SSB may have different frequency locations, and UE is not required to measure both AO-SSB and OD-SSB in this case [4].

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Observation 3: RAN1#120 agreed that A-SSB and OD-SSB may have different frequency locations, and UE is not required to measure both AO-SSB and OD-SSB in this case.

On its impacts on L3 measurement, we think there are two solutions:

- Solution 1: Frequency of OD-SSB is added in servingCellMO identified by frequency of A-SSB

- Solution 2: Introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB (similar to Redcap)

We have concern on Solution 1. If it is supported, we believe it will introduce significant changes in RAN2 spec, including TS 38.331 [8] and TS 38.300 [9]. As follows, we provide our analysis on required spec changes:

- L3 RRM framework and configuration (Section 5.5.1/5.5.2): it challenges the basic L3 RRM framework which is per-frequency configured and per-cell reporting. Essentially, it requires either per-cell L3 measurement configuraiton with two ssbFrequency.

- Performing measurement (Section 5.5.3): existing Section 5.5.3 uses loop structure to specify the UE to perform L3 measurements independently for each measId (and its associated single MO). However, it requires the UE to derive cell measurement result with SSB across different measId, and thereby the structure of procedure text needs to be changed.

- Measurement report triggering (Section 5.5.4): similar to Section 5.5.3, existing Section 5.5.4 uses loop structure to specify the UE to trigger measurement report independently for each measId based on L3 cell level measurement derived from one measId. It may also require change of whole structure of procedure text to allow the UE to determine whether to trigger event based on cell measurement result derived from SSB across different measId.

- Measurement reporting (Section 5.5.5): existing Section 5.5.5 specify the UE to use measId to identify the triggered measurement reporting. However, it is not clear whether one measId is sufficient if always-on SSB and OD-SSB of the same serving cell are configured in the same MOs.

Observation 4: If frequency of OD-SSB is added in servingCellMO identified by frequency of A-SSB, it will introduce significant spec changes in TS 38.331:

- It challenges the basic L3 RRM framework which is per-frequency configured.

- It challenges the basic loop structure of independent processing of L3 measurements for each measId in Section 5.5.3 / 5.5.4 / 5.5.5 of TS 38.331.

We think Solution 2 can work. Please note that Rel-17 Redcap has made similar specification work in RRC spec [8], as illustrated in Figure. 4.

Observation 5: In Rel-17 Redcap, a separate servingCellMO is added under ServingCellConfig to indicate MO of NCD-SSB.

Figure. 4 Illustration of RRC spec change to configure another servingCellMO for NCD-SSB in Rel-17 Redcap

Its further spec impact is smaller. Thus, we propose to support Solution 2.

Proposal 8: When A-SSB and OD-SSB have different center frequency in Case 2, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB (similar to Redcap).

# 3 Conclusion

In this contribution, we discuss remaining issues on-demand SSB for SCell. Our observations are:

Observation 1: RAN1 agreed to support both implicit and explicit deactivation. However, RAN1 didn’t agree both types of deactivation indications can be included in the same MAC-CE, and it doesn’t bring any benefit.

Observation 2: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

Observation 3: RAN1#120 agreed that A-SSB and OD-SSB may have different frequency locations, and UE is not required to measure both AO-SSB and OD-SSB in this case.

Observation 4: If OD-SSB and always-on SSB can have different SSB carrier frequencies for one serving cell, it will introduce significant spec changes in RAN2 spec, including TS 38.331 and TS 38.300:

- It challenges the basic UE assumption that SSBs in two MOs are regarded as different cells and definition of intra/inter-frequency SSB measurement captured in Section 9.2.4 of TS 38.300.

- It challenges the basic L3 RRM framework which is per-frequency configured. Essentially, it requires either per-cell configuration with two ssbFrequency, or one cell can be included in two different MOs.

- It challenges the basic loop structure of independent processing of L3 measurements for each measId in Section 5.5.3 / 5.5.4 / 5.5.5 of TS 38.331. Coupled processing across different measId is needed to derive L3 cell measurement result with SSBs across different measId.

Observation 5: In Rel-17 Redcap, a separate servingCellMO is added under ServingCellConfig to indicate MO of NCD-SSB.

Based on above observations, our proposals are:

MAC-CE format

Proposal 1: The OD-SSB MAC-CE includes one fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 3: OD-SSB MAC-CE includes another bitmap to indicate the index of candidate configurations of OD-SSB in RRC, where each candidate configuration may include SSB periodicity and other RAN1 agreed OD-SSB parameters (e.g. od-ssb-PositionsInBurst). FFS its detailed format.

Proposal 4a: For one Rel-19 UE, the Network configures whether to use either implicit deactivation or explicit deactivation (but not both) of OD-SSB transmission in all its SCells (i.e. per UE rather than per SCell config).

Proposal 4b: If Proposal 4a can be agreed, RAN2 confirm that it is not necessary to introduce two separate MAC-CEs for implicit deactivation and explicit deactivation respectively.

L3 measurement in NES case 1

Proposal 5: In L3 measurement in OD-SSB case 1, if RRC based activation / deactivation, no specification change is foreseen because NW can de-configure servingCellMO via RRC when the OD-SSB is deactivated.

Proposal 6: In L3 measurement in OD-SSB case 1, if MAC-CE based activation / deactivation, introduce the following RRC specifications changes:

- The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation MAC-CE

- The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

L3 measurement in NES case 2

Proposal 7: When A-SSB and OD-SSB share the same center frequency in Case 2, RAN2 wait RAN4 conclusion before evaluation of RAN2 specification impacts on L3 measurement.

Proposal 8: When A-SSB and OD-SSB have different center frequency in Case 2, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB (similar to Redcap).

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#129, Chair Notes.

[3] RAN2#127, Chair Notes.

[4] RAN1#120, Chair Notes.

[5] RAN2#127b, Chair Notes.

[6] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[7] RAN1#118, Chair Notes.

[8] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[9] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.