# 3GPP TSG RAN WG2 Meeting #129b      	                           R2-2502131
Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.3

Source:	Apple

Title:	Remaining issues on on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective on on-demand SIB1 (OD-SIB1) for UEs in IDLE/INACTIVE state is copied as below:

In this contribution, we share our view on remaining issues from below aspects:

- Remaining issue on CONNECTED UE

- Remaining issue on OD-SIB1 request procedure

- Key RRC open issues

- Key MAC open issues

- Key 38.304 open issues

# 2 Discussion

## 2.1 Remaining issue on CONNECTED UE

In RAN2#129 [2], OD-SIB1 procedure for RRC_CONNECTED UE when T311 is running was supported. The only remaining issue is whether/how to support OD-SIB1 procedure for RRC_CONNECTED when searchSpaceSIB1 is configured.

Proposal 5: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1 as baseline.

[Vivo]: NW cannot know when the UE wants to request OSI, then proposal 5 cannot work in the case. [Ericsson]: Want to have more time to think of two options.

We will make conclusion on P5 next meeting.

As defined in legacy TS 38.331 [3], if the CONNECTED UE (when T311 is not running) is configured with searchSpaceSIB1, the UE monitors the broadcasted SIB1. And if some SIB(s) are not broadcasting as indicated in the acquired SIB1, the UE may trigger OD-SIB procedure to acquire these SIB(s).

From Section 5.2.3.1 of TS 38.331 [3]: 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

According to offline / online discussion of RAN2#129 [2], the OD-SIB1 specific issue is that the CONNECTED UE may not trigger OD-SIB procedure if SIB1 is not acquired because it doesn’t know the broadcasting status of interested SIB(s).

Observation 1: For Rel-19 CONNECTED UE configured with searchSpaceSIB1, it may not trigger OD-SIB procedure if SIB1 is not acquired because it doesn’t know the broadcasting status of interested SIB(s).

We have the following 4 alternatives to resolve this issue:

- Alt-1: Rely on NW implementation (e.g. dedicated RRC message to deliver SIB1, or not configure searchSpaceSIB1 for the UE not supporting OD-SIB1)

- Alt-2: Allow the UE to trigger OD-SIB procedure, irrespective of broadcasting status of interested SIB

- Alt-3: Allow the UE to trigger the same OD-SIB1 procedure as IDLE/INACTIVE UE

- Alt-4: Allow the UE to send request for SIB1 via dedicated RRC message DedicatedSIBRequest

We provide our analysis in Table.1:

| Solution | NW impact | UE impact | Required spec impacts |
| --- | --- | --- | --- |
| Alt-1 | It may lead to unnecessary DL RRC message for SIB1 delivery because some UE may not need to acquire SIB1. | N/A | N/A |
| Alt-2 | It may lead to unnecessary UL congestion caused by multiple UE sending requests for SIB(s) which are broadcasting | It may lead to unnecessary UE OD-SIB request (and power consumption) when the requested SIB(s) are broadcasting | Procedure text changes are required in Section 5.2.2.3.3 of TS 38.331. |
| Alt-3 | NW may have to configure UL WUS resource in each BWP. | N/A | Procedure text changes are required in running CR of TS 38.331 |
| Alt-4 | N/A | N/A | ASN.1 change on DedicatedSIBRequest to allow UE to request SIB1 |

Table.1 Comparison among candidate solutions for CONNECTED UE

According to the Table 1, we don’t support Alt-2 / Alt-3 because they require further spec work and will bring further potential issues to NW/UE. Between Alt-1 and Alt-4, we think Alt-1 can be baseline and Alt-4 is enhancement. Considering the further spec work on Alt-4 is manageable, we can support it if majority prefer. Thus, we propose:

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting:

- Rely on the NW implementation (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1) as baseline.

- If majority prefer enhancement, allow the UE to send request for SIB1 via dedicated RRC message DedicatedSIBRequest.

## 2.2 Remaining issue on OD-SIB1 request procedure

### 2.2.1 How to check NES cell broadcasting status

This is to follow the agreement in RAN2#127b [4]:

Agreements on OD-SIB1

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

We noticed RAN1#120 has identified 5 candidate solutions for further discussion [5]:

FL Proposal 6-1-9 (from vivo2) If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following: - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission. - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy. Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements Note: The above cases are for SSB on the sync raster.

In the following, we proved analysis of the spec impacts and corresponding pros and cons for each alternative.

For Alt 1, we think this provides a clear picture of NW transmitting behavior and UE monitoring behavior. When K_SSB=30 for FR1 and K_SSB=14 for FR2, it indicates that NW is providing OD-SIB1 based on other UE’s request so that the UE could directly monitor Type 0 PDCCH on the NES cell. However, the UE monitoring behaviour should be further clarified on what is the start time and ending time for the PDCCH monitoring.

On the other hand, based on the agreement in last meeting, that NW can indicate that the OD-SIB1 is transmitted only via the SSB associated with the UL WUS, this means that OD-SIB1 is no longer provided in all SSB beams, which reduces the possibility that a UE could successfully receive the OD-SIB1 provided based on another UE’s request. Therefore, the

For Alt 2, the UE is always required to monitor Type 0 PDCCH before sending UL WUS. This not only wastes UE power but also increases UE access latency.

There was discussions in last RAN1 meeting that UE anyway blindly detects Type 0 PDCCH on a normal cell, however, we would like to clarify that from UE’s perspective, monitoring Type 0 PDCCH before transmitting UL WUS on an NES cell is different from monitoring Type 0 PDCCH on a normal cell. For the latter (on normal cell), UE is aware that there is SIB1 scheduling on that cell while for the former (on NES cell), UE has no information on whether the cell is transmitting or not, then it is a waste of UE power to perform such kind of blind detection. Therefore, if we need to follow RAN2 agreement on checking SIB1 transmitting status, such information should be explicitly indicated by the network.

For Alt 3 and Alt 4, we think the spec impact is similar, while for Alt 3, UE may change PDCCH monitoring behavior based on its own implementation. We think Alt 3 and Alt 4 are still consistent with RAN2 agreement, in a way that when NW is transmitting SIB1 no matter whether it is periodically transmitting in legacy way or transmitting based on other UE’s request, the NW changes the k_SSB value to be K_SSB<=23 for FR1 or K_SSB<=12 for FR2.

However, when K_SSB<=23 for FR1 or K_SSB<=12 for FR2, the cell maybe accessible and detected by legacy UEs, which may cause problem for legacy UEs if they intend to decode SIB1 while the NW is not transmitting.

For Alt 5, according to our understanding, the motivation is similar to Alt 1 that NW explicitly indicates that the NW is transmitting OD-SIB1 to other UE so that the UE could monitor Type 0 PDCCH before it sends UL WUS. However the spec change may impact the spared bit in MIB or the reserved bit in PBCH. It is noted that for FR1, there are 2 reserved bits in the PBCH payload while for FR2, no reserved bits is available. Thus, this would lead to different designs for FR1 and FR2.

The above analysis is summarized in the following Table.2

|  | Spec impact | Pros | Cons |
| --- | --- | --- | --- |
| Alt 1 | - UE behavior when K_SSB=30 for FR1 and K_SSB=14 for FR2 - UE monitoring window definition | Legacy UEs will not select/reselect to NES cell if NES cell is only providing OD-SIB1 for a period of time | - Larger spec impact - If OD-SIB1 is transmitted only via the SSB associated with UL WUS, detection probability is low for another UE |
| Alt 2 | - UE PDCCH monitoring before sending UL WUS - UE monitoring window definition |  | - UE power wasting and latency increase - Larger spec impact - Impact to legacy UEs when NW is only broadcasting SIB1 for a short period of time based on other UE’s request |
| Alt 3 | No other spec impact except for OD-SIB1 request behaviors | Smaller spec impact | Impact to legacy UEs when NW is only broadcasting SIB1 for a short period of time based on other UE’s request |
| Alt 4 |  |  |  |
| Alt 5 | - Reinterpretation of MIB or PBCH payload - UE monitoring window definition | Same as Alt 1 | - Large spec impact and may impact spared bits in MIB or reserved bits in PBCH - Different solutions for FR1 and FR2 if reserved bits in PBCH is used |

Table.2 Spec impacts of Alt 1-Alt 5 on how UE determines broadcasting status of OD-SIB1

Based on the above analysis on spec impacts and on further understanding of RAN2’s agreement, we think Alt 4 is consistent with RAN2’s agreement and corresponding spec impact is minimized; therefore, Alt 4 is preferred for the way-forward.

Proposal 2: RAN2 assume that Rel-19 UE determines the broadcasting status of OD-SIB1 in NES cell explicitly from the K_SSB value of its SSB (i.e. no need to specify blind detection behavior):

- When K_SSB>=24 for FR1 or K_SSB>=13 for FR2, the UE determines that SIB1 is not broadcasted (i.e. the UE does not monitor Type 0 PDCCH, as Alt-4 in RAN1#120 agreement).

- When K_SSB<24 for FR1 or K_SSB<13 for FR2, the UE determines that SIB1 is being broadcasted (i.e. NES cell needs to reset its K_SSB value when broadcasting SIB1).

### 2.2.2 Paging interruption during OD-SIB1 acquisition

This issue was raised by some company contribution in study phase [6], but it was not treated. In details, after the UE sends OD-SIB1 request in NES cell, it can’t monitor paging in NES Cell until OD-SIB1 is acquired. This is an extra paging interruption compared with legacy cell reselection. We think it is a valid issue because TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”) [7]:

4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell.

These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

Observation 2: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

We think there are two alternatives:

- Alt-1: leave it to UE implementation. But RAN2 need to send LS to RAN4 to check whether the paging interruption requirement needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

Since RAN2 has not discussed this issue before, we suggest RAN2 to discuss these 2 alternatives.

Proposal 3: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

## 2.3 Key RRC open issues

In current RRC running CR [8], one FFS is captured on whether the UL-WUS configuration applies to one cell or several cells:

FFS: Whether the UL-WSU configuration applies to one cell or several cells

We do an analysis on payload size of UL WUS configuration based on latest RAN1 agreement [5] in Appendix of this contribution. It shows that at least 369 bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2952 bit if 8 NES cells’ UL WUS configurations are included. However, according to TS 38.331, SIB1 or a SI message is limited to 2976 bit, which may not be sufficient if only cell specific UL WUS configuration is allowed. It will cause coverage issue.

Observation 3: RAN1#118 has agreed to have a unified configuration format for cell specific UL WUS configuration and area specific UL WUS configuration.

Observation 4: According to analysis on payload size of RAN1 agreed UL WUS configuration, at least 369bit is required for one single UL WUS configuration. Thus, if only cell specific UL WUS configuration is allowed, at most 7 NES cell’s UL WUS configuration can be included, which may cause coverage issue.

Based on Observation 3 and 4, we support UL WUS configuration can be applied to multiple cells. Then, we think the simplest solution is to allow more than one PCI(s) to be associated with one UL WUS configuration. For example:

ULWUSConfigInfoList ::=     SEQUENCE (SIZE (1..maxULWUS)) OF ULWUSConfigInfo

ULWUSConfigInfo ::=         SEQUENCE {

CarrierFreq               ARFCN-ValueNR,

NESCellList ::=          SEQUENCE (SIZE (1..maxNESCell)) OF PhysCellId,

ULWUSConfig               UlWUSConfig

}

As PCI information is included in PBCH, the UE can determine whether the stored UL WUS configuration is applicable via decoding PBCH during OD-SIB1 acquisition. In this way, there is no spec impacts on MIB. This solution is applicable to both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs (when T311 is running).

Proposal 4: Support UL-WUS configuration common to several cells by associating more than one PCI with one UL WUS configuration.

## 2.4 Key MAC open issues

In current MAC running CR [9], one FFS is captured on whether any changes are required for handling preamble backoff during SIB1 request.

Editor’s note: FFS whether any changes are required for handling preamble backoff during SIB1 request procedure.

This FFS is added because some company proposed that PREAMBLE_BACKOFF should be set to 0 during SIB1 request procedure. The justification is because only Msg1 based SIB1 request is supported, and RACH resources are dedicatedly configured.

From running 38.321 CR [9]:

1>	else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms

We agree that it makes sense to set PREAMBLE_BACKOFF as 0 during SIB1 request procedure. However, we think the current text can already work (i.e. rely on NW implementation not to indicate any backoff in the RAR):

As legacy OD-SIB procedure has the same issue, we don’t think it is necessary to have a new procedure text to fix backoff to 0 for OD-SIB1 procedure. Thus, we propose:

Proposal 5: Rely on NW implementation to indicate absence of backoff in the RAR during OD-SIB1 procedure (i.e., it is not necessary to have a new procedure text to fix backoff to 0 for OD-SIB1 procedure).

## 2.5 Key 38.304 open issues

### 2.5.1 How to capture new barring behaviour

In current 38.304 running CR [10], one EN is captured on whether to capture new barring behavior in TS 38.331 or/and TS 38.304.

Editor’s note: whether to capture the barring (including no UL WUS configuration and max number of preamble transmission for OD-SIB1 request) in TS 38.331 or/and TS 38.304.

Agreement (RAN2#127b)

1	For Rel-18, remove detailed description on IFRI from 38.331, and keep only pointers to 38.304

2	At least for Rel-18, adopt the approach proposed in R2-2408293  (Huawei, HiSilicon, Samsung), as baseline for further work.

R2-2408293	Discussion on the cell barring across R18 features	Huawei, HiSilicon, Samsung	discussion	Rel-18	TEI18, NR_NTN_solutions-Core, NR_redcap-Core, NR_redcap_enh-Core, NR_IAB_enh-Core, Netw_Energy_NR-Core, NR_netcon_repeater-Core

Noted

Proposal 1:	Apply below principle on the responsibility of capturing cell barring and IFRI handlings between 38.331 and 38.304 since R18, i.e., adding some few missing parts rather than removing redundant parts.

| Specification part | Handling of cellBarred in MIB | Handling of feature specific cell baring indicator | Handling of intra-Frequency Reselection |
| --- | --- | --- | --- |
| 38.331 procedure in 5.2.2.4 | Specify how to use cellBarred, only if applied | Specify how to use the feature specific indicator to determine the cell barring | Specify the applied value, e.g. in according to some RRC fields or the default value (e.g. “as if xxx field is set to allowed”); Give the reference to TS 38.304; |
| 38.331 field description | Specify the “ignore” behaviour, if cellBarred is not applied | Clarify the indicator application to specific feature(s) or the “ignore” behaviour by other features | Specify the “ignore” behaviour, if some IFRI related field is not applied |
| 38.304 in 5.3.1 | 5.3.1 Beginning part: List of the fields which may impact the cell barring |  | 5.3.1 Ending part: The actual UE operation on how to “select to another cell on the same frequency” |

Please note that RAN2 recently had “barring behavior clean-up” effort (i.e. how to handle the the cell barring and associated behaviours are defined in TS38.331 and TS38.304, which are not completely aligned between the specifications).

As highlighted above, RAN2 agreed to keep duplicated descriptions between 38.331 and 38.304 regarding the UE determination whether the cell is considered barred or not.

Observation 5: RAN2 recently had “barring behavior clean-up” effort and finally agreed to keep duplicated descriptions between 38.331 and 38.304 regarding the UE determination whether the cell is considered barred or not.

Thus, we propose to follow this guidance.

Proposal 6: Following agreement on “barring behavior clean-up”, capture the new UE barring behavior of OD-SIB1 in both TS 38.331 and TS 38.304.

### 2.5.2 How to capture unbarring behaviour

In current 38.304 running CR [10], one EN is captured on whether to capture unbarring behavior in TS 38.331 or/and TS 38.304.

Editor’s note: whether to capture the unbarring behavior in TS 38.331 or/and TS 38.304.

From our side, one concern is that existing Section 5.3.1 of TS 38.304 only captures UE barring behavior (i.e. UE unbarring behavior was not captured before).

Observation 6: Existing Section 5.3.1 of TS 38.304 only captures UE barring behavior (i.e. UE unbarring behavior was not captured before).

Thus, we think there are 3 alternatives:

- Alt-1: capture it as one NOTE in Section 5.3.1 of TS 38.304

- Alt-2: capture it as normative text in a new section of TS 38.304

- Alt-3: capture it only in TS 38.331.

Thus, we propose:

Proposal 7: RAN2 discuss how to capture the unbarring behavior of OD-SIB1:

- Alt-1: capture it as one NOTE in Section 5.3.1 of TS 38.304

- Alt-2: capture it as normative text in a new section of TS 38.304

- Alt-3: capture it only in TS 38.331 (i.e. no need to capture it in TS 38.304).

### 2.5.3 Whether to capture trigger condition of OD-SIB1 acquisition

In current 38.304 running CR [10], one EN is captured on whether to capture trigger conditon of OD-SIB1 acqusition in TS 38.331 or/and TS 38.304.

Editor’s note: whether to capture trigger condition of OD-SIB1 acquisition procedure for RRC_IDLE / RRC_INACTIVE UE (i.e. reusing cell reselection criteria) in TS 38.331 or/and TS 38.304.

Duing running CR discussion, there were some comments that above condition is legacy conditon of SIB1 acqusition, and thereby no need to captured in spec. However, we disagree these comments. Please note that legacy specifications (TS38.331/38.304/38.300) don’t mandate UE to acquire SIB1 during cell reselection (i.e. it is up to UE implementation whether to acquire SIB1, only MIB acqusition is mandatory). We understand that the following RAN2#127b agreement mandates UE to trigger OD-SIB1 acqusition when cell reselection critrion is met.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

Observation 7: Legacy specifications (TS38.331/38.304/38.300) don’t mandate UE to acquire SIB1 during cell reselection (i.e. it is up to UE implementation whether to acquire SIB1, only MIB acqusition is mandatory). However, RAN2#127b agreed to mandate UE to trigger OD-SIB1 acqusition when cell reselection critrion is met.

Thus, we don’t think the trigger condition is legacy behavior of SIB1 acqusition. So, it should be captured in specfication.

Proposal 8: Trigger condition of OD-SIB1 acquisition (i.e. reusing cell reselection criteria) is different from legacy. So, RAN2 capture it in both TS 38.331 and TS 38.304.

# 3 Conclusion

In this contribution, we discuss remaining issues of on-demand SIB1. Our observations are:

Observation 1: For Rel-19 CONNECTED UE configured with searchSpaceSIB1, it may not trigger OD-SIB procedure if SIB1 is not acquired because it doesn’t know the broadcasting status of interested SIB(s).

Observation 2: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

Observation 3: RAN1#118 has agreed to have a unified configuration format for cell specific UL WUS configuration and area specific UL WUS configuration.

Observation 4: According to analysis on payload size of RAN1 agreed UL WUS configuration, at least 369bit is required for one single UL WUS configuration. Thus, if only cell specific UL WUS configuration is allowed, at most 7 NES cell’s UL WUS configuration can be included, which may cause coverage issue.

Observation 5: RAN2 recently had “barring behavior clean-up” effort and finally agreed to keep duplicated descriptions between 38.331 and 38.304 regarding the UE determination whether the cell is considered barred or not.

Observation 6: Existing Section 5.3.1 of TS 38.304 only captures UE barring behavior (i.e. UE unbarring behavior was not captured before).

Observation 7: Legacy specifications (TS38.331/38.304/38.300) don’t mandate UE to acquire SIB1 during cell reselection (i.e. it is up to UE implementation whether to acquire SIB1, only MIB acqusition is mandatory). However, RAN2#127b agreed to mandate UE to trigger OD-SIB1 acqusition when cell reselection critrion is met.

Based on observations, our proposals are:

Remaining issue on RRC_CONNECTED UE

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting:

- Rely on the NW implementation (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1) as baseline.

- If majority prefer enhancement, allow the UE to send request for SIB1 via dedicated RRC message DedicatedSIBRequest.

Proposal 2: RAN2 assume that Rel-19 UE determines the broadcasting status of OD-SIB1 in NES cell explicitly from the K_SSB value of its SSB (i.e. no need to specify blind detection behavior):

- When K_SSB>=24 for FR1 or K_SSB>=13 for FR2, the UE determines that SIB1 is not broadcasted (i.e. the UE does not monitor Type 0 PDCCH, as Alt-4 in RAN1#120 agreement).

- When K_SSB<24 for FR1 or K_SSB<13 for FR2, the UE determines that SIB1 is being broadcasted (i.e. NES cell needs to reset its K_SSB value when broadcasting SIB1).

Remaining issue on OD-SIB1 request procedure

Proposal 3: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

Key RRC open issues

Proposal 4: Support UL-WUS configuration common to several cells by associating more than one PCI with one UL WUS configuration.

Key MAC open issues

Proposal 5: Rely on NW implementation to indicate absence of backoff in the RAR during OD-SIB1 procedure (i.e., it is not necessary to have a new procedure text to fix backoff to 0 for OD-SIB1 procedure).

Key 38.304 open issues

Proposal 6: Following agreement on “barring behavior clean-up”, capture the new UE barring behavior of OD-SIB1 in both TS 38.331 and TS 38.304.

Proposal 7: RAN2 discuss how to capture the unbarring behavior of OD-SIB1:

- Alt-1: capture it as one NOTE in Section 5.3.1 of TS 38.304

- Alt-2: capture it as normative text in a new section of TS 38.304

- Alt-3: capture it only in TS 38.331 (i.e. no need to capture it in TS 38.304).

Proposal 8: Trigger condition of OD-SIB1 acquisition (i.e. reusing cell reselection criteria) is different from legacy. So, RAN2 capture it in both TS 38.331 and TS 38.304.

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#129, Chair Notes.

[3] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[4] RAN2#127b, Chair Notes.

[5] RAN1#120, Chair Notes.

[6] R2-2406470, Remaining details to enable on-demand SIB1, Intel Corporation.

[7] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.

[8] R2-2501462, Running RRC CR for enhancements for network energy efficiency, Ericsson.

[9] R2-2501463, Running MAC CR for network energy saving, Interdigital.

# Appendix: Analysis of overhead of UL WUS configuration

The following Table.3 provides an estimation based on latest RAN1 agreement in RAN1#120 [5]:

| Purpose | Approximate payload size analysis | Note | Total payload size |
| --- | --- | --- | --- |
| Identify NES cell(s) | PhysCellId (10bit) ARFCN-ValueNR (22bit) | This is only value of one NES cell. Please note that RAN2 agreed multiple NES cells’ UL WUS config can be included in on Cell A. | 32bit |
| RACH resource parameter for SIB1 request | rach-ConfigGeneric (40bit) - prach-ConfigurationIndex (8bit) - msg1-FDM (2bit) - msg1-FrequencyStart (9bit) - zeroCorrelationZoneConfig (4bit) - preambleReceivedTargetPower (8bit) - preambleTransMax (4bit) - powerRampingStep (2bit) - ra-ResponseWindow (3bit) ssb-perRACH-Occasion (3bit) si-RequestPeriod (3bit) SI-RequestResources (14bit) - ra-PreambleStartIndex (6bit) - ra-AssociationPeriodIndex (4bit) - ra-ssb-OccasionMaskIndex (4bit) Other RACH config (22bit) - msg1-SubcarrierSpacing (3bit) - prach-RootSequenceIndex (10bit) - rsrp-ThresholdSSB (7bit) - n-TimingAdvanceOffset (2bit) - TDD-UL-DL-ConfigCommon (~30bit) - ss-PBCH-BlockPower (7bit) - restrictedSetConfig (1bit) | - Assume up to 275 PRBs. - n-TimingAdvanceOffset is missed. | 120bit |
| BWP info of initial UL BWP | BWP (~20bit), including locationAndBandwidth, SCS, cyclicPrefix. | It is necessary to identify PRACH frequency location because PRACH resource is configured per BWP. | 20bit |
| UL frequency info | FrequencyInfoUL-SIB (~133bit) - MultiFrequencyBandListNR (80bit) - absoluteFrequencyPointA (22bit) - scs-SpecificCarrierList (~24bit) - p-Max (6bit) - frequencyShift7p5khz (1bit) | - It is necessary to obtain carrier information of PRACH transmission and related setting (e.g. UL power). - Assume up to 8 bands - Assume only 1 SCS. | 133bit |
| SIB1 reception | pdcch-ConfigSIB1 (8bit) | It seems conflicted with latest RAN1 agreement | 8bit (TBD) |
| RAR reception | RA-SearchSpace (6bit) monitoringSlotPeriodicityAndOffset (16bit) duration (12bit) monitoringSymbolsWithinSlot (14bit) nrofCandidates (6bit) | - Assume CORESET#0 is used for RAR reception (i.e. no extra common CORESET configuration). - Assume only one search space is configured for RAR. | 52bit |
| Total payload size: 369bit×Num of NES Cell (e.g. If 8 NES Cells’ UL WUS config are included, 2952 bit is required) * * Optimization of area-specific configuration is not considered, i.e. one NES cell has one UL WUS config in this analysis. |  |  |  |

Table.3 Analysis on required payload size of UL WUS configurations