3GPP TSG-RAN WG2 Meeting #129bis	R2-2502145

China, Wuhan, April 7th – 11th, 2025

Source:	CATT

Title:	Analysis on LP-WUS for RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In previous RAN1 meetings and RAN2 meetings, great progress was made for LP-WUS in RRC_CONNECTED mode. Based on the agreements, we will continue to analyze the LP-WUS monitoring procedure in RRC_CONNECTED mode for LP-WUS including the following aspects:

- Short DRX cycle;

- LP-WUS monitoring restriction;

- Dual DRX group;

- LP-WUS enabling/disabling and activation/deactivation.

And our proposals are provided.

Discussion

2.1	Short DRX cycle

In RAN2#129 meeting, RAN2 discussed whether LP-WUS was applied to short DRX cycle [1]. Based on the discussion, we analyze the Option 1-1 and Option 1-2 respectively.

Option 1-1

Based on the discussion, the following assumption was made [1]:

Observation: Companies have the same understanding on the DCP behaviour, i.e., DCP is only applied when UE is using long DRX cycle.

For LP-WUS, RAN2 has assumed solutions/ operations introduced for DCP mechanism was taken as baseline for Option 1-1. Following this assumption, LP-WUS is not applied to short DRX cycle. And from technique point of view, if LP-WUS is applied to short DRX cycle, the following issues need to be discussed.

Regarding the configuration of LP-WUS for Option 1-1, RAN1 has made the following agreements:

Agreement For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z - Time offset2 is RRC configured - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined.

According to the agreements, it can be seen that for Option 1-1, the network will configure the periodicity of the LP-WUS which is independently from the C-DRX periodicity. And the UE will monitor the LP-WUS at or after Time Offset2 which is also configured by the network. If it is agreed that the LP-WUS can be applied to short DRX cycle, whether the same mechanism is used for short DRX cycle should be discussed.

In addition, the short DRX cycle can be configured as small as millisecond level [2], which may be smaller than of the offset between LP-WUS and the start of the drx-onDurationTimer. This may result in LP-WUS can’t be applied to short DRX with small values, which brings restriction on application for LP-WUS together with short DRX cycle.

shortDRX SEQUENCE { drx-ShortCycle ENUMERATED { ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }, drx-ShortCycleTimer INTEGER (1..16) } OPTIONAL, -- Need R

Furthermore, it is still questionable on the power saving gain for LP-WUS applying to short DRX cycle.

Taking the above into account, it is proposed that:

Proposal 1: LP-WUS is not applied to short DRX cycle for Option 1-1.

Option 1-2

For Option 1-2, the LP-WUS is configured independent of the DRX cycle actually. And the drx-onDurationTimer will not be started as agreed in RAN2 last meeting. Hence, it is meaningless to configure short DRX cycle for Option 1-2. Hence, it is proposed that:

Proposal 2: LP-WUS is not applied to short DRX cycle for Option 1-2.

2.2	LP-WUS Monitoring restriction

For DCP, it was captured in 38.300[3] that

# 11 UE Power Saving /Omitted/ A UE can only be configured to monitor DCP when connected mode DRX is configured, and at occasion(s) at a configured offset before the on-duration. More than one monitoring occasion can be configured before the on-duration. The UE does not monitor DCP on occasions occurring during active-time, measurement gaps, BWP switching, or when it monitors response for a CFRA preamble transmission for beam failure recovery (see clause 9.2.6), in which case it monitors the PDCCH during the next on-duration. If no DCP is configured in the active BWP, UE follows normal DRX operation. /Omitted/

For LP-WUS it was captured in the latest running CR of TS 38.321[4]that:

4> if all LP-WUS monitoring occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last LP-WUS occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4): 5> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

In RAN1#120 meeting for the LP-WUS monitoring during DRX Active Time [5]:

Agreement For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time - FFS FAR and MDR impact

The C-DRX active time is total duration that the UE monitors PDCCH. But it does not include that time such as, measurement, PDSCH reception and PUSCH transmission.

Based on the progress above, considering the LP-WUS is monitored via a different receiver and it is FFS whether the UE monitors LP-WUS when MR is on. Hence, we continue to analyze whether LP-WUS can be monitored for the following cases:

- Measurement gaps;

- BWP switching;

- Monitoring CFRA RAR for BFR;

- SPS/CG occasion.

- 2.2.1	Measurement gap

The UE performs intra/inter-frequency measurement during measurement gap in which the UE does not perform data transmission and reception. And it has been agreed in RAN1[6]-[7] that no impact on RRM/RLM/BFD measurement requirements is assumed for Option 1-1 and Option 1-2, hence the UE will turn on MR during measurement gap to perform measurement.

- 2.2.2	BWP switching

The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell. During BWP switch case, it is unclear whether MR will be involved. For example, whether there are different MR behaviors for different sleep mode. From RAN2’s perspective, this can be left to RAN1 for more details.

- 2.2.3	Monitoring CFRA RAR for BFR

According to the latest running CR for TS38.321[4], the UE will prioritize CFRA RAR for BFR and turns on drx-onDurationTimer when LP-WUS collides with CFRA RAR. However, the UE turns on MR to monitor CFRA RAR for BFR but this may not within DRX Active Time. Hence, the current RAN1 agreements do not cover this case.

- 2.2.4	SPS/CG occasion

For SPS, the gNB allocates downlink resources for downlink data transmissions to UEs and RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it. For CG, the gNB allocates uplink resources for uplink data transmissions from UEs. And MR is switched on to perform PUSCH transmission or PDSCH reception.

- 2.2.5	Summary

As analyzed above, the UE switches on MR during the measurement gap, monitoring CFRA RAR for BFR and SPS/CG occasion. But there is no conclusion whether the UE needs to monitor LP-WUS when MR is on based on RAN1 agreements and it is FFS on whether MR behavior when BWP switch happens. Hence, it is suggested that:

Proposal 3: RAN2 asks RAN1 whether the UE monitors LP-WUS during the following cases:

-	Measurement gaps;

-	BWP switching;

-	Monitoring CFRA RAR for BFR;

-	SPS/CG occasion.

2.3	Dual DRX group

In order to reduce the power consumption when both FR1 and FR2 cells are configured via Carrier Aggregation, a secondary DRX group was introduced with a separate drx-InactivityTimer and drx-onDurationTimer configured for the FR2 cells enabling FR2 to go to sleep more quickly. However, it is FFS whether LP-WUS can be configured with dual DRX group. In RAN1#119 meeting[2] this issue and the following alternatives were discussed:

- For the case when UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS is configured on a serving cell per cell-group - LP-WUS indication is applicable to all activated serving cells in the cell-group - For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode, - Opt.1 - LP-WUS is configured on a serving cell per cell-group - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all activated serving cells - Note: This requires RAN2 coordination via LS - Opt.2 - LP-WUS is configured on a serving cell per DRX group per cell-group - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group - Note: This requires RAN2 coordination via LS - Opt.3 - LP-WUS is configured on a serving cell per cell-group - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group. FFS: Whether the serving on which the LP-WUS is configured is only PCell/PSCell or RRC configured cell other than PCell/PSCell

In the following, RAN2 continue to discuss these options when UE is configured with NR-DC and/or CA without/with dual DRX groups in RRC CONNECTED mode provided by RAN1.

  - Opt.0

    - LP-WUS is not configured with dual DRX group simultaneously.

In this option, LP-WUS function and dual DRX group are not configured simultaneously which follows DCP mechanism.

  - Opt.1

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all activated serving cells

In this option, one LP-WUS is configured per Cell group. It can control the UE to monitor PDCCH separately via separate indications, e.g. DRX group ID can be indicated in the LP-WUS. When the UE is indicated to wake up by LP-WUS with the DRX group ID, the UE monitors the PDCCH for the corresponding DRX group.

  - Opt.2

    - LP-WUS is configured on a serving cell per DRX group per cell-group

    - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group

In this option, LP-WUS triggers the UE to wake up to monitor PDCCH separately for different DRX groups. It requires the LP-WUS to be configured for each DRX group, i.e. separate LP-WUS is configured on the corresponding serving cell in each DRX group.

  - Opt.3

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group.

In this option, the LP-WUS controls all the activated serving cells, e.g., when the UE is indicated to wake up by LP-WUS, the UE wakes up to monitor the PDCCH of all the activated serving cells of all DRX groups but with different timer lengths.

Compared with these options:

- Option 0 follows the DCP principle brings the least spec impacts;

- Option 2 brings benefits on control flexibity but also causes higher resouce consumption, while on the opposite, Option 3 consumes less resource and brings less spec impact but lacks control flexiblity. Option 1 is one compromised solution with less resource consumption but flexibity.

However, it is within RAN1 scope to discuss the LP-WUS payload and the LP-WUS location. Hence, it is proposed that:

Proposal 4: RAN2 postpone discussion on simultaneous configuration of LP-WUS and dual DRX group until RAN1 make further progress.

2.4	LP-WUS enabling/disabling and activation/deactivation

- 2.4.1	Enabling/disabling

In RAN1#115 meeting, RAN1 it was agreed that:

RAN1#115 Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

For RRC CONNECTED mode, from RAN1 perspective, PDCCH monitoring triggered by LP-WUS is enabled or disabled by gNB RRC signaling. But it is FFS whether to support UE assistance to enable/disable PDCCH monitoring triggered by LP-WUS. In our understanding, the gNB can determine when/whether to enable/disable PDCCH monitoring triggered by LP-WUS based on UE capability and the network load. Hence, no UE assistance information is needed.

Proposal 5: UE assistance is not needed to enable/disable PDCCH monitoring triggered by LP-WUS.

- 2.4.2	Activation/deactivation

In RAN1 previous meetings, it was agreed that:

RAN1#118 meeting Agreement - In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of - by gNB RRC signaling, with or without UE assistance. - by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance. - based on pre-configured condition(s), such as timer. - LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB. - other options are not precluded.

RAN1#119 meeting [Wed]Proposal 4-1: When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS - FFS the condition for the fallback - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior - For LP-WUS operation option 1-2, - FFS what UE fallbacks to - FFS how to avoid misalignment between gNB and UE on the UE LP-WUS/PDCCH monitoring

It can be seen that RAN1 is still studying LP-WUS activation/deactivation/fallback to PDCCH monitoring when UE monitors LP-WUS. RAN2 can wait RAN1 for more progress for these aspects.

Proposal 6: For LP-WUS activation/deactivation/fallback to PDCCH monitoring when UE monitors LP-WUS aspects, RAN2 wait for more details from RAN1 for further discussion.

Conclusion

In this document, we analyze the remaining issues in RRC_CONNECTED for LP-WUS, and the following and proposals are provided:

Short DRX cycle

Proposal 1: LP-WUS is not applied to short DRX cycle for Option 1-1.

Proposal 2: LP-WUS is not applied to short DRX cycle for Option 1-2.

LP-WUS monitoring restriction

Proposal 3: RAN2 asks RAN1 whether the UE monitors LP-WUS during the following cases:

-	Measurement gaps;

-	BWP switching;

-	Monitoring CFRA RAR for BFR;

-	SPS/CG occasion.

Dual DRX group

Proposal 4: RAN2 postpone discussion on simultaneous configuration of LP-WUS and dual DRX group until RAN1 make further progress.

LP-WUS enable/disable and activation/deactivation

Proposal 5: UE assistance is not needed to enable/disable PDCCH monitoring triggered by LP-WUS.

Proposal 6: For LP-WUS activation/deactivation/fallback to PDCCH monitoring when UE monitors LP-WUS aspects, RAN2 wait for more details from RAN1 for further discussion.

Reference

- R2-2501444, [AT129][204][LPWUS] Proposals for reply to R1 LS R2-2500012/R1-2410909 CATT;

- TS 38.331, Radio Resource Control (RRC) protocol specification;

- TS 38.300, NR and NG-RAN Overall Description; Stage 2;

- R2-2502307, Running MAC CR for LP-WUS;

- R1-2501682, Report of RAN1#120 meeting;

- R1-2407587, RAN1#118 meeting report;

- R1-2409341, RAN1#118bis meeting report.