3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2502146

Wuhan, China, 7th - 11th April 2025

Source:	CATT

Title:	Discussion on Asymmetric DL sTRP/UL mTRP

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- 1. Introduction

For Rel-19 MIMO, during the RAN2#129 meeting, the following agreements are made for Asymmetric DL sTRP/UL mTRP [1].

One PL offset value is indicated for each TCI state included in the new MAC CE. The new MAC CE contains one serving cell ID and one BWP ID TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) The new MAC CE can include flexible number of PL offset values. - Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In this contribution, we will further discuss the detail spec impacts for asymmetric DL sTRP/UL mTRP deployment.

- 2. PL offset

During the post email discussion on running CR for TS 38.321, there are several issues left for PL offset MAC CE [2]. In the following, we will analyze the leftover issues on MAC CE impact for the asymmetric DL sTRP/UL mTRP deployment scenarios.

What channels are applicable to

During the last RAN2 meeting, a new MAC CE was agreed to be introduced for PL offset update for asymmetric DL sTRP/UL mTRP. And for this MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset. Based on RAN1 agreements, the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH. For example, for SRS, the DCI-indicated TCI state is applied if followUnifiedTCI-StateSRS is configured, otherwise for periodic SRS resource TCI state is configured by RRC, and for SP/AP SRS resource TCI state is indicated by MAC CE.

Proposal 1: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Whether to limit the maximum number of TCI states in the MAC CE

During the post email discussion on running CR for TS 38.321, there is an Editor’s Note on whether to limit the maximum number of TCI states in the MAC CE [3].

We share the same view as Rapporteur that the flexible number can be determined by UE based on the length indicated in the MAC subheader based on the format of Pathloss Offset Update MAC CE. Thus, we think there is no need to limit the maximum number of TCI states in the Pathloss Offset Update MAC CE.

Proposal 2: It’s not necessary to limit the maximum number of TCI states in the Pathloss Offset Update MAC CE.

- 3. 2TA for single-DCI mTRP

During RAN1#119 meeting, RAN1 sent another LS[4] to check whether the following agreement would cause any MAC impact.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. “RAN2 could check whether this RAN1 agreement would cause any MAC impact”

From RAN2 perspective, the current 2TA operation is associated with the TA of a TAG. For example, there is no description of restriction on coresetPoolIndex in MAC spec. Thus, there is no big MAC impact due to 2TA for single-DCI mTRP. There may be some impacts on RRC spec for the field description, e.g., the coresetPoolIndex restriction and a single n-TimingAdvanceoffset configuration.

Observation 1: There is no big MAC impact on 2TA for single-DCI mTRP.

Proposal 3: RAN2 confirm the RAN1 agreements on 2TA for single-DCI mTRP and further discuss MAC and RRC impacts during Stage 3 running CR phase on the following aspects:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- 4. PHR

During the last meeting, the following agreement was made and one FFS is left for further discussion, i.e., FFS whether/how to capture this.

- Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

For PHR aspect, RAN1 has made the following agreements.

RAN1#118 Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

From RAN2 perspective, whether PHR trigger is impacted for the asymmetric DL sTRP/UL mTRP scenarios should be discussed. In TS 38.321 clause 5.4.6, the key trigger of PHR is that any path loss changes more than a threshold.

### 5.4.6 Power Headroom Reporting … A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

In asymmetric DL sTRP/UL-only mTRP scenario, for a joint/UL TCI state configured with a PL offset, the UL transmission power is determined based on the PL and the PL offset of the PL RS. Thus, the PHR should be triggered when the change of PL of a PL RS is larger than a threshold, where the PL takes into account the PL offset of the PL RS.

The PL value for the PL RS is calculated within the PHY layer, which is transparent to the MAC layer. Thus, the PL used to determine the PHR trigger would already be taken into account the PL offset in PHY layer. There is no spec impact on PHR triggering condition in MAC.

Proposal 4: It’s not necessary to capture the RAN2 understanding on PHR triggering condition in MAC spec.

- 5. Conclusion

This paper discusses the spec impacts for Asymmetric DL sTRP/UL mTRP. The contribution of this paper is summarized as follows.

Observation 1: There is no big MAC impact on 2TA for single-DCI mTRP.

Proposal 1: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Proposal 2: It’s not necessary to limit the maximum number of TCI states in the Pathloss Offset Update MAC CE.

Proposal 3: RAN2 confirm the RAN1 agreements on 2TA for single-DCI mTRP and further discuss MAC and RRC impacts during Stage 3 running CR phase on the following aspects:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

Proposal 4: It’s not necessary to capture the RAN2 understanding on PHR triggering condition in MAC spec.

- 6. Reference

- R2-2501332	, Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD	Vice Chairman (CATT) report

- [Post129][206][MIMO_Ph5] Running CR for TS 38.321 (Samsung)

- R2-2502664, Introduction of Rel-19 MIMO, Samsung

- R2-2500008, LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5 (R1-2410758; contact: Samsung)