3GPP TSG-RAN WG2 Meeting #129bis		R2-2502155

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In previous meetings, some agreements for LP-WUS/WUR in RRC_IDLE/INACTIVE were made [1] [2] [3] as below.

RAN2#126: - The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) RAN2#127: - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. RAN2#128: - FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In this contribution, we will further discuss the remaining issues for RRC_IDLE/ INACTIVE procedures with LP-WUR.

- Discussion

  - LP-WUS configuration

In RAN2#126 meeting, it was agreed that the LP-WUS related configuration is provided via system information.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Regarding the specific system information for LP-WUS configuration, both the existing SIB(s) and new SIB are feasible. However, considering paging-related configuration and PEI information are included in SIB1, it is straightforward to follow the same design for LP-WUS configuration. Besides, introducing a new SIB will result in more specification work, e.g., the scheduling information of the new SIB and the on-demand request for the new SIB should be futhur discussed. Thus, SIB1 is preferred.

Proposal 1: LP-WUS configuration is provided in SIB1.

In RAN2#126 meeting, it was agreed that the entry/exit condition for LP-WUS monitoring is included in LP-WUS configuration. However, it is still FFS whether the entry/exit condition for LP-WUS monitoring is always configured in SIB. Besides, RAN1 concluded that it is up to RAN2 discussion [4].

It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

Some companies thought if the MR and LR have the same coverage, the entry/exit condition for LP-WUS monitoring seems not always need to be configured in RAN2#125bis meeting.

According to the WID [5], the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for Msg3. However, it depends on the real deployment on whether the coverage of LP-WUS and LP-SS is full coverage or partial coverage. For example, if the size of a cell is small, the coverage of LP-WUS and LP-SS can be full coverage.  On the contrary, if the size of a cell is large, the coverage of LP-WUS and LP-SS can be partial coverage. In our understanding, the entry/exit condition should be mandatorily configured to make UE correctly check whether the entry/exit condition for LP-WUS monitoring is met or not.

Some companies may argue it should be optional to leave the flexibility to network configuration. But our understanding is network could either provide a very small value in case the network always allows UE to perform LP-WUS monitoring or provide the same value as the threshold of condition for RRM relaxation/offloading, which provides enough flexibility for network configuration.

Proposal 2: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

In RAN2#126 meeting, it was agreed that the LP-WUS related configuration includes LP-SS configuration, LP-WUS configuration, and the entry/exit condition for LP-WUS monitoring. Besides, in RAN2#127 meeting, RAN2 agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. If the network supports the OOK-based WUR, the LP-SS configuration should be configured in the LP-WUS related configuration. Otherwise, after UE with OOK-based WUR enters LP-WUS monitoring, OOK-based WUR cannot receive LP-WUS without the synchronization from LP-SS.

Proposal 3: If the network supports the OOK-based WUR, the LP-SS configuration should be always provided.

  - Entry/exit condition for LP-WUS monitoring

According to the following RAN1 and RAN2 agreements, it could be observed that both RAN1 and RAN2 agreed that separate entry/exit thresholds can be configured for different types of  LR.

RAN2 agreement in RAN2#127 meeting Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. RAN1 agreement in RAN1#119 meeting Confirm the following working assumption with the addition of the note at the bottom: Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption.

Observation 1: RAN1 and RAN2 agreed that separate entry/exit thresholds can be configured for different types of LR.

This WI introduces the following LR types:

- LRType#1: OOK-based LP-WUR measuring LP-SS with/without overlaid OFDM sequence

- LRType#2: OFDM-based LP-WUR measuring only LP-SS with and without overlaid OFDM sequence

- LRType#3: OFDM-based LP-WUR measuring SSB and may also measure LP-SS with and without overlaid OFDM sequence

It could be easily agreed that separate entry/exit thresholds will be configured for LRType#1 and LRType#3, while for LRType#2 which is OFDM-based WUR and the measured signalling for this LR type is LP-SS, it is uncertain whether to follow the entry/exit thresholds designed for LRType#1 or for  LRType#3 or introducing separate new entry/exit thresholds.

In RAN4#114 meeting [6], RAN4 agreed the same requirement shall be applied for LRType#1 and LRType#2 based on the following conclusion. Thus, in our understanding, same entry/exit condition with same threshold(s) should be configured for LRType#1 and LRType#2 considering the measured RS(s) are the same.

Issue 1-1-3-1: On RRM requirements to be specified for OOK-based LP-WUR based on LP-SS with overlaid OFDM sequence Agreement: - In Rel-19, the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not. Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence Agreement: - Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS - Reuse the same requirement for LP-SS OOK based

Observation 2: The requirement for OFDM-based LP-WUR measuring only LP-SS has same requirement for OOK-based LP-WUR measuring LP-SS.

Proposal 4: The entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS.

Whether MR threshod(s) in entry/exit condition for LP-WUS monitoring can be configured separately for different types of LR

According to our above RAN2 agreement, different companies may have different understanding on the meaning of separate entry/exit thresholds for LP-WUS monitoring for different types of LR:

- Understanding#1: MR threshold(s) in the entry/exit condition for LP-WUS monitoring are the same for different types of LR while the LR threshold(s) in the entry/exit condition are separate for different types of LR;

- Understanding#2: Both MR threshold(s) and LR threshold(s) in the entry/exit condition for LP-WUS monitoring are separate for different types of LR.

However, according to the above highlighted part in RAN1 agreements, it could be observed that RAN1 only agreed the MR threshold(s) in entry conditions for LP-WUS monitoring and agreed entry/exit thresholds can be configured separately for different types of LR at the same time, which means the RAN1’s intention is understanding#2.

Observation 3: RAN1 agreed both MR threshold(s) and LR threshold(s) in the entry/exit condition for LP-WUS monitoring are separate for different types of LR.

Thus, RAN2 can confirm the RAN1 agreement directly:

Proposal 5: RAN2 confirms the MR thresholds in entry/exit condition can be configured separately for different types of LR.

How OFDM-based WUR which supports measuring SSB and LP-SS chooses the entry/exit thresholds for LP-WUS monitoring

Based on the analysis above, different entry/exit thresholds can be configured for OFDM-based WUR measuring SSB, i.e., LRType#3 above, and OFDM-based WUR measuring only LP-SS i.e., LRType#2 above. Then, one raised issue is how OFDM-based WUR which supports measuring both SSB and LP-SS chooses the entry/exit thresholds for LP-WUS monitoring when both kinds of entry/exit thresholds are configured in the camped cell. In our understanding, there are two options as follows.

- Option 1: OFDM-based WUR always chooses to measure SSB and uses the thresholds for OFDM-based WUR monitoring SSB, i.e., for LRType#3 above.

- Option 2: It is up to UE implementation to choose which signalling to be measured and which thresholds to be used.

In our understanding, both options can work and suggest RAN2 to discuss which option is selected.

Proposal 6: RAN2 to discuss how OFDM-based WUR chooses the entry/exit thresholds for LP-WUS monitoring between the thresholds for OFDM-based WUR measuring SSB (LRType#3) and OFDM-based WUR measuring LP-SS (LRType#2).

  - Other wake up conditions

As legacy PEI design, the MR in ultra-deep sleep/monitoring LP-WUS should also wake up in case there is UL data/signalling arrival.

Proposal 7: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT, due to UL data and signaling arrival when monitoring LP-WUS.

  - LP-WUS Subgrouping

Network indication to indicate whether UE to monitor both LP-WUS and PEI

Regarding the subgrouping method for LP-WUS, RAN2 agreed that the PEI subgrouping method is taken as baseline, i.e. CN assigned and UE_ID based subgrouping. In the following part, we will analyse the specific detail for subgrouping methods.

The first issue is whether a network indication should be introduced to indicate the UE to monitor LP-WUS and PEI or not.

At the network side, PEI could be configured in the case that the coverage of LP-SS/LP-WUS is the partial coverage. According to legacy design, if there is PEI configuration, all UE supporting PEI could monitor PEI, which means that network needs to always provide PEI if it indicates supporting PEI. Similarly, if network supports LP-WUS and provides the LP-WUS configuration, all UE supporting LP-WUS could monitor LP-WUS, which means that network needs to always provide LP-WUS if it indicates support LP-WUS.

At the UE side, there is a case that PEI and LP-WUS are applicable at one UE when the entry condition of LP-WUS monitoring is fulfilled. In RAN1#116 meeting [7], RAN1 made the following agreement:

Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

It could be observed that, when UE supports PEI and PEI is configured by the gNB, it is up to UE implementation whether to monitor PEI or not after the UE receives LP-WUS indicating wake-up.

Observation 4: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Based on the above design at network side, regardless of whether the UE monitors PEI or not after receiving LP-WUS indicating wake-up, the network would provide PEI and LP-WUS in case it provides the PEI and LP-WUS configuration. Thus, there is no additional effort for network to provide both PEI and LP-WUS. In this way, there is no need for network to provide an indication to indicate the UE to monitor both PEI and LP-WUS or not.

Proposal 8: No need to introduce a network indication to indicate UE to monitor both LP-WUS and PEI or not.

Subgrouping design for LP-WUS

In the last RAN2 meeting, RAN2 agreed on the specific formula for UE_ID based subgrouping. Similarly, for UE_ID based LP-WUS subgrouping, the UE_ID for the calculation would also be extended compared to the UE_ID for PEI, PF and PO calculation. Regarding UE_ID in the formula, it could be derived from 5G-S-TMSI. What is more, RAN1 agreed that the maximum number of LP-WUS subgroups per PO is 31 in RAN1#120 meeting [8] as follows.

Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32.

Since the PEI and LP-WUS may be used together, to achieve the purpose that LP-WUS subgroup ID is separate from the PEI subgrouping ID, the UE_ID for LP-WUS subgrouping should be extended considering the maximum number of PEI subgrouping, i.e., 8. Thus, additional 8 bits from 5G-S-TMSI are used for the UE_ID for LP-WUS subgrouping in case LP-WUS and PEI are used together.

Therefore, if eDRX is not configured, UE_ID=5G-S-TMSI mode (8192* 31).

Proposal 9: For LP-WUS UE_ID based subgrouping, UE_ID = 5G-S-TMSI mode (8192* 31).

Subgrouping method selection

Considering UE's LP-WUS subgroup can be either assigned by CN or derived based on UE_ID, the UE needs to know how to obtain its subgroup when camping on a cell. In Rel-17 PEI, a cell can indicate its capability of CN assigned subgrouping and UE_ID based subgrouping to UE by the configuration. Specifically, the detail is as follows.

- If  PEI is configured and both subgroupsNumPerPO and subgroupsNumForUEID are equal to 1, the cell supports PEI without subgrouping, i.e., there is only one subgroup and all UEs supporting PEI belong to the same subgroup.

- If PEI is configured and subgroupsNumForUEID has the same value (except 1) as subgroupsNumPerPO, the cell only supports the UE_ID based subgrouping.

- If PEI is configured and subgroupsNumForUEID is absent, the cell only supports the CN assigned subgrouping.

- If PEI is configured and subgroupsNumForUEID < subgroupsNumPerPO, the cell supports both CN assigned subgrouping and UE_ID based subgrouping.

Correspondingly, according to TS 38.304 [9], the UE behaviors for the PEI subgroup are captured as follows.

- If UE only supports UE_ID based/CN assigned subgrouping and the cell supports both CN assigned subgrouping and UE_ID based subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If the cell only supports UE_ID based/CN assigned subgrouping and the UE supports both CN assigned subgrouping and UE_ID based subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If both UE and cell support CN assigned subgrouping and UE_ID based subgrouping, UE uses the CN assigned subgrouping.

- If both UE and cell only support UE_ID based/CN assigned subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If both UE and cell only support CN assigned subgrouping and UE has no CN assigned subgroup ID, UE monitors the legacy PO.

- If the cell supports PEI without subgrouping, UE uses the UE_ID based subgrouping.

- If UE only supports one kind of subgrouping and the cell only supports another kind of subgrouping, UE monitors the legacy PO.

For LP-WUS subgrouping, we think it is straightforward to reuse the PEI mechanism. That is, the network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration. For UE supporting both UE_ID based and CN assigned subgrouping, it is up to network configuration to use UE_ID based or CN assigned subgrouping, similar to the PEI mechanism.

Proposal 10: The network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration, similar as PEI design.

Proposal 11: For UE supporting both UE_ID based and CN assigned subgrouping, it is up to network configuration to use UE_ID based or CN assigned subgrouping, similar as PEI design.

  - Emergency PDU session

In Rel-17 PEI, RAN2 discussed whether UE monitors PEI when it has an emergence PDU session [10]. In details, for UE supports CN assigned subgrouping, if there is an active emergency PDU session, the UE won’t inform the CN that it has the corresponding UE capability on CN assigned subgrouping.

When the UE has an active emergency PDU Session: - The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

In this way, the CN won’t assign the subgroup ID for this UE [11]. Hence there won’t be paging latency for the CN assigned subgrouping. We think the similar method could be used for LP-WUS to avoid the paging latency during the emergency PDU session. However, it should be up to the SA2/CT1 decision.

Proposal 12: For CN assigned subgrouping, RAN2 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

For UE_ID based subgrouping, in the case of CN triggered paging for a UE with an emergency PDU session, the RAN doesn’t know if there is an emergency PDU session, thus the UE_ID based subgrouping for PEI is not able to be restricted which will introduce the paging latency. After the discussion within a few meetings in RAN2, RAN2 concluded that disabling UE_ID based PEI is not supported. It means that UE still monitors the PEI with UE_ID based subgrouping when it has an emergency PDU session. Considering the time offset between PEI and its corresponding PO is small, e.g., 50ms, the probability that the emergency PDU session arrives within the time offset is small. Besides, SA2 confirmed that the service impact is limited even without any enhancement on UE_ID based subgrouping since it only has impacts on the emergency call back scenario. Besides, other features in addition to the PEI may also cause the MT call set up delay, thus it is not a big deal.

Observation 5: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

However, since the time offset between LP-WUS and its corresponding PO may be much larger than the PEI scenario, the paging latency may not be acceptable when UE has an emergency PDU session. We need to discuss and collect the companies’ views on whether to resolve this paging latency issue during an emergency PDU session.

If the latency issue on the use of UE_ID based subgrouping for LP-WUS during the PDU session needs to be addressed, we think the following options could be considered:

- Option 1: Introducing an indication in NG paging, Xn paging (if needed) and F1 paging to disable the LP-WUS function.

- Option 2: Leaving it to UE implementation, i.e., if a UE supporting UE_ID based subgrouping indicates its capability to gNB, when there is an active emergency PDU session, the UE should update its UE capability via indicating the gNB that it doesn’t support the UE_ID based subgrouping to gNB by UE implementation. After the emergency PDU session is released, the UE could update its UE capability again and indicate it supports the UE_ID based subgrouping to gNB by UE implementation. Similar to the legacy, when UE reports the capability, the RAN will indicate UE RADIO CAPABILITY INFO INDICATION to the CN, and CN will indicate this capability in CN paging for RRC_IDLE UEs, or in NGAP core Networking assistance information for RRC_INACTIVE UEs.

Proposal 13: RAN2 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation, e.g. UE updates its capability of UE_ID based subgrouping for LP-WUS.

  - UE capability

For CN/RAN paging, the network needs to know the UE capability for paging to decide which kind of paging mechanism is used, e.g. PEI/LP-WUS or legacy paging. In Rel-17 PEI, separate  UE capabilities of CN assigned  subgrouping and UE_ID based subgrouping is introduced. Besides, the UE capability to indicate supporting PEI and UE_ID based subgrouping are same, i.e., if UE supporting PEI it will also support the UE_ID based subgrouping, and this combined UE capability is reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message. While UE’s capability of supporting the CN assigned subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure.

Observation 6: For UE capability design in Rel-17 PEI, the UE capability supporting PEI is also used to indicate the capability supporting UE_ID based subgrouping.

Observation 7: For UE capability design in Rel-17 PEI, the UE capabilities for UE_ID based subgrouping and CN assigned subgrouping are separate.

Considering LP-WUS is also used for CN/RAN paging, UE capability for LP-WUS should also be reported to the network like PEI. Different from PEI, besides two kinds of subgrouping methods, there are two kinds of LR receivers for LP-WUS, i.e. OOK-based WUR and OFDM-based WUR. Since RAN2 has agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs,  correspondingly, it is reasonable to introduce separate UE capabilities for OFDM-based WUR and OOK-based WUR. But the final decision should be determined in RAN1.

Proposal 14: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Regarding the UE capability for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, it is straightforward to follow the capability design method for PEI. That is, the UE capabilities on supporting CN assigned subgrouping and UE_ID based subgrouping are separately indicated. And no need to introduce additional UE capability for UE_ID based subgrouping, while using the UE capability supporting the LP-WUS to indicate whether supporting the UE_ID based subgrouping. Though there are separate UE capabilities for different types of LR, i.e., the UE capability supporting OFDM-based WUR and OOK-based WUR, we think no matter UE indicates which LR type among these two capabilities, it will also support the UE_ID based subgrouping.

Proposal 15: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, similar as PEI design.

Proposal 16: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Regarding the capability reporting for LP-WUS, we think the similar reporting mechanism for PEI could be reused directly. In detail, UE’s capability of supporting the CN assigned  subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure. And for the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping is reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message, and gNB will report this UE capability to AMF.

Proposal 17: The capability reporting design for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

After the AMF acquired the UE capabilities for LP-WUS, for RRC_IDLE UEs, the CN will include these UE capabilities in NG paging message to inform the gNB. For RRC_INACTIVE UEs, the CN will inform the gNB about these UE capabilities via NGAP core Networking assistance information IE, then the last serving gNB could store this UE capabilities. After the gNB acquire the UE capabilities for LP-WUS, it could determine whether to send LP-WUS with CN assigned subgrouping ID or UE_ID based subgrouping ID and which kind of WUR (i.e. OFDM-based WUR or OOK-based WUR) is used based on the included UE capability information.

Proposal 18: The CN paging message, includes UE’s OFDM/OOK-based WUR capability.

- Conclusion

In this contribution, we discuss the remaining issues for RRC_IDLE/ INACTIVE procedures with LP-WUR. Based on the discussion, we have the following observations and proposals.

LP-WUS configuration

Proposal 1: LP-WUS configuration is provided in SIB1.

Proposal 2: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

Proposal 3: If the network supports the OOK-based WUR, the LP-SS configuration should be always provided.

Entry/exit condition of using LP-WUS

Observation 1: RAN1 and RAN2 agreed that separate entry/exit thresholds can be configured for different types of LR.

Observation 2: The requirement for OFDM-based LP-WUR measuring only LP-SS has same requirement for OOK-based LP-WUR measuring LP-SS.

Observation 3: RAN1 agreed both MR threshold(s) and LR threshold(s) in the entry/exit condition for LP-WUS monitoring are separate for different types of LR.

Proposal 4: The entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS.

Proposal 5: RAN2 confirms the MR thresholds in entry/exit condition can be configured separately for different types of LR.

Proposal 6: RAN2 to discuss how OFDM-based WUR chooses the entry/exit thresholds for LP-WUS monitoring between the thresholds for OFDM-based WUR measuring SSB (LRType#3) and OFDM-based WUR measuring LP-SS (LRType#2).

Other wake up conditions

Proposal 7: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT, due to UL data and signaling arrival when monitoring LP-WUS.

LP-WUS Subgrouping

Observation 4: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Proposal 8: No need to introduce a network indication to indicate UE to monitor both LP-WUS and PEI or not.

Proposal 9: For LP-WUS UE_ID based subgrouping, UE_ID = 5G-S-TMSI mode (8192* 31).

Proposal 10: The network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration, similar as PEI design.

Proposal 11: For UE supporting both UE_ID based and CN assigned subgrouping, it is up to network configuration to use UE_ID based or CN assigned subgrouping, similar as PEI design.

Emergency PDU session

Observation 5: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

Proposal 12: For CN assigned subgrouping, RAN2 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

Proposal 13: RAN2 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation, e.g. UE updates its capability of UE_ID based subgrouping for LP-WUS.

UE capability

Observation 6: For UE capability design in Rel-17 PEI, the UE capability supporting PEI is also used to indicate the capability supporting UE_ID based subgrouping.

Observation 7: For UE capability design in Rel-17 PEI, the UE capabilities for UE_ID based subgrouping and CN assigned subgrouping are separate.

Proposal 14: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Proposal 15: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, similar as PEI design.

Proposal 16: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Proposal 17: The capability reporting design for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Proposal 18: The CN paging message, includes UE’s OFDM/OOK-based WUR capability.

- References

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- R2-2410912, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD

- R1-2403821, Final Report of 3GPP TSG RAN WG1 #116b v1.0.0

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO

- R4-2502600, WF on RRM requirements for NR_LPWUS

- R1-2401937, Final Report of 3GPP TSG RAN WG1 #116 v1.0.0

- Chair notes RAN1#120 eom0

- TS38.304 v18.4.0

- R2-2209018 Report of [AT119-e][004] [ePowSav] Subgrouping/PEI (MediaTek)

- TS24.501 v19.2.0