3GPP TSG-RAN WG2 Meeting #129bis		R2-2502156

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meetings, the following agreements for RRM measurement relaxation and offloading in RRC_IDLE and INACTIVE have been achieved [1] [2] [3] [4].

RAN2#127: - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis: - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128: - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. RAN2#129: Agreement on serving cell RRM relaxation criteria - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

In this contribution, we will further discuss the issues for MR serving/neighboring cell RRM measurements relaxation and serving cell RRM measurement offloading from MR to LP-WUR for RRC_IDLE/INACTIVE mode.

- Discussion

  - RRM relaxation and offloading criteria

Regarding the exit condition for serving cell measurement relaxation, RAN2 made one FFS in RAN2#128 meeting as follows.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements.

In R16/17 RRM relaxation, no separate exit condition is defined. That is, if the entry condition for neighboring cell measurement relaxation is not fulfilled, neighboring cell measurement relaxation is exited and UE performs the normal neighboring cell measurement. We think the same principle can be applied to R19 serving and neighboring cell measurement relaxation.

Proposal 1: For serving/neighboring cell measurement relaxation, no need to define a separate exit condition other than ‘not fulfilling the entry condition’.

According to the previous agreement, for the entry condition of serving cell measurement offloading, the MR-based threshold is always configured and the LR-based threshold is optionally configured.

Observation 1: For the entry condition of serving cell measurement offloading, the MR-based threshold is always configured and the LR-based threshold is optionally configured.

In case the LR-based threshold is also configured together with the MR-based threshold for the entry condition, how UE determines the entry condition for serving cell measurement offloading is fulfilled should be decided, i.e. when either condition or both conditions are fulfilled. Considering the MR-based entry condition has been agreed as the baseline, and in some case, MR-based measurement can not reflect the channel condition of LR measurement, then, LR-based measurement should be considered as the complimentary on top of MR-based measurement for the entry condition. Thus, it is more reasonable that the entry condition is fulfilled when both the MR-based threshold and LR-based threshold are met. Besides, the decision on the entry condition for LP-WUS monitoring/serving cell measurement relaxation has been made as below. Similar conclusion should be made for the condition of serving cell offloading.

For the entry condition of LP-WUS monitoring: If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). For the entry condition of serving cell measurement relaxation: If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

Proposal 2: If NW configures thresholds for both MR and LR measurements for the entry condition of serving cell offloading, then the entry condition is met when all the measured results are above the configured threshold(s).

In RAN2#127bis meeting, RAN2 made WA as below.

Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

Besides, RAN4 achieved the following agreements on serving and neighboring cell relaxation factors in RAN4#114 meeting [5].

Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

Observation 2: RAN4 agreed that scaling factor(s) for serving and neighboring cell measurement relaxation is 16.

Based on the RAN4 agreement, we think RAN2 WA can be confirmed and the thresholds of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation.

Proposal 3: Confirm the WA ‘For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’’.

Proposal 4: The corresponding threshold of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation.

For the case when MR serving cell measurement results are not available (i.e. serving cell measurement offloading), RAN2 made the following FFS for neighboring cell measurement relaxation in RAN2#127bis meeting.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

In RAN4#111 meeting [6], RAN4 agreed the following case#1.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

Based on case #1 above, we think MR does not need to perform neighboring cell measurement when serving cell measurement via MR is offloaded. In other words, the threshold of the entry/exit condition for serving cell measurement offloading should be higher than or equal to SIntraSearchP/SnonIntraSearchP. Therefore, LR measurement for evaluating neighboring cell measurement relaxation is not needed for the case when serving cell measurement offloading.

Proposal 5: The threshold of the criteria for serving cell measurement offloading should be higher than or equal to SIntraSearchP/SnonIntraSearchP, and LR measurement for evaluating neighboring cell measurement relaxation is not needed for the case when serving cell measurement via MR is offloaded.

Regarding the relationship between the entry condition for serving cell measurement relaxation and that for neighboring cell measurement relaxation, RAN2 made one FFS in RAN2#128 meeting as follows.

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

In R16/17 RRM relaxation, only neighboring cell measurement is relaxed. In our understanding, only serving cell measurement relaxation does not bring too much power saving for UE. Thus, a reasonable NW implementation will guarantee that the threshold of entry condition for serving cell measurement relaxation is higher than or equal to that of entry condition for neighboring cell measurement relaxation.

Proposal 6: It is up to network to configure the threshold of entry condition for serving and neighboring cell measurement relaxation, i.e. no restriction between the threshold for serving and neighboring cell measurement relaxation.

In RAN2#127 meeting, RAN2 agreed that separate entry/exit thresholds for LP-WUS monitoring can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Following the same logic, we think it can also apply to RRM relaxation/offloading. That is, if a cell supports both types of LRs, separate entry/exit thresholds of RRM relaxation/offloading can be configured for OFDM-based and OOK-based WUR.

Besides, as we descriped in our accompanied paper [7], RAN1 agreed that the entry/exit thresholds for  LP-WUS monitoring can be configured separately for different types of LR, which includes both MR thresholds and LR thresholds. Similarly, We think the same principles can be applied to serving cell RRM measurement relaxation/offloading and neighboring cell RRM measurement relaxation, i.e., separate entry/exit thresholds which includes both MR threshold(s) and LR threshold(s) can be configured separately for different types of LR for serving cell RRM measurement relaxation/offloading and neighboring cell RRM measurement relaxation.

Proposal 7: Separate thresholds, including MR thresholds and LR thresholds, for RRM relaxation/offloading criteria can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

For the entry/exit condition for LP-WUS monitoring, as we descriped in our accompanied paper [7], both RAN1 and RAN2 agreed that separate entry/exit thresholds can be configured for different types of LR, and for OFDM-based WUR measuring LP-SS only, entry/exit thresholds could be the same as the OOK-based WUR measuring LP-SS following the same logic as RAN4 agreement that the requirement for OOK-based WUR measuring LP-SS is reused for OFDM-based WUR monitoring LP-SS. For simplicity, the same principle can be applied to serving cell RRM measurement relaxation/offloading and neighboring cell RRM measurement relaxation.

Proposal 8: The entry/exit thresholds for OFDM-based WUR measuring LP-SS only, are the same as that for OOK-based WUR measuring LP-SS for serving cell RRM measurement relaxation/offloading and neighboring cell RRM measurement relaxation.

Regarding the specific metrics for RRM measurement offloading/relaxation criteria, in our view, the metrics for legacy RRM relaxation criteria and entry/exit condition for LP-WUS monitoring can be considered as baseline., i.e., the measurement metric is based on RSRP and optionally on RSRQ. In detail, in some cases, RSRP is enough to reflect the cell quality, while it is up to network to configure whether RSRQ is needed.

Proposal 9: The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

In the TS 38.304 running CR for LP-WUS [8], there are some FFSs related to the specific parameters of the serving cell quality by MR and LR for RRM relaxation and offloading as below. The Qrxlevmeas/Qqualmeas is the measured RSRP/RSRQ for the serving cell. While the Srxlev/Squal considers some additional parameters on top of the Qrxlevmeas/Qqualmeas for cell selection and re-selection. In the legacy Rel-16/17 RRM relaxation, the existing Srxlev/Squal is used for the serving cell quality by MR. We think it can be reused for the serving cell quality by MR measurement of Rel-19 RRM relaxation and offloading. As for the serving cell quality by LR measurement, the measured performance should be used. Because the additional parameters are configured for MR measurement and used for cell selection and re-selection, which should not be applied to LR.

Editor’s NOTE: FFS serving cell quality by MR is existing Srxlev/ Squal or Qrxlevmeas/Qqualmeas (i.e. measured value). Editor’s NOTE: FFS serving cell quality by LR is measured value.

Proposal 10: The serving cell quality by MR is the existing Srxlev/Squal, and the serving cell quality by LR is the measured performance, e.g. QLPrxlevmeas/QLPqualmeas (i.e. measured value) for the RRM measurement offloading/relaxation criteria.

Except for the criteria, the specific signalling to carry the RRM offloading/relaxation conditions and parameters should be also discussed. For R16/17 neighboring cell measurement relaxation, the corresponding conditions and parameters are configured in SIB2. We think it is straightforward to follow the same design for R19 RRM offloading/relaxation conditions and parameters.

Proposal 11: The R19 RRM offloading/relaxation conditions and parameters are configured in SIB2.

  - Relationship between RRM relaxation/offloading and LP-WUS monitoring

Regarding whether the relaxation criteria/threshold for serving cell measurement relaxation/offloading and neighbor cell measurement relaxation are the same as or separate from the entry condition/threshold for LP-WUS monitoring, which is described as the following directions, RAN2 needs further discussion.

- Direction 1: the criteria/threshold for RRM relaxation/offloading are the same as the entry condition for LP-WUS monitoring.

- Direction 2: the criteria/ threshold for RRM relaxation/offloading are separate from the entry condition for LP-WUS monitoring.

After the study in Rel-18, the following conclusion was made in TR 38.869 [9]:

UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore MR serving and neighbour cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or at least serving cell RRM offloaded from MR to LR are beneficial.

It could be observed that there is no power saving gain if UE starts to monitor LP-WUS while the serving cell measurement relaxation/offloading or neighbor cell measurement relaxation has not been performed. In this way, if the criteria/threshold for RRM relaxation/offloading are separate from the entry condition for LP-WUS monitoring, i.e. Direction 2, there may be a risk that the entry condition for LP-WUS monitoring is fulfilled, but the RRM relaxation/offloading criteria have not been fulfilled, which is not expected.

Observation 3: There is no power saving gain that UE monitors LP-WUS without RRM measurement relaxation/offloading.

Thus, network configuration on the specific threshold of the RRM relaxation/offloading criteria and entry condition for LP-WUS monitoring should avoid such case without any power saving gain. In order to guarantee serving/neighboring cell measurement is relaxed when UE monitors LP-WUS, thresholds of the criteria for the serving/neighboring cell measurement relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring. But for simplicity, it is better to adopt the same threshold.

As for serving cell measurement offloading, considering the entry conditions of LP-WUS monitoring include at least serving cell quality via MR, serving cell measurement via MR can not be offloaded to LR before UE enters the LP-WUS monitoring state. Thus thresholds of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Based on the analysis above, we provide two examples to illustrate the thresholds and UE behaviors for the serving cell measurement relaxation/offloading and neighboring cell measurement relaxation comparing to the threshold of entry condition on LP-WUS monitoring in the following figures.

Figure 1: Thresholds and UE behaviors for serving cell offloading/neighboring cell measurement relaxation and LP-WUS monitoring

Figure 2: Thresholds and UE behaviors for serving/neighboring cell measurement relaxation and LP-WUS monitoring

For simplicity, the serving cell measurement offloading and serving cell measurement relaxation are provided in Figure 1 and Figure 2, respectively. All the criteria for serving cell measurement relaxation/offloading and neighboring cell measurement relaxation and the entry condition for LP-WUS monitoring are based on the serving cell quality via MR.

For neighbor cell measurement relaxation, it refers to the neighboring cell measurement for intra-frequency/inter-frequency with equal or lower priority frequency. As for the inter-frequency with higher priority, it will be more complicated and can be further discussed later. When the serving cell quality based on the measurement via MR exceeds the legacy S-measure threshold, there is no neighboring cell measurement to be performed according to the current specification. Besides, it is plain and obvious that the threshold of the legacy S-measure threshold is higher than or equal to the threshold for the neighboring cell measurement relaxation (i.e. further relaxation is used to differentiate the RRM relaxation in Rel-16/17), since the neighboring cell measurement can be further relaxed only in case UE needs to perform neighboring cell measurement.

In Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring with respect to the S-measure threshold, which could either be higher (i.e. Case 1) or lower (i.e. Case 2) than legacy S-measure threshold with the further details as below. Besides, as the analysis above, the threshold of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring and the S-measure threshold. For the green arrow, it represents that UE performs serving cell offloading if the serving cell offloading criteria are fulfilled.

- For Case 1, i.e. the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell offloading criteria is fulfilled, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR and UE won’t perform neighboring cell measurement.

- For Case 2, i.e. the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR when the corresponding conditions are fulfilled.

In Figure 2, similar to Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring and serving cell measurement relaxation criteria with respect to the S-measure threshold, i.e., Case 1 and Case 2 with the further details as below.

- For Case 1, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell measurement relaxation is fulfilled, UE will monitor LP-WUS and perform serving cell measurement relaxation and no neighboring cell measurement.

- For Case 2, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement relaxation, and MR neighboring cell measurement relaxation.

Based on the analysis and examples above, we have the following proposals:

Proposal 12: The threshold of the criteria for serving/neighboring cell measurement relaxation should be at least equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 13: The threshold of the criteria for serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

- Conclusion

In this contribution, we discuss the RRM relaxation of UE MR for both serving and neighboring cell measurements, and UE serving cell RRM measurement offloading from MR to LP-WUR. Based on the discussion, we have the following observations and proposals.

RRM relaxation and offloading criteria

Observation 1: For the entry condition of serving cell measurement offloading, the MR-based threshold is always configured and the LR-based threshold is optionally configured.

Observation 2: RAN4 agreed that scaling factor(s) for serving and neighboring cell measurement relaxation is 16.

Proposal 1: For serving/neighboring cell measurement relaxation, no need to define a separate exit condition other than ‘not fulfilling the entry condition’.

Proposal 2: If NW configures thresholds for both MR and LR measurements for the entry condition of serving cell offloading, then the entry condition is met when all the measured results are above the configured threshold(s).

Proposal 3: Confirm the WA ‘For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’’.

Proposal 4: The corresponding threshold of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation.

Proposal 5: The threshold of the criteria for serving cell measurement offloading should be higher than or equal to SIntraSearchP/SnonIntraSearchP, and LR measurement for evaluating neighboring cell measurement relaxation is not needed for the case when serving cell measurement via MR is offloaded.

Proposal 6: It is up to network to configure the threshold of entry condition for serving and neighboring cell measurement relaxation, i.e. no restriction between the threshold for serving and neighboring cell measurement relaxation.

Proposal 7: Separate thresholds, including MR thresholds and LR thresholds, for RRM relaxation/offloading criteria can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Proposal 8: The entry/exit thresholds for OFDM-based WUR measuring LP-SS only, are the same as that for OOK-based WUR measuring LP-SS for serving cell RRM measurement relaxation/offloading and neighboring cell RRM measurement relaxation.

Proposal 9: The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

Proposal 10: The serving cell quality by MR is the existing Srxlev/Squal, and the serving cell quality by LR is the measured performance, e.g. QLPrxlevmeas/QLPqualmeas (i.e. measured value) for the RRM measurement offloading/relaxation criteria.

Proposal 11: The R19 RRM offloading/relaxation conditions and parameters are configured in SIB2.

Relationship between RRM relaxation/offloading and LP-WUS monitoring

Observation 3: There is no power saving gain that UE monitors LP-WUS without RRM measurement relaxation/offloading.

Proposal 12: The threshold of the criteria for serving/neighboring cell measurement relaxation should be at least equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 13: The threshold of the criteria for serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

- References

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD

- R2-2410912, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD

- R2-2501332, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others

- R4-2502600, WF on RRM requirements for NR_LPWUS

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR

- R2-2502155, Discussion on LP-WUS WUR in RRC_IDLE INACTIVE

- TS38.304 v18.4.0

- 3GPP TR 38.869 v18.0.0