3GPP TSG-RAN WG2 Meeting #129bis		R2-2502157

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we will further discuss the remaining issues on LP-WUS/WUR in RRC_CONNECTED from RAN2 perspective.

- Discussion

  - LP-WUS with short C-DRX

In RAN2#129 meeting, we replied an LS to RAN1 and have an FFS on whether short DRX can be configured and used with Option 1-1 and Option 1-2[1]:

- Overall Description: RAN2 thanks RAN1 for the LS on LP-WUS operation in CONNECTED mode. RAN2 confirmed as the following - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer. RAN2 will continue to discuss whether short DRX can be configured and used with Option 1-1 and Option 1-2.

For Option 1-1, since RAN2 has the agreement:

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

Considering DCP is only applied when UE uses Long DRX cycle, then it should be confirmed that Option 1-1 is also applied only to Long DRX cycle. In this case, when DRX Command MAC CE and Long DRX Command MAC CE is received to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, the UE behaviour should be:

- If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

- If UE receives Long DRX Command MAC CE or DRX Command MAC CE in case Short DRX cycle is not configured, UE will use Long DRX cycle according to current specification, since UE will leave the C-DRX Active time when Long DRX Command MAC CE is received, the LP-WUS will be monitored in this case.

Proposal 1: RAN2 confirms in Option 1-1, LP-WUS is only applied when UE using Long DRX cycle.

- If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

For Option 1-2, RAN2 has the following agreements [3]:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

It could be observed that for LP-WUS Option 1-2, there is no difference for UE behaviour on LP-WUS monitoring and PDCCH monitoring between Long DRX cycle and Short DRX cycle. No matter Long DRX cycle or Short DRX cycle is used, UE configured with LP-WUS Option 1-2 has the same behaviour, i.e., UE monitors LP-WUS outside C-DRX Active Time and starts to monitor PDCCH after receiving LP-WUS during the timer triggered by LP-WUS.

In this way, it is up to network to configure UE to use short or long DRX cycle. While in typical case, network will only configure long DRX cycle when using LP-WUS Option 1-2. Thus, there is no restriction on Long DRX cycle and Short DRX cycle for the use of LP-WUS Option 1-2, and the LP-WUS operation will not be affected by either long and short DRX configuration, i.e. no spec impact.

Proposal 2: RAN2 confirms in Option 1-2, LP-WUS operator is not affected by short DRX configuration, i.e. no spec impact. LP-WUS is monitored outside active time, and UE starts to monitor PDCCH in the new timer after receiving LP-WUS, as captured in MAC.

  - CSI report behaviour within the Active time triggered by LP-WUS

For LP-WUS Option 1-2, in RAN2#128 meeting, we agreed that the drx-ondurationTimer is not started if Option 1-2 LP-WUS is configured and in this case, during the non-Active time configured by drx-onDurationTimer UE doesn’t report the semi-periodic CSI and report the periodic CSI/L1-RSRP based on network configuration [3], for example:

- If lpwus-TransmitOtherPeriodicCSI is configured with value true, UE reports periodic CSI;

- If lpwus-TransmitPeriodicL1-RSRP is configured with value true, UE reports periodic L1-RSRP.

- The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

Observation 1: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

However, we have not discussed the UE behaviour on CSI report during the Active Time triggered by LP-WUS Option 1-2. Since when LP-WUS Option 1-2 is configured, the drx-onDurationTimer is not started, then the Active Time triggered by LP-WUS is most frequently compared with Active Time caused by other triggers, thus we think the LP-WUS triggered C-DRX Active Time is more similar to the legacy drx-onduraionTimer rather than C-DRX Active Time caused by other triggers. Accordingly, the CSI/L1-RSRP reporting behaviour in LP-WUS triggered C-DRX Active Time should be same as the legacy drx-ondurationTimer. Specifically, UE reports the periodic CSI on PUCCH, Semi-periodic CSI report on PUCCH and Semi-periodic CSI report on PUSCH during this timer triggered by LP-WUS is running. Otherwise, insufficient CSI reporting may cause the failure of data transmission during Active Time when CSI-mask is set to true.

Proposal 3: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

  - LP-WUS MO configuration and collision issue

UAI report time offset between LP-WUS and PDCCH monitoring

In RAN1#118bis and RAN1#120 meeting, RAN1 agreed that UE reports the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting, besides, UE could indicate via UAI its preference for the configured time offset to the network [4].

Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Note: Other values are not precluded - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities - FFS whether the same set of candidates are used for different SCSs - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - FFS: candidate values that can be indicated by UAI

In our understanding, if the UE reports the time offset via the UAI, the network will configure the time offset between LP-WUS and PDCCH monitoring according to the UAI, and the time when MR is waked up is determined by the UE capability. The figure1 shows the following 3 cases:

- Case#1: MR wakes up earlier than the configured time offset, e.g., t1 in figure1. In this case, the value of time offset reported via UAI is longer than the value reported by UE capability.

- Case#2: MR wakes up just as the configured time offset, e.g., t2 in figure1. In this case, the value of time offset reported via UAI is longer than or same as the value reported by UE capability.

- Case#3: MR wakes up later than the configured time offset, e.g., t3 in figure1. In this case, the value of time offset reported via UAI is shorter/longer than or same as the value reported by UE capability.

Figure 1: Time offset between Option 1-2 LP-WUS and its corresponding timer

Case#2 is the ideal scenario which reached the best balance on PDCCH monitoring latency and power saving gain. For Case#1, the power saving gain is not as good as Case#2, however considering the whole time offset between LP-WUS and PDCCH monitoring will not be long, the gain difference is marginal between the two cases. Thus Case#1 and Case#2 are both acceptable for LP-WUS.

Observation 2: When LP-WUS triggers PDCCH monitoring, the scenario in which MR wakes up earlier than the configured time offset couldn’t reach the best power saving gain than the scenario in which MR wakes up same as the configured time offset.

For case#3, since the start time for PDCCH monitoring is passed for the UE, UE could only monitor PDCCH from t3 in the above example. While for the network, it will consider t2 as the start of PDCCH monitoring time for the UE, thus it will send the PDCCH from t2. In this case, the PDCCH will be missed by the UE, which is not accepted.

Observation 3: When LP-WUS triggers PDCCH monitoring, UE may miss the PDCCH if MR wakes up later than the configured time offset.

With the above, we think case#3 should be avoided as much as possible. Thus, we suggest RAN2 to restrict that the value reported via UAI is longer than UE capability.

Proposal 4: The preferred value of time offset between LP-WUS and PDCCH reported via UAI is expected to be longer than the value for the minimum time gap between LP-WUS and MR to start PDCCH monitoring corresponding to the SCS reported via UE capability.

LP-WUS occasion Collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR

In Rel-16 Power Saving, RAN2 has discussed the collision scenarios for DCP [2]:

- DCP occasion is overlapped with the C-DRX Active Time;

- DCP occasion is overlapped with the measurement gap or within BWP switching interruption length;

- DCP collides with the RAR addressed to C-RNTI, i.e., DCP occasion is during ra-ResponseWindow for BFR via RACH.

When the above collision occurs, UE couldn’t monitor the DCP and will start the corresponding drx-onDurationTimer directly. Similarly, one issue needs to be discussed is for LP-WUS Option 1-1 and Option 1-2, whether a UE is expected or not expected to monitor LP-WUS in C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Figure 2: LP-WUS occasion collides with C-DRX Active Time

This issue is related to whether LR and MR are able to work simultaneously. In our view, it highly depends on UE implementation. In case some hardware e.g., antenna is shared between MR and LR, the UE can only use one receiver at a time. For such UE implementation, the LP-WUR and MR are not able to work simultaneously. Therefore, it may be safer to assume that as a baseline design, LP-WUR is not able to receive signal(s) when MR transmits/receives signals/channels. Thus, the collision issue should be considered for LP-WUS Option 1-1 and Option 1-2.

Proposal 5: For LP-WUS Option 1-1 and Option 1-2, RAN2 should consider the UE behaviour when all LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

For LP-WUS Option1-1, we have the following agreement in RAN2#126 meeting [5] ：

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

Thus, if all LO collides with the C-DRX Active Time/measurement gap/BWP switching interruption length/RAR for BFR, UE couldn’t monitor the LP-WUS and UE will start the corresponding drx-onDurationTimer.

For LP-WUS Option 1-2, it is also possible that all LP-WUS occasion collides with the C-DRX Active Time/measurement gap/BWP switching interruption length/RAR for BFR, in this case, since the UE couldn’t monitor the LP-WUS, one issue is whether UE starts the new timer corresponding to the LP-WUS should be considered. In our understanding, a similar mechanism for DCP could be reused, i.e., UE starts the new timer for PDCCH monitoring to avoid missing the potential PDCCH.

Proposal 6: For LP-WUS Option 1-1 and Option 1-2, in case all the LP-WUS occasion of an LP-WUS collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR, UE starts the corresponding drx-onDurationTimer for LP-WUS Option 1-1 and the corresponding new timer for LP-WUS Option 1-2.

Miss-detection for Option 1-1

For DCP, without any collision, it is still possible that UE cannot detect the DCP, e.g. the channel quality is not good enough for DCP reception. In DCP design, it is up to the network to configure whether UE should start the corresponding drx-onDurationTimer when DCP is not detected, i.e. based on the RRC parameter ps-WakeUp.

Since there is no agreement on whether there is an explicit indication carried in LP-WUS to indicate whether to start/not start the drx-onDurationTimer for Option 1-1 and start the new timer for Option 1-2, or just when receiving the LP-WUS, UE needs to start the corresponding timer otherwise if UE doesn’t receive the LP-WUS, UE won’t start the timer. We couldn’t conclude whether UE could determine whether the mis-detection scenario occurs. However, since RAN2 agreed, the solutions/ operations introduced for DCP mechanism is taken as baseline for Option 1-1, we think the former method will be applied to Option 1-1, thus UE could determine whether miss the LP-WUS. To handle the mis-detection of Option 1-1, we could introduce a similar RRC parameter, e.g. lpwus-WakeUp, to control whether UE should start the corresponding drx-onDurationTimer for LP-WUS Option 1-1. But the final decision should be based on LP-WUS design in RAN1. For Option 1-2, we could wait for further progress on the LP-WUS design from RAN1.

Proposal 7: For LP-WUS Option 1-1, in case UE cannot detect all LP-WUS occasion of a LP-WUS, RAN2 assumes it is up to the network configuration for UE to determine whether to start the corresponding drx-onDurationTimer, e.g. lpwus-Wakeup.

  - LP-WUS operation for UE configured with dual DRX

In Release 16, secondary DRX group configuration is introduced for one cell group. And RAN2 concluded that the DCP could not be configured when the secondary DRX group is configured for simplicity since there is not enough time in Release 16 to discuss the coexistence issue [6].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

However, for LP-WUS Option 1-1 and Option 1-2, we think the restriction for DCP is not needed since we have enough time to consider the coexistence between LP-WUS and dual DRX. In this case, except the length of drx-onDurationTimer and drx-InactivityTimer, other DRX parameters are common for two DRX groups and the start of the drx-onDurationTimer is also the same for two DRX groups. Thus, the Active Time for two DRX groups is overlapped. When there is traffic, the Active Times of the two DRX groups will probably be aligned. In addition, LP-WUS monitoring on both primary and secondary DRX groups in parallel will increase the UE complexity and power consumption. So, we slightly prefer using one set of LP-WUS configuration for dual DRX groups per MAC entity. However, whether there is a need to include the DRX group specific indication, e.g.  the LP-WUS is for secondary DRX group and/or for default DRX group could be further discussed. Besides, for Option 1-2, whether the timer triggered by LP-WUS for PDCCH monitoring, e.g. the lpwus-PDCCHMonitoringTimer is configured per MAC entity or per DRX group should also be further discussed.

Proposal 8: For both Option 1-1 and Option 1-2, LP-WUS could co-exist with dual DRX. And one set of LP-WUS configuration is configured per MAC entity. FFS whether introducing an indication in LP-WUS to indicate the DRX group and FFS whether the lpwus-PDCCHMonitoringTimer is configured per MAC entity or per DRX group.

  - LP-WUS Option3

RAN1 has agreed to support LP-WUS Option 1-1 and Option 1-2, however, for Option 1-3, RAN1 didn’t have any conclusion on it. Since Option 1-3 is transparent to MAC, which only impacts the physical layers design, we think it is better to wait for RAN1 progress, if any. RAN2 should focus on Option 1-1 and Option 1-2 and won’t discuss Option 1-3 unless RAN1 triggers.

Proposal 9: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

  - Enable and disable for LP-WUS monitoring

RAN1 has also agreed that gNB RRC signalling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID, current RRM/RLM/BFD/CSI/BM measurement and corresponding report are performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary.

Proposal 10: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Regarding the RRC message to carry the LP-WUS configuration, RAN2 agreed the dedicated RRC message is used.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In our understanding, it is straightforward to use the RRCResume and RRCReconfiguration messages as usual.

Proposal 11: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

From RAN2 perspective, we want to clarify the following terms firstly:

- Enable for LP-WUS monitoring: Which is to enable the LP-WUS functionality while UE could use LP-WUS only if the LP-WUS functionality is enabled and is activated if there is some activation mechanisms for LP-WUS;

- Activation for LP-WUS monitoring: When LP-WUS function is enabled, it is possible for network to activate the LP-WUS monitoring, and in this case UE could monitor LP-WUS; otherwise if the LP-WUS is not enabled, the activation procedure for LP-WUS is not expected to happed;

- Deactivation for LP-WUS monitoring: After LP-WUS monitoring is activated, upon the corresponding deactivation mechanism is performed, UE stops using the LP-WUS functionality.

Considering the LP-WUS configuration overhead is comparable to other dedicated configurations, e.g. L3 measurement configuration, and the LP-WUS monitoring is not an urgent behaviour, there is no motivation to introduce additional activation/deactivation procedure for LP-WUS monitoring. UE could use the LP-WUS when the LP-WUS monitoring is enabled, i.e., when UE receives the RRC dedicated signalling with LP-WUS configuration. And if the network doesn’t expect the UE to use the LP-WUS, it could just send the RRC signalling to disable the LP-WUS monitoring for the UE.

Besides, according to the following RAN1 agreement [7], it could be observed LP-WUS could be used if it is enabled, thus there is no other additional activation/deactivation mechanism for LP-WUS.

Conclusion From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1

Proposal 12: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

  - LP-WUS impact on SR/PRACH/CG

According to the above agreement in RAN1, it could be observed that LP-WUS won’t impact the UE bahaviour on SR, PRACH and CG-PUSCH transmission.

Observation 4: RAN1 agreed LP-WUS won’t impact the UE bahaviour on SR, PRACH and CG-PUSCH transmission.

From RAN2 perspective, in case uplink transmission such as SR, PRACH, Configured Grant (CG) PUSCH are triggered by UE itself, the UL transmission should be transmitted irrespective WUS monitoring, and MR PDCCH monitoring is started after the UL transmission according to the legacy UE behavior. From our understanding, it is the legacy UE behavior and should not be impacted after the introduction of LP-WUS.

Proposal 13: RAN2 confirms LP-WUS won’t impact the SR, PRACH and CG-PUSCH transmission, unless critical issue is identified.

  - LP-WUS impact on Cell DTX

According to the following RAN1 agreements, it could be observed LP-WUS could be configured together with Cell DTX and RAN1 has an FFS that whether UE is expected to monitor the LP-WUS during the Cell DTX inactive time.

For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX - FFS During Cell DTX inactive time, whether the UE is expected to monitor LP-WUS or not

According to the TS 38.321, it could be observed that the Cell DTX won’t impact the DCP monitoring:

For each Serving Cell configured with cell DTX, the MAC entity need not: 1> if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period: 2> monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause; 2> instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS; 2> indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity; 2> set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment; 2> consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

Observation 5: Cell DTX won’t impact the DCP monitoring, i.e., UE could monitor the DCP during the cell DTX inactive time.

Following the same design, LP-WUS in Option 1-1 could also be monitored during the cell DTX inactive time. And since the network should ensure that there is at least partial overlapping between UE's connected mode DRX on-duration and cell DTX/DRX active duration, UE could monitor the PDCCH after LP-WUS indicating UE to start the drx-OndurationTimer.

Proposal 14: During Cell DTX inactive time, UE is expected to monitor LP-WUS for Option 1-1.

Regarding to the Option 1-2, it is straightforward to follow the same mechanism as Option 1-1. However, since the timer triggered by LP-WUS couldn’t be always overlapped with the Cell DTX active duration, in case the LP-WUS cycle is shorter than the DRX cycle, UE may monitor the LP-WUS in vain since reasonable network implementation is to only provide LP-WUS to trigger the PDCCH monitor during the cell DRX active time. Therefore, for LP-WUS operation Option 1-2, it is reasonable that UE monitor WUS only within cell DTX active time for both network and UE energy savings. When LP-WUS is detected, the UE starts the new timer for PDCCH monitoring and enters the C-DRX active time.

Proposal 15: For LP-WUS operation Option 1-2 configured with Cell DTX, a UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell.

- Conclusion

In this contribution, we discuss the remaining issues on LP-WUS triggers PDCCH monitoring in RRC_CONNECTED. Based on the discussion, we have the following observations and proposals.

Observation 1: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

Observation 2: When LP-WUS triggers PDCCH monitoring, the scenario in which MR wakes up earlier than the configured time offset couldn’t reach the best power saving gain than the scenario in which MR wakes up same as the configured time offset.

Observation 3: When LP-WUS triggers PDCCH monitoring, UE may miss the PDCCH if MR wakes up later than the configured time offset.

Observation 4: RAN1 agreed LP-WUS won’t impact the UE bahaviour on SR, PRACH and CG-PUSCH transmission.

Observation 5: Cell DTX won’t impact the DCP monitoring, i.e., UE could monitor the DCP during the cell DTX inactive time.

LP-WUS with short C-DRX

Proposal 1: RAN2 confirms in Option 1-1, LP-WUS is only applied when UE using Long DRX cycle.

- If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

Proposal 2: RAN2 confirms in Option 1-2, LP-WUS operator is not affected by short DRX configuration, i.e. no spec impact. LP-WUS is monitored outside active time, and UE starts to monitor PDCCH in the new timer after receiving LP-WUS, as captured in MAC.

CSI report behaviour within the Active time triggered by LP-WUS

Proposal 3: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

LP-WUS MO configuration and collision issue

Proposal 4: The preferred value of time offset between LP-WUS and PDCCH reported via UAI is expected to be longer than the value for the minimum time gap between LP-WUS and MR to start PDCCH monitoring corresponding to the SCS reported via UE capability.

Proposal 5: For LP-WUS Option 1-1 and Option 1-2, RAN2 should consider the UE behaviour when all LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Proposal 6: For LP-WUS Option 1-1 and Option 1-2, in case all the LP-WUS occasion of an LP-WUS collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR, UE starts the corresponding drx-onDurationTimer for LP-WUS Option 1-1 and the corresponding new timer for LP-WUS Option 1-2.

Proposal 7: For LP-WUS Option 1-1, in case UE cannot detect all LP-WUS occasion of a LP-WUS, RAN2 assumes it is up to the network configuration for UE to determine whether to start the corresponding drx-onDurationTimer, e.g. lpwus-Wakeup.

LP-WUS operation for UE configured with dual DRX

Proposal 8: For both Option 1-1 and Option 1-2, LP-WUS could co-exist with dual DRX. And one set of LP-WUS configuration is configured per MAC entity. FFS whether introducing an indication in LP-WUS to indicate the DRX group and FFS whether the lpwus-PDCCHMonitoringTimer is configured per MAC entity or per DRX group.

LP-WUS Option3

Proposal 9: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

Enable and disable for LP-WUS monitoring

Proposal 10: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 11: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

Activation and deactivation procedure for LP-WUS monitoring

Proposal 12: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

LP-WUS impact on SR/PRACH/CG

Proposal 13: RAN2 confirms LP-WUS won’t impact the SR, PRACH and CG-PUSCH transmission, unless critical issue is identified.

LP-WUS impact on Cell DTX

Proposal 14: During Cell DTX inactive time, UE is expected to monitor LP-WUS for Option 1-1.

Proposal 15: For LP-WUS operation Option 1-2 configured with Cell DTX, a UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell.

- References

- R2-2501395, Reply LS on LP-WUS operation in CONNECTED mode

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

- RAN2#128_ChairNotes.

- 3GPP RAN1#118bis meeting, Chairman notes

- RAN2#126_ChairNotes.

- 3GPP TS 38.331:"NR; Radio Resource Control (RRC) protocol specification".

- 3GPP RAN1#120 meeting, Chairman notes