3GPP TSG-RAN WG2 Meeting #129bis		R2-2502159

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreements were made for inter-CU LTM [1]:

Agreements on inter-CU LTM:

- Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s).

- Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- How to support shared RACH resources for inter-CU LTM is left to RAN3.

In this paper, we first discuss the remaining open issues for inter-CU LTM in the SA scenario. Then we discuss the remaining issues of inter-CU LTM with DC.

- Discussion

  - Remaining issues for Inter-CU LTM in SA

Security key handling

According to the LS [2], SA3 has agreed that NCC can be transmitted to the UE in plaintext via MAC CE message, and SA3 will not consider the protection of MAC CE messages in this release. Therefore, we propose that RAN2 confirms support for this SA3 solution.

Proposal 1: RAN2 confirms that the SA3 solution is supported, i.e., NCC can be transmitted to the UE in plaintext via the LTM cell switch command message.

If Proposal 1 is agreed, RAN2 needs to discuss whether a new Rel-19 LTM cell switch command MAC CE is required to include the NCC for inter-CU LTM. According to current TS 38.321 [3], the Rel-18 LTM cell switch command MAC CE has been defined as follows:

Figure 1. Rel-18 LTM Cell Switch Command MAC CE

Since all fields in the legacy Rel-18 LTM Cell Switch Command MAC CE are needed for inter-CU LTM, we believe that the legacy Rel-18 LTM Cell Switch Command MAC CE can be reused for Rel-19 inter-CU LTM by utilizing a R bit to indicate whether the NCC field is present (e.g., using the R bit in Oct 3 to indicate whether NCC field is present). There is no need to introduce a new MAC CE for inter-CU LTM cell switch. We show the MAC CE format in Figure 2 below:

Figure 2. LTM Cell Switch Command MAC CE with NCC

Proposal 2: If proposal 1 is agreed, the Rel-18 LTM cell switch command MAC CE should be reused for inter-CU LTM with an NCC field added by utilizing a R bit to indicate whether the NCC field is present.

For inter-CU LTM cell switch, the network should ensure that the NCC is included in the LTM cell switch command MAC CE. If the NCC is not included for an inter-CU LTM cell switch, UE shall ignore the LTM Cell Switch Command or trigger RRC re-establishment.

Proposal 3: Network ensures that NCC is included in the LTM Cell Switch Command MAC CE for an inter-CU LTM cell switch. If the NCC is not included for an inter-CU LTM cell switch, UE shall ignore the LTM Cell Switch Command or trigger RRC re-establishment.

For intra-CU LTM cell switch, security key update is not needed, therefore, the UE shall ignore the NCC field for intra-CU LTM cell switch if indicated in the LTM Cell Switch Command.

Proposal 4: UE shall ignore the corresponding NCC field for intra-CU LTM cell switch if indicated in the LTM cell switch command.

LTM fast recovery

There are mainly two new cases that need to be discussed for Rel-19 LTM fast recovery:

- Case 1: Perform intra-CU LTM fast recovery (i.e., the target cell of LTM fast recovery is an intra-CU LTM candidate cell) after an inter-CU LTM cell switch failure

- Case 2: Perform inter-CU LTM fast recovery (i.e., the target cell of LTM fast recovery is an inter-CU LTM candidate cell) after RLF or reconfiguration with sync failure

As for case 1, in Rel-18, it was agreed that LTM fast recovery is not supported for T304 expiry after L3 HO and CHO:

RAN2#125bis Agreement

- LTM recovery is not supported for T304 expiry after HO (L3 HO) and CHO, but supported for T304 expiry after LTM cell switch (and RLF as earlier agreed). This agreement supersedes earlier agreements.

RAN2#125bis Agreement

- LTM recovery is not supported for T304 expiry after HO (L3 HO) and CHO, but supported for T304 expiry after LTM cell switch (and RLF as earlier agreed). This agreement supersedes earlier agreements.

The motivation of the agreement is to avoid performing an intra-CU LTM fast recovery after an inter-CU handover. This is because, following an inter-CU handover, PDCP re-establishment and RLC re-establishment will be performed, making it difficult for a UE to revert back to the source cell configuration once buffers and state variables have been reset. Therefore, we believe that Rel-19 can follow Rel-18’s principle, i.e., LTM fast recovery targeting an intra-CU LTM candidate cell should not be supported for T304 expiry after inter-CU LTM cell switch.

Proposal 5: Following the Rel-18 principle, LTM fast recovery to an intra-CU LTM candidate cell is not supported for T304 expiry after an inter-CU LTM cell switch.

As for case 2, whether inter-CU LTM fast recovery can be supported depends on two factors:

- Whether there exists a valid NCC at UE

- Whether candidate cells have security keys corresponding to this NCC.

According to the LS from SA3 [2], SA3 has requested RAN3 to discuss details regarding key distribution between the source gNB and target gNB. Therefore, whether candidate cells other than the target cell have available security keys for the inter-CU LTM fast recovery depends on the key distribution approach discussed in RAN3. We propose that RAN2 should postpone the discussion on inter-CU LTM fast recovery until the key distribution approach is finalized in RAN3.

Observation 1: The feasibility of supporting LTM fast recovery to an inter-CU LTM candidate cell depends on the key distribution approach between the source gNB and the target gNB.

Proposal 6: RAN2 should postpone the discussion on LTM fast recovery to an inter-CU LTM candidate cell until the key distribution approach is finalized in RAN3.

UE-based TA measurement

In RAN2#126 meeting, whether UE-based TA measurement can be supported for inter-CU LTM was discussed. Some companies believe that RAN4 has already defined the UE requirement for UE-based TA measurement and it applies to both intra-CU and inter-CU scenarios. From our understanding, UE-based TA measurement is supported in synchronized cases in Rel-18 according to the reply LS by RAN4 [4]. Therefore, when the serving cell and candidate cells in Rel-19 from different CUs are synchronized, UE-based TA measurement is supported obviously and the Rel-18 mechanism can be reused.

Observation 2: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 7: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

However, for Rel-19 inter-CU LTM, as the serving cell and the candidate cells may belong to different CUs and are more likely to be unsynchronized, it seems highly beneficial to support UE-based TA measurement for unsynchronized scenarios. The decision on whether UE-based TA measurement for unsynchronized cases can be supported and how to support it should be made by RAN1 and RAN4. Therefore, an LS should be sent to RAN1 and RAN4 to inquire about the feasibility and approach for supporting UE-based TA when the serving cell and the candidate cell are unsynchronized.

Proposal 8: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 9: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

CFRA resource in LTM Cell Switch Command MAC CE

To support the CFRA resource configuration included in the LTM cell switch command MAC CE for inter-CU LTM, two approaches can be considered:

Approach 1: Allocated by Target DU

- The CFRA resources are allocated by T-DU (Target Distributed Unit);

- Before sending an LTM Cell Switch Command MAC CE, S-DU (Source Distributed Unit) retrieves dedicated CFRA resources per UE from T-DU.

Approach 2: Allocated by Source DU

- During the handover preparation phase, S-DU obtains a shared CFRA resource pool from T-DU for multiple UEs within a single DU unit;

- S-DU then allocates specific CFRA resources to UEs requiring an LTM Cell Switch.

For approach 1 (CFRA resource allocated by Target DU), specific CFRA resources need to be reserved for each UE. This could lead to significant resource wastage when subsequent LTMs are supported since CFRAs would have to remain reserved over extended periods of time. Additionally, if CFRA resources are allocated by Target DU, there would be no practical difference between including them in an LTM Cell Switch Command MAC CE versus including them in preconfigured candidate RRC configurations. This goes against our original motivation behind including CFRA resources within an LTM Cell Switch Command MAC CE.

For approach 2 (CFRA resource allocated by source DU), CFRA resources are shared among UEs within a single DU, which is similar to the early-RACH resources sharing mechanism supported in Rel-18 LTM. The S-DU is responsible for allocating specific CFRA resources for each UE to perform RACH-based LTM cell switch while simultaneously avoiding the CFRA resource conflicts. As a result, the resource wastage can be largely reduced. Therefore, we prefer using Approach 2.

Proposal 10: For inter-CU LTM, CFRA resources for RACH-based LTM could be shared among multiple UEs within a single gNB-DU and assigned by S-DU in the LTM cell switch command MAC CE.

If CFRA resources for RACH-based LTM are shared among multiple UEs, the source DU needs to inform the target DU about the CFRA resource information (e.g., preamble index and RA-RNTI) indicated in the LTM cell switch command. This ensures that the target cell knows which UE is performing RACH-based LTM and which CFRA resources are being used.

Observation 3: For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used, when the CFRA resources are shared among multiple UEs.

Proposal 11: Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

Coexistence of inter-CU LTM with MIMO 2TA

In the last meeting, there was an FFS regarding whether Rel-19 inter-CU LTM can coexist with MIMO two TA. In our view, the analysis can be divided into two parts:

- Coexistence of Rel-19 inter-CU LTM and Rel-18 MIMO two TA

For this part, we believe no fundamental differences exist between this scenario and the coexistence of Rel-18 intra-CU LTM with Rel-18 MIMO two TA. Therefore, we propose that RAN2 confirms that R19 inter-CU LTM can coexist with Rel-18 MIMO two TA.

Proposal 12: RAN2 confirms that R19 inter-CU LTM can coexist with Rel-18 MIMO two TA, following the same principle as for Rel-18 intra-CU LTM and Rel-18 MIMO two TA.

- Coexistence of Rel-19 inter-CU LTM and Rel-19 MIMO two TA

As for the second part, Rel-19 MIMO two TA introduces UL-only TRP. For the UL-only TRP, there is no DL TCI state configured, and DL transmission is not supported. Therefore, the joint/DL TCI state and the TA value indicated in the LTM Cell Switch Command for RACH-less LTM can only be associated with the TRP that supports both DL and UL. This ensures that R19 inter-CU LTM with network provided TA can coexist with Rel-19 MIMO two TA without any mismatch issue.

Proposal 13: RAN2 confirms that R19 inter-CU LTM with network provided TA can coexist with Rel-19 MIMO two TA.

As for the coexistence of UE-based TA measurement and MIMO two TA, for simplicity, we believe that the agreement from Rel-18 can be followed: UE-based TA measurement should not be configured if candidate cell is configured with Rel-19 MIMO 2TA.

RAN2#126 Agreement

- UE-based TA measurement should not be configured if candidate cell is configured with Rel-18 MIMO 2TA

RAN2#126 Agreement

- UE-based TA measurement should not be configured if candidate cell is configured with Rel-18 MIMO 2TA

Proposal 14: For Rel-19 inter-CU LTM, UE-based TA measurement should not be configured if candidate cell is configured with Rel-18/Rel-19 MIMO 2TA.

As for RACH-based LTM and MIMO two TA, the same TAG mismatch issue exists as we discussed in Rel-18 [5]. Specifically, if a TCI state ID included in the LTM Cell Switch Command MAC CE is associated with TRP 1, while the TAG of TA in the RAR for RACH-based LTM is associated with TRP 2, after completing the LTM cell switch, UE shall use the TCI state in the LTM Cell Switch Command MAC CE for data transmission without having any available TA value associated to the TCI state.

Observation 4: Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and R19 MIMO two TA is supported.

In [5], we proposed four options to resolve the problem:

- Option 1: After the completion of LTM, the target DU initiates a PDCCH order triggered RACH procedure via the indicated TCI state in LTM Cell Switch Command. The RACH procedure is performed to obtain the TA value associated with the indicated TCI state.

- Option 2: The target DU sends a new TCI state in RACH Msg4. The new TCI state is associated with the TAG indicated in the RACH RAR.

- Option 3: If the mismatch issue occurs, after the completion of RACH procedure, UE follows the TCI state associated to the RACH procedure, i.e. the DL TCI state is associated to the SSB for RACH procedure and the UL TCI state is associated with the TCI state for Msg3 transmission. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the TAG ID same as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM.

Due to time constraints, it was agreed that network implementation (e.g., option 1) would handle this issue in Rel-18.

RAN2#127 discussion for RACH based LTM and 2 TAs

- RAN2 understand NW can handle this situation, e.g. by PDCCH order. No need of new option.

RAN2#127 discussion for RACH based LTM and 2 TAs

- RAN2 understand NW can handle this situation, e.g. by PDCCH order. No need of new option.

Actually, initiating a PDCCH order-triggered RACH procedure via the indicated TCI state after LTM cell switch completion would result in additional latency. To ensure the UE can perform data transmission in the target cell immediately, other options need to be considered.

Proposal 15: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 16: If Option 3 in Proposal 15 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

Coexistence of inter-CU LTM with CHO

The coexistence of inter-CU LTM with CHO was supported based on the agreement reached in RAN2#127 meeting.

RAN2#127 Agreement

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

However, since CHO is triggered by the UE and RRCReconfiguraiton is initiated from the network, there may be cases where the RRCReconfiguration from the source gNB conflicts with an ongoing conditional handover. This can result in the UE failing to receive the RRCReconfiguration message. If the RRCReconfiguraiton message includes information related to LTM configuration update, the target gNB of CHO must be aware of the updated LTM configuration in order to reconfigure the UE accordingly. Otherwise, a misalignment may occur between the UE and the network, potentially leading to the failure of subsequent LTM cell switch. To illustrate this issue, we refer to the example presented in the following figure.

Figure 3. Conflict of CHO and RRCReconfiguration for inter-CU LTM

To address the issue, interaction between the source gNB and the target gNB of CHO is essential. RAN3 is expected to design the corresponding signaling enhancement.

Proposal 17: Send an LS to ask RAN3 to consider the LTM configuration mismatch issue between UE and the network caused by CHO execution.

  - Remaining issues for inter-CU MCG LTM with DC

In RAN2#127 meeting, the following agreements were made for the case of inter-CU MCG LTM with SN released or unchanged:

RAN2#127 Agreements

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Security key handling

During inter-CU MCG LTM with DC, the SN key is updated concurrently with any MN key change. As per TS 33.501[6], the MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. Therefore, when the KNG-RAN is updated due to inter-CU MCG LTM, the sk-Counter utilized by UE for generating the SN key should be set to zero.

TS 33.501

6.10.3.1	SN Counter maintenance

The MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN.

In our view, there are two options for ensuring that sk-Counter = 0 is used when inter-CU MCG LTM with SN unchanged or SN addition is performed:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

Both the options work well and RAN2 can select one for handling the security key in this scenario.

Proposal 18: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

In addition, similar to Rel-17 CHO with SCG, inter-CU MCG LTM with SN unchanged also requires the signaling enhancement between the candidate MN and SN to make the SN recognize that the Addition/Modification is related to an LTM. For instance, an indicator should be added in the SN Addition Request and SN Modification Request message. Furthermore, the source MN ID and UE ID should also be included in the SN Addition Request to help the SN to optimize resource allocation.

Proposal 19: Network signaling enhancements between MN and SN are needed for inter-CU MCG LTM with SN unchanged (e.g., the candidate MN informs SN the Addition/Modification is related to an LTM, includes source MN ID and UE ID in the SN Addition Request to help the SN to optimize resource allocation). Send an LS to inform RAN3.

- Conclusion

In this paper, we discuss the inter-CU LTM remaining open issues and also the specification impact of inter-CU LTM in DC scenarios. We have the following observations and proposals:

Remaining issues for Inter-CU LTM in SA

Security key handling

Proposal 1: RAN2 confirms that the SA3 solution is supported, i.e., NCC can be transmitted to the UE in plaintext via the LTM cell switch command message.

Proposal 2: If proposal 1 is agreed, the Rel-18 LTM cell switch command MAC CE should be reused for inter-CU LTM with an NCC field added by utilizing a R bit to indicate whether the NCC field is present.

Proposal 3: Network ensures that NCC is included in the LTM Cell Switch Command MAC CE for an inter-CU LTM cell switch. If the NCC is not included for an inter-CU LTM cell switch, UE shall ignore the LTM Cell Switch Command or trigger RRC re-establishment.

Proposal 4: UE shall ignore the corresponding NCC field for intra-CU LTM cell switch if indicated in the LTM cell switch command.

LTM fast recovery

Observation 1: The feasibility of supporting LTM fast recovery to an inter-CU LTM candidate cell depends on the key distribution approach between the source gNB and the target gNB.

Proposal 5: Following the Rel-18 principle, LTM fast recovery to an intra-CU LTM candidate cell is not supported for T304 expiry after an inter-CU LTM cell switch.

Proposal 6: RAN2 should postpone the discussion on LTM fast recovery to an inter-CU LTM candidate cell until the key distribution approach is finalized in RAN3.

UE-based TA measurement

Observation 2: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 7: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

Proposal 8: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 9: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

CFRA resource in LTM Cell Switch Command MAC CE

Observation 3: For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used, when the CFRA resources are shared among multiple UEs.

Proposal 10: For inter-CU LTM, CFRA resources for RACH-based LTM could be shared among multiple UEs within a single gNB-DU and assigned by S-DU in the LTM cell switch command MAC CE.

Proposal 11: Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

Coexistence of inter-CU LTM with MIMO 2TA

Observation 4: Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and R19 MIMO two TA is supported.

Proposal 12: RAN2 confirms that R19 inter-CU LTM can coexist with Rel-18 MIMO two TA, following the same principle as for Rel-18 intra-CU LTM and Rel-18 MIMO two TA.

Proposal 13: RAN2 confirms that R19 inter-CU LTM with network provided TA can coexist with Rel-19 MIMO two TA.

Proposal 14: For Rel-19 inter-CU LTM, UE-based TA measurement should not be configured if candidate cell is configured with Rel-18/Rel-19 MIMO 2TA.

Proposal 15: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 16: If Option 3 in Proposal 15 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

Coexistence of inter-CU LTM with CHO

Proposal 17: Send an LS to ask RAN3 to consider the LTM configuration mismatch issue between UE and the network caused by CHO execution.

Inter-CU MCG LTM with DC

Proposal 18: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

Proposal 19: Network signaling enhancements between MN and SN are needed for inter-CU MCG LTM with SN unchanged (e.g., the candidate MN informs SN the Addition/Modification is related to an LTM, includes source MN ID and UE ID in the SN Addition Request to help the SN to optimize resource allocation). Send an LS to inform RAN3.

- References

- RAN2#129_ChairNotes.

- S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases. CATT.

- TS 38.321 v18.4.0, Medium Access Control (MAC) protocol specification (Release 18).

- R4-2314455, Reply LS beam application time and UE based TA measurement for LTM. Ericsson.

- R2-2406439, Discussion on MAC open issue for LTM. vivo.

- TS 33.501 v19.0.0, Security architecture and procedures for 5G system (Release 19).