3GPP TSG-RAN WG2 Meeting #129bis		R2-2502168

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on UE-initiated/event-driven beam management

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

According to the previous RAN1 meetings, some progress on UE-initiated/event-driven beam management were made in RAN1 [1][2][3], e.g.

Agreement in RAN1#116bis Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. Agreement in RAN1#118bis Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed. Agreement in RAN1#119 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1. Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4. Final LS in R1-241XXXX Agreement in RAN1#120 Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

In this contribution, we will discuss the RAN2 aspects related to UE-initiated/event-driven beam management, including MAC and RRC impacts.

- Discussion

  - UE-initiated/event-driven beam report - MAC impacts

In RAN2#129 meeting, the following WA was concluded:

=> Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.

If the event evaluation for UE-initiated beam reporting is captured in TS 38.321, we need to consider:

- For the basic event evaluation procedure agreed in RAN1, i.e., UE initiated beam report occurs once the L1-RSRP of the new beam becomes a threshold value better than the current beam, no additional issue needs to be resolved. The MAC entity shall perform event evaluation based on the measurement results provided by the physical layer, if the event condition is satisfied, the MAC entity will indicate the physical layer to trigger the beam reporting procedure;

- For the time-window-based event evaluation procedure agreed in RAN1, a timer and counters for new beam RSs need to be introduced.  Upon determining that the event condition is satisfied based on the measurement results provided by the physical layer, and if the timer has expired or has not been started, the MAC entity shall start/restart the timer and increment the value of the counter corresponding to the new beam RS(s) which satisfies the event condition by 1. However, during the timer is running, the MAC entity shall only increment the value of the counter corresponding to the new beam RS(S) which satisfies the event condition by 1. Furthermore, the duration of the timer is configured by the network and the initial value of each counter is 0. If the value of at least one counter is equal to or larger than M before the expiration of the timer, the MAC entity will indicate the physical layer to trigger the beam reporting procedure.

  - According to RAN1 agreement, when the current beam RS is updated within the duration of the timer, values of all counters corresponding to new beam RSs need to be reset to 0.

  - When the timer expires, the values of all counters corresponding to new beam RSs need to be reset to 0.

Proposal 1: If the event evaluation for UE-initiated beam reporting is captured in TS 38.321, a timer and multiple counters needs to be introduced.

Proposal 2: Upon determining that the event condition is satisfied based on the measurement results provided by the physical layer, and if the timer has expired or has not been started, the MAC entity shall start/restart the timer.

Proposal 3: Upon determining that the event condition is satisfied based on the measurement results provided by the physical layer, and during the timer is running, the MAC entity shall increment the value of the counter corresponding to the new beam RS(s) that satisfies the event condition by 1.

Proposal 4: If the value of at least one counter is equal to or larger than M before the expiration of the timer, the MAC entity will indicate the physical layer to trigger beam reporting procedure.

Regarding the measurement RS determination of new beam(s), RAN1 has agreed that the measurement RS(s) for new beam(s) are explicitly configured in one RS resource set associated with a CSI reporting configuration in RAN1#118 meeting [2].  But whether the RS in the RS resource set can be updated by MAC-CE is still being discussed in RAN1.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

In our view, new beam(s) changes as the current beam changes, so it is necessary to update the measurement RS(s) of new beam(s). In addition to dynamic update by MAC CE, RRC reconfiguration is another approach. However, as RRC reconfiguration is a long-time operation, it is not suitable for UE-initiated/event-driven beam reporting, which is intended to reduce TCI switch delay. To avoid the update of measurement RS(s) of new beam(s), all of the current beam(s) applied for transmission and reception, the activated TCI states other than the indicated one, and inactive TCI states should be included in the measurement resource set. However, it may result in a large measurement overhead and increase the UE computation complexity. In summary, the dynamic update of measurement RS within the new beam resource set by MAC CE is more reasonable. But anyway, the final decision should be up to RAN1 discussion.

Proposal 5: RAN2 assume to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

  - Configuration – RRC impacts

In RAN1#116 bis meeting[1], RAN1 has agreed to introduce Event-2 as the trigger event for beam report.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

In our understanding, the trigger event should be configured by RRC for beam report. Thus, the corresponding configuration as well as the definition of the trigger event should be captured in RRC specification. If there is configuration for trigger event in RRC, the corresponding measurement report should be triggered by the configured event; otherwise, it will follow the legacy periodic report.

In addition to the trigger Event 2 defined as above, RAN1 has working assumption on Event 1 and Event 7 in RAN1#118 meeting [2] as below, which has not been final decided:

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise The additionally supported events will be lower priority compared to event 2.

If these events are agreeable, it should be up to RRC to configure which trigger event(s) should be applied for beam report.

Proposal 6: It is up to RRC to configure which/whether trigger event(s) should be applied for beam report.

Proposal 7: The corresponding RRC configuration as well as the definition of trigger events should be captured in RRC specification.

- Conclusion

In this contribution, we discuss the RAN2 aspects on UE-initiated/event-driven beam management, including RRC and MAC impacts. Based on the discussion, we have the following observations and proposals.

MAC impacts:

Proposal 1: If the event evaluation for UE-initiated beam reporting is captured in TS 38.321, a timer and multiple counters needs to be introduced.

Proposal 2: Upon determining that the event condition is satisfied based on the measurement results provided by the physical layer, and if the timer has expired or has not been started, the MAC entity shall start/restart the timer.

Proposal 3: Upon determining that the event condition is satisfied based on the measurement results provided by the physical layer, and during the timer is running, the MAC entity shall increment the value of the counter corresponding to the new beam RS(s) that satisfies the event condition by 1.

Proposal 4: If the value of at least one counter is equal to or larger than M before the expiration of the timer, the MAC entity will indicate the physical layer to trigger beam reporting procedure.

Proposal 5: RAN2 assume to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

RRC impacts:

Proposal 6: It is up to RRC to configure which/whether trigger event(s) should be applied for beam report.

Proposal 7: The corresponding RRC configuration as well as the definition of trigger events should be captured in RRC specification.

- References

- Draft_Minutes_report_RAN1#116bis

- Draft_Minutes_report_RAN1#118

- Draft_Minutes_report_RAN1#119

- Draft_Minutes_report_RAN1#120