3GPP TSG-RAN WG2 Meeting #129bis		R2-2502172

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on capability mis-match issue for LP-WUS, PEI, (e)RedCap, 2Rx XR, NTN, inactiveStatePO-Determination

Agenda Item:	8.19

Document for:	Discussion and Decision

- Introduction

In SA2#166 meeting, SA2 sent an LS [1] to RAN2 and RAN3 for the Rel-19 LP-WUS work item. In this LS, SA2 asked whether LP-WUS related aspects of UE Radio Capability for Paging Information may be lost when Initial Registration is performed on a pre-Rel-19 gNB (or a later gNB that does not support LP-WUS). Besides, SA2 also proposed other features signalled within the UE Radio Capability for Paging Information for this issue. The involved text is extracted as below.

2) Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

The issue was discussed in LP-WUS session, and the following reply LS and conclusion were agreed in the last RAN2 meeting [2].

On the issue of UE radio capability for paging information, RAN2 had the following agreement: - Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for Release 15 gNB and Release 16 gNB and for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the coming RAN2 meetings based on company contributions.

In this contribution, we will further discuss the issue on UE radio capability for paging information for Release 15 gNB and Release 16 gNB, and other cases mentioned by SA2 LS, e.g. (e)RedCap, PEI, etc.

- Discussion

## 2.1 Issue identified for different cases

From the LS from SA2, considering the UE reports the paging related radio capability in the initial registration via AS signalling, if this is performed on a pre-Rel 19 gNB (or a later release gNB that does not support the corrseponding feature), the gNB does not understand the corresponding capability, and thus cannot include the capability in the UERadioPagingInformation inter-node message to CN or other gNBs for paging, i.e. the paging related radio capability reported by the UE is not maintained by the network, which results in inconsistent paging operations between the UE and the network and causes paging issues.

According to TS 38.331 [3], the inter-node message to transfer UE’s radio paging information, i.e.   UERadioPagingInformation is extracted as below.

-- ASN1START -- TAG-UE-RADIO-PAGING-INFORMATION-START UERadioPagingInformation ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ ueRadioPagingInformation UERadioPagingInformation-IEs, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } UERadioPagingInformation-IEs ::= SEQUENCE { supportedBandListNRForPaging SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v15e0-IEs OPTIONAL } UERadioPagingInformation-v15e0-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1700-IEs OPTIONAL } UERadioPagingInformation-v1700-IEs ::= SEQUENCE { ue-RadioPagingInfo-r17 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17) OPTIONAL, inactiveStatePO-Determination-r17 ENUMERATED {supported} OPTIONAL, numberOfRxRedCap-r17 ENUMERATED {one, two} OPTIONAL, halfDuplexFDD-TypeA-RedCap-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1800-IEs OPTIONAL } UERadioPagingInformation-v1800-IEs ::= SEQUENCE { numberOfRxERedCap-r18 ENUMERATED {one, two} OPTIONAL, supportOf2RxXR-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1840-IEs OPTIONAL } UERadioPagingInformation-v1840-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } -- TAG-UE-RADIO-PAGING-INFORMATION-STOP -- ASN1STOP

It can be noted that in the UERadioPagingInformation message, some radio capability is included in a container ue-RadioPagingInfo-r17, e.g. PEI, LP-WUS, while some other radio capability is included directly in this message but not in the container, e.g. (e)RedCap, 2RxXR, NTN, inactiveStatePO-Determination-r17.

The container ue-RadioPagingInfo-r17 was introduced in Rel-17 in UERadioPagingInformation measage. For gNB in Rel-17 and later release, this container can be transparently transferred from gNB to AMF, or from one gNB to another gNB. Thus, for the radio capability in the container in UERadioPagingInformation message, no matter whether the gNB in Rel-17 and late release supports the corresponding feature, i.e. whether it could identify the corresponding capability in the container or not, the gNB will forward these capabilities to CN or other gNB for paging. Thus, there is no capability mis-match issue. While for gNB before Rel-17, it cannot support this container, so it cannot identify the corresponding capbilty and forward to CN and other gNB. Thus, there is capability mis-match issue between the UE and the gNB which sends its paging.

Observation 1: For the capability in the container ue-RadioPagingInfo-r17, i.e. PEI and LP-WUS, there is no capability mis-match issue for Rel-17 and later release. But there is capability mis-match issue for Rel-15 and Rel-16.

For the capability not in the container ue-RadioPagingInfo-r17, only the gNB supporting the corresponding feature could identify the capability and forward it to AMF. For (e)RedCap, 2RxXR, NTN, only the gNB supporting the corresponding feature allows the (e)RedCap, 2RxXR, NTN UE to camp on or to access. In this way, when UE reports the paging related radio capability in the initial registration, the gNB can anyway understand the corresponding capability, which could be transfterred to CN or to other gNB via CN/anchor gNB during paging procedure. There is also no capability mis-match issue for these features.

Observation 2: For the capability not in the container ue-RadioPagingInfo-r17 about (e)RedCap, 2RxXR, NTN, there is no capability mis-match issue.

But for inactiveStatePO-Determination-r17, it was introduce late in Rel-17 for all types of UEs. There is no restriction for camping and access. Thus, when the UE supporting inactiveStatePO-Determination-r17 accesses a gNB not supporting this feature, the gNB cannot understand the corresponding UE capability, so it cannot be transferred to CN or other gNB. There will be capability mis-match issue.

Observation 3: For the capability not in the container ue-RadioPagingInfo-r17 about inactiveStatePO-Determination-r17, there is capability mis-match issue for the gNB not supporting inactiveStatePO-Determination-r17.

## 2.2 Solution

Regarding how to resolve this capability mis-match issue, in fact, it was discussed for PEI in Rel-17 in RAN2#120 meeting as follows [4].

RAN2#120 meeting R2-2211603 Discussion on the PEI capability maintenance Huawei, HiSilicon discussion Rel-17 NR_UE_pow_sav_enh-Core - Xiaomi think that the network deployment fixes this. The network is required to support homogenously, OPPO agrees, VDF agrees as well. HW think we didn’t agree homogenous requirement for this case. - Ericsson think there is some truth to the HW observation. Legacy gNB us required to support the latest ASN.1 Noted, not agreeable (no support)

To resolve the capability mis-match issue between the gNB and the UE which will result in the paging lost as above, R2-2211603 proposed the following 2 solutions:

- Solution#1: Introduce a capability filter for the PEI capability reporting where the UE only reports the PEI capability

- Solution#2: Introduce an additional bit to indicate whether the gNB has been upgraded to support the PEI feature, UE decides whether to report the PEI capability based on this indication.

However, during the online discussion, most companies thought the issue does not exist considering the network is required to have the same capability homogenously, or legacy gNB may be required to support the latest ASN.1. Finally, no conclusion on the solutions was agreed for this issue.

In our understanding, for the case identified above, i.e. for PEI and LP-WUS in Rel-15 and Rel-16 gNB, or for inactiveStatePO-Determination in the gNB not supporting inactiveStatePO-Determination, similar logic could be followed, i.e. it could be up to the network implementation to avoid the capability mis-match issue, e.g. the network supporting PEI/LP-WUS/inactiveStatePO-Determination-r17 is homogenously deployed in the TA/RNA or all legacy gNBs are upgraded to support the latest ASN.1 for the UE capability on paging.

Proposal 1: It is up to network implementation to address the paging capability mis-match issue between the gNB and the UE for PEI/LP-WUS/inactiveStatePO-Determination-r17, e.g., all legacy gNBs are upgraded to support the latest ASN.1 for the UE capability on paging. No specification impact is foreseen.

- Conclusion

In this contribution, we discuss the capability mis-match issue for LP-WUS, PEI, (e)RedCap, 2Rx XR, NTN, inactiveStatePO-Determination. Based on the discussion, we have the following observations and proposal.

Issue identified for different cases

Observation 1: For the capability in the container ue-RadioPagingInfo-r17, i.e. PEI and LP-WUS, there is no capability mis-match issue for Rel-17 and later release. But there is capability mis-match issue for Rel-15 and Rel-16.

Observation 2: For the capability not in the container ue-RadioPagingInfo-r17 about (e)RedCap, 2RxXR, NTN, there is no capability mis-match issue.

Observation 3: For the capability not in the container ue-RadioPagingInfo-r17 about inactiveStatePO-Determination-r17, there is capability mis-match issue for the gNB not supporting inactiveStatePO-Determination-r17.

Solution

Proposal 1: It is up to network implementation to address the paging capability mis-match issue between the gNB and the UE for PEI/LP-WUS/inactiveStatePO-Determination-r17, e.g., all legacy gNBs are upgraded to support the latest ASN.1 for the UE capability on paging. No specification impact is foreseen.

- References

- R2-2500050/S2-2412876, LS Reply on LP-WUS subgrouping

- R2-2501388, Reply LS on LP-WUS subgrouping

- TS38.331 v18.5.1

- R2-2300002, Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France