3GPP TSG-RAN WG2 Meeting #129bis													     R2-2502178

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.3.2.1

Source:	Apple

Title:	SLS evaluation results for RRM measurement prediction

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our SLS evaluation results for RRM measurement prediction.

We are aware that in RAN2#129 we’ve agreed that no additional simulation results are expected in RAN2#129bis and therefore we are not seeking presentation or discussions based on this contribution.

We are only asking to include the results (uploaded to the corresponding FTP folder) to be included in the TR.

# 2 	Discussion

As mentioned in the introduction section, this contribution is only submitted to provide background and relevant explanations to the SLS results we’ve submitted in the agreed manner (to the corresponding FTP folder). As such, this contribution is brief as the only intention is to provide the relevant information about the simulation assumptions we’ve used and the results we’ve submitted.

As not so many companies provided SLS evaluation results (in terms of HOF rate), we believe that additional results (even submitted somewhat late) are useful to strengthen our confidence in the conclusions of the study.

Observation 1: since limited number of companies provided SLS results, additional results are useful to strengthen our confidence in the conclusions of the study.

In this exercise we focused on temporal domain prediction in FR1 and case B. This scenario was selected as arguably the most common and the most relevant to practical deployments.

Observation 2: we focused on temporal domain prediction in FR1 and case B as the most relevant to actual deployments.

Simulation assumptions are aligned with what we’ve agreed in RAN2 and what we’ve used in the previous submissions. They are summarized in Appendix A below.

We simulated case B with MRR of 50%, with two lengths of Observation Window (OW): 600ms and 1000ms.

As in the previous contributions, we’ve evaluated the following models:

- A simple Autoregressive model, and

- A more complex Feedforward NN model

Furthermore, for comparison, we used the following:

- Baseline HO, i.e. without predictions, and

- Sample and hold

Regarding HO modelling, we used the following approach:

- A Handover Failure (HOF) is considered if:

  - A HO command is transmitted to the UE while T310 is running [TR 36.839]

  - T310 expires while TTT is running [TR 36.839]

  - T310 expires while TTT is not running and there exists at least one strong neighbour cell [TR 36.839]

  - RACH to target cell fails [TR 36.839]

- A Handover Success is considered if:

  - RACH to target cell is successful [TR 36.839]

- RACH is modeled with the PDCCH-based RACH model [TR 36.839].

And finally we used HOF rate and HOF/UE/sec as the KPIs. The results are summarized in the Table 1 below.

| KPI | Baseline Handover | HO Sample and Hold | HO FNN OW3 (OW 600ms) | HO AR OW3 (OW 600ms) | HO FNN OW5 (OW 1000ms) | HO AR OW5 (OW 1000ms) |
| --- | --- | --- | --- | --- | --- | --- |
| HOF Rate | 11.00 % | 17.13 % | 12.48 % | 11.99 % | 12.29 % | 11.96 % |
| HOF/UE/Sec | 0.0133 | 0.0193 | 0.0166 | 0.0163 | 0.0165 | 0.0163 |

Table 1: SLS Results for temporal prediction case B in FR1 with 50% MRR

One thing that pops up immediately is the relatively thigh HOF rate even in the baseline scenario, i.e. without predictions. This certainly looks suspicious; however we observe that some other companies (e.g. [2]) reported similarly high HOF rates.

First off, this probably deserves further study, which the current AI/ML SI may or may not be the right time and place. At any rate, based on our initial analysis, one reason for such high HOF rate might be the following (the list is most likely not exhaustive):

- In the TR 36.839 [3], it is assumed that if a HO command is transmitted while the T310 has been triggered or is running at the UE (indicating PDCCH failure), then the HO command is not successfully received and it counts as HOF, which is not necessarily true. Even if T310 is running, there is a likelihood that PDCCH could be decoded and HO command could be received.

- In addition, no HARQ and RLC ARQ retransmissions are assumed for handover modeling according to TR 36.839 [3]

Observation 3: high baseline (i.e. without predictions) HOF rate indicates that a potential issue with RAN2 simulation assumptions; this does not necessarily negate the study outcome, but deserves further analysis (not necessarily in this particular SI).

The first and perhaps the main observation is that the difference in both HOF rate and HOF/UE/sec between the best performing models is rather small (i.e. 11% vs 12% HOF and 0.013 vs. 0016 in HOF/UE/sec). Having said that, there is a certain performance degradation, which may or may not be acceptable – depending on various factors such as where UE is actively transmitting/receiving traffic, QoS, etc.

Observation 4: while the performance degradation (in terms of SLS KPIs such as HOF rate) is small (in case B with MRR of 50%), it is non-zero; such small (e.g. 1%) performance degradation may be acceptable in some case, but maybe not in others.

One practical consideration that follows from this, is that the decision of when and whether to use AI/ML prediction depends, among other things, on the importance of the situation the UE is in (e.g. whether it is in an emergency call or even just high importance over-the-top video call) or simply transmitting/receiving occasional best-effort traffic.

Observation 5: the decision of when and whether to use AI/ML prediction depends, among other things, on the importance of the situation the UE is in and the type of traffic.

While, as we’ve already established, the most important (in order to decide how to proceed to the normative phase) results are the best results we can achieve, some differences between various models simulated may also be of interest.

Here we would like to note that simple Autoregressive (AR) models actually outperform (slightly) complex FNN. While the gap is probably insignificant, what’s important is that simple AR models provide very competitive (and in fact identical if not better) performance compared to complex FNN models. We’ve observed similar results in our previous submissions as well.

Observation 6: simple Autoregressive (AR) models provide very competitive (and in fact identical if not better) performance compared to complex FNN models.

We realize that there is no opportunity to discuss these observations during RAN2#129bis, therefore we are only asking to include the evaluation results (uploaded to FTP) in the TR.

Proposal 1: to include the results of this contribution (uploaded to FTP) in the TR.

# 3	Proposals

Observation 1: since limited number of companies provided SLS results, additional results are useful to strengthen our confidence in the conclusions of the study.

Observation 2: we focused on temporal domain prediction in FR1 and case B as the most relevant to actual deployments.

Observation 3: high baseline (i.e. without predictions) HOF rate indicates that a potential issue with RAN2 simulation assumptions; this does not necessarily negate the study outcome, but deserves further analysis (not necessarily in this particular SI).

Observation 4: while the performance degradation (in terms of SLS KPIs such as HOF rate) is small (in case B with MRR of 50%), it is non-zero; such small (e.g. 1%) performance degradation may be acceptable in some case, but maybe not in others.

Observation 5: the decision of when and whether to use AI/ML prediction depends, among other things, on the importance of the situation the UE is in and the type of traffic.

Observation 6: simple Autoregressive (AR) models provide very competitive (and in fact identical if not better) performance compared to complex FNN models.

Proposal 1: to include the results of this contribution (uploaded to FTP) in the TR.

# 4 	References

[1]				RAN2#129 	meeting minutes

[2]				R2-2500245	Simulation results of AI based Handover performance, CATT, Turkcell

[3]				TR 36.839	Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks

# Appendix A – Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario |
| Deployment Scenario | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel Model | UMA, LOSsoft is enabled |
| System bandwidth | 20MHz |
| UE Speed | 30 kmph |
| UE distribution | 100% outdoor |
| BS Antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1, 2, 4), (dH, dV) = (0.5, 0.8) λ, 1 beam |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE Antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1, 1, 2), (dH, dV) = (0.5, 0.8) λ, 1 beam |
| UE Antenna radiation pattern | Omni-directional |
| BS Tx power | 44 dBm |
| Maximum UE Tx power | 23 dBm |
| Inter site distance | 500 m |
| BS Antenna height | 25 m |
| UE height | 1.5 m |
| Spatial Consistency | Procedure A in 38.901 |
| UE Trajectory model | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Boundary-terminated model |

| Sampling period | 40ms |
| --- | --- |
| SSB period | 40ms |
| RRC Measurement period | 200ms |
| L1 filtering | Block filter with 5 samples |
| L3 Filter Coefficient | 4 |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| PW length | 200 |
| MRRT | 50% |
| T310 duration | 1000ms |
| Qout | 10% BLER of out-of-sync PDCCH configuration |
| Qin | 2% BLER of in-sync PDCCH configuration |
| Qout evaluation period | max(200ms, 10 SSB periods) = 400ms |
| Qin evaluation period | max(100ms, 5 SSB periods) = 200ms |
| N310 | 1 |
| N311 | 1 |
| Handover preparation delay | 40 ms |
| Handover execution delay | 40 ms |