3GPP TSG-RAN WG2 Meeting #129bis													     R2-2502179

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.3.5

Source:	Apple

Title:	On standards impacts other than LCM

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on standards impacts of AI/ML mobility beyond LCM.

# 2   	Discussion

While LCM is important, we should first discuss the standards impacts for the “core” AI/ML for mobility functionality. To recap, we seem to have concluded in the evaluation phase that at least the following AI/ML functions work:

- RRM measurement prediction

- Measurement event prediction

The question now becomes, how can we exploit AI/ML capability to predict RRM measurements and measurement events.

Observation 1: we need to discuss how can we exploit AI/ML capability to predict RRM measurements and measurement events; this, rather than LCM, is the main (i.e. “core”) outcome of the SI.

### 2.1 RRM prediction

According to SID [2], we are supposed to study both UE-based and network-based RRM measurement prediction.

Besides LCM (discussed in a separate paper [5]), and capability signalling for UE-based models, we have not identified any standards impacts needed to exploit AI/ML RRM measurement prediction.

Observation 2: we have not identified any standards impacts needed to exploit AI/ML RRM measurement prediction.

### 2.2 Measurement event prediction

As we have shown, a UE can rather easily predict a measurement event (e.g. A3) at least 400ms ahead of the event occurrence. In our evaluation, we focused on case A, where the intention is to improve handover performance. Admittedly, in the results submitted to this meeting we evaluated case A in FR1 – this is because in our view FR1 is the most important frequency range (in terms of practical deployments) by far. Regardless of whether the results are for FR1 or FR2, the point is that if prediction works at least in some (important) use cases, it is worth progressing the related functionality to the normative phase.

As a side note, we think that case B also has merits (in the context of event prediction, as it does in the context of RRM prediction), but in this contribution we focus on case A.

Observation 3: in this section we focus on standards impacts in support of case A with the task of HO performance optimization.

To analyze where performance gains can be realized, we refer to the following high level schematic call flow of a handover process and signalling in Figure x. We use Xn handover for illustration, with the understanding that the same applies to N2 handover as well – where, as we show below, the gains a potentially even bigger.

Figure 1: high level Xn HO call flow

This figure is well familiar and needs no explanation. We’d just note that it can be roughly divided into three parts:

- Measurements

- HO preparation (steps 2, 3 and 4)

- HO execution

In this section we focus on potential optimizations of the second part – HO preparation. According to field data, the delay between a UE reporting A3 till the UE gets ReconfigurationWithSync from the networks can be as high as 50ms. Granted, HO preparation doesn’t always takes 50ms – but in some cases it does. We have no visibility into the reasons for such large delay, it can be for example high latency backhaul or some other reason. Regardless of the cause for such delay, as we show below, AI/ML prediction can help to eliminate it entirely – with minimal cost to UEs and the network.

Observation 4: in real networks, HO preparation can take up as much as 50ms.

This wait time normally does not result in HO failure, as very few UEs move fast enough to go out of coverage pf the serving cell in 50ms, but even if the HO is successful, the relatively long wait in suboptimal radio conditions results in performance degradation, which is sometimes visible to the user.

Observation 5: 50ms wait in suboptimal radio conditions rarely results in HO failure but does affect performance.

With this in mind, we propose a simple HO enhancement (as illustrated by the Figure X below) which allows to eliminate the 50ms handover preparation wait time. The (minor) differences compared to the legacy flow are shown with “double arrows”.

Figure 2: AI/ML enhanced Xn HO

The enhancements are as follows:

- The UE performs measurements and also runs AI/ML measurement event prediction

- When an event is predicted, it is reported to the network (with the indication that it’s a predicted event)

- At this point the network (up to the network discretion) may start HO preparation (towards the predicted cell)

- The UE continues performing measurements and when the real measurement event is triggered (based on the real measurements) the UE reports it to the network (as in legacy)

- At this point, if the network has already done HO preparation towards the correct cell it can send ReconfigurationWithSync immediately

Now let’s consider the following scenarios:

- Successful measurement event prediction

- Wrong measurement event prediction

- Missed measurement event prediction

In the first case of the UE successfully predicting the correct A3 event (with the correct target cell), the HO preparation is triggered ahead of time (the network doesn’t need to do so 400ms in advance, as might be possible, but hopefully it would be somewhere between 100ms to 50ms). The HO then succeeds, and the UE spends 50ms less in suboptimal radio conditions.

Now let’s consider the rare case of a UE predicting a wrong A3. This may result in one unnecessary HO preparation which indeed would waste a tiny bit of network resources (in a single cell) for a very short period of time (of a few hundreds ms). After which the UE would detect the real A3 event based on real measurements and the legacy procedure would kick in.

Finally, if the UE prediction algorithm misses an event – nothing happens bad, as we simply fall back to the legacy procedure (i.e. HO based on real measurement event only without a prediction).

Proposal 1: consider measurement event reporting enhancements to allow the UE to report predicted events to the network, so that the network may start HO preparation ahead of time; in such a framework new “predicted measurement events” can be defined.

It is important stress that with this optimization the actual HO is always based on the actual measurement event – predicted measurement events are only used for HO preparation. In particular, this means that it doesn’t affect RAN4 requirements.

Observation 6: it is important to point out that in the proposed enhancement the HO is triggered based on real measurements (only the preparation is triggered by prediction) and therefore this does not affect RAN4 requirements and therefore does not need RAN4 TU.

Finally, on the topic of SLS, which is important to assess the gains of case B optimizations, for this particular optimization SLS (under the assumption that we don’t simulate traffic, which we’ve already agreed on) is not essential. This is because, as pointed out already, this optimization focuses on HO preparation (and not the HO decision itself) with the intention of optimizing performance (i.e. throughput and latency) during the HO preparation process.

# 3	Proposals

Observation 1: we need to discuss how can we exploit AI/ML capability to predict RRM measurements and measurement events; this, rather than LCM, is the main (i.e. “core”) outcome of the SI.

Observation 2: we have not identified any standards impacts needed to exploit AI/ML RRM measurement prediction.

Observation 3: in this section we focus on standards impacts in support of case A with the task of HO performance optimization.

Observation 4: in real networks, HO preparation can take up as much as 50ms.

Observation 5: 50ms wait in suboptimal radio conditions rarely results in HO failure but does affect performance.

Observation 6: it is important to point out that in the proposed enhancement the HO is triggered based on real measurements (only the preparation is triggered by prediction) and therefore this does not affect RAN4 requirements and therefore does not need RAN4 TU.

Proposal 1: consider measurement event reporting enhancements to allow the UE to report predicted events to the network, so that the network may start HO preparation ahead of time; in such a framework new “predicted measurement events” can be defined.

# 5	References

[1]				TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2]				RP-240082 Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR

[3]				RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[4]				RAN2#129 meeting minutes

[5]				R2-2502177 On LCM for AI/ML mobility