3GPP TSG-RAN WG2 Meeting #129bis	R2-2502182

Wuhan, China, 7 – 11 April 2025

Agenda item:	8.7.5

Source:	Apple, Ericsson

Title:	Discussions on Fast RLC Retransmission

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

In order to fulfil the Rel-19 XR work item objective related to RLC-AM enhancements for packets with small delay budget [1], RAN2 has reached many agreements related to faster retransmission over the last few meetings:

RAN2 #126 Agreements on Timely RLC Retransmissions [2]: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic RAN2 #127bis Agreements on Timely RLC Retransmissions [3]: RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. Exclude enhanced status reporting. Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay RAN2 #128 Agreements on Timely RLC Retransmissions [4]: Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. RAN2 #129 Agreements on Timely RLC Retransmissions [5]: Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC Only a single autonomous retransmission will be triggered per RLC SDU. There is no dynamic activation/deactivation of the autonomous retransmission mechanism. We have separate thresholds for autonomous reTx and for polling

Based on the highlighted parts in the agreements listed above, we think RAN2 should at least address the following three issues:

- Whether the discardTimer in PDCP should be reused for both enhanced polling and autonomous retransmission?

- How to strike a balance between capacity impacts of Autonomous Retransmission and XR user experience, when PDU Set discarding is considered?

- How to perform retransmission of a RLC SDU as soon as possible if only polling enhancement is configured?

This paper presents some of our views on these issues.

# Discussions

## Timer for Triggering of Delay-Based Auto ReTX and Polling

It is confirmed that both autonomous retransmission and delay-based polling can be triggered based on the remaining time of RLC SDUs. However, it is still FFS if the PDCP discard timer should be used as the basis of remaining time measurement, just like the other delay-related features such as DSR and LCH priority adjustment, or if a new type of timer should be introduced for delay-based enhancements in RLC layer. In particular, it has been proposed that a new timer at RLC could be considered.

The PDCP discard timer for a packet starts when the packet arrives from the upper layer. The value of which essentially indicates the maximum length of time that the transmitter should keep this packet in the buffer, and it is typically configured in accordance with the required PDB. Thus, it is reasonable to use the remaining value of the running discard timer to measure the “urgency level” of a packet. From this point of view, the existing PDCP discard timer fulfil the intentions of autonomous retransmission and delay-based polling well. Whenever the remaining time of discard timer of a packet becomes smaller than a threshold, the PDCP can send an indication to RLC to trigger the required operations. Such cross-layer interaction is simply an extension of the existing discard indication from PDCP to RLC.

One potential argument of introducing a new timer in RLC layer is that, the PDCP layer is already sending many different indications to the lower layer, and so it may be desirable to make this as a “standalone” operation of RLC, in order to minimize cross-layer interaction. However, we think there may be some specification complexity if a new timer in RLC is introduced, as we need to further define UE behaviors such as when this new timer should be started and/or stopped. Currently, we do not see a very strong justification to have a new timer in RLC when the existing PDCP discard timer can already serve the purpose.

Proposal 1: PDCP discard timer is used to determine the remaining time of a packet for triggering of RLC autonomous retransmission and delay-based polling.

## Auto ReTX with Considerations of PDU Set Discarding

With Autonomous RLC retransmission, the transmitter can autonomously consider a packet for retransmission when the remaining time of this packet becomes smaller than a threshold, even if the status feedback is absent. From the latency perspective, such scheme may be beneficial as a more urgent RLC SDU could be retransmitted more proactively and quickly. Nonetheless, some concerns on impacts to the system capacity have also been brought up in the previous RAN2 discussions, especially in uplink scenarios. For instance, the UE may not know the network status in general, blind retransmission may cause further congestions to other traffic flows and/or users. RAN2 has also agreed that the impacts to capacity should be considered.

This issue is even trickier when XR PDU Sets are taken into account. Specifically, when PDU Set discarding is configured, all packets of a PDU Set will be discarded jointly as soon as one packet of this PDU Set is lost. To achieve the best user experience by avoiding or minimizing PDU Set discarding, all packets belonging to the same PDU Set should be delivered quickly when at least one packet becomes urgent.  This turns out to be dilemma, as triggering autonomous retransmission for a large number of packets simultaneously can cause significant impacts to the capacity, while this may be required to ensure XR user experience is not severely degraded. With this in mind, we think RAN2 can further discuss the following options:

- Option 1: Autonomous retransmission is triggered only based on remaining time each individual RLC SDU, regardless if PDU Set discarding is configured or not.

- Option 2: If PDU Set discarding is configured the radio bearer, autonomous retransmission is triggered for all RLC SDUs belonging to the same PDU Set.

- Option 3: If PDU Set discarding is configured the radio bearer, autonomous retransmission is triggered for up to N RLC SDUs belonging to the same PDU Set, where the value of N is pre-configured.

- Option 4: If PDU Set discarding is configured the radio bearer, autonomous retransmission is triggered for all RLC SDUs belonging to the same PDU Set, only if the PDU Set is considered important.

In Option 1, basically there is no dependency between PDU Set and autonomous retransmission. The UE considers each RLC SDU independently even if they belong to the same PDU Set. The impact to capacity is minimized, but from Application point of view the operation of autonomous retransmission may be triggered in vain.

In Option 2, the UE handles all RLC SDUs corresponding to the same PDU Set jointly, so autonomous retransmission is triggered for all RLC SDUs as soon as one of the packets has a remaining time smaller than the threshold. This can facilitate the transmitter to deliver the whole PDU Set within its PSDB, but it may impact the capacity significantly if the number of packets to be retransmitted together in this PDU Set is large.

In Option 3, instead of retransmitting all packets of a PDU Set as in Option 2, the UE may be pre-configured with a maximum number of packets that can be retransmitted autonomously in one PDU Set. This may provide a more flexible control, but RAN2 may have to discuss further about the reasonable value of “N” that can be configured.

In Option 4, we could have some differentiated handling for PDU Sets with different importance levels. In particular, joint autonomous retransmission of multiple RLC SDUs is only applied to PDU Sets that considered to be more important. Considering that typically there could be more low-importance PDU Sets than high-importance PDU Sets in the queue, we may reach a good trade-off between capacity and user experience. However, this option is only feasible when the UE is capable of identifying PSI on the corresponding traffic flow.

Since this problem has never been brought up in RAN2 previously, we suggest that RAN2 can discuss if this issue should be handled in Rel-19, and what would be the best approach.

Proposal 2: RAN2 should discuss if/how PDU Set discarding should be considered for autonomous retransmission.

## RLC SDU Selection for ReTX upon Expiration of Poll Retransmit Timer

We have also agreed that a poll can be triggered when the remaining time becomes smaller than a threshold, and ideally this would facilitate the transmitter to receive the status report in a timely manner. For UL where the receiver is a gNB, it is up to network implementation when to send the status report. Thus, as long as the gNB is properly implemented, we think faster RLC retransmission can be achieved with delay-based polling. Nevertheless, we cannot guarantee the status report is always received on time. For instance, how quickly the transmitter can receive the status report may be constrained by the instantaneous DL channel quality and congestion status. We think some complementary mechanism for delay-based polling can be specified to facilitate faster retransmission in situations where the transmitter does not receive the status report on time, especially when autonomous retransmission is not configured. We must highlight that, configuration of autonomous retransmission is not always appropriate, especially in scenarios where the radio resource is scarce, and congestion is more likely to occur. Since we are talking about the cases where the UE cannot receive status report before t-PollRetransmit expiration, we presume congestion may be present.

According to TS 38.322, if the UE does not receive the status report and the t-PollRetransmit expires, the UE would select a RLC SDU for retransmission (which allows the UE to include a poll in the corresponding AMD PDU), when no new transmission can be performed:

#### 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

With delay-based polling in Rel-19, if the poll which started or restarted the expired t-PollRetransmit was triggered by the delay status of a RLC SDU, we think it makes sense for the UE to deliberately select a RLC SDU with remaining time smaller than a poll-triggering threshold for retransmission, so at least we can make sure the more urgent RLC SDU can seize this opportunity to perform retransmission as early as possible.

Proposal 3: Upon expiration of t-PollRetransmit that was started/restarted by a delay-based polling, the transmitting side of an AM RLC entity should consider a RLC SDUs with remaining time smaller than the poll-triggering threshold for retransmission, if needed.

# Conclusions

In this paper, we have presented some of our views on RLC-AM enhancements for faster retransmission, with the following proposals:

Proposal 1: PDCP discard timer is used to determine the remaining time of a packet for triggering of RLC autonomous retransmission and delay-based polling.

Proposal 2: RAN2 should discuss if/how PDU Set discarding should be considered for autonomous retransmission.

Proposal 3: Upon expiration of t-PollRetransmit that was started/restarted by a delay-based polling, the transmitting side of an AM RLC entity should consider a RLC SDUs with remaining time smaller than the poll-triggering threshold for retransmission, if needed.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #126 Chair’s Notes, Fukuoka, Japan, May 2024.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.

- 3GPP RAN2 #128 Chair’s Notes, Orlando, USA, November 2024.

- 3GPP RAN2 #129 Chair’s Notes, Athens, Greece, November 2024.